Dynamics Revision Questions

Q1.	A vehicle is capable of a maximum acceleration of 5.2 m/s2 and a maximum retardation of 7.2 m/s2.  Calculate the maximum distance it can travel in 10s, starting from rest, and finishing at rest, assuming there is no upper limit to its speed.  Also sketch a velocity-time graph of the journey, showing all given information.  What other assumptions have been made in the analysis?

[150.94m]

Q2.	A train passes station A with a speed of 80 kph and acceleration 3 m/s2.  After a time of 30s the train passes a signal which instructs the driver to slow down to a halt.  The driver decelerates the train at 2.4 m/s2 until it eventually stops as instructed.  Find the total time and total distance covered from station A until the point where it comes to rest.  Sketch a velocity-time graph to depict the journey and the given information.  Also determine the average velocity (in kph) of the entire journey as described, and if the train had a mass of 72 tonnes, find the kinetic energy possessed by the train at the point when the signal was given.

[76.76s; 4640.3m; 217.62kph; 453.36MJ]

Q3.	For a reciprocating piston moving with simple harmonic motion in a straight line, and given conditions:

- Velocity of 15 m/s at 85 mm from the mid-position

- Velocity of 4.5 m/s at 105 mm from the mid-position

Determine the amplitude of oscillation and frequency of the motion.  Also determine the maximum acceleration experienced by the reciprocating piston.

[r = 106.78mm; 5751.8m/s]

Q4.	A solid disc flywheel of diameter 1.2 m and mass 92 kg, is accelerated from rest by a motor to a speed of 1450 rpm in a period of 45s.

- Neglecting any losses, calculate the necessary accelerating torque to achieve the above acceleration.							[55.87Nm]

- If the bearings supporting the flywheel partially collapsed, causing a constant frictional torque of 2.2 Nm acting on the flywheel, what would be the speed of the flywheel 45s after it had reached the speed of 1450 rpm?	[2842.7rpm]

- If the original flywheel was to be replaced by one of a composite design, having the same mass and moment of inertia, find the required radius of gyration.

[0.424m]

- Find the loss in kinetic energy of the original flywheel, when considering the partial failure of the supporting bearings, when the flywheel has been rotating for a total time of 90s.						[295.6J]

Q5.	A body of mass 3 kg rests on a rough inclined plane at 60º to the horizontal.  If the coefficient of static friction between the body and the plane is 0.35, determine:

- The force which must be applied parallel to the plane to just prevent motion (i.e. sliding) down the plane.						[20.34N]

- The force which must be applied horizontally to the body to just prevent motion down the plane.							[25.32N]