Report: Engine Performance Characteristics

Name: Greig Western

Student Number: 1804348

Subject Tutor: Abigail Summerfield

Module Number: M4X00394

University of Wales Trinity St David

School of Engineering

Course: Motorsport Engineering (BEng)

Contents

Introduction	3

Experimental Procedure	5

Results	6

Analysis	10

Conclusion	12

Evaluation	13

Further Work	14

Bibliography	15

# Introduction

An Engine dynamometer (dyno) is a device that generally uses one of two methods to measure the performance output in terms of both power and torque, whilst also measuring a range of other inputs and outputs, depending on the dyno itself and the engine. For instance, a carburetted engine will only have a few measurements available to measure due to a lack of ECU (Engine Control Unit), whereas the latest engine management systems will be able to give feedback on a long list of parameters, from fuel pressure, to lambda (air fuel ratio) and beyond.

The two main types of Engine dyno are the water brake type which uses water as resistance to the torque output of the engine, and Eddy current type, which transfers the torque into heat via a magnetic coil situated about a toothed rotor as shown in Fig 1.

Fig 1.                                                (Moodle.uwtsd.ac.uk, 2020)

This report will study the raw data retrieved from an eddy current dyno, and from there a host of more useful data can be extrapolated, such as engine power, Brake Specific Fuel Consumption (BSFC), Brake Thermal efficiency (BTE) and Brake Mean Effective Pressure (BMEP). This data will show how efficient an engine is at all loads, and, by looking at this data one can conclude not only the health of an engine, but how to increase efficiency and performance in the future. It is important to not only increase the power for motorsport, but also the BSFC as this will allow the same performance, or better for less fuel at any given engine load and speed. Any increase in this parameter alone will pay huge dividends due to reduced fuel weight of a vehicle, longer intervals between pit stops, and to meet tighter regulations on emissions imposed by ruling bodies.

The engine to be tested in this report will be a Honda D16v3 4-cylinder 1.6 litre (1600cc) single overhead cam with 4 valves per cylinder with VTEC (Variable Valve Timing and Lift Electronic Control). The engine is not a performance model so predictions for this engine would be a modest output in terms of both torque and power with a fairly high efficiency according to the BSFC. The torque produced should be fairly flat with an increase in the midrange from lower engine speeds, and then a slight tail off at the higher rpm range. The power, due to a lack of forced induction should be fairly linear, increasing from the lower end of the rpm range, then tailing off at the top end as the peak efficiency of the cylinder head is reached and surpassed. Although this report calls for the power and torque to be recorded in metric values, these values will also be converted to imperial units as a sanity check as this will confirm if the output is correct as if the two outputs are put on the same scale against rpm in a graph then the torque and power will intersect at 5252rpm if recorded correctly due to the calculation used to measure power from the torque and rpm of an engine: hp=torque x rpm/5252 (Bell, 1997). If this happens then it can be taken that other readings and calculations are correct. The industry as a whole in the world also uses the imperial system, therefore giving more meaning to the results.

# Experimental Procedure

The equipment used to carry out the test were:

1 x Schenk dynamometer

1 x Honda D16v3 Engine

2 x Apicom DSG torque arms

1 x mass of 20.45 kg

1 x computer with 3 screens, DataQ Pro software, keyboard and mouse

2 x rotary dials to increase dyno load and engine rpm independently

It is important to note that the engine and the dynamometer are situated in a separate cell from the computer and associated paraphernalia, with a window placed between the two so that visual checks can be carried out during the test. The separation is for both noise and health and safety reasons, separating the user from harm via a thick wall and window.

The first step of the test was to calibrate the dyno. This is done by attaching the torque arms to the dyno, one on each side to counter each other. The mass is then attached to the arm which place load onto the dyno in the same direction of travel as the rotation of the engine. Once the mass is attached the torque output calculated from the readings taken at the load cell on the dyno is recorded using the computer. This is then checked against the mass multiplied by the length of the torque arm to calibrate the system. In this case the arm is 815.776mm in length and the mass is 20.45kg. The torque recorded by the computer was 163.66NM. Torque is calculated by multiplying the load by the distance from the fulcrum, this would be 20.45x9.81 (load is subject to gravity) multiplied by 0.815776 (distance in metres) which gave 163.656NM which is correct when rounded.

With the dyno now ready to be used the mass is removed from the torque arm and the dyno cell door is closed separating the user from the cell except for the computer controls and the window.

A ‘sweep’ test is carried out at this time, which, according to parameters set by the user, runs the engine from one set rpm level to another (in this case 1000rpm to 5500rpm) at a set ramp rate, recording at set rpm intervals (every 50 rpm in this test) to give results across the board at each rpm level specified. The DataQ Pro software has this ability built in, requiring the user to define a few parameters to start the test (Dsgroup.uk.com, 2020). The dyno, working with the computer software then controls both the load output of the dyno against the rpm input of the engine at 100% throttle (Wide Open Throttle or WOT) to create a sweep of the designated rpm range. The dyno increases the load at each point in the rpm range until the load is overcome by the engine, this gives the torque reading at each specified rpm point, from which other calculations are made to extrapolate power output etc...

The raw data received was the Load in N, the Engine Speed in RPM and the Fuel Flow Rate in l/min.

# Results

The results given were the first 3 columns, Load (N), Engine speed (RPM) and fuel flow rate (l/min). From this raw data the rest of the required data can be calculated. Engine Torque was also calculated in imperial units (lb/ft) so that engine power (BHP, Brake Horse Power) could also be calculated as a sanity check.

|  |  |  |  | Torque Arm Radius | Barometric Pressure | Humidity | Ambient temp |  | Piston Area |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | m | mBar | % | Deg C |  | m^2 |  |  |  |
|  |  |  |  | 0.815775 | 1024.65 | 32.37 | 26 |  | 0.441786466 |  |  |  |
| Load | Dyno Speed | Fuel Flow Rate | Air/Fuel Ratio | Torque | Brake Power | Corrected Power | SFC | Corrected Imperial Power | Imperial Torque | BMEP | SFC | BTE |
| Newtons | rpm | l/min | Lambda | NM | W | W | g/kWh | BHP | lb/ft | KPa | kg/kWh | % |
| 111.1509914 | 1000 | 0.0704 | 1.3277 | 90.67420001 | 9495.380021 | 9485.390232 | 328.7426088 | 12.72009801 | 66.87766296 | 716.438125 | 0.328742609 | 21.88771262 |
| 111.447887 | 1050 | 0.0724 | 1.32655 | 90.91640002 | 9996.780303 | 9986.263007 | 321.1249925 | 13.39177842 | 67.0563 | 718.3518041 | 0.321124993 | 22.40692539 |
| 112.1839968 | 1100 | 0.0737 | 1.31367 | 91.51689999 | 10541.99009 | 10530.8992 | 309.9849242 | 14.12214644 | 67.49920476 | 723.0964952 | 0.309984924 | 23.2121732 |
| 113.2253379 | 1150 | 0.0728 | 1.29262 | 92.36640003 | 11123.47481 | 11111.77216 | 290.1927729 | 14.9011087 | 68.125762 | 729.8085943 | 0.290192773 | 24.79532373 |
| 114.5089026 | 1200 | 0.0744 | 1.27289 | 93.41350002 | 11738.68662 | 11726.33672 | 281.0276914 | 15.72525206 | 68.89806107 | 738.081977 | 0.281027691 | 25.60396704 |
| 115.972664 | 1250 | 0.07895 | 1.25459 | 94.60759997 | 12384.10588 | 12371.07695 | 282.6722441 | 16.58986161 | 69.77878144 | 747.5168408 | 0.282672244 | 25.45500628 |
| 117.625203 | 1300 | 0.079 | 1.2336 | 95.95569998 | 13062.99463 | 13049.25146 | 268.1513773 | 17.4993072 | 70.77308608 | 758.1684952 | 0.268151377 | 26.83343947 |
| 119.4125831 | 1350 | 0.0796 | 1.2044 | 97.41379998 | 13771.55153 | 13757.06291 | 256.2865915 | 18.44849651 | 71.84852231 | 769.6892854 | 0.256286591 | 28.07569334 |
| 121.2052343 | 1400 | 0.0835 | 1.1824 | 98.87620001 | 14496.00803 | 14480.75724 | 255.4075571 | 19.41898508 | 72.92713008 | 781.2440511 | 0.255407557 | 28.17232125 |
| 122.8819221 | 1450 | 0.0847 | 1.1716 | 100.244 | 15221.41434 | 15205.40037 | 246.7312114 | 20.39074601 | 73.93596464 | 792.0513597 | 0.246731211 | 29.1630058 |
| 124.1445251 | 1500 | 0.08865 | 1.1616 | 101.274 | 15908.08271 | 15891.34632 | 247.0908073 | 21.31061325 | 74.69565141 | 800.189631 | 0.247090807 | 29.12056433 |
| 125.0332506 | 1550 | 0.0932 | 1.1624 | 101.999 | 16556.03097 | 16538.61289 | 249.6062014 | 22.17861066 | 75.23038245 | 805.9180264 | 0.249606201 | 28.82710329 |
| 125.7761025 | 1600 | 0.0959 | 1.16441 | 102.605 | 17191.63276 | 17173.54599 | 247.3416027 | 23.03006864 | 75.67734381 | 810.7061746 | 0.247341603 | 29.09103714 |
| 126.3632742 | 1650 | 0.09875 | 1.17361 | 103.084 | 17811.63654 | 17792.89748 | 245.8266532 | 23.86063138 | 76.03063505 | 814.490866 | 0.245826653 | 29.27031571 |
| 126.7677975 | 1700 | 0.10005 | 1.18462 | 103.414 | 18410.13089 | 18390.76217 | 240.9660761 | 24.66237989 | 76.27402984 | 817.0982733 | 0.240966076 | 29.86073337 |
| 127.1232877 | 1750 | 0.10305 | 1.19295 | 103.704 | 19004.7506 | 18984.75631 | 240.4260438 | 25.4589379 | 76.48792226 | 819.3896315 | 0.240426044 | 29.92780498 |
| 127.5658117 | 1800 | 0.1093 | 1.19775 | 104.065 | 19615.79038 | 19595.15323 | 247.0643245 | 26.27749238 | 76.75418143 | 822.2419773 | 0.247064325 | 29.12368576 |
| 128.0867886 | 1850 | 0.11415 | 1.20142 | 104.49 | 20243.00934 | 20221.71231 | 250.0325379 | 27.11772064 | 77.06764438 | 825.6000015 | 0.250032538 | 28.7779495 |
| 128.5146027 | 1900 | 0.11745 | 1.2008 | 104.839 | 20859.55738 | 20837.6117 | 249.6569273 | 27.94365404 | 77.32505285 | 828.3575327 | 0.249656927 | 28.82124613 |
| 128.9877724 | 1950 | 0.12055 | 1.20058 | 105.225 | 21487.31566 | 21464.70953 | 248.7601097 | 28.78460478 | 77.60975102 | 831.4074094 | 0.24876011 | 28.92515106 |
| 129.5957832 | 2000 | 0.12555 | 1.19565 | 105.721 | 22142.15447 | 22118.85941 | 251.4157783 | 29.66183285 | 77.97558079 | 835.3264218 | 0.251415778 | 28.61961885 |
| 130.2614079 | 2050 | 0.12785 | 1.19355 | 106.264 | 22812.27713 | 22788.27705 | 248.5007949 | 30.55953529 | 78.37607586 | 839.6167921 | 0.248500795 | 28.95533494 |
| 131.0067114 | 2100 | 0.1309 | 1.19322 | 106.872 | 23502.3803 | 23477.65419 | 246.9582198 | 31.48400383 | 78.82451231 | 844.4207424 | 0.24695822 | 29.13619865 |
| 131.8255646 | 2150 | 0.13435 | 1.18951 | 107.54 | 24212.35929 | 24186.88624 | 246.0346358 | 32.43509818 | 79.31720237 | 849.6987669 | 0.246034636 | 29.24557238 |
| 132.5978364 | 2200 | 0.1377 | 1.19055 | 108.17 | 24920.579 | 24894.36085 | 245.0030555 | 33.38383579 | 79.78186519 | 854.6765449 | 0.245003055 | 29.36871026 |
| 133.3652049 | 2250 | 0.14105 | 1.18686 | 108.796 | 25634.45358 | 25607.48438 | 243.9746562 | 34.34014871 | 80.24357778 | 859.622718 | 0.243974656 | 29.49250492 |
| 134.0823144 | 2300 | 0.14495 | 1.1866 | 109.381 | 26345.00854 | 26317.29179 | 243.9582812 | 35.29201463 | 80.67505038 | 864.2449403 | 0.243958281 | 29.49448453 |
| 134.5898072 | 2350 | 0.14985 | 1.18464 | 109.795 | 27019.60795 | 26991.18147 | 245.9084163 | 36.19571418 | 80.98040018 | 867.5160509 | 0.245908416 | 29.2605835 |
| 135.0274279 | 2400 | 0.1537 | 1.18001 | 110.152 | 27684.21712 | 27655.09143 | 246.171238 | 37.08603071 | 81.24370912 | 870.3367919 | 0.246171238 | 29.2293438 |
| 135.3473691 | 2450 | 0.15565 | 1.17538 | 110.413 | 28327.9347 | 28298.13177 | 243.6295153 | 37.94836067 | 81.4362123 | 872.3990144 | 0.243629515 | 29.53428586 |
| 135.6440195 | 2500 | 0.1596 | 1.17344 | 110.655 | 28969.41126 | 28938.93346 | 244.2805598 | 38.80768855 | 81.61470181 | 874.311113 | 0.24428056 | 29.45557254 |
| 136.0779627 | 2550 | 0.1626 | 1.16821 | 111.009 | 29643.33001 | 29612.1432 | 243.2143756 | 39.71047628 | 81.87579806 | 877.1081501 | 0.243214376 | 29.58469758 |
| 136.5278416 | 2600 | 0.16805 | 1.16209 | 111.376 | 30324.49536 | 30292.59191 | 245.720066 | 40.62297161 | 82.14648255 | 880.0079029 | 0.245720066 | 29.28301244 |
| 136.7791364 | 2650 | 0.17145 | 1.16202 | 111.581 | 30964.54774 | 30931.97092 | 245.5095764 | 41.48039164 | 82.29768236 | 881.6276561 | 0.245509576 | 29.30811846 |
| 136.8722993 | 2700 | 0.17465 | 1.15796 | 111.657 | 31570.27297 | 31537.05889 | 245.2934445 | 42.29182671 | 82.35373689 | 882.2281497 | 0.245293444 | 29.33394231 |
| 136.768104 | 2750 | 0.1784 | 1.15788 | 111.572 | 32130.42944 | 32096.62603 | 246.192041 | 43.04221744 | 82.29104435 | 881.5565453 | 0.246192041 | 29.22687395 |
| 136.5719714 | 2800 | 0.18005 | 1.15674 | 111.412 | 32667.70459 | 32633.33593 | 244.3825515 | 43.76195615 | 82.1730347 | 880.2923472 | 0.244382552 | 29.44327942 |
| 136.5119059 | 2850 | 0.1831 | 1.15398 | 111.363 | 33236.43237 | 33201.46537 | 244.2697192 | 44.52382909 | 82.13689431 | 879.9051873 | 0.244269719 | 29.45687977 |
| 136.3721614 | 2900 | 0.1879 | 1.1482 | 111.249 | 33784.9073 | 33749.36327 | 246.6037845 | 45.25857113 | 82.05281241 | 879.004446 | 0.246603785 | 29.17807512 |
| 136.2226104 | 2950 | 0.19255 | 1.14152 | 111.127 | 34329.71707 | 34293.59986 | 248.6961073 | 45.98840329 | 81.96283012 | 878.0404956 | 0.248696107 | 28.932595 |
| 136.0571236 | 3000 | 0.199 | 1.13561 | 110.992 | 34869.16518 | 34832.48044 | 253.0505091 | 46.71105292 | 81.86325952 | 876.973829 | 0.253050509 | 28.43473335 |
| 135.847507 | 3050 | 0.20605 | 1.12749 | 110.821 | 35395.70135 | 35358.46266 | 258.1176993 | 47.41640559 | 81.73713678 | 875.622718 | 0.258117699 | 27.8765221 |
| 135.8989918 | 3100 | 0.21005 | 1.11813 | 110.863 | 35989.59327 | 35951.72976 | 258.7863922 | 48.21198864 | 81.76811431 | 875.95457 | 0.258786392 | 27.80449036 |
| 135.92596 | 3150 | 0.21605 | 1.10783 | 110.885 | 36577.32765 | 36538.8458 | 261.9015006 | 48.999323 | 81.78434061 | 876.128397 | 0.261901501 | 27.47377825 |
| 136.0228004 | 3200 | 0.21795 | 1.09644 | 110.964 | 37184.3933 | 37145.27278 | 259.8913722 | 49.8125537 | 81.84260784 | 876.7525944 | 0.259891372 | 27.68627403 |
| 136.3059667 | 3250 | 0.22305 | 1.08326 | 111.195 | 37844.0178 | 37804.20331 | 261.3368658 | 50.69619272 | 82.01298419 | 878.5777795 | 0.261336866 | 27.53313708 |
| 136.5830039 | 3300 | 0.2255 | 1.07172 | 111.421 | 38504.33346 | 38463.82427 | 259.6764858 | 51.58075762 | 82.17967276 | 880.3634587 | 0.259676486 | 27.70918486 |
| 136.7386841 | 3350 | 0.2285 | 1.06105 | 111.548 | 39132.28548 | 39091.11564 | 258.9087215 | 52.42196789 | 82.2733429 | 881.3669156 | 0.258908721 | 27.7913533 |
| 136.8061046 | 3400 | 0.2342 | 1.05089 | 111.603 | 39735.93202 | 39694.1271 | 261.3359615 | 53.23061833 | 82.31390867 | 881.8014831 | 0.261335962 | 27.53323235 |
| 136.831847 | 3450 | 0.23605 | 1.04119 | 111.624 | 40327.87091 | 40285.44323 | 259.5340831 | 54.02358508 | 82.32939743 | 881.9674091 | 0.259534083 | 27.7243885 |
| 136.824492 | 3500 | 0.24195 | 1.03708 | 111.618 | 40910.13368 | 40867.09342 | 262.2348556 | 54.80358962 | 82.32497205 | 881.9200015 | 0.262234856 | 27.43885336 |
| 136.8097821 | 3550 | 0.24545 | 1.0315 | 111.606 | 41490.10311 | 41446.45269 | 262.3096156 | 55.58052198 | 82.31612135 | 881.8251869 | 0.262309616 | 27.4310331 |
| 136.6651344 | 3600 | 0.25505 | 1.02668 | 111.488 | 42029.98582 | 41985.7674 | 269.0678281 | 56.3037538 | 82.22908929 | 880.8928414 | 0.269067828 | 26.74204419 |
| 136.3206767 | 3650 | 0.25755 | 1.02521 | 111.207 | 42506.32981 | 42461.61025 | 268.6603866 | 56.94186857 | 82.02183495 | 878.6725947 | 0.268660387 | 26.7826003 |
| 135.8793785 | 3700 | 0.26445 | 1.02133 | 110.847 | 42949.12157 | 42903.93616 | 273.0140355 | 57.53503647 | 81.75631332 | 875.8281499 | 0.273014035 | 26.35550856 |
| 135.3964022 | 3750 | 0.2651 | 1.01813 | 110.453 | 43374.79167 | 43329.15843 | 270.9992036 | 58.10526803 | 81.46571468 | 872.7150636 | 0.270999204 | 26.55145718 |
| 134.9600074 | 3800 | 0.2693 | 1.01898 | 110.097 | 43811.45736 | 43765.36471 | 272.5488427 | 58.69022938 | 81.20314335 | 869.9022243 | 0.272548843 | 26.40049276 |
| 134.4402562 | 3850 | 0.27375 | 1.01933 | 109.673 | 44216.97936 | 44170.46007 | 274.5116283 | 59.23347037 | 80.89041788 | 866.5521006 | 0.274511628 | 26.2117266 |
| 133.7697282 | 3900 | 0.2759 | 1.01933 | 109.126 | 44567.8272 | 44520.9388 | 274.4896211 | 59.70346935 | 80.48697258 | 862.2301254 | 0.274489621 | 26.21382813 |
| 133.0330054 | 3950 | 0.28285 | 1.01901 | 108.525 | 44890.61012 | 44843.38213 | 279.380676 | 60.1358723 | 80.04369899 | 857.4814831 | 0.279380676 | 25.75490851 |
| 132.422543 | 4000 | 0.28455 | 1.01819 | 108.027 | 45250.24395 | 45202.6376 | 278.8260548 | 60.61764108 | 79.67639413 | 853.5466686 | 0.278826055 | 25.80613836 |
| 131.7471116 | 4050 | 0.2871 | 1.02146 | 107.476 | 45582.18461 | 45534.22904 | 279.2760836 | 61.06231182 | 79.26999853 | 849.1930879 | 0.279276084 | 25.76455405 |
| 131.259232 | 4100 | 0.28895 | 1.02547 | 107.078 | 45974.04594 | 45925.6781 | 278.6799103 | 61.58725285 | 78.97644967 | 846.0483967 | 0.27867991 | 25.81967154 |
| 130.7468358 | 4150 | 0.29305 | 1.02721 | 106.66 | 46353.04768 | 46304.2811 | 280.3232506 | 62.09496704 | 78.66814958 | 842.7456806 | 0.280323251 | 25.66830876 |
| 130.4268947 | 4200 | 0.2949 | 1.02756 | 106.399 | 46796.72436 | 46747.491 | 279.4184033 | 62.68932039 | 78.47564646 | 840.6834587 | 0.279418403 | 25.75143106 |
| 130.2381171 | 4250 | 0.29715 | 1.02669 | 106.245 | 47285.28912 | 47235.54176 | 278.6412275 | 63.34380621 | 78.36206218 | 839.4666682 | 0.278641227 | 25.823256 |
| 129.9120468 | 4300 | 0.3022 | 1.02417 | 105.979 | 47721.80818 | 47671.60158 | 280.7845828 | 63.92857115 | 78.16587122 | 837.3649398 | 0.280784583 | 25.62613544 |
| 129.5957832 | 4350 | 0.30605 | 1.02372 | 105.721 | 48159.18597 | 48108.51922 | 281.7792022 | 64.51448644 | 77.97558079 | 835.3264218 | 0.281779202 | 25.53568075 |
| 129.2721645 | 4400 | 0.3092 | 1.02014 | 105.457 | 48591.09735 | 48539.97619 | 282.1489686 | 65.09307887 | 77.78086492 | 833.2404955 | 0.282148969 | 25.50221532 |
| 128.8578346 | 4450 | 0.3099 | 1.01663 | 105.119 | 48985.75994 | 48934.22357 | 280.5093974 | 65.62177249 | 77.53156966 | 830.5698784 | 0.280509397 | 25.6512752 |
| 128.1517575 | 4500 | 0.31445 | 1.01144 | 104.543 | 49264.72811 | 49212.89825 | 283.0161362 | 65.9954808 | 77.10673506 | 826.0187669 | 0.283016136 | 25.42407598 |
| 127.2176764 | 4550 | 0.3171 | 1.00696 | 103.781 | 49449.04011 | 49397.01634 | 284.3374506 | 66.24238685 | 76.54471433 | 819.9980261 | 0.284337451 | 25.30593045 |
| 126.172045 | 4600 | 0.3194 | 1.00597 | 102.928 | 49581.5368 | 49529.37363 | 285.6344703 | 66.41988063 | 75.91557569 | 813.2582734 | 0.28563447 | 25.19102033 |
| 125.0651221 | 4650 | 0.32055 | 1.00795 | 102.025 | 49680.75352 | 49628.48597 | 286.0904071 | 66.55279225 | 75.24955899 | 806.1234583 | 0.286090407 | 25.15087388 |
| 124.2070424 | 4700 | 0.3207 | 1.00892 | 101.325 | 49870.42719 | 49817.96009 | 285.1356766 | 66.80688084 | 74.73326701 | 800.5925944 | 0.285135677 | 25.2350875 |
| 123.2153474 | 4750 | 0.32485 | 1.00934 | 100.516 | 49998.55181 | 49945.94992 | 288.085324 | 66.97851776 | 74.13658098 | 794.2004957 | 0.288085324 | 24.97671054 |
| 121.9572799 | 4800 | 0.32455 | 1.00944 | 99.48970001 | 50008.97771 | 49956.36484 | 287.7592716 | 66.99248438 | 73.37962314 | 786.0914585 | 0.287759272 | 25.00501099 |
| 120.7238516 | 4850 | 0.3257 | 1.00926 | 98.48350004 | 50018.86484 | 49966.24157 | 288.7218262 | 67.00572927 | 72.63749029 | 778.1412365 | 0.288721826 | 24.92164809 |
| 119.4265576 | 4900 | 0.3277 | 1.0087 | 97.42520003 | 49991.48114 | 49938.88668 | 290.6538808 | 66.96904582 | 71.85693053 | 769.7793598 | 0.290653881 | 24.75598719 |
| 118.018081 | 4950 | 0.32845 | 1.0075 | 96.27620003 | 49905.99945 | 49853.49493 | 291.8180812 | 66.85453377 | 71.00947409 | 760.7008413 | 0.291818081 | 24.65722384 |
| 116.7255677 | 5000 | 0.3287 | 1.00504 | 95.22179999 | 49858.01789 | 49805.56384 | 292.3212478 | 66.79025723 | 70.2317908 | 752.3697793 | 0.292321248 | 24.61478187 |
| 115.3262848 | 5050 | 0.32985 | 1.00511 | 94.08029998 | 49752.93318 | 49700.58969 | 293.9635528 | 66.64948479 | 69.38986606 | 743.3505199 | 0.293963553 | 24.47726489 |
| 113.8229291 | 5100 | 0.3297 | 1.00273 | 92.85389999 | 49590.55211 | 49538.37946 | 294.7919992 | 66.43195762 | 68.48532247 | 733.6604459 | 0.294791999 | 24.40847705 |
| 112.5014863 | 5150 | 0.33095 | 1.00172 | 91.77589999 | 49495.36133 | 49443.28883 | 296.4787529 | 66.30443918 | 67.69023279 | 725.142915 | 0.296478753 | 24.26961015 |
| 111.2676902 | 5200 | 0.3338 | 1.00156 | 90.76939997 | 49427.81656 | 49375.81511 | 299.4405384 | 66.21395558 | 66.94787864 | 717.1903223 | 0.299440538 | 24.02955788 |
| 110.019552 | 5250 | 0.3313 | 1.00219 | 89.75120003 | 49343.29937 | 49291.38684 | 297.7069265 | 66.10073559 | 66.1968951 | 709.1452857 | 0.297706927 | 24.1694872 |
| 108.6849928 | 5300 | 0.3324 | 1.00391 | 88.6625 | 49208.99103 | 49157.21981 | 299.5106319 | 65.92081491 | 65.3939135 | 700.5432113 | 0.299510632 | 24.02393432 |
| 107.2166958 | 5350 | 0.3308 | 1.00258 | 87.46470002 | 49002.15853 | 48950.60491 | 299.3270591 | 65.64374019 | 64.51046414 | 691.0791127 | 0.299327059 | 24.03866784 |
| 106.0250682 | 5400 | 0.3388 | 1.00254 | 86.49260001 | 48910.41302 | 48858.95592 | 307.1409762 | 65.52083707 | 63.79348206 | 683.3983225 | 0.307140976 | 23.42710451 |
| 105.0576446 | 5450 | 0.3438 | 1.00377 | 85.70340002 | 48912.8729 | 48861.41321 | 311.6580789 | 65.52413234 | 63.21139972 | 677.1626682 | 0.311658079 | 23.08755728 |
| 104.398149 | 5500 | 0.3488 | 1.00188 | 85.1654 | 49051.74908 | 49000.14329 | 315.2954235 | 65.71017215 | 62.81459242 | 672.9118039 | 0.315295424 | 22.82121215 |

Table 1

Graph 1                                               Torque and BSFC (Y axis) vs Engine speed (X axis)

Graph 2                                         Engine power and Torque (Y axis) vs Engine speed (X axis)

# Analysis

As can be seen from graph 2, once the metric power and torque data is converted to Imperial the lines do indeed cross at 5252rpm, proving that the data received and calculated is sound (Glucker et al., 2020). The BSFC is shown on the graph and in the table as g/kWh instead of the requested kg/kWh (Mdpi.com, 2020), the reason for this is twofold, firstly, from research, the industry standard is g/kWh, making comparison to real world results far easier, and also, it allows the data in graph 1 to be read more easily in the same scale.

The engine itself is a rare one (D16v3), seemingly only available in remote areas of Eastern Europe with little data available for comparison. There are other Honda engines which are similar however, and these units and their data would suggest that the test engine has endured a lot of wear and is not running efficiently. The D16 engine is rated at between 77-96 KW power with between 135-145 Nm (Engineswork, 2020) whereas this engine produced peak figures of only 50KW and 111 Nm.

All calculations made to create table 1 were taken from the booklet handed out and made available on Moodle (Moodle.uwtsd.ac.uk, 2020) with the exception of the conversion to Imperial units.

The corrected figures are present to account for the atmospheric conditions at the time and place of the dyno test, this gives the results more weight when comparing to other tests.

Several units had to be researched:

The specific gravity of fuel (Sunoco Race Fuels, 2020) to calculate the BSFC.

The Calorific Value of Petrol (known as motor spirit) to calculate Brake Thermal Efficiency (BTE) (GOV.UK, 2020)

During research into BTE figures for motor vehicles an equation was given (Moodle.uwtsd.ac.uk, 2020) however, it was decided to use other methods to correlate this answer, as used by others in the field so another equation was found:

(Dergipark.org.tr, 2020) which worked according to expected results of between 25-40% efficiency (X-engineer.org, 2020).

The BMEP should be at its highest when peak torque is reached (Bell, 1997), this is a true indication of the efficiency of an engine, this is borne out by reading across Table 1 at peak torque, which is generated at 2700 rpm. This correlates exactly with the peak BMEP figure, proving the calculations are correct. Average BMEP readings are in the region of 130-145 psi (Bell, 1997), by converting KPa to psi in any given cell gives us figures consistent with this: at 2600 rpm the BMEP is 880 KPa which converts to 127.6 psi, which, given the fact that this engine seems to be low on expected power and torque would support this.

The BSFC vs Torque graph is interesting in that they seem to almost mirror each other, indicating that the maximum torque of an engine is indeed its most efficient area of operating. This makes sense as this is where it is producing the most turning force. The BSFC then dies away at either side of the peak torque figure.

Graph 3.                                                                                                                                                                   (X-engineer.org, 2020)

Graph 3 represents the same phenomenon in a different manner, as torque increases so the BSFC figures decrease, denoting a more efficient combustion process. In this case it is showing how to make the best use of BSFC and torque with gearing.

# Conclusion

The results indeed showed that the torque and power were as expected from this type of engine. This is due to the fairly flat torque curve, this is a result of a few factors, firstly this is a naturally aspirated (NA) engine subject to only atmospheric pressure (14.7psi at sea level is 1 bar, the standard for atmospheric pressure on earth), it is also a low revving, economy-based engine. In general, with NA engines the amount of airflow available through the cylinder head will denote whereabouts in the rev range the maximum torque will occur. The VTEC in this engine will aid even further, using its combination of variable valve timing and camshaft profiles to boost the lower end of the torque range by using 8 valve levels of airflow and increasing the top end torque by engaging the VTEC to create 16v levels of torque further up the range (Hondalists.blogspot.com, 2020). This would indicate that for any given maximum torque level possible from an engine it is desirable to increase the torque either side of the peak by decreasing the BSFC either side of its lower point to flatten out the torque curve, this gives the best overall performance for the engine as power is a result of torque and engine speed. Therefore, with a similar torque figure throughout the range the power will increase with engine speed in a linear fashion until the breathing capabilities of the cylinder head are overcome and as the torque is maximised at each point either side of the peak torque, then the maximum possible power is extracted from the engine at any given rpm point.

With the correct research one can create a complete dyno table and graph from the raw data using only a few initial parameters and then verifying these results against other results and figures from other sources. The results show how the torque and BSFC are linked, showing that as the engine speed is increased beyond the maximum torque figure, so the BSFC deteriorates. This is due to a less perfect combustion efficiency due to all manner of factors from combustion chamber type, to port lengths, shapes and volumes, to camshaft profiles etc...

# Evaluation

The test went as planned, there was little room for error as the software in the computer controlled the sweep of the dyno and the whole process was overlooked by the technician. There is little that could have been changed from this. It would have been preferable to have completed the test from start to finish as a solo test but due to time and scheduling constraints this was not possible.

# Further Work

If this test were to be repeated then there are a few things that could be improved upon for more ongoing information and better results to prove more theories:

Multiple Engines: It would be interesting to compare different engines to the one on the stand, something designed for performance use to see how the torque and power figures differ for different applications. It would also be beneficial to see how forced induction in the form of a supercharger or turbocharger would affect the outcome of the tables and charts.

Mappable ECU: Another interesting option would be to implement a mappable ECU into the test. This could show how changing fuelling and ignition tables affects the power, torque and efficiency of an engine, and how switching between maps on the fly could give one engine different characteristics depending on the situation the driver finds themselves in. One could alter the BSFC about the peak torque figure to give different readings, backing up the claims and findings of this report.

Different Fuels: With different fuels of different octane levels, from low octane fuels available in poorer countries through to motorsport specific fuels high in chemicals such as Toluene (Vonkdevelopment.com, 2020) one could compare the effects of different calorific values of fuels on the BSFC, Torque and power outputs of an engine, along with other information, such as the effects of octane levels on the cooling effect of the charge and resultant BMEP of the engine with different octane levels.

Engine dyno vs Chassis dyno: If available, it would make sense to compare the readings collated in this test vs readings when the engine is installed in a vehicle with all of the ancillaries and transmission attached, the results would give more understanding to how much efficiency is lost once an engine is installed into a vehicle, and what changes could be made to reduce any losses.

Engine Dyno vs Real world: The ultimate test here is to correlate these results against real world results. As good as any dyno, engine or chassis, is, the results cannot be expected to be correct until validated with real world results. It is extremely difficult to replicate the conditions and airflow of the real world in a dyno cell.

# Bibliography

Moodle.uwtsd.ac.uk. (2020). UWTSD Moodle 2019/20: Log in to the site. [online] Available at: https://moodle.uwtsd.ac.uk/pluginfile.php/328107/mod_resource/content/1/W230%20Dynamometer.pdf [Accessed 2 Jan. 2020].

X-engineer.org. (2020). [online] Available at: https://x-engineer.org/wp-content/uploads/2017/08/The-effect-of-gear-ratios-on-BSFC.png?41ab8b&41ab8b [Accessed 2 Jan. 2020].

Moodle.uwtsd.ac.uk. (2020). UWTSD Moodle 2019/20: Log in to the site. [online] Available at: https://moodle.uwtsd.ac.uk/mod/resource/view.php?id=213510 [Accessed 2 Jan. 2020].

Sunoco Race Fuels. (2020). Sunoco Race Fuels. [online] Available at: https://www.sunocoracefuels.com/tech-article/specific-gravity-matter [Accessed 2 Jan. 2020].

GOV.UK. (2020). Digest of UK Energy Statistics (DUKES): calorific values. [online] Available at: https://www.gov.uk/government/statistics/dukes-calorific-values [Accessed 2 Jan. 2020].

Dergipark.org.tr. (2020). [online] Available at: https://dergipark.org.tr/tr/download/article-file/225867 [Accessed 2 Jan. 2020].

Engineswork. (2020). Honda D16 Engine Problems and Specs | D16A D16B D16W D16Y. [online] Available at: https://engineswork.com/engines/honda-engine/d16-engine.html [Accessed 2 Jan. 2020].

Glucker, J., Glucker, J., Contributor, J., Editor, V., Editor, V., Editor, V., Editor, V., Editor, V., Manager, J., Manager, J., Editor, V. and Editor, V. (2020). Why do horsepower and torque cross at 5,252 rpm?. [online] Motor Authority. Available at: https://www.motorauthority.com/news/1115177_why-do-horsepower-and-torque-cross-at-5252-rpm [Accessed 2 Jan. 2020].

Mdpi.com(2020).[online]Availableat:https://www.google.co.uk/urlsa=t&rct=j&q=&esrc=s&source=web&cd=16&ved=2ahUKEwiw3fmtueXmAhXJXhUIHS6QCrQQFjAPegQIChAC&url=https%3A%2F%2Fwww.mdpi.com%2F1996-1073%2F11%2F10%2F2531%2Fpdf&usg=AOvVaw0x2N7WZHhSPWyOiXvJNdhw [Accessed 2 Jan. 2020].

Hondalists.blogspot.com. (2020). How VTEC work. [online] Available at: https://hondalists.blogspot.com/2014/01/how-vtec-work.html#.Xg4zUm52uKM [Accessed 2 Jan. 2020].

Bell, A. (1997). Four-stroke performance tuning in theory and practice. Sparkford: Haynes, p.202 & 203.

Dsgroup.uk.com. (2020). DSG Ltd - DaTAQ Pro software suite - dsgroup.uk.com. [online] Available at: https://www.dsgroup.uk.com/products-and-services/control-and-data-acquisition/dataq-pro-software-suite [Accessed 3 Jan. 2020].

Vonkdevelopment.com. (2020). [online] Available at: http://www.vonkdevelopment.com/pdf/octanbooster.pdf [Accessed 3 Jan. 2020].