Module: M6X00408

Lecturer: Tim Tudor

Author: Harry Farnhill

P number: 148876

Computational fluid dynamics of an Ahmed body

Table of Contents

2	Introduction	3

3	Testing plan	4

4	Results and analysis	5

4.1	Domain size study	5

4.2	Scale factor and global element size	7

4.2.1	Scale factor	8

4.2.2	Global element size	10

4.3	Element size of the Ahmed body	14

4.4	Element size of the density box	16

4.5	Prism layers	20

4.5.1	Ahmed body	20

4.5.2	Ahmed body and floor	21

4.6	Accuracy study	23

4.7	Turbulence models in fluent	25

5	Conclusions	26

6	References	27

# Introduction

Figure 2-1 2-D CAD drawing of the Ahmed body

This report is a study into the aerodynamic affects of the Ahmed body using Computational fluid dynamics (CFD). The study will need to be refined using knowledge of aerodynamic principles and adjusting in ANSYS to analyse results and comparisons made to real life. CFD is like a virtual Wind Tunnel and is lot cheaper to use which breaks down the object into elements in a triangular shape creating a mesh, like FEA. The smaller the mesh the more accurate the study but increased time. To get more accurate results from the project finding the compromise between element size and time, where the small elements needs to be placed will show high level of understanding for using CFD.

# Testing plan

Figure 3-1 Testing plan of the Ahmed body

600 iterations as it could get accurate results without excessive computing time

When conducted the testing plan it was important that the accuracy of the simulation needed to be done in steps so that it would be easier to understand every setting change and what affect that had. The testing started with the scale factor, then the result of adjusting that would lead into the next test so gradually the results got more accurate the further the test session went. For example, most accurate scale factor/computing time would be the set value for adjusting the global element size and so on.

This method would get the simulation accurate to an extent because the settings were a compromise between accuracy and computing time. After the turbulence model settings there would be 1 very accurate study at the expense of computing time just to see how close the simulation could be compared to real life.

# Results and analysis

Figure 4-1 Testing plan and results for analysing domain study

Domain size study

Figure 4-2 Time vs accuracy graph of adjusting domain size

Figure 4-3 Graph showing variance in domain size

Figure 4-4 selection of different domain sizes

The domain size figures were chose based off increasing the domain size big enough show the difference in lift and drag without increasing computing time excessively. For example, to show the effects of using a larger domain the height was set to 50m, that would increase in computing time excessively and would be unnecessary amount to adjust the domain.

The graphs shows that making the domain too large increases the computing time as there are more elements in the study but also decreases the accuracy, this is the same for a smaller domain size as the air flow from the Ahmed body is restricted by the size of the domain. (Thabet, 2018)Therefore, the domain size selected will be the medium as it’s the most accurate results but also the computing time is less than the biggest domain. Also, the scale factor was set at 1 so the amount of elements were the same for all domains otherwise the smallest domain would have too small elements making it more accurate. (W.Meile, 2011)

## Scale factor and global element size

Adjusting the scale factor adjusts the size of all the domain elements instead of adjusting them separately, therefore if you wanted to make an accurate mesh quickly you can adjust the scale factor, however by doing this it makes the meshing process more inefficient as you are making every node smaller which doesn’t need to be accurate wasting time. So, when conducting the scale factor study, it was decided to go from maximum to minimum size nodes to show that to improve accuracy the scale factor is not the ideal way to do it.

Scale factor reducing all the element size making the big elements to big and small elements to small

Figure 4-5 Volume mesh of the largest scale factor

The global element size is the adjustment of the elements instead of scaling them. This means that when you have a mixture of small nodes and big nodes, you can adjust the size of the big nodes which don’t need to be accurate, compared to the scale factor which will adjust the size of every element.

Figure 4-6 Volume mesh of the smallest scale factor

Figure 4-7 table plan of adjusting scale factor

Scale factor

Figure 4-8 Adjusting the scale factor of the entire domain

Figure 4-4 shows that decreasing the scale factor decreases the lift and drag result getting them closer to the real-life coefficients, this is because decreasing the scale factor reduces the size of the nodes as can be seen in figure 4-8. This increases the accuracy of the airflow around and underneath the body which affect the lift and drag coefficients. (Khan, 2014)As the density box has smaller elements than the main domain this reduces the size of the nodes within the density box making the air flow more accurate. However, on the smallest scale factor this increases the computing solving time which reduces efficiency as can be seen in figure 4-9.

Figure 4-9 Time vs accuracy graph for adjusting the scale factor of the entire domain

Figure 4-9 shows the accuracy of the simulation compared to real life, 100% been exactly accurate. By reducing the scale factor, the accuracy increases as the entire domain has smaller nodes reducing any errors with large elements mixing with small elements which can be seen later.  This creates a more accurate study at the expensive of computing time as can be seen by decreasing the scale factor from 0.4 to 0.2. This makes you question whether it is the most reliable and efficient way to improve the accuracy is by adjusting the scale factor, as the computing time increases I would say no. What can be seen is that the Lift coefficient value is a lot closer than the drag value on a larger scale factor setting.

Figure 4-10 Volume mesh of the largest element size

Global element size

Larger elements increase and decrease the height of the boundary layer

Figure 4-11 Volume mesh of the smallest element size

Figure 4-12 Graph of adjusting the global element size of the entire domain

Results start to flat line, reducing the element size but not more accurate results

Results flat lining, increasing the computing time but no increase in accuracy

Figure 4-13 Time vs accuracy graph of adjusting the global element size

Figure 4-12 shows that adjusting the global elements size increases the accuracy but only to a certain degree because on the smallest global element size this was still far of the real-life values. This is because to get an accurate result you need to use a large domain so that you can see the wake region after the air hits the Ahmed body. (Banga, 2015)This means that there are certain areas of the domain where the air flow is insignificant and air flow around the floor of the Ahmed body which is. However, the global element size can only be adjusted for the whole domain which means that to get accurate results you need to reduce the size of the elements around the Ahmed body and density box. (W.Meile, 2011)

Figure 4-13 shows that when reducing the global element size from 0.2 to 0.1 the results start to flat line and increasing the computing time out ways the accuracy results, therefore going forward to the global element size shouldn’t be anything less than the 0.2.

Figure 4-14 Pressure plot of the smallest element size of the domain

Figure 4-15 Pressure plot of the largest element size of the domain

Figure 4-16 Turbulent kinetic energy of the smallest element size

Figure 4-17 Turbulent kinetic energy of the largest global element size

Post processing results

##### Analysis

Figure 4-17 and 16 show that by using the smaller element size the wake region shows higher pressures and the air floor sticking to ground closer than with larger elements. This is because when a boundary layer forms along the Ahmed body the air flow wants to follow the surface because of the friction between the Ahmed body and air flow (this can be seen in figure 4-16 along the top surface). (Bernard, 2009)The air attaches to the slant angle and when the angle drops away to the vertical surface the air tries to follow the surface, but the velocity is too fast. This creates vortices (turbulence) which is chaotic non-laminar air flow which increases kinetic energy which can be seen in the wake region in figures 4-17 and 16. The smaller elements are able to replicate the air flow more accurately which is why the kinetic energy plot is more turbulent and higher pressures in figure 17 compared to 16.

Figures 4-14 and 15 shows the pressure plot acting on the Ahmed body and the smaller element size show a bigger drop in pressure around the front face on the Ahmed body. This pressure drop comes from as soon as the air hits the vertical surface the velocity drops and the pressure increases. The air tries to increase to recover to the initial velocity and escapes around the sides, this causes the velocity to increase and pressure decrease, this pressure decrease is the blue next to the front face on the Ahmed body. (Banga, 2015)The smaller elements are able to show this in more depth as the air flow is split up into smaller nodes and show that there is greater pressure decrease than the larger elements can predict.  Therefore, going forward it will be important use a fine element size around the Ahmed body as to show this but this element size will not be necessary for the entire domain, but the density box and Ahmed body will need small element size (between 0.1 to 0.01) for an accurate result.

## Element size of the Ahmed body

Figure 4-18 Prism mesh of the largest element size on the Ahmed body

Trying to get the smallest elements for the computing time to get the most accurate results

Adjusting the node sizes on the Ahmed body will be important as this where the air will be flowing over the body and hitting the surfaces, the smaller the nodes the more accurate the Lift and drag results will be but will take longer to solve and vice versa for using larger nodes. Therefore, to reduce computing time it will be important to use the smallest elements on the ahmed body and largest elements in the domain where the air flow has a neglible effect on the Ahmed body.

Figure 4-19 Prism mesh of the smallest element size on the Ahmed body

Figure 4-20 Graph of adjusting the element size of the Ahmed body

Figure 4-21 testing plan of adjusting the size of the elements on the Ahmed body

Could be an error in the results as drag coefficient trend is constant

Lift and drag starting to converge as the elements are smaller and more accurate

Figure 4-22 Time vs accuracy graph by adjusting the element size on the Ahmed body

Figure 4-20 shows that by decreasing the element size on the Ahmed body the closer the results get to the real coefficients. As the Ahmed body is small compared to the domain you can use very small elements which doesn’t increase the computing time excessively, these nodes also need to be small as it’s where the main air flow around the body and can see that on average the drag coefficient decreases by 0.1. On the element size of 0.1 it can be seen that lift coefficient doesn’t follow the same trends as by using smaller nodes, this could because the element size is too big that causes inaccuracy in the results as it can’t replicate accurate airflow.

## Element size of the density box

The density box is an area containing the wake region and airflow around the Ahmed body, this means that you can run smaller nodes in the wake box without adjusting nodes in the whole domain which improves the accuracy in the results. This reduces the computing time but can still produce accurate results. Therefore, you want these nodes in the density box to be as small possible.

Larger elements increase the boundary layer height and reduce accuracy of results

Figure 4-23 Prism mesh of the largest elements of the density box

Figure 4-24 Prism mesh of the smallest element size in the density box

Much smaller elements increase the accuracy of the wake region but there’s to much variance in the boundary layer height with makes the study less accurate as the boundary layer height should remain consistent unless it collides with another surface

Figure 4-25 Time vs accuracy graph of adjusting the global element size of the density box

Figure 4-26 Graph of adjusting the element sizes of the density box

Figure 4-27 Testing plan of the adjusting the element size of the density box

Figure 4-25 and 26 shows that by reducing the element size that the accuracy does increase but only marginally, this could because the scale used to adjust the element size of the density box was a lot smaller compared to the element size of the entire domain. The results start to flat line between 0.6 to 0.4, this shows that this the most accurate the density box can be without increasing computing time.  By decreasing the size of the elements in the density box it will create an inconsistent boundary layer, this means there will need to be a compromise between the element size of the domain and density box. This would be between 0.4-0.6.

Figure 4-28 Velocity vectors of the smallest element size of the density box

Figure 4-29 Velocity vectors of the largest element size of the density box

Figure 4-30 Turbulent kinetic energy of the smallest element size of the density box

Figure 4-31 Turbulent kinetic energy of the largest element size of the density box

Post processing of the density box

##### Analysis

From making the density box finer (smaller elements) it can be seen the wake region shows the air flow to be more streamlined, this is because the smaller elements break the air flow down which show it be more laminar. This can be seen in the turbulent kinetic energy plot and velocity vectors in figures 28-29 where the velocity vectors show the turbulence in more detail, however both element sizes in the density box predict the same trends but the smaller elements can do it in more detail. This shows that adjusting previous variable (element size, scale factor) has resulted in an accurate base case as no matter how fine the density box is there is still a wake region around the density box.

## Prism layers

Prism layers are ways of inputting boundary layers into the study, there will be boundary layers forming around the Ahmed body but adjusting the amount and height of these will affect the results. As I’m comparing my simulation to a wind tunnel, the wind tunnel has a stationary floor which means that a boundary layer will form on the floor which will interact with the Ahmed body and affect the results, therefore I would need to adjust the number of prims layers to find the most accurate result in replicating real life boundary layers.

Figure 4-32 Graph of adjusting the amount of prism layers on the Ahmed body

Ahmed body

Figure 4-33 Graph of time vs accuracy for adjusting the prism layers on the Ahmed body

Figure 4-32 and 33 show that by adjusting the prism layers from 4-6 affects the accuracy of the results but no trends saying less elements is more accurate and vice versa, this could because the once the Ahmed body has at least 3 prism layers it is forming the boundary layer and by increasing the prism layers it reducing the amount of turbulent airflow caused by the Ahmed body.

The element size of the density box is 0.1 and Ahmed body is 0.01 which means that the prism layer is going to be small, this means that by increasing the prism layers from 4-6 isn’t a big enough scale to make a significant difference to the coefficients. The reason to use 4-5 prism layers is because that seemed like a realistic height for the boundary layer around the Ahmed body and 6 prism layers should be too high to see a drop off of accuracy. Figure 4-32 shows that using 6 prism layers on Ahmed body causes the drag coefficient to decrease in accuracy showing 5 prism layers is the most accurate amount for the scale that was used.

Figure 4-34 Time vs accuracy graph of adjusting the amount of prism layers on the floor and Ahmed body

Figure 4-35 graph showing the amount of prism layers on the floor and Ahmed body

Ahmed body and floor

Figure 4-35 and 34 show that increasing the prism layers on the floor increases the accuracy of the simulation but affects the lift coefficient more. This is because when a boundary layer height increases the air velocity decreases and in turn pressure increases according to Bernoulli’s equation. (Bernoullis equation, n.d.) When this happens between the Ahmed body and the floor, pressure increases below the Ahmed and decreases because of the Venturi affect. (Huminic, 2012) This decreases the coefficient of Lift as the pressure above the Ahmed body decreases and reduces the aero load on the Ahmed body. By increasing the prism layers, it increases this process.

The prism layer height also depends on the mesh element size, for example a small element size on the density box will compress the prism layers and reduce the height of the boundary layer which means that trying to get the accuracy of the prism layers depends on the element size. For the element size of the density box of 0.1, 6 prism layers result in the most accurate setting.

## Accuracy study

| Mesh Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Scale factor | Global element size | Ahmed body element size | Density box size element size | Prism layers |
| 1 | 0.2 | 0.01 | 0.1 | 3 |

Figure 4-36 Table of the mesh parameters used for accuracy study

These values were decided on the previous study and then by increasing from 30-50% more accurate as computing time wouldn’t be considered too important as long as it wasn’t over 6 hours as this wouldn’t be useful because the solver time would be too long.

Figure 4-37 Volume mesh of the accurate study

| Table of results |  |  |
| --- | --- | --- |
|  | drag | Lift |
| Coefficient | 0.303 | 0.36 |
| Accuracy % (100 being exact) | 101.33 | 104.34 |

Figure 4-38 Turbulent kinetic energy plot of the accuracy study

Smaller elements in the wake region

Figure 4-39 Turbulence vortex from the accurate run

Figure 4-40 Velocity vector of the accurate run

Figure 4-41 Pressure plot of the accurate run

When conducting the accurate study, it was important to increase the amount of elements in the density box to refine the wake region. As there are multiple Ahmed bodies with different slant angles which change the turbulence, there needs to small elements in the density box to show this (figure 4-38)

From figure 4-37 the turbulence region shows the different size elements between the density box and the entire domain, this can reduce the accuracy of the turbulence as its less refined at long distance, this could be more accurate by reducing of the element size of the entire domain, but this will increase computing time. (Thabet, 2018)As the elements in the density box were very small this seemed more important to get an accurate study by decreasing the elements in the wake region instead reducing elements after the density box region as this would result in a less accurate study.

From figure 4-39 it can be seen that the pressure region around the “feet” of the ahmed body show the same trends as the flat face surface on the Ahmed body, this is because the elements are so small in this region they can replicate air flow the same as front surface whereas using larger elements in this region cant show this. (W.Meile, 2011)Therefore, reducing the size of the density box elements was the largest difference between the previous studies and the accurate one which increased the accuracy in lift by 5% and drag by 7%.

## Turbulence models in fluent

Figure 4-42 Table of plan and results of turbulence models

K-epsilon

- Is the most widely used engineering turbulence model for industry

- Does not perform well for air flow with strong separation, large streamline curvatures and large pressure gradient (ANSYS model turbulent flows, n.d.)

K-omega

- Most common turbulence model solver

- Two equation model which mean that include extra transport equations which can account for convection and diffusion of turbulent energy. (ANSYS model turbulent flows, n.d.)

Transition omega

Figure 4-43 Bar chart showing adjusting turbulence models with Fluent

Transition attempts to predict air flow between laminar and turbulent. (ANSYS model turbulent flows, n.d.)

The bar chart shows the difference between turbulence model and there is minimal difference between K-omega and K-epsilon as they are very similar in the way they calculate turbulence with K-omega been more accurate. (ANSYS model turbulent flows, n.d.) Whereas transitional turbulence model decreases in accuracy and would not be the correct turbulence model to use for the Ahmed body as the accuracy decreases by 45% for lift and 70% for drag.

# Conclusions

The conclusion made from the study were that breaking down each variable and starting with larger element to small elements was the best way to get the accurate study as you were able to understand each change and what affect that had. Once the domain size and density box size were selected increasing the accuracy would be by reducing the element size on the Ahmed body and the density to further refine the wake region. As the slant angle caused the biggest change in laminar to turbulent air flow the density box region would need to be the most accurate area. To make comparisons to wind tunnel data prism layers on the floor (for a stationary floor) and ahmed body would need to be selected but the amount would depend on the element size, finding a compromise by element size and amount of prism layers would lead a more accurate air flow representation on the body and floor.

# References

ANSYS model turbulent flows. (n.d.). Retrieved from http://www.southampton.ac.uk/~nwb/lectures/GoodPracticeCFD/Articles/Turbulence_Notes_Fluent-v6.3.06.pdf

Banga, S. (2015). CFD of simulation flow around an extrnal vehicle: Ahmed body.

Bernard, R. (2009). Road vehicle Aerodynamic design . MechAero publishing .

Bernoullis equation. (n.d.). Retrieved from https://www.princeton.edu/~asmits/Bicycle_web/Bernoulli.html

Huminic, A. (2012). Study of aerodynamics for a simplified car model with the underbody shaped as a Venturi nozzle.

Khan, R. S. (2014). CFD aerodynamic Analysis of the Ahmed body . Engineering trends and technology .

Thabet, S. (2018). CFD simulation of the air flow around a car model (Ahmed body).

W.Meile. (2011). Experiments and numerical simulations on the aerodynamics of the Ahmed body.