# COUNTERMOVEMENT JUMP (CMJ) – VIDEO RECORDING INSTRUCTIONS

## Preparation

## Purpose of the Recording

Film a side-on (sagittal plane) view of a participant performing a countermovement jump to analyse joint angles, displacement, and movement phases.

## Required Equipment

- Smartphone/camera (60–240 fps)

- Tripod

- 1 m calibration object

- Floor tape

- Optional anatomical markers (hip, knee, ankle, shoulder)

## Preparing the Filming Area

- Choose a well-lit area with an uncluttered background.

- Mark the jump line and camera position with tape.

- Ensure the full body stays in frame during the jump.

## Camera Setup

- Position camera perpendicular to the participant (side view).

- Distance: 3–5 m away.

- Height: hip height.

- Record in landscape.

- Calibration object must be in the same plane of motion.

## Participant Preparation

- Wear tight clothing.

- Apply anatomical markers if available.

- Complete warm-up.

## Recording

## Recording the Jump

- Ensure participant stands still for 3 seconds.

- Cue: 'Perform a maximal CMJ with hands on hips.'

- Record full jump including landing.

- Capture 3 maximal trials with rest periods.

- Do not move the camera.

## Naming and Saving Videos

- Use filename format: CMJ_ParticipantID_Trial#.mp4

- Example: CMJ_05_T1.mp4

## Considerations

## Common Recording Mistakes to Avoid

- Camera tilted or handheld

- Calibration object not visible

- Athlete moving out of plane

- Jump cut off in frame

- No still frames at start

- Filming too close or too far

## 9. Final Checks Before Leaving

- Full body visible

- Markers visible

- Calibration object visible

- Lighting adequate

- No obstructions

- Participant remains in plane

# CMJ Data Analysis Tasks

To be completed after capturing the video and importing it into Dartfish.

# Part A - Data Extraction Tasks

### 1. Identify Key Movement Events

Using Dartfish frame-by-frame playback, record the frame number (or timestamp) for the following:

- Standing baseline

- Start of countermovement

- Maximum knee flexion (lowest point)

- Take-off

- Apex of jump

- Landing contact

Record in a table.

### 2. Joint Angle Measurements

Using the angle tool, measure:

- Hip angle at: standing, lowest point, take-off

- Knee angle at: standing, lowest point, take-off

- Ankle angle at: standing, lowest point, take-off

Create a table and calculate Δangle (change) for each joint.

### 3. Vertical & Horizontal Displacement

Track the hip (or shoulder) marker and record:

- Horizontal displacement between standing and take-off

- Vertical displacement between standing and apex

Use the calibration tool so values are in metres.

# Part B - Interpretation Questions

# Joint Contribution Analysis

Based on joint angle changes, which joint appears to contribute the most to vertical propulsion?
Justify using your measured Δangles.

### Movement Plane & Axis Identification

For each joint movement during the CMJ, identify:

- Movement name (flexion/extension, etc.)

- Plane of movement

- Axis of rotation

Then explain why the sagittal plane dominates the CMJ.

### Technique Evaluation

Compare two trials with different countermovement depths.

- How does countermovement depth affect jump height?

- How does it influence joint loading?

- Did deeper countermovement improve take-off velocity?

### Frontal & Transverse Plane Deviations

Did the participant show knee valgus, hip shift, trunk rotation, or asymmetry?
If yes:

- Identify the plane

- Identify the axis

- Discuss potential injury implications

- Suggest one corrective strategy

### Practical Application

Suggest two coaching cues or technique modifications that could meaningfully improve the participant’s CMJ performance based on your analysis.