# ELECTRONICS: Lab Exercise  1

Perform the following exercise to confirm:

- In a series connected circuit the same current is observed in all parts

of  the circuit.

- The sum of the voltage drops across the resistors is equal to the supply voltage. (Kirchhoff’s Voltage Law).

- The effective resistance of a number of resistors in series is equal to the sum of the resistor values.

Procedure:

  - Connect the circuit as shown

  - Start the simulation and note the circuit currents at all ammeters.

  - Note the voltage across each resistor.

  - Place your readings in the table below.

  - Ensure ammeter resistance is set to 1e-009, DC and voltmeter resistance to 100M,DC.

10M.

| V1 = | A1 = |
| --- | --- |
| V2 = | A2 = |
| V3 = | A3 = |

V1 + V2 + V3 =………………

Compare the battery voltage with V1 + V2 + V3. What do you observe?

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Why are they the same?

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Now change R2 to 2k and R3 to 3k. Re- run the simulation and record V1,V2 and V3 below.

| R1 = 1k | R2 = 2k | R3 = 3k |
| --- | --- | --- |
| V1 = | V2 = | V3 = |

What do you observe about the relationship between the voltages and the resistances now?

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Write down a sentence that defines the relationship between resistor voltage and resistor value in a series circuit.

……………………………………………………………………………………………………………………………………………………………………………………

Now remove all meters and the supply and connect  an ohmmeter  across the three resistors .What is the measured total effective resistance.

……………………………………………………………………………………………………………………………………………………………………………………

How does this compare with the individual resistors in the circuit?

……………………………………………………………………………………………………………………………………………………………………………………

Summarise all of your results in a few simple sentences.

Exercise 1 confirms:

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………