| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 6 | LEFEL 6 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| December 2019 | Rhagfyr 2019 |

MODULE CODE: ACCA6010

COD Y MODWL: ACCA6010

ADVANCED ROUTING AND SWITCHING SOLUTION

## VLANs

| VLAN | Name | Network |
| --- | --- | --- |
| 10 | Student | 192.168.10.0/24 |
| 20 | Staff | 192.168.20.0/24 |
| 30 | Guest | 192.168.30.0/24 |
| 99 | Management and Native | 192.168.99.0/24 |

## Port Assignments

## DS1 (VTP Server)

| Ports | Assignment | IP Address |
| --- | --- | --- |
| Fa0/1 – 0/5 | 802.1q Trunks (Native VLAN 99) | 192.168.99.2 |

## AS1 (VTP Client)

| Ports | Assignment | IP Address |
| --- | --- | --- |
| Fa0/1 – 0/4 | 802.1q Trunks (Native VLAN 99) | 192.168.99.3 |
| Fa0/5 – 0/11 | VLAN 10 (Student) |  |
| Fa0/12 – 0/18 | VLAN 20 (Staff) |  |
| Fa0/19 – 0/24 | VLAN 30 (Guest) |  |

## AS2 (VTP Client)

| Ports | Assignment | IP Address |
| --- | --- | --- |
| Fa0/1 – 0/4 | 802.1q Trunks (Native VLAN 99) | 192.168.99.4 |
| Fa0/5 – 0/11 | VLAN 10 (Student) |  |
| Fa0/12 – 0/18 | VLAN 20 (Staff) |  |
| Fa0/19 – 0/24 | VLAN 30 (Guest) |  |

Final configuration

R1#sh run

Building configuration...

Current configuration : 1350 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R1

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

!

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

!

interface GigabitEthernet0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

!

interface GigabitEthernet0/0.99

encapsulation dot1Q 99 native

ip address 192.168.99.1 255.255.255.0

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/0/0

ip address 1.1.1.2 255.255.255.252

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

router ospf 1

log-adjacency-changes

network 1.1.1.0 0.0.0.3 area 0

network 192.168.99.0 0.0.0.255 area 0

network 192.168.10.0 0.0.0.255 area 0

network 192.168.20.0 0.0.0.255 area 0

network 192.168.30.0 0.0.0.255 area 0

!

ip classless

!

end

R2#sh run

Building configuration...

Current configuration : 1260 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname R2

interface Loopback0

ip address 209.165.200.225 255.255.255.248

!

interface Loopback60

ip address 172.16.60.1 255.255.255.0

!

interface Loopback61

ip address 172.16.61.1 255.255.255.0

!

interface Loopback62

ip address 172.16.62.1 255.255.255.0

!

interface Serial0/0/0

ip address 1.1.1.1 255.255.255.252

clock rate 64000

!

interface Serial0/0/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

log-adjacency-changes

network 1.1.1.0 0.0.0.3 area 0

network 172.16.60.0 0.0.0.255 area 60

network 172.16.61.0 0.0.0.255 area 60

network 172.16.62.0 0.0.0.255 area 60

default-information originate

!

ip classless

ip route 0.0.0.0 0.0.0.0 Loopback0

!

end

DS1#sh run

Building configuration...

Current configuration : 2375 bytes

!

version 12.2(37)SE1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname DS1

!

spanning-tree mode rapid-pvst

!

interface Port-channel1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport trunk encapsulation dot1q

!

interface Port-channel2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport trunk encapsulation dot1q

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport trunk encapsulation dot1q

channel-group 1 mode active

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport trunk encapsulation dot1q

channel-group 1 mode active

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport trunk encapsulation dot1q

channel-group 2 mode active

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport trunk encapsulation dot1q

channel-group 2 mode active

!

interface FastEthernet0/5

switchport trunk native vlan 99

switchport trunk encapsulation dot1q

switchport mode trunk

!

interface FastEthernet0/6

shutdown

!

interface FastEthernet0/7

shutdown

!

interface FastEthernet0/8

shutdown

!

interface FastEthernet0/9

shutdown

!

interface FastEthernet0/10

shutdown

!

interface FastEthernet0/11

shutdown

!

interface FastEthernet0/12

shutdown

!

interface FastEthernet0/13

shutdown

!

interface FastEthernet0/14

shutdown

!

interface FastEthernet0/15

shutdown

!

interface FastEthernet0/16

shutdown

!

interface FastEthernet0/17

shutdown

!

interface FastEthernet0/18

shutdown

!

interface FastEthernet0/19

shutdown

!

interface FastEthernet0/20

shutdown

!

interface FastEthernet0/21

shutdown

!

interface FastEthernet0/22

shutdown

!

interface FastEthernet0/23

shutdown

!

interface FastEthernet0/24

shutdown

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

mac-address 00d0.d33a.2401

ip address 192.168.99.2 255.255.255.0

!

ip classless

!

line con 0

!

line aux 0

!

line vty 0 4

password cisco

login

end

AS1#sh run

Building configuration...

Current configuration : 2899 bytes

!

version 12.2

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname AS1

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

interface Port-channel1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

!

interface Port-channel3

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

channel-group 3 mode active

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

channel-group 3 mode active

!

interface FastEthernet0/5

switchport access vlan 10

switchport mode access

shutdown

!

interface FastEthernet0/6

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/7

switchport access vlan 10

switchport mode access

shutdown

!

interface FastEthernet0/8

switchport access vlan 10

switchport mode access

shutdown

!

interface FastEthernet0/9

switchport access vlan 10

switchport mode access

shutdown

!

interface FastEthernet0/10

switchport access vlan 10

switchport mode access

shutdown

!

interface FastEthernet0/11

switchport access vlan 10

switchport mode access

shutdown

!

interface FastEthernet0/12

switchport access vlan 20

shutdown

!

interface FastEthernet0/13

switchport access vlan 20

shutdown

!

interface FastEthernet0/14

switchport access vlan 20

shutdown

!

interface FastEthernet0/15

switchport access vlan 20

shutdown

!

interface FastEthernet0/16

switchport access vlan 20

shutdown

!

interface FastEthernet0/17

switchport access vlan 20

shutdown

!

interface FastEthernet0/18

switchport access vlan 20

shutdown

!

interface FastEthernet0/19

switchport access vlan 30

shutdown

!

interface FastEthernet0/20

switchport access vlan 30

shutdown

!

interface FastEthernet0/21

switchport access vlan 30

shutdown

!

interface FastEthernet0/22

switchport access vlan 30

shutdown

!

interface FastEthernet0/23

switchport access vlan 30

shutdown

!

interface FastEthernet0/24

switchport access vlan 30

shutdown

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

mac-address 0003.e436.7701

ip address 192.168.99.3 255.255.255.0

!

line con 0

!

line vty 0 4

password cisco

login

line vty 5 15

login

end

AS2#sh run

Building configuration...

Current configuration : 2761 bytes

!

version 12.2

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname AS2

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

interface Port-channel2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

!

interface Port-channel3

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

channel-group 2 mode active

!

interface FastEthernet0/2

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

channel-group 2 mode active

!

interface FastEthernet0/3

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

channel-group 3 mode active

!

interface FastEthernet0/4

switchport trunk native vlan 99

switchport trunk allowed vlan 10,20,30,99

switchport mode trunk

channel-group 3 mode active

!

interface FastEthernet0/5

switchport access vlan 10

shutdown

!

interface FastEthernet0/6

switchport access vlan 10

shutdown

!

interface FastEthernet0/7

switchport access vlan 10

shutdown

!

interface FastEthernet0/8

switchport access vlan 10

shutdown

!

interface FastEthernet0/9

switchport access vlan 10

shutdown

!

interface FastEthernet0/10

switchport access vlan 10

shutdown

!

interface FastEthernet0/11

switchport access vlan 10

shutdown

!

interface FastEthernet0/12

switchport access vlan 20

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/13

switchport access vlan 20

shutdown

!

interface FastEthernet0/14

switchport access vlan 20

shutdown

!

interface FastEthernet0/15

switchport access vlan 20

shutdown

!

interface FastEthernet0/16

switchport access vlan 20

shutdown

!

interface FastEthernet0/17

switchport access vlan 20

shutdown

!

interface FastEthernet0/18

switchport access vlan 20

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/19

switchport access vlan 30

shutdown

!

interface FastEthernet0/20

switchport access vlan 30

shutdown

!

interface FastEthernet0/21

switchport access vlan 30

shutdown

!

interface FastEthernet0/22

switchport access vlan 30

shutdown

!

interface FastEthernet0/23

switchport access vlan 30

shutdown

!

interface FastEthernet0/24

switchport access vlan 30

shutdown

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

mac-address 00e0.b04c.db01

ip address 192.168.99.4 255.255.255.0

!

line con 0

!

line vty 0 4

login

line vty 5 15

login

end