Q1.

A)

- Inverting amplifier

- Gain = -R1/R2

- Gain = -29/2 = -14.5

- Vout = 0.345 x -14.5 = -5.0025v

- Output is 180° out of phase

B)

- Non-inverting amplifier

- Gain = 1+R1/R2

- Gain = 1+27/2 = 14.5

- Vout = 0.62 x 14.5 = 8.99v

- Output is in phase

Q2.

A)

- 0100000101    =   0  1000 0010 1  Data = 0100 0001 = 41hex

- Ascii Character = ‘A’

B)

- 0101010101    =   0  1010 1010  1  Data = 0101 0101 = 55hex

- Ascii Character = ‘U’

Q3.

- I2C Serial transmission protocol.

- Multi point to multi point transmission system.

- Serial Data – (SDA) is the signal line for incoming data bits in series from a master or slave.

Serial Clock – (SCK) is the signal line that carries the clock signal and is always controlled by the master.

- Low-speed devices like microcontrollers, and A/D and D/A converters in embedded systems also memory devises in computers.

Q4.

- RS232

- Point-to-point Transmission System

- Data collection of scientific control instruments.

- See next page – Hex A5 = 10100101, Signal transmitted 0 10100101 1

| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Start Bit | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | Stop Bit |  |  |  |
|  |  | Data |  |  |  |  |  |  |  |  |  |  |  |  |

End of exam