| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| May 2022 | Mai 2022 |

MODULE CODE: ACDA4006

Electrical & Electronic Principles

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator, Formulae Sheet

Adnoddau neu ddefnyddiau sydd eu hangen – Cyfrifiannell, Taflen Fformiwla

5 Questions

Total marks available = 50

ANSWER ALL QUESTIONS

- For the circuit shown in Fig Q1, given the following component values R1=100Ω, R2=500Ω, R3=300Ω, R4=150Ω, and R5=470Ω. If the battery terminal voltage is 15V, calculate:

  - The total circuit resistance

[4 Marks]

  - Write the current I2 in terms of the currents  I1 and  IT

[2 Marks]

  - Write the current I2 in terms of the currents  I3 and  I4

[2 Marks]

  - Total circuit current IT

[2 Marks]

  - The value of current flowing I5

[2 Marks]

  - Voltage across R3

[2 Marks]

  - Total power dissipated

[2 Marks]

  - Power dissipated by R1

[2 Marks]

  - Power dissipated by R3

[2 Marks]

Fig Q1

  - Define the term “time constant” in relation to a RC circuit

[2 Marks]

  - For a RC circuit consisting of a 1kΩ resistance and a 47µF capacitance connected to a 24Vdc power supply: calculate the voltage on the capacitor at a time of 20ms after the supply was connected, assuming the capacitor had an initial charge of 0V.

[3 Marks]

  - For the circuit described in part b, plot a graph of charging current against time, for times between 0s and 235ms

[10 Marks]

- Design and sketch an LED circuit consisting of a 9V battery, an on-off switch and a resistor.  Assuming the forward voltage across the LED is 2V and the current through the LED should be limited to a maximum of 20mA. Determine a suitable value of the resistor.

[5 Marks]

- An oscilloscope display shows the waveform in Fig Q4. The oscilloscope settings are shown in the box next to the waveform.

Oscilloscope settings

5 V/div

2 µs/div

Fig Q4

- Estimate the period of the waveform

[1 Mark]

- From the estimated period, determine its frequency

[1 Mark]

- Determine its peak to peak amplitude

[1 Mark]

- Determine the RMS value of the waveform

[2 Marks]

- For an AC voltage V=50sin100πt+1.57, find the following quantities:

  - Peak value

[1 Mark]

  - RMS value

[1 Mark]

  - Frequency

[1 Mark]

  - Time period

[1 Mark]

  - Phase angle relative to V=50sin100πt and state if it is leading or lagging this waveform

[1 Mark]

Diwedd/End

# Formulae Sheet

|  |  | RT=R1+R2+…+Rn 1RT=1R1+1R2+…+1Rn 1CT=1C1+1C2+…+1Cn CT=C1+C2+…+Cn LT=L1+L2+…+Ln 1LT=1L1+1L2+…+1Ln | VOut=VinRBRA+RB I=IinRBRA+RB |
| --- | --- | --- | --- |
|  |  |  | XC=12πfC XC=-j1ωC XL=2πfL XL=jωL |
| Z=R+jXL-XC Z=R2+XL-XC2 | ω=2πf XT=XL-XC | φ=tan-1XTR pf=cosφ | fres=12πLC |
| VC=Vs1-e-tτ IC=VRe-tRC VC=V0e-tτ IC=-VRe-tRC τ=RC | τ=LR VL=Ve-RtL VL=-Ve-RtL IL=VR1-e-RtL IL=VRe-RtL | f=12πRC AV=20logVoutVin AP=10logPoutPin | Q=XLR Q=XCR Q=fRfU-fL Q=1RLC |
|  |  | E=12CV2 E=12LI2 |  |
| VB =RB2(RB1 + RB2)× VCC VE=VB-0.7 IC=IE VC=VCC-IC×RC Av=vcvb VE=10%VCC RIN=11RB1+1RB2+1hFE×RE ROUT=RC AV≈-RCRE |  | Vout=-R2R1×Vin Vout=1+R2R1×Vin Vout=R2R1×V2-V1 Vout=-R2R1×V1+V2 Vout=-RR1×V1+RR2×V2+…+RRn×Vn |  |

Detach this page and place in answer book

Student Number ………………………….	Question Number ……………………….