Lab - Accessing Network Devices with SSH

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
| S1 | VLAN 1 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |

Objectives

Part 1: Configure Basic Device Settings

Part 2: Configure Basic Security Measures on the Router and Switch

Part 3: Configure the Router for SSH Access

Part 4: Configure the Switch for SSH Access

Part 5: SSH from the CLI on the Switch

Background / Scenario

In the past, Telnet was the most common network protocol used to remotely configure network devices. Telnet does not encrypt the information between the client and server. This allows a network sniffer to intercept passwords and configuration information.

Secure Shell (SSH) is a network protocol that establishes a secure terminal emulation connection to a router or other networking device. SSH encrypts all information that passes over the network link and provides authentication of the remote computer. SSH is rapidly replacing Telnet as the remote login tool of choice for network professionals. SSH is most often used to log in to a remote device and execute commands; however, it can also transfer files using the associated Secure FTP (SFTP) or Secure Copy (SCP) protocols.

The network devices that are communicating must be configured to support SSH in order for SSH to function. In this lab, you will enable the SSH server on a router and then connect to that router using a PC with an SSH client installed. On a local network, the connection is normally made using Ethernet and IP.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). The switches used are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and the output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the correct interface identifiers.

Note: Make sure that the routers and switches have been erased and have no startup configurations. If you are unsure, contact your instructor.

Required Resources

1 Router (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

1 PC (Windows 7 or 8 with terminal emulation program, such as Tera Term, and Wireshark installed)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords on the router.

Cable the network as shown in the topology.

  - Initialize and reload the router and switch.

  - Configure the router.

    - Console into the router and enable privileged EXEC mode.

    - Enter configuration mode.

    - Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

    - Assign class as the privileged EXEC encrypted password.

    - Assign cisco as the console password and enable login.

    - Assign cisco as the VTY password and enable login.

    - Encrypt the plaintext passwords.

    - Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

    - Configure and activate the G0/1 interface on the router using the information contained in the Addressing Table.

    - Save the running configuration to the startup configuration file.

Configure PC-A.

Configure PC-A with an IP address and subnet mask.

Configure a default gateway for PC-A.

Verify network connectivity.

Ping R1 from PC-A. If the ping fails, troubleshoot the connection.

- Configure Basic Security Measures on the Router

  - Encrypt the clear text passwords.

R1(config)# service password-encryption

  - Strengthen passwords.

An administrator should ensure that passwords meet the standard guidelines for strong passwords. These guidelines could include combining letters, numbers and special characters in the password and setting a minimum length.

Note: Best practice guidelines require the use of strong passwords, such as those shown here, in a production environment. However, the other labs in this course use the cisco and class passwords for ease in performing the labs.

    - Change the privileged EXEC encrypted password to meet guidelines.

R1(config)# enable secret Enablep@55

    - Require that a minimum of 10 characters be used for all passwords.

R1(config)# security passwords min-length 10

  - Secure the console and VTY lines.

    - You can set the router to log out of a connection that has been idle for a specified time. If a network administrator was logged into a networking device and was suddenly called away, this command automatically logs the user out after the specified time. The following commands cause the line to log out after five minutes of inactivity.

R1(config)# line console 0

R1(config-line)# exec-timeout 5 0

R1(config-line)# line vty 0 4

R1(config-line)# exec-timeout 5 0

R1(config-line)# exit

R1(config)#

    - The following command impedes brute force login attempts. The router blocks login attempts for 30 seconds if someone fails two attempts within 120 seconds. This timer is set especially low for the purpose of this lab.

R1(config)# login block-for 30 attempts 2 within 120

What does the 2 within 120 mean in the above command?

The 2 within 120 means 2 incorrect attempts within 120 seconds

What does the block-for 30 mean in the above command?

Block-for 30 means it will block any other attempt for 30 seconds

  - Verify that all unused ports are disabled.

Router ports are disabled by default, but it is always prudent to verify that all unused ports are in an administratively down state. This can be quickly checked by issuing the show ip interface brief command. Any unused ports that are not in an administratively down state should be disabled using the shutdown command in interface configuration mode.

R1# show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

Embedded-Service-Engine0/0 unassigned      YES NVRAM  administratively down down

GigabitEthernet0/0         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/1         192.168.1.1     YES manual up                    up

Serial0/0/0                unassigned      YES NVRAM  administratively down down

Serial0/0/1                unassigned      YES NVRAM  administratively down down

Configure the Router for SSH Access

Using Telnet to connect to a network device is a security risk because all information is transmitted in a clear text format. SSH encrypts the session data and provides device authentication, which is why SSH is recommended for remote connections. In Part 2, you will configure the router to accept SSH connections over the VTY lines.

Configure device authentication.

The device name and domain are used as part of the crypto key when it is generated. Therefore, these names must be entered prior to issuing the crypto key command.

Configure the domain for the device.

R1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

R1(config)# crypto key generate rsa modulus 1024

The name for the keys will be: R1.ccna-lab.com

% The key modulus size is 1024 bits

% Generating 1024 bit RSA keys, keys will be non-exportable...

[OK] (elapsed time was 1 seconds)

R1(config)#

*Jan 28 21:09:29.867: %SSH-5-ENABLED: SSH 1.99 has been enabled

Configure a local database username.

R1(config)# username admin privilege 15 secret adminpass

Note: A privilege level of 15 gives the user administrator rights.

Enable SSH on the VTY lines.

Enable Telnet and SSH on the inbound VTY lines using the transport input command.

R1(config)# line vty 0 4

R1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

R1(config-line)# login local

R1(config-line)# end

R1#

Save the running configuration to the startup configuration file.

R1# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

R1#

Establish an SSH connection to the router.

Start Tera Term from PC-A.

Use Tera Term to telnet to R1.

Does R1 accept the Telnet connection? Explain.

Initially R1 didn’t accept the connection and was giving me an error saying it closed by foreign host. Upon further investigation I had to use the command “transport input telnet” within the line vty 0 4. I also had to re configure the entire router due to human error.

Use Tera Term, establish an SSH session to R1. Use the username admin and password adminpass. You should be able to establish an SSH session with R1.

Does R1 accept the SSH connection? The connection was accepted.

Intentionally mistype the user and password information to see if login access is blocked after two attempts.

What happened after you failed to login the second time?

After attempting to login 2 times incorrectly the error message changed from closed by foreign host to refused by remote host.

From your console session on the router, issue the show login command to view the login status. In the example below, the show login command was issued within the 30 second login blocking period and shows that the router is in Quiet-Mode. The router will not accept any login attempts for 14 more seconds.

R1# show login

A default login delay of 1 second is applied.

No Quiet-Mode access list has been configured.

Router enabled to watch for login Attacks.

If more than 2 login failures occur in 120 seconds or less,

logins will be disabled for 30 seconds.

Router presently in Quiet-Mode.

Will remain in Quiet-Mode for 14 seconds.

Denying logins from all sources.

R1#

After the 30 seconds has expired, SSH to R1 again and login using the SSHadmin username and Admin1p@55 for the password.

After you successfully logged in, what was displayed? After logging in successfully I was greeted with the login banner we’d set earlier.

Enter privileged EXEC mode and use Enablep@55 for the password.

If you mistype this password, are you disconnected from your SSH session after two failed attempts within 120 seconds? Explain. It wont disconnect you from the SSH session as the password blocker only works on connecting via SSH.

Issue the show running-config command at the privileged EXEC prompt to view the security settings you have applied.

Configure the Switch for SSH Access

In Part 3, you will configure the switch in the topology to accept SSH connections. After the switch has been configured, establish an SSH session using Tera Term.

Configure the basic settings on the switch.

Console into the switch and enable privileged EXEC mode.

Enter configuration mode.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the plain text passwords.

Create a banner that will warn anyone accessing the device that unauthorized access is prohibited.

Configure and activate the VLAN 1 interface on the switch according to the Addressing Table.

Save the running configuration to the startup configuration file.

Configure the switch for SSH connectivity.

Use the same commands that you used to configure SSH on the router in Part 2 to configure SSH for the switch.

Configure the device name as listed in the Addressing Table.

Configure the domain for the device.

S1(config)# ip domain-name ccna-lab.com

Configure the encryption key method.

S1(config)# crypto key generate rsa modulus 1024

Configure a local database username.

S1(config)# username admin privilege 15 secret adminpass

Enable Telnet and SSH on the VTY lines.

S1(config)# line vty 0 15

S1(config-line)# transport input telnet ssh

Change the login method to use the local database for user verification.

S1(config-line)# login local

S1(config-line)# end

Establish an SSH connection to the switch.

Start Tera Term from PC-A, and then SSH to the SVI interface on S1.

Are you able to establish an SSH session with the switch? I was successfully able to establish and login to a SSH session with the switch.

SSH From the CLI on the Switch

The SSH client is built into the Cisco IOS and can be run from the CLI. In Part 4, you will SSH to the router from the CLI on the switch.

View the parameters available for the Cisco IOS SSH client.

Use the question mark (?) to display the parameter options available with the ssh command.

S1# ssh ?

-c    Select encryption algorithm

-l    Log in using this user name

-m    Select HMAC algorithm

-o    Specify options

-p    Connect to this port

-v    Specify SSH Protocol Version

-vrf  Specify vrf name

WORD  IP address or hostname of a remote system

SSH to R1 from S1.

You must use the –l admin option when you SSH to R1. This allows you to log in as user admin. When prompted, enter adminpass for the password.

S1# ssh -l admin 192.168.1.1

Password:

***********************************************

Warning: Unauthorized Access is Prohibited!

***********************************************

R1#

You can return to S1 without closing the SSH session to R1 by pressing Ctrl+Shift+6. Release the Ctrl+Shift+6 keys and press x. The switch privileged EXEC prompt displays.

R1#

S1#

To return to the SSH session on R1, press Enter on a blank CLI line. You may need to press Enter a second time to see the router CLI prompt.

S1#

[Resuming connection 1 to 192.168.1.1 ... ]

R1#

To end the SSH session on R1, type exit at the router prompt.

R1# exit

[Connection to 192.168.1.1 closed by foreign host]

S1#

What versions of SSH are supported from the CLI?

Using the ssh -v? command I was able to find that the different versions supported are protocol versions 1 and 2.

How would you provide multiple users, each with their own username, access to a network device?

You could potentially add all the logins to the local database. This would include the passwords and usernames.

Reflection (max 800 words):

Write a research report comparing various Encryption standards, e.g AES, SHA, DES, RSA.

# Encryption Standards

## AES

AES which is short for Advanced Encryption Standard is one of many different Encryption standards you will find keeping our data safe. The way it encrypts the data is a very complex process and it goes something like this. Key expansion, add round key, Byte substitution, Shift rows, Mix Columns, Add round key, x the key whether the key is 128, 192 or 256-bit. Then byte substitution, Shift rows, Add round key. This process is essentially several mathematical operations being applied to the data repeatedly [1].

Although there seems to be a lot of steps within this method making it look very strong and hard to crack over the years there have been some issues with the method. These issues have not been ground-breaking as they would potentially take billions of years to crack which would mean AES is technically unbreakable as of now. The reason it isn’t entirely unbreakable is due to AES itself can still be vulnerable if it hasn’t been implemented properly [1].

## SHA

SHA which stands for Secure Hash Algorithms is another type of encryption method that is used on the internet every day. Similar to AES it also has a lot of different versions of the algorithm itself. This method uses hash algorithms which essentially takes the piece of data and then turns it into hashed text which is irreversible. These hash keys are then used in digital signatures. Digital signatures are an important part of how an SSL certificate will provide authentication. These digital signatures are also extremely sensitive so any change to the file will cause the signatures to change. This makes its essentially impossible for a hacker to modify any legitimate certificate or to make one seem legitimate as the hash will change and will make it invalid [2].

SHA-1 and SHA-2 are somewhat alike however, you’d see SHA-2 as a successor to SHA-2 as it is an overall improvement. The only real difference between the two is that SHA-1 will have a smaller bit hash in comparison to SHA-2. Reasoning behind this being an improvement is that having a larger bit hash will provide you will more combinations to your key which will guarantee more uniqueness to your key. The security within this method lies within the uniqueness of the key [2].

## DES

DES which stands for Data Encryption Standard is known as an outdated symmetric-key method of data encryption. This method works by using the same key to encrypt and decrypt the message. This encryption method was known as a block cypher which meant you’d have a cryptographic key and algorithm applied to a block of data simultaneously rather than one bit at a time. Because DES encrypted plain text messages into 64bit blocks it made it quite susceptible to brute force attacks. The process of this method involved 16 rounds that could be run in 4 different modes encrypting blocks individually or making each cypher block dependant on all previous ones. The only downside of this was the reversing the method and this was possible as the key was essentially only 56 bits long as eight of those bits were used as parity checks. Comparing this method to something like AES you are better off not using this method as there are other methods which are a lot more secure and widely used and just do a better job [3].

## RSA

RSA stands for Rivest-Shamir-Adleman which are the name of the individuals which created this encryption method. This encryption method is essentially having a message for example, being sent already as an encryption for them then to be decrypted by the person you were aiming for the message to be received by. This would be done by having a public key. The public key once obtained can be shared openly as it will only be able to be opened by another key which will be known as the private key. Generating this private key will consist of a lot of mathematics as the entire process of generating these keys are under Carmichael’s totient function[4].

This method of encryption is very old and may have some flaws such as poor key generation. This will happen if some of the numbers generated are too close together which will cause the key to be found easily. There are also ways in which RSA can’t be directly broken but can give off information that a encryption process is underway. This method is deemed safe despite the possible attacks that may occur during encryption. If the encryption is performed correctly and the key falls within the correct parameters, it is more than likely you’ll be safe using this method [4].

# References

| [1] | J. Lake, “Comparitech,” 5 October 2018. [Online]. Available: https://www.comparitech.com/blog/information-security/what-is-aes-encryption/#How_does_AES_work. [Accessed 10 December 2019]. |
| --- | --- |
| [2] | P. Nohe, “thesslstore,” 9 November 2018. [Online]. Available: https://www.thesslstore.com/blog/difference-sha-1-sha-2-sha-256-hash-algorithms/. [Accessed 10 December 2019]. |
| [3] | M. Rouse, “Data Encryption Standard (DES),” November 2014. [Online]. Available: https://searchsecurity.techtarget.com/. [Accessed 10 December 2019]. |
| [4] | J. Lake, “What is RSA encryption and how does it work?,” 10 December 2018. [Online]. Available: https://www.comparitech.com/blog/information-security/rsa-encryption/. [Accessed 10 December 2019]. |

‌