| Drawing Name/No. |  |  |  | CAM_Assign_Task1_Part |  |  |  |  |  |  |  |  |  |  |  |  | Tooling and Set up |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  |  |  | Work Piece |  |  |  |  |  |  |  |  |  |  |  |  | T1 |  | 5 Drill Bit |  |
| Program No. |  |  |  | P1000 (Main Program) |  |  |  |  |  |  |  |  |  |  |  |  | T2 |  | 4 Drill Bit |  |
| Material |  |  |  | Aluminium |  |  |  |  |  |  |  |  |  |  |  |  | T3 |  | 12 Slot Drill |  |
| Program By |  |  |  | Leon Knight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date |  |  |  | 15/5/2020 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N | G | M |  | X |  | Y | Z |  | R |  | F |  |  |  | Comments |  |  |  |  |
| Circular Holes | 10 | G21 | G90 |  | G54 |  | G40 |  |  |  |  |  |  |  |  | Program co-ordinate set up |  |  |  |  |
|  | 20 |  | M06 |  | T1 |  |  |  |  |  |  |  |  |  |  | Selecting tool 1 |  |  |  |  |
|  | 30 | G43 | H01 |  | M08 |  |  |  |  |  |  |  |  |  |  | Setting compensation and turning on coolant |  |  |  |  |
|  | 40 |  | M03 |  | S7639 |  |  |  |  |  |  |  |  |  |  | Set spindle speed |  |  |  |  |
|  | 50 | G00 |  |  |  |  |  | Z20.0 |  |  |  |  |  |  |  | Set tool height |  |  |  |  |
|  | 60 | G81 | X48.5 |  | Y23.0 |  | Z-10.0 | G98 |  | L0 |  |  |  |  |  | Setting the central point for circular pattern |  |  |  |  |
|  | 70 | G70 | I4.0 |  | J0 |  | L4 |  |  |  |  |  |  |  |  | Setting hole distance from centre nd number of holes |  |  |  |  |
| Linear Holes | 80 | M06 | T2 |  |  |  |  |  |  |  |  |  |  |  |  | Tool Change to tool 2 |  |  |  |  |
|  | 90 | G43 | H02 |  | M08 |  |  |  |  |  |  |  |  |  |  | Setting compensation and turning on coolant |  |  |  |  |
|  | 100 |  | M03 |  | S9549 |  |  |  |  |  |  |  |  |  |  | Set spindle speed |  |  |  |  |
|  | 110 | G00 | Z20.0 |  |  |  |  |  |  |  |  |  |  |  |  | Set tool height |  |  |  |  |
|  | 120 | G81 | X20.0 |  | Y10.0 |  | Z-10.0 | R2.0 |  | G98 |  | L1 |  | F954 |  | Drill first linear hole location using correct feed speed |  |  |  |  |
|  | 130 | Y20.0 |  |  |  |  |  |  |  |  |  |  |  |  |  | Second linear hole location |  |  |  |  |
|  | 140 | Y30.0 |  |  |  |  |  |  |  |  |  |  |  |  |  | Third linear hole location |  |  |  |  |
|  | 150 | G80 |  |  |  |  |  |  |  |  |  |  |  |  |  | Canned cycle ended |  |  |  |  |
| Perimeter | 160 | M06 | T3 |  |  |  |  |  |  |  |  |  |  |  |  | Tool Change to tool 3 |  |  |  |  |
|  | 170 | G43 | H03 |  |  |  |  |  |  |  |  |  |  |  |  | Setting tool length compensation |  |  |  |  |
|  | 180 | G41 | D12 |  | M08 |  |  |  |  |  |  |  |  |  |  | Setting diameter compensation and turning on coolant |  |  |  |  |
|  | 190 | M03 | S3183 |  |  |  |  |  |  |  |  |  |  |  |  | Set spindle speed |  |  |  |  |
|  | 200 | G00 | X0.0 |  | Y0.0 |  |  |  |  |  |  |  |  |  |  | Set to datum location |  |  |  |  |
|  | 210 | G00 | Z20.0 |  |  |  |  |  |  |  |  |  |  |  |  | Set tool height |  |  |  |  |
|  | 220 | G00 | Z2.0 |  |  |  |  |  |  |  |  |  |  |  |  | Rapid speed to above workpiece |  |  |  |  |
|  | 230 | G01 | Z-3.0 |  |  |  |  |  |  |  |  |  |  |  |  | Cut into material by a depth of 3mm using correct feed speed |  |  |  |  |
|  | 240 | M98 | P2000 |  |  |  |  |  |  |  |  |  |  |  |  | Activate sub program |  |  |  |  |
|  | 250 | G01 | Z-6.0 |  |  |  |  |  |  |  |  |  |  |  |  | Cut into the material further to 6mm depth |  |  |  |  |
|  | 260 | M98 | P2000 |  |  |  |  |  |  |  |  |  |  |  |  | Activate sub program |  |  |  |  |
|  | 270 | G01 | Z-9.0 |  |  |  |  |  |  |  |  |  |  |  |  | Cut into the material further to 6mm depth |  |  |  |  |
|  | 280 | M98 | P2000 |  |  |  |  |  |  |  |  |  |  |  |  | Activate sub program |  |  |  |  |
|  | 290 | G00 | Z50.0 |  |  |  |  |  |  |  |  |  |  |  |  | Return tool out of the way of work piece |  |  |  |  |
|  | 300 | M05 | M09 |  | G40 |  |  |  |  |  |  |  |  |  |  | Shutting down coolant and spindle |  |  |  |  |
|  | 310 | G28 |  |  |  |  |  |  |  |  |  |  |  |  |  | Return to zero |  |  |  |  |
|  | 320 | M02 |  |  |  |  |  |  |  |  |  |  |  |  |  | End Program |  |  |  |  |

| Drawing Name/No. |  |  | CAM_Assign_Task1_Part |  |  |  |  |  |  |  |  |  |  | Tooling and Set up |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  |  | Work Piece |  |  |  |  |  |  |  |  |  |  | T1 |  | 5 Drill Bit |  |
| Program No. |  |  | P2000 (Sub Program) |  |  |  |  |  |  |  |  |  |  | T2 |  | 4 Drill Bit |  |
| Material |  |  | Aluminium |  |  |  |  |  |  |  |  |  |  | T3 |  | 12 Slot Drill |  |
| Program By |  |  | Leon Knight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date |  |  | 15/5/2020 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N | G | M |  | X |  | Y | Z |  | R |  | F |  | Comments |  |  |  |  |
| 10 | G01 | X75.0 |  | Y0.0 |  |  |  |  |  |  |  |  | Travel along bottom edge |  |  |  |  |
| 20 | G03 | X85.0 |  | Y10.0 |  | R10 |  |  |  |  |  |  | Cutting out 10mm outer radius |  |  |  |  |
| 30 | G01 | Y36.0 |  |  |  |  |  |  |  |  |  |  | Travel up the right-hand edge |  |  |  |  |
| 40 | G02 | X79.0 |  | Y42.0 |  | R6 |  |  |  |  |  |  | Cutting out 6mm inner radius |  |  |  |  |
| 50 | G01 | X18.0 |  |  |  |  |  |  |  |  |  |  | Travel along top edge |  |  |  |  |
| 60 |  | X0.0 |  | Y24.0 |  |  |  |  |  |  |  |  | Cutting the 45 angle |  |  |  |  |
| 70 |  |  |  | Y0.0 |  |  |  |  |  |  |  |  | Reset to datum edge |  |  |  |  |
| 80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Speeds, Feeds, and other calculations

Tan 45= x / 18

x=Tan 45 × 18

x=18

Therefore, the Y co-ordinate for the piece is

42 (total height) - 18 (x value) = 24mm from datum

45

x

18

Type of Operation: 	Drilling

Stock/Part material: 	Aluminium

Tool Type: 		Twist Drill

Tool Material: 		High speed steel

Tool Size: 		4

Number of flutes: 	2

Spindle Speed: 1000 × 120π × 4 = 9,549.29 RPM

Feed Rate: 0.10 x 9,549.29 = 954.93mm/min

Type of Operation: 	Milling

Stock/Part material: 	Aluminium

Tool Type: 		End Mill

Tool Material: 		High speed steel

Tool Size: 		12

Number of flutes: 6

Spindle Speed: 1000 × 120π × 12 = 3,183.10 RPM

Feed Rate(mm/rev): 0.17 x 6 = 1.02mm/rev

Feed Rate (mm/min): 1.02 x 3183.10 = 3,246.76mm/min

Type of Operation: 	Drilling

Stock/Part material: 	Aluminium

Tool Type: 		Twist Drill

Tool Material: 		High speed steel

Tool Size: 		5

Number of flutes: 	2

Spindle Speed: 1000 × 120π × 5 = 7,639.44 RPM

Feed Rate: 0.11 x 7,639.44 = 840.33mm/min