School of Engineering

Aerodynamic Vehicle Testing and Analysis

Zhaocong Sun

1805164

Automotive Engineering Beng

09/01/2019

Advanced Vehicle Dynamics M6X00415

Content

Notations	3

0. Introduction	3

1. Research and study	3

1.1 Aerodynamic principles	3

1.2 Some research about CL and CD	4

1.3 Frontal Area	5

2. Test	6

2.1 Overall Test Plan	6

2.2 Detailed Test schedule	6

2.3. Instructions for the driver	7

3. Methodology	7

3.1 Math Channels in MoTeC	7

3.1.1 Define Static load	7

3.1.2 Calculate Corner Radius	8

3.1.3 Calculate Theoretical Steer	8

3.1.4 Calculate Longitude Weight Transfer	9

3.1.5 Calculate Aero Load	9

3.1.6 Calculate the center of pressure	9

3.1.7 Created Math Channel	10

3.2 Methodology used in MATLAB	10

4. Analysis	12

4.1 The aim of the analysis	12

4.2 Calculate CL & CD	12

4.2.1 Calculate CL	12

4.2.2 Calculate CD with EXCEL	13

4.2.3 Calculate CD with MATLAB	16

4.2.4 Analysis	19

4.3 Data traces to show lift and drag	20

4.4 Total lift versus speed x-y plot	21

4.5 Lateral acceleration versus speed x-y plot	22

4.6 Indicate the downforce at f and r axles.	23

4.7 Center of Pressure	24

4.8 The effect of static ride height and rake angle	25

5. Conclusion	26

6. Reference	27

Notations

CL-Aerodynamic lift coefficient

CD-Aerodynamic drag coefficient

Cop-Centre of pressure

0. Introduction

Race car performance depends on many elements such as engine, geometry, suspension, drivers and aerodynamics. In recent years, vehicle aerodynamics has attracted increased attention. In addition, mass of vehicle, tire slip angle, wings angle and the static ride height has a great influence on vehicle performance.

In order to analyze the handling performance and aerodynamic performance of the vehicle, our group design and undertake practical vehicle tests to collect suitable data from the simulator. The coasting test is designed to calculate rolling resistance, CL and CD, and to analyze the handling performance and aerodynamic performance. There are also many circuit tests with different ride height designed to analyze the overall performance.

In this report, MoTeC is used to create several math channels to do the majority of calculations and analysis. MATLAB is used to handle the data extracted from MoTeC. All in all, this report will focus on the aerodynamic performance and handling performance of the vehicle using data collected from Base Performance Simulator.

1. Research and study

1.1 Aerodynamic principles

When the vehicle is on motion, in addition to the force from the ground, it is also affected by the aerodynamic force and moments of the airflow around it. The force from the ground is influenced by the mass, rolling resistance coefficient and center of gravity while the aerodynamic force is influenced by the velocity, geometry and wings angle and the ride height.

1.2 Some research about CL and CD

The two basic measures of aerodynamic performance are downforce(lift) and drag. An increase in downforce provides the car with extra grip force, which improves the cornering capability of the vehicle. A decrease in drag lowers the resistance to accelerate, which improves the speed through high speed corners and on long straights. But if we want to increase the downforce, the drag would increase at the same time due to the increase of the rolling resistance. On most circuits, the engineer is usually balancing this trade-off to optimize lap time, or to give the car better performance at an overtaking point.

When the race car is on motion, the air would flow around its body and produce an aerodynamic force on the car. According to a lot of trails and researches[1]:

F=12ρv∞2ACF                                                                (1)

Where CF is coefficient of aerodynamic force. The acting point of the aerodynamic force is called center of pressure(C.P). The aerodynamic force could be divided into three parts: Drag, Lift and Side.

Drag=12ρv∞2ACD

Lift=12ρv∞2ACL

Side=12ρv∞2ACS

Drag will influence the total resistance and Lift will influence the maximum longitude force. In conclusion, Lift and Drag is essential to the aerodynamic performance. That is why we have to calculate the CL and CD.

Figure 1 Differential Pressure

The aerodynamic drag is made of several parts: some from pressure difference, some from the friction between the air and the car body and some from Lift force.

The differential pressure resistance is formed by the moving air causing pressure difference in the front and rear of the car. It accounts for 50%~60% of the total aerodynamic drag of the car and is the main component of aerodynamic drag, which is shown in figure 1. The resistance caused by the friction between air and vehicle body contributes about 6% to 11% of the total aerodynamic drag. The separation of lift in the horizontal direction count 8%~15% of the total aerodynamic drag[2].

1.3 Frontal Area

Figure 2 The Frontal Area

The frontal area is defined as figure 2. When the parallel light shines into the front of the car, the shadow is the frontal area. We should use the frontal area as the area when calculating CL because we are calculating CL for the whole vehicle

2. Test

2.1 Overall Test Plan

To characterize the aerodynamic and handling performance of the GP2 Race car model, a set of vehicle tests is designed to utilize the Base Performance Simulator

Several coast down tests and circuit test were performed. The details about the test will be introduced below. In addition, some circuit tests with different wing angles and ride height are carried out for the analysis[3].

2.2 Detailed Test schedule

Test data: 2018/10/30

People: Group2 Students of AVD course

Detailed test schedule:

Table 1 Detailed test schedule

| Coast down | Low f max r |
| --- | --- |
|  | Low f mid r |
|  | Max f Low r |
|  | Max f mid r |
|  | Mid f low r |
|  | Mid f max r |
|  | Max wings |
|  | Mid wings |
|  | Min wings |
|  | Std wings |
| Snett | Low f max r |
|  | Max f low r |
|  | Min wings |
|  | Mid wings |
|  | Max wings |
|  | Std 58% front brake balance |
|  | Std gp2 |
|  | Std wings low front rh max rear rh |
|  | Std wings max front rh low rear rh |
|  | Steady state |

The detailed test schedule is shown in Table 1. Two out of the coast down tests is chosen to calculate the total aerodynamic lift coefficient (CL) and the coefficient of drag(CD). In this report, I choose And the circuit tests with different ride heights and wings angle are chosen to analyze the overall vehicle performance.

2.3. Instructions for the driver

1) When doing the coast test, the vehicle should drive in a straight road. The driver should release the pedal until the velocity become steady and reached the set velocity value.

2) When doing the circuit test, the driver should remain their usual driving habits.

3) There should be someone helping the driver to set the parameters such as ride height, rolling resistance coefficient and so on.

3. Methodology

3.1 Math Channels in MoTeC

3.1.1 Define Static load

When the vehicle is on motion, there exits longitude weight transfer. The static load should be defined when the velocity is 0 as figure 3.

Figure 3 Tire Load data from MoTeC

Thus, define the static load(constant) as follows.

FL_Static=1801.3N

FR_Static=1795.4N

RL_Static=2025.4N

RR_Static=2035N

3.1.2 Calculate Corner Radius

The equation of centripetal force is

F=ma   a=v2r                                                               (2)

Thus

Corner Radius=Mass×Velocity2Lateral force=Velocity2Lateral forceMass×g×g=Velocity2G Lateral force×g

3.1.3 Calculate Theoretical Steer

According to the trigonometric function,

Figure 4 Principle of theoretical steer

this equation could be obtained:

cosθ=lr                                                                        (3)

If the x is approximated to 0, x is approximated to sinx. Thus

Theoretical Steer θ=sin-1wheelbaseCorner Radius[rad]≈wheelbaseCorner Radius×180π

3.1.4 Calculate Longitude Weight Transfer

According to the profile from Tim, the Load Transfer could be calculated with formula 4.

Figure 5 Load Transfer

Load Transfer=m×a×ht                                               (4)

Thus, longitude weight transfer can be calculated with :

Longitude weight transfer=Mass×G Force LongG×g×CoGWheelbase

3.1.5 Calculate Aero Load

Tire load is changing due to the changes of Aero Load and Longitude weight transfer. Thus, the Aero Load could be calculated as[4]:

Front Aero Load=Tire LoadFL+Tire LoadFR-FLStatic-FRStatic-Long WT

Rear Aero Load=Tire LoadRL+Tire LoadRR-RLStatic-RRStatic+Long WT

3.1.6 Calculate the center of pressure

As is taught in the course, the center of pressure could be calculated as below function[5].

Center of pressure=Front Aero load÷Total Aero Load             (5)

3.1.7 Created Math Channel

Figure 6 Created Math Channel

3.2 Methodology used in MATLAB

In the test, the rolling resistance Ff, the aerodynamic resistance Fw and the accelerating resistance Fj are the main resistances. During the driving process, the balance equation is:

Ff+Fw+Fj=0                                                             (6)

The vehicle dynamics shows that the rolling resistance, aerodynamic resistance, inertial resistance can be expressed as follows[6].

Ff= Gfcosα=Gf0+f1uacosα                                           (7)

Fw=ρCDAua22                                                                    (8)

Fi=δmdudt                                                                    (9)

Assume the rolling resistance is constant and take formula (7), (8), (9) into formula (6):

Gfcosα+ρCDAV22+δmdVdt=0                                                  (10)

In the equation, G is the total mass of the racing car, N; f are the rolling resistance coefficient, in this report, f is assumed as constant; α is the slope angle, it is zero in flat road& this report; A is the frontal area of the vehicle, in this report A is 1.2 m2; V is the driving speed of the vehicle, m/s;  ρ is the air density, in this report,  ρ=1.223 Kg∙m-3; CD is the aerodynamic resistance coefficient. The general truck CD value is 0.7-0.9, the passenger car CD value is 0.5~0.7, normal car CD value is 0.3 ~ 0.5, and the CD of the racing car in the test will be calculated and analysed later; δ is the conversion coefficient of rotation mass, δ = 1.03 ~ 1.04 usually[7].

Integrate formula (10) and get formula (11)

V1V2dVGfcosα+CDAV221.15=t2t1-dtδm                                       (11)

This formula will be used in MATLAB to calculate rolling resistance coefficient and aerodynamic drag coefficient[8]

4. Analysis

4.1 The aim of the analysis

Requirements & schedules for the tests:

(1) Understand, analyze and quantify the difference between 2 performances. As is required, I will calculate aerodynamic lift coefficient CL and aerodynamic drag coefficient CD.

(2) Produce data traces to indicate the downforce (lift) at the front axles and rear axles.

(3) Produce data traces to show how the total lift and drag varies over the course of one complete lap.

(4) Produce a corresponding total lift versus speed x-y plot as well as a lateral acceleration versus speed x-y plot for each setup and discuss what these show.

(5) Calculate and plot the centre of pressure for each aero setup.

(6) Investigate the effect of static ride height and rake angle on aerodynamic performance and discuss the implications on overall vehicle performance. Run extra tests or create additional math channels to obtain specific data such as vehicle handling balance for this investigation.

4.2 Calculate CL & CD

4.2.1 Calculate CL

As is shown in part 1, below formula could be used to calculate CL

Lift=12ρv∞2ACL

Air density ρ is 1.223 Kg/m3 and the frontal area is 1.2 m2. Then, it could be calculated directly in MoTeC.

CL='Total Aero load' [N]*2/1.223/1.2/('Speed' [m/s]^2)

Figure 7 CL of ‘Coast down max f low r’

Figure 8 CL of ‘Coast down mid wings’

From figure 7 &8, it could be calculated out that CL is 2.96 for ‘Coast down max f low r’ and 2.87 for ‘Coast down mid wings’ for this racing car.

4.2.2 Calculate CD with EXCEL

Choose the ‘Coast down mid wings’ and ‘Coast down max f low r’ profile, and export the coast data of velocity and time to MATLAB & EXCEL, which is shown as figure 9.

Figure 9 Export data to MATLAB

Firstly, extract the data(which is marked in green in figure 9) from MoTeC to EXCEL. Secondly, use the following formula to calculate the acceleration

a1=Rolling resistance coefficient×9.81+12CDρAV2mass

a2=Rolling resistance coefficient×9.81

Figure 10 The data from ‘Coast down mid wings’ in EXCEL

When the vehicle is fast, the resistance is consisted of rolling resistance and aerodynamic resistance(it is very big and make up the majority of the resistance). When the vehicle is slow, aerodynamic resistance becomes very small(it can be ignored).

Figure 11 The data from ‘Coast down mid wings’ in EXCEL

From figure 10, we could know when the velocity is about 53 m/s, the acceleration is about -1.99/s2.

From figure 11, we could know when the velocity is about 27 m/s, the acceleration is about -0.71m/s2.

As calculated, CD is 0.48 while the rolling resistance coefficient is 0.072 in the test called ‘Coast down mid wings’. This method is used to be compared to the method of MATLAB.

4.2.3 Calculate CD with MATLAB

After that, use the methodology in 3.2 to calculate the rolling resistance and the aerodynamic drag of this racing car in MATLAB.

Gfcosα+ρCDAV22+δmdVdt=0                                                  (10)

Actually, formula 10 is a differential equation about V and t. We could use MATLAB to handle this equation and get the result as:

V=f(t)

Where constant CD and f is unknown. The programme is shown below:

Figure 12 Programme in MATLAB

After solving the differential equation and getting the function ‘V=f(t)’[9], the data from MoTeC is used to fit this function and get the value of CD and f.

(1)Coast down with mid wings

Figure 13 V-t of 'Coast down mid wings'

In figure 13, the green one is the original data from MoTeC and the red one is the fitted curve.

Figure 14 Result of ‘Coast down mid wings’ from MATLAB

From MATLAB, the rolling resistance coefficient f is 0.02976 and the aerodynamic drag coefficient CD is 0.5951 of 'Coast down mid wings'

Figure 15  V-t of 'Coast down max f low r '

(2)Coast down Max f low r

Figure 16 Result of ‘Coast down mid wings’ from MATLAB

From MATLAB, the rolling resistance coefficient f is 0.02946 and the aerodynamic drag coefficient CD is 0.6207 of ' Coast down max f low r '.

As the calculation is very convenient, I calculated all data sets.

Table 2 f and CD of the coast test

| Parameters | f | CD |
| --- | --- | --- |
| Min f Max r | 0.03082 | 0.6012 |
| Min f Mid r | 0.03019 | 0.4742 |
| Max f Low r | 0.02946 | 0.6207 |
| Max f mid r | 0.02941 | 0.7341 |
| Mid f low r | 0.02935 | 0.4839 |
| Mid f max r | 0.03015 | 0.7225 |
| Max wings | 0.02996 | 0.86 |
| Mid wings | 0.02976 | 0.5951 |
| Min wings | 0.02967 | 0.3641 |
| Std wings | 0.02967 | 0.7939 |

The yellow one is chose to finish the analysis of the requirement, which includes the parameters of ‘mid wings’& ‘max f low r’.

4.2.4 Analysis

From table 2, rolling resistance coefficient f is about 0.0297. CD is changing with the ride height. It could be seen that CD(Max wings)＞CD(Std wings)＞CD(Mid wings)＞CD(Min wings). More angle wings brings more aerodynamic resistance.

If the front height of the vehicle is stable, the higher the rear height, the bigger the aerodynamic resistance coefficient. If the rear height of the vehicle is stable, the higher the front height, the bigger the aerodynamic resistance coefficient. The reason is that the frontal area becomes bigger when the ride is high.

In my opinion, the rolling resistance coefficient calculated by MATLAB is correct. There is a common sense that the coefficient of good asphalt road is about 0.0105. Thus 0.072(calculated by EXCEL) may be wrong because the velocity is not slow enough to ignore the aerodynamic resistance.

Figure 0-1 Circuit mid wings

4.3 Data traces to show lift and drag

Figure 17 Trace to show aerodynamic lift and drag

The red curve is from the test ‘Circuit mid wings’ while the black curve is from the test ‘Circuit max f low r’. From figure 17, the average velocity of ‘Circuit mid wings’ is a little bit faster than that of ‘Circuit max f low r’. When crossing the corner, the vehicle of ‘Circuit max f low r’ seems a little bit steady.

Figure 18 Total lift versus speed

4.4 Total lift versus speed x-y plot

The red curve is from the test ‘Circuit mid wings’ while the black curve is from the test ‘Circuit max f low r’. The red curve is more concentrated, which means the vehicle with ‘Circuit mid wings’ is loaded with more steady lift force and have a better handling and driving performance.

With the increase of velocity, the aerodynamic lift will increase faster and faster. The reason is that the lift is In proportion to the square of the speed

Figure 19 Lateral acceleration versus speed

4.5 Lateral acceleration versus speed x-y plot

The orange curve is from the test ‘Circuit mid wings’ while the black curve is from the test ‘Circuit max f low r’.

From figure 19, the orange curve is more concentrated when the vehicle is fast while the black curve is more concentrated when the vehicle is a little bit slow. It means that the vehicle with ‘mid wings’ will endure more lateral force when the vehicle increase the velocity.

Both curves shows that the lateral force is small when the velocity is high.

Figure 20 Downforce at front and rear axles

4.6 Indicate the downforce at f and r axles.

The red curve is from the test ‘Circuit mid wings’ while the black curve is from the test ‘Circuit max f low r’.

Longitudinal weight transfer is calculated with the formula in part 3.1.4. It could be seen from the figure that vehicle of ‘Circuit mid wings’ has more rear aero load and less front aero load than that of ‘Circuit max f low r’.

Figure 21 Scatter figure of center of pressure

4.7 Center of Pressure

Figure 22 Center of pressure versus distance

The red curve is from the test ‘Circuit mid wings’ while the black curve is from the test ‘Circuit max f low r’.

From figure 21 & 22, the CoP of ‘Circuit mid wings’ is bigger than that of ‘Circuit of max f low r’. This is because that the percentage of front aerodynamic load of ‘Circuit mid wings’ is bigger.

4.8 The effect of static ride height and rake angle

The rake angle is calculated with:

Rake angle=atand⁡((Average rear height-Average front height)/Wheelbase)

The result of the calculation is shown as figure 23

Figure 23 CoP versus rake angle

The red curve is from the test ‘Circuit mid wings’ while the black curve is from the test ‘Circuit max f low r’.

When the rake angle increases the CoP of the red curve doesn’t change a lot while the black changes a lot. It means that the load of the ‘Circuit mid wings’ is more steady than that of the ‘Circuit max f low r.’

5. Conclusion

When it comes to different corners, the maximum speed at which the vehicle could travel around this corner is changing. The key here is that the coefficient of friction is produced over 4 tires and changes dynamically. And the load across these 4 tires is not equal during cornering due to weight transfer.

Figure 24 The figure to explain something

When the vehicle is on motion, the tire will have a slip angle at which coefficient of friction reduces. Due to the exits of slip angle, the tire begins sliding as well as slipping. The reason of OVERSTEER & UNDERSTEER is that there are difference between rear axle acceleration and front axle acceleration.

Why the Longitude force decrease when the vehicle crosses a corner? In my opinion, the lateral force and longitude force comes from the friction between the tire and the ground. The total force is limited by the vertical load. The needed lateral force increases when the vehicle crosses a corner. Thus the longitude force used to overcome the resistance decreases.

In my opinion, the CL & CD which was calculated by MATLAB is more accurate. And the differential equation is used.

6. Reference

[1] https://wenku.baidu.com/view/f348305e77232f60ddcca134.html

[2] https://www.researchgate.net/publication/284720454_DRAG_AND_LIFT_REDUCTION

[3] Aerodynamics Application Note

[4] Estimation of aerodynamic performance from constant speed and coast down testing, Aerodynamic Analysis..

[5] Joseph Katz, Aerodynamics of Race Cars, Annu. Rev. Fluid Mech,. 2006.38:27-63

[6] Wu Gaogui, Quadratic sliding measurement of air resistance coefficient of automobile. Journal of South China University of Technology (Natural Science Edition), 2000 (06): p.44-48.

[7] Han Zongqi and Li Liang, Method for Determining the Drag Coefficient of a Vehicle. Automotive Engineering, 2002 (04): pp. 364-366.

[8] Liu Fucai, Pan Jianghua and Han Zongqi, Method for Determining the Coefficient of Coasting Resistance of Automobile Based on Genetic Algorithm. Automotive Engineering, 2003 (06): p.610-612 + 616.