Construction Design LTD

| Construction Method Statement | Caretakers House |
| --- | --- |

Roof Tiling

| Step | Significant Hazards | Risks | Existing Control Measure | Additional Recommendations |
| --- | --- | --- | --- | --- |
| Arrival on Site | Slips trips and falls | LOW | Sign site register Complete toolbox talks and site progress meetings for update on site. | Correct PPE to be worn |
| Work at height register | Slips trips and falls | LOW | Read risk Assessments and method statements. Sign working at height register. Ensure all tools, equipment and materials are available prior to starting to avoid leaving the area unattended. Check scaffolding is at the correct height and check Scaff-tags. | Correct PPE to be worn All power tool protection to be used. |
| Materials delivered to site and delivered to working area. | Pedestrians, manual handling, use of machinery, tip hazards. | MED | Use of mechanical lifting where practically possible. Ground conditions to be level and free from debris. All waste to be disposed of at regular intervals and working areas kept clean. | Correct PPE to be worn. Use of ladders to be class 1 and checked and used on level ground. |
| Storage of materials | Working area being overcrowded, materials falling, damage to materials, manual handling, trip hazards. | MED | Delivery of materials to be stored correctly and checked for damage. Materials stored in the correct areas to avoid damage. | Manual Handling procedures carried out correctly and where practically possible use of mechanical lifting aids used. |
| Scaffolding errection | Falls from height, manual handling, falling objects AND Hand Tools. | HIGH | Read risk Assessments and method statements to be submitted and checked by Site Manager. Sign working at height register. Ensure all tools, equipment and materials are available prior to starting to avoid leaving the area unattended. | Manual Handling procedures carried out correctly and use of lanyards. Correct PPE to be worn. |
| Fall arrest equipment in place | Falls From Height of operatives, materials and power tools. | HIGH | Equipment checked and in place before work commences. | Correct procedures for fitting to be carried out. |
| Lifting Tiles up to roof area. | Falls from height, manual handling, falling objects. | MED | Manual lifting adhering correct method and weights/the use of an electric tile covaying hoist to be used. Restrict operatives working in the area and any access needed to the building will need to be sealed off and alternative access used. Correct use of scaffolding used and checked before us. All tools and materials to be in place and checked before work commences. Area not to be left unsupervised if tools and equipment is left out. | Manual Handling procedures carried out correctly and where practically possible use of mechanical lifting aids used. Correct PPE to be worn All power tool protection to be used |
| Cutting of tiles | Manual handling, silica dust damaging the lungs. Sharp pieces of tiles entering the eyes. | HIGH | Use of water suppression to be use when cutting. All waste and debris to be removed and whilst complying with waste management plan. | Manual Handling procedures carried out correctly and where practically possible use of mechanical lifting aids used. Correct PPE and RPE to be worn, powerhood to be worn with beards. All power tool protection to be used. |
| Clean up | Manual handling, dust. | MED | All waste and debris to be removed and whilst complying with waste management plan. | PPE to be worn at all times. |
| Completion | Slips trips and falls | LOW | Inform Site manager of completion. | PPE to be worn at all times |