| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |

MODULE CODE: M5X02201

MECHANICAL PRINCIPLES

PHASE TEST 2 RESIT

TIME:  2 HOURS

answer all questions

Equipment or materials required – None

Adnoddau neu ddefnyddiau sydd eu hangen – Dim

Calculators are permitted –show your working

A ‘formula sheet’ is provided for reference.

Assume ‘g’  =  9.81 m/s2.

and  1 bar  =  105 N/m2 (Pa)

ANSWER ALL QUESTIONS

Show all your workings and state any assumptions or approximations you have made.

QUESTION ONE

Calculate:

  - The maximum torque and power which can be transmitted at 1800 RPM by a brand new clutch if it has an outer diameter of 120mm, an inner diameter of 40mm and a coefficient of friction of 0.45. The axial force may be taken as 900N.

(5 Marks)

  - The reduction in torque and power which can be transmitted by the same clutch at 1800 RPM when worn by 1.55mm, if the axial force is provided by 6 springs, each with a rate of 25 N/mm.

(5 Marks)

QUESTION TWO

A belt drive must transmit 5 kW of power at 2500 rev/min. The pulley wheel is 130 mm diameter and the coefficient of friction is 0.4. The lap angle is 128o. Assuming the initial belt tension is 250 N and the belt mass may be taken to be negligible;

- Calculate the linear velocity of the belt

(2 Marks)

- Calculate the maximum and minimum belt tensions

(4 Marks)

- Calculate the maximum power transmitted per belt

(2 Marks)

- Calculate the number of belts required

(2 Marks)

QUESTION THREE

45°

95°

110°

Three masses A, B and C are placed on a balanced disc as shown, at radii of 80 mm, 120 mm and 95 mm respectively.

The masses are 0.87 kg, 0.55 kg and 0.58 kg respectively.

A fourth balancing mass, D, is to be added at a radii of 40mm to statically balance the system.

- Construct a table to calculate the magnitude of the forces created by the rotating masses

(2 Marks)

- Construct a vector diagram using a suitable scale to represent the resultant force.

(4 Marks)

- Calculate the mass of the unknown mass D

(2 Marks)

- Calculate the angular position at which the balancing mass D should be placed

(2 Marks)

QUESTION FOUR

The diagram above shows a torque - angle diagram for a certain machine. If the speed of the shaft must be maintained between 2400 and 2500 rev/min, calculate

- the coefficient of fluctuation of speed					  (2 Marks)

- the maximum fluctuation of energy (βW)			              (3 Marks)

- the moment of inertia of a suitable flywheel. 				  (2 Marks)

- The mass and thickness of the flywheel if it is to take the form of a simple steel disc of radius 150 mm and density 7850Kg/m3.

(3 Marks)

QUESTION FIVE

The diagram below shows a compound gear train.

Gear A has 32 teeth

Gear B has 64 teeth

Gear C has 24 teeth

Gear D has 36 teeth

Gear A is the input gear and revolves at 2500 rev/min anticlockwise when viewed for the left end. If the input torque is 20 Nm and the gear train efficiency is 85%

- State which gears must have the same module/pitch

(1 Mark)

- Calculate the output speed and direction

(3 Marks)

- Calculate the output power

(3 Marks)

- Calculate the holding torque required

(3 Marks)