| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2019 | mai 2019 |

MODULE CODE: M4X00386

ENGINEERING SCIENCE 2

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator, Thermodynamics Tables

FORMULAE BOOKLET FOR ELECTRICAL ENGINEERING

| Q=CV | Q=It | V=IR | V=V0e-tRC | I=I0e-tRC |
| --- | --- | --- | --- | --- |
| Q=Q0e-tRC | P=IV | P=I2R | P=V2R | E=12CV2 |
| E=12QV | ρ=RAl | RT=R1+R2+R3 | 1RT=1R1+1R2+1R3 | V=ε-Ir |
| Vrms=V02 | Irms=I02 | Vrms=IrmsR |  |  |

FORMULAE BOOKLET FOR THERMODYNAMICS

- BASIC CONCEPT:

The change in the energy content of a body or any other system is equal to the difference between the energy input and the energy output, and the energy balance is expressed as:

∆E=Ein-Eout

E2 – E1

WORK:

Work done by the system is considered: POSITIVE

Work done on the system is considered: NEGATIVE

HEAT:

Heat transfer into the system is considered: POSITIVE.

Heat transfer out of the system is considered: NEGATIVE.

- CLOSED SYSTEMS:

Non-flow energy equation:

∆U=Q-W

- ENTHALPY:

H=U+P.V           (J)

Where:

- U = internal energy

- P = pressure

- V = volume

- FLOW WORK:

The flow work, or flow energy, or again displacement energy:

Wflow=P×V

- OPEN SYSTEMS / Steady-flow systems

- Mass Balance:

inm=outm                       (kg/s)

- Energy Balance:

Q-W=outm.h+12.c2+g.z-inm.h+12.c2+g.z

- CONTINITY OF MASS FLOW:

m=mass flow in kgs                                     A=cross sectional area in m2

v=specific volume in m3kg                       c=velocity in ms

Consider the volume of substance passing the section per second = V in m3/s

V=m.v      also     V=A.c   (Continuity equation)

Therefore:

m.v=A.c

From which

m=A.cv

- OPEN SYSTEMS / Unsteady-flow systems

- Mass Balance:

mi-me=m2-m1CV

Where:

- i = inlet, and e = exit.

- 1 = initial state, and 2 = final state of the control volume.

- Often one or more terms in this equation are zero.

- Energy Balance: (when kinetic and potential energy changes associated with the control volume and fluid streams are negligible)

Q-W=outmh-inmh+m2u2-m1u1CV

- IDEAL GAS:

We defined an ideal gas as a gas whose temperature, pressure, and specific volume are related by:

P.v=R.T

where:

- R is the gas constant (kJ/kg.K) or (kPa.m3/kg.K)

- P the absolute pressure (kPa)

- T the absolute temperature (K)

- v is the specific volume (m3/kg)

The change in internal energy or enthalpy for an ideal gas during a process from state 1 to state 2 are defined as:

u2-u1=cvT2-T1       (kJkg)

h2-h1=cpT2-T1       (kJkg)

where:

- cv is the specific heat at constant volume

- cp is the specific heat at constant pressure

Reminder:

°C=K-273                                K=°C+273

ELECTRICAL ENGINEERING

- QUESTION ONE (10 marks)

Figure 1 shows the circuit in question 1 where internal resistance is not neglected.

a)	Given that the reading on A1 is 4.5A and R1 = 2R2, what is the reading on A2		[2]

b)	Calculate the value of R1 and R2 given that the potential difference across R2 is 13.6V	[2]

c)	Calculate the power used by R1								[2]

c)	Given that the total resistance of the circuit is 10Ω, calculate the emf of the battery	[4]

Figure 1

r

R1

R2

- QUESTION TWO (20 marks)

Figure 2 shows an ac waveform that is displayed on an oscilloscope screen.

Figure 2

a)	The time base of the oscilloscope is set at 1.5ms per division and the y-gain at 1.5V per division.

i	Calculate the frequency of the waveform						[2]

ii	Calculate the peak voltage								[2]

iii	Calculate the period									[2]

iv	Calculate the rms voltage								[2]

b)	State and explain the effect on the oscilloscope if the time base is switched off	[2]

c)	In the UK, domestic users are supplied with mains electricity at a root mean square voltage of 230V.

Figure 3

i	State what is meant by root mean square voltage					[2]

ii	Calculate the peak value of the supply voltage					[2]

iii	For the circuit shown in figure 3, calculate Irms if the peak current is 5A		[2]

iv	If Vrms is 110V, calculate the peak voltage and the value of resistor R		[4]

- QUESTION THREE (20 marks)

a)	Figure 4 shows a capacitor which was charged to a pd of 12V and discharged through a 150kΩ resistor.

i	Calculate the discharge current							[3]

ii	Use the graph to determine the time constant of the circuit, giving the answer to an appropriate number of significant figures						[5]

iii	Calculate the capacitance of the capacitor						[3]

iv	Show that the capacitor lost approximately 90% of its energy in 2.3s		[3]

Figure 4

b)	In order to produce a time delay, a set of traffic lights contains a capacitor identical to the one used in part (a). The capacitor is charged from a 24V supply and then discharged through a 150kΩ resistor.

i	State and explain the effect of this higher pd on the energy stored by the capacitor	[3]

ii	State and explain the effect of this higher pd on the time taken for the capacitor to lose 90% of its energy									[3]

THERMODYNAMICS

- QUESTION FOUR / Closed System (10 marks)

A domestic refrigerator is loaded with food and the door closed. During a certain period the machine consumes 1 kWh of energy and the internal energy of the system drops by 5103 kJ. Find the net heat transferred in the system.

- QUESTION FIVE / Opens System, Steady-Flow Process (20 marks)

An adiabatic turbine receives 20.6103 kg of steam per hour as shown in Figure 5. The steam enters the turbine as superheated water at a velocity of 3103 m/min, an elevation of 10 m, a pressure of 2 MPa, and a temperature of 400C. It leaves the turbine as saturated liquid-vapour mixture with a quality of 0.9 at a velocity of 10.8103 m/min, an elevation of 6 m, and a pressure of 15 kPa. Determine the power output of the turbine.

STEAM

TURBINE

Wout=?

Superheated water

m1=20.6×103kgh

P1=2 MPa

T1=400°C

c1=3×103mmin

z1=10 m

Saturated water

Liquid-vapour mixture x = 0.9

P2=15 kPa

c2=10.8×103mmin

z2=6 m

Figure 5: Steam turbine schematic

Assumptions:

- The turbine operates under steady-flow conditions.

- The system is adiabatic and thus there is no heat transfer.

- The properties of the fluid can be found in Tables A-5 and A-6.

- In a saturated liquid-vapour mixture, the mass fraction of vapour is called the quality and is expressed as:

x=mvapourmtotal

Quality may have values between 0 (saturated liquid) and 1 (saturated vapour). It has no meaning in the compressed liquid or superheated vapour regions. In the saturated mixture region, the average value of any intensive property y is determined from:

y=yf+x.yfg

where f stands for saturated liquid, g for saturated vapour and y stands for v, u or h.

- QUESTION SIX / Open System, Unsteady-Flow Process (20 marks)

A tank with an internal volume of 0.1 m3 contains nitrogen gas at 200 kPa and 25C as shown in Figure 6. The tank is connected to a supply line containing nitrogen at 800 kPa and 25C. A valve is opened allowing nitrogen to flow into the tank until the pressure reaches 800 kPa. Determine,

- The final nitrogen temperature T2 in the tank if the internal energy of nitrogen at initial and final stages are 221.41 kJ/kg and 282.34 kJ/kg respectively,	[4]

- The mass of nitrogen that has entered the tank during the process,	[8]

- The heat transfer between the tank and the surroundings during the process.	[8]

Nitrogen Pi = 800 kPa, Ti = 25°C

Initial time		   Final time

P1 = 200 kPa		 P2 = 800 kPa

T1 = 25C	  T2 = ?

u1 = 221.42 kJ/kg         u2 = 282.34 kJ/kg

Figure 6: nitrogen tank connected to a supply line schematic.

Assumptions:

- Nitrogen can be considered as an ideal gas.

- Gas constant and specific heat values can be found in Table A-2.

- Kinetic and potential energies are negligible.

- There are no work interactions involved.

Diwedd/End