Assignment J Solution Sheet

Use of this sheet

This is an indicative staff solution. Credit equivalent scientifically defensible approaches, provided assumptions are stated and calculations are internally consistent.

Numerical agreement with dosimetry does not need to be exact. Order-of-magnitude correctness and well-defended reasoning are the key assessment criteria.

Key indicative numerical answers

| Quantity | Indicative value | Marker note |
| --- | --- | --- |
| Source activity (19 Dec 2024) | ≈ 5.0–5.3 TBq | Accept reasonable rounding |
| Dose rate at 1 m | ≈ 0.65–0.70 Sv h⁻¹ | Γ ≈ 0.13 mSv m² GBq⁻¹ h⁻¹ |
| Approach phase (0.7 m) | ≈ 0.40–0.60 Sv h⁻¹ | 30–45% of nominal |
| Close phase (0.18 m) | ≈ 15–19 Sv h⁻¹ | 75–95% of nominal |
| Total effective dose | ≈ 100–130 mSv | Accept ~80–160 mSv |

1. Framing expected from a good report

The strongest submissions will present a coherent reconstruction of the exposure scenario rather than a sequence of isolated calculations.

Students should justify:

- decay correction of source activity

- use of a tabulated gamma constant

- inverse-square behaviour

- assumptions regarding shielding fractions and geometry

Recognition that the radiation field is highly non-uniform, particularly at very short distances, is essential.

2. Worked scientific solution

2.1 Decay-corrected activity

Given:

- Initial activity = 18 TBq

- Elapsed time ≈ 135 days

- Half-life (Ir-192) = 73.8 days

A=A0⋅2-t/T1/2
A≈18×2-135/73.8≈5.1 TBq

Accept values in the range 5.0–5.3 TBq.

2.2 Unshielded dose rate at 1 m

Using:

D=ΓAr2

Convert activity:

5.1 TBq=5100 GBq
D1m=0.13×5100≈663 mSv/h≈0.66 Sv/h

A good answer may note that this assumes a point source and does not account for shielding or geometry.

2.3 Dose rate at 8 m (contextual check)

D8m=0.6682=0.6664≈0.010 Sv/h=10 mSv/h

The reported measurement of >1 mSv/h should be interpreted as a lower-bound field indication, not a precise measurement. Differences may arise from:

- shielding and obstruction

- measurement location

- instrument limitations

This value is not inconsistent with the calculation.

2.4 Approach phase

Distance ≈ 0.7 m

Nominal unshielded field:

D=0.660.72≈1.35 Sv/h

Apply attenuation (30–45%):

D≈0.40–0.61 Sv/h

Estimated dose (representative exposure):

Dose≈6–14 mSv

2.5 Close inspection phase

Distance ≈ 0.18 m

Nominal field:

D=0.66(0.18)2≈20 Sv/h

Apply attenuation (75–95%):

D≈15–19 Sv/h

Estimated dose:

Dose≈75–150 mSv

A strong answer will recognise that at such short distances:

- the point-source approximation becomes less reliable

- small positional changes significantly affect dose

- exposure is highly localised and non-uniform

2.6 Total effective dose

Combining phases:

Total dose≈100–130 mSv

Accept a broader range of ~80–160 mSv if well justified.

3. Comparison with dosimetry

Reported value:

- TLD ≈ 138 mSv

The calculated dose is consistent with the measured value, particularly considering:

- non-uniform radiation field

- very steep spatial dose gradients at short range

- body orientation during exposure

- differences between localised and whole-body dose

Agreement at this level is strong evidence of a valid reconstruction.

4. Health implications

At doses of order 100 mSv:

- deterministic whole-body effects are unlikely

- the primary consequence is increased stochastic risk

- this represents a serious occupational overexposure

If localised exposure occurred (e.g. hands), higher tissue doses may be possible and should be acknowledged.

Students should avoid overstating acute radiation syndrome.

5. IRR17 compliance

Likely issues include:

- failure to adequately restrict exposure under abnormal conditions

- inadequate verification of source position

- failure to identify abnormal radiation fields before close approach

- insufficient use of radiation monitoring equipment

- deficiencies in training, supervision, or procedural compliance

Better answers will apply the regulations directly to the incident rather than listing them.

Dose greatly exceeds dose limits and so Regulation 26 applies.

6. ONR SAP evaluation (indicative)

EKP.5 Defence in Depth

Multiple layers failed:

- mechanical failure or incomplete source return

- failure to detect abnormal radiation field

- unsafe close approach without verification

- absence or misuse of dosimetry

Engineering safety / source control

Source position should be unambiguous and verifiable.

Human factors / procedures

The incident reflects reliance on assumption rather than radiological confirmation.

7. ALARP improvements

Credit proportionate and justified improvements, such as:

- improved source-return verification procedures

- mandatory radiological survey before approach

- enhanced maintenance and inspection of drive systems

- improved training in abnormal condition recognition

- improved use and availability of personal dosimetry

Improvements should clearly reduce risk with reasonable effort.

8. Marker guidance on common errors

| Issue | Treatment |
| --- | --- |
| Failure to convert TBq → GBq | Penalise units, not method |
| Incorrect inverse-square use | Penalise accuracy but credit structure |
| Ignoring shielding factors | Lose interpretation marks |
| Overstating health effects | Correct firmly |
| Treating calculation as exact | Reward recognition of uncertainty |

9. Indicative concluding judgement

A strong conclusion would identify the incident as arising from failure to correctly identify the true source position and radiation field, leading to a close-range exposure in a highly non-uniform field.

The resulting dose is significant and clearly above normal occupational limits, though not expected to cause deterministic whole-body injury.

The event represents a failure of engineering controls, procedural discipline, and radiological verification, and highlights the importance of robust defence in depth in industrial radiography operations.