Question 1:

A) SPI – Serial Peripheral Interface

B) It is a synchronous serial communication interface mostly used in embedded systems. It communicates in full duplex mode using the master-slave system. The system can communicate with multiple slaves via a slave select line.

C)

SCLK - System Clock – A continuous clock used to synchronise the systems and its transmissions

MOSI - Master Out Slave In – Data transmission from the master to the slave systems

MISO - Master In Slave Out – Data transmission from the slaves to the master system

/SS - Slave Select – Used to select which slave the data is being transmitted to

Question 2:

A)

a) Inverting Amplifier

b) Gain = - RF/R1

c) -4

d) -0.8 Vp-p

e) Output is 180 out of phase with the input

B)

a) Non-Inverting Amplifier

b) Gain = 1 + RF/R1

c) 5

d) 0.55 Vp-p

e) The output is in phase with the input

Question 3)

- RS232

- It is a point-to-point serial communication interface with relatively low transmission rates, but it is cheap and easy to set-up.

- Connecting peripherals (printer, monitors) to a personal computer

Question 4:

A)

a) 10001100 – 0011 0001 – 3 1

b) ASCII character 1

B)

a) 11000100 – 0010 0011 – 2 3

b) ASCII character #