In this lesson, you will learn how to create and customize thematic maps

MapInfo tutorial 3

Beverley Holland

Thematic mapping.

MapInfo allows you to create seven different types of thematic maps. Each type has its own purpose and can add uniqueness to you GIS map layers.

Objectives

- Express Numeric Values with a Thematic Map.

- Customize a Thematic Map and Legend.

- Express Character Values with a Thematic Map.

- Edit a Legend.

- Save a Thematic Map.

- Manage Thematic Templates.

- Understand Thematic Layers.

- Shade a Map with Data from a Separate Table.

- Use a Geographic Join.

- Embed a Legend.

In order for you to use MapInfo efficiently, you must understand the concept thematic maps. With thematic mapping it is possible to create many different representations of the same data. A thematic map is a map whose appearance is altered to express some underlying information.

Understand Types of Thematic Maps

- Ranges: Each colour represents a range of numeric values.

- Bar Charts: A bar graph is placed over each object being shaded.

- Pie Charts: A pie chart is placed over each object being shaded.

- Graduated Symbol: Symbols of different sizes are used to represent the magnitude of the data. That is, large symbols represent large values and small symbols represent small values.

- Dot Density: Dots are placed in a boundary map so that the total number of dots represent that region's data value.

- Individual: Each unique value is given its own colour or symbol.

- Grid: A surface thematic interpolating point data over a region.

MapInfo allows you to express numeric values with a thematic map, such as populations.

In this type of map a single colour or style represents a user-specified range of values. For example, a country shaded red might represent a population between two and four million. Whereas, a country shaded pink could represent a population range of one to two million, and so on for each range of the thematic map.

Try to create a thematic maps based on data from the Swansea output are map (with 700 + land polygons) table. It is possible to thematically shade the output areas in different ways. Open up the workspace that you have created. The workspace should contain the Swansea output map and the Townsend index data. If you have managed to download the other boundary data then they should be in you workspace or you can add them to the workspace.

- Open the File menu and select Open.

- From the Files of Type drop down, choose Workspace (*.wor).

- Select your…...WOR. (if you have all your maps in the same folder as the workspace this should work

- Press Open. The following maps should appear:

Your layer window may look like this if you have downloaded addition map boundary data.

Based on the tables opened by this workspace, we can begin creating thematic maps using the Townsend data. Start by creating a ranged map. We will shade the Swansea output area table (swanseaoutput in this example) based on the Townsend index of deprivation. Creating a thematic map involves three steps. Each step has its own dialog. Remember you must click on the map page to make it active and see the map in the map option on the dropdown menu

- Open the Map menu and select Create Thematic Map.

The first step is to select the type of thematic map you wish to create.

You also need to select a thematic template. A thematic template contains thematic map settings, such as colour choice, range method, and legend settings. These templates are to be used during current and future sessions. A template must be chosen each time a thematic map is created.

For each type of thematic map, there are a variety of predetermined templates to choose from. In the case of ranges or individual value thematic maps, you need to know the type of map object you wish to shade. These types of thematic maps have templates for point, line, and region type objects.

For this first map choose Ranges, using numeric information available in the underlying table. This is the default thematic map type.

- Verify the Ranges button is selected.

- Press Sort by Name to view templates alphabetically.

- Choose the template sequential brow-light yellow.

- Press Next.

Next, the Create Thematic Map - Step 2 of 3 dialog allows you to specify the table you are shading and the field you will get values from. The data can either be in the table that you are shading, or it can come from another table. The data we are using is in the same table.

When the dialog appears, the Table drop-down choose your Swansea output are map. In the Field drop-down list choose in the first instance join .

- Press Next.

On the next dialog choose Townsend (or whatever you have called you spreadsheet data)

- Choose Townsend from the get value for Table drop-down list.

- Choose townsednindex from the Field drop-down list.

- Click ok

- You are now back at the step 2 of 3 dialog box

Next, the Create Thematic Map - Steps allows you to specify the ranges and styles to be used. It is also possible to customize the theme legend accompanying the thematic map. For now we will look at the map with the current settings. Later we can modify the map.

- Press OK.

The map is now shaded according to the Townsend index. Note the Theme Legend window that appears. In the legend the numbers in parentheses indicate how many records are in each range. If desired, it is easy to move the legend window or resize it.

If your Theme Legend window does not appear, do one of the following:

- Press the Show/Hide Legend button.

- Choose Show Theme Legend Window from the Options menu.

MapInfo Professional allows you to improve a thematic map and legend by making changes to the settings of the map. Thematic maps allow for the quick visualization of the data, it can be improved for the benefit of the map interpreter, using customization. For example, the foreground colours for a map.

This example will demonstrate changing the styles used in shading the map. To customize the thematic map and legend, open the MAP menu and choose MODIFY THEMATIC MAP.

From the Map menu, choose Modify Thematic Map.

Click the Styles button.

This dialog box has two sections. The first, Auto Spread, allows you to automatically spread the colours from the colour selected for the top range to the colour selected for the bottom range. You may also turn this feature off to choose your own colours for each range.

Click on the red polygon at the top of the Customize Range Styles dialog (it is marked 3.2 to 10.1 yours may vary slightly).

The Region Style dialog displays. You may change the pattern, foreground, and background of the shading used for the objects that fall in the selected range.

Change the foreground of the top range. Click on the Foreground drop-down list.

Click on a dark colour. Remember that the colour you pick here will be used for the highest ranges; lower ranges will have lighter variations of this colour.

Press OK to close the Region Style dialog.

Notice that the colour is now spread from dark red for the top range to light yellow for the bottom range. Next, the shading for the bottom range will be changed. Repeat this process for the gray polygon at the bottom.

Click the gray polygon representing the bottom range.

Click the Foreground drop down list.

Click on yellow. Note that the colour selected here will be used for the lowest range.

The map is now shaded with your new colours, graduating from yellow at the lower ranges to dark blue at the higher ranges.

This shading reflects a general principle of thematic mapping. It is considered intuitive that darker colours often are used to represent higher values or intensities, while lighter colours are often used to represent lower values or lower intensities.

If you want to create your own colour scheme, you do not have to use the Auto Spread feature. Auto Spread may be turned off. Simply check None at the top of the Customize Range Styles dialog. Also, if you change one of the colours in the middle ranges, Auto Spread is automatically disabled.

MapInfo allows you to change the ranges themselves in order to help improve a thematic map and legend by making changes to the settings of the map. You are able to choose how many ranges there are, and how they are calculated. It is even possible to create your own custom ranges.

In this example, the number of ranges and the method by which those ranges are calculated will be changed.

To make these changes, double click the LEGEND WINDOW. This has the same effect as opening the Map menu and clicking Modify Thematic map.

Click Ranges. The Customize Ranges dialog displays:

There are six different range calculation methods to choose from.

Range Calculation Methods:

- Equal Count - places the same number of records in each range. The result will be an even distribution of records in each range. The values of the ranges themselves may vary greatly in this calculation method.

- Equal Ranges - makes the magnitude of each range the same. In this method, the values of the ranges are evenly distributed, but the record count may vary greatly.

- Natural Break - makes each range contain data values that are close together.

- Standard Deviation - means that the ranges meet at the mean and at standard deviation intervals from the mean.

- Quantile - ranges are distributed across another segment of your data by a percentage.

- Custom Ranges - allow you to define your own ranges.

For this example, from the METHOD drop down select EQUAL RANGES.

Notice that the values have been removed from the range listings below because they are no longer valid. To recalculate the ranges based on the new information, From the # of Ranges drop-down list select 5.

Click Recalc.

Pressing RECALC causes the ranges to be recalculated. Once the calculation has been performed, the RECALC button changes back to OK. This way you can experiment with many different range calculation methods before you choose the one you want.

Click OK to close the Customize Ranges dialog.

Click OK to close the Modify Thematic Map dialog.

The map displays:

Note the difference between this map and the first. Each range is the same size according to the Townsend index. By changing the ranges, we have a different distribution of colours across the map.

Some of the data on which you create thematic maps will be character data. MapInfo allows you to express character values with a thematic map.

In this example, each polygon group has been classified as being of the following different types under the OAC_GROUP_ in the Swansea output map:

Ageing City Dwellers

Ageing Rural Dwellers

Ageing Urban Living

Asian Traits

Challenged Asian Terraces

Challenged Diversity

Challenged Terraced Workers

Comfortable Cosmopolitans

Constrained Flat Dwellers

Endeavouring Ethnic Mix

Ethnic Dynamics

Ethnic Family Life

Farming Communities

Hard-Pressed Ageing Workers

Industrious Communities

Inner-City Students

Migration and Churn

oac_group_

Rented Family Living

Rural Tenants

Semi-Detached Suburbia

Students Around Campus

Suburban Achievers

Urban Professionals and Families

White Communities

The points representing each site will be thematically shaded so that you can quickly see the type of each site by just looking at the map.

Look at  the information included in the Swansea output  table by displaying it in a Browser window.

There will be several records that are of the same type.

Click the Individual button in the Create Thematic Map - Step 1 of 3 dialog.

.

The SITES table contains point type objects. Choose Point IndValue Default from the Template Name list.

Click Next.

In the Step 2 dialog we will specify which table to thematically shade and which field to use. The Table list should be your Swansea map. The Field list should be OAC_GROUP_  .

An individual value thematic map will allow us to shade with either character or numeric information.

Click Next.

In Step 3, we can examine how MapInfo categorizes the data based on the Type field. The number in parenthesis is how many sites are of that particular type. It is possible to customize the map. The Style button allows you to change the type of symbol, its size and colour, while the Legend button allows you to customize the Legend window.

The STYLE button allows you to change the type of symbol, its size and colour.

Click the STYLES button and choose a different symbol for each of the categories.

Click OK to return to the Create Thematic Map - Step 3 of 3 dialog.

Click OK to create the map.

The MAP redraws itself with the sites shaded according to their type using the colours that were set. If you wish to alter the thematic map at this point you can do so by double-clicking in the legend window, or choose MODIFY THEMATIC MAP from the Map Menu.

When a thematic map is created, and you wish to formally present it to someone else to read, it is a good idea to customize the Legend

While the legend provides us with enough information to understand the shading on the map, it could use some improvement so that it is more understandable to someone viewing the information for the first time. The legend can be improved by modifying the information that it contains, for example, its title

To modify a thematic legend double click the LEGEND window on the legend that you wish to modify.

Click Legend. The Customize Legend dialog displays.

Change the legend to something sensible. Save you work to a workspace.