Contents

Figure table	2

Equation table	3

Tables	3

Introduction	4

Task 1a:	4

Task 1b:	7

Comparing cut off frequency results	9

Task 2:	9

Half wave dipole antenna	9

Results	12

Conclusion	12

Task 3: RADAR	12

The birth of the radar – 1930’s.	12

What is a radar	13

Traditional radar use	13

Technological advances	13

Challenges and failures	14

Societal and environmental impacts	15

potential hazards	15

Positives	15

Recent developments	15

Radars and drones	16

Example of RCs	16

Proposed improvements	17

Conclusion	17

References	18

# Figure table

Figure 1, rectangular wave guide results	5

Figure 2, Cut off frequency calculation	5

Figure 3, Cut off frequency answer	5

Figure 4, Cut off frequency results	6

Figure 5, E and H field for wave guide	7

Figure 6, TEmn mode equation	7

Figure 7, Simulated Teflon results	8

Figure 8, Teflon final results	8

Figure 9, radiating surface	9

Figure 10, length equation	10

Figure 11, operating frequency calculation	10

Figure 12, S-parameter	10

Figure 13, Finished dipole design	11

Figure 14, Gain results	11

Figure 15, Listening ears,	12

Figure 16, Radar echo waves	13

Figure 17, Guinness world record	15

Figure 18, Radar cross section	16

# Equation table

Equation 1, Cut off frequency calculation	4

Equation 2, Cut off frequency answer	5

Equation 3, TEmn mode equation	7

Equation 4, length equation	9

Equation 5, operating frequency calculation	9

Equation 6, Listening ears	12

Equation 7, Radar echo waves	13

Equation 8, Guinness world record	14

Equation 9, Radar cross section	16

# Tables

Table 1, Field values	6

Table 2, Teflon results	7

Table 3, comparison of cut off frequencies	8

Table 4, Example RCs [9]	16

Table 5, Radar bands	17

# Introduction

This report is structured into four main sections, each addressing different aspects of electromagnetic wave propagation and radar technology. The first section focuses on the design of a rectangular waveguide, incorporating mathematical, statistical, and electromagnetic principles. The second section presents a modified waveguide, featuring PEC (Perfect Electric Conductor) walls and a Teflon-filled interior, with a comparative analysis of its performance against the initial design. The third section explores the design of a half-wave dipole antenna, examining theoretical predictions and their alignment with simulated results. The final section discusses radar systems and their limitations in detecting drones, particularly due to the small size and high manoeuvrability of modern UAVs. Given the rapid rise in drone technology, traditional radar systems face challenges in effective detection and tracking. This report reviews recent advancements in radar-based drone detection, their societal and environmental impacts, and potential improvements to enhance detection capabilities in the future.

This report explores the design and simulation of a rectangular waveguide and a halfwave dipole antenna. Two waveguide configurations were analysed. Air-filled (ɛr=1) and Teflon-filled (ɛr=2.1) with results showing lower cut off frequencies in the Teflon-filled waveguide due to increased permittivity. The halfwave dipole antenna’s simulated results deviated from theoretical predictions, suggesting discrepancies due to modelling assumptions or material losses. These findings emphasize the impact of dielectric materials on waveguide performance and the challenges in achieving theoretical resonance in antenna design.

# Task 1a:

The following rectangular waveguide model was designed with the following in mind.

PEC walls filled with a vacuum and the set-up frequency sweep from 2GHz to 30GHz.

Dimensions of the waveguide 23mm (a) Height: (0)6mm, Length 50mm along the Z-axis:

These dimensions where determined by (a) the 1st two numbers of my student ID, 2 and 3, (b) the 3rd and fourth number 0 and 6 divided by two. Results are shown in Figure 1.

Figure 1, rectangular wave guide results

The dominant mode in a rectangular waveguide is the lowest-order mode that can propagate. TE10 mode has the lowest cut off frequency, meaning it propagates first before other modes. The equation for the cut off frequency is given in Equation 1 and the answer is given in Equation 2.

fcmn= c2ma2+nb2

Figure 2, Cut off frequency calculation

c = speed or light (3.0 x 103 m/s)

a = waveguide width (23mm)

b = waveguide height (6mm)

m, n = Mode indices (for  TE10: m = 1, n = 0)

For TE10:  fc10 = 3.0 x 1082 x 0.023

fc10 = 3.0 x 1082 x 0.046

fc10= 6.52 GHz

Figure 3, Cut off frequency answer

Figure 4, Cut off frequency results

The results shown in Figure 2, show a cut off frequency of 6.5500. Which is very similar to the theoretical prediction.

The cut off frequencies for the first three waveguide modes were determined using the theoretical waveguide cut off frequency equation. The dominant mode TE10 ​ has the lowest cut off frequency of 6.52 GHz, meaning it is the first mode to propagate when the operating frequency exceeds this value. The next two higher-order modes, TE20 and TE01 ​, have cut off frequencies of 12.99 GHz and 19.50 GHz. if the frequency increases beyond 12.99 GHz, the TE20 mode will also propagate, leading to multimode operation. At 19.50 GHz and beyond, the TE01 mode is also supported, further increasing wave propagation.

- Mode one – Ensures efficient wave propagation at lower frequencies

- Mode two – Introduces the possibility of multimode operation

- Mode three – Further increases the complexity, potentially leading to interference or power issues.

The port field display was recorded for both port 1 and port 2. The results are shown in Figure 3, the results were as expected for TE10 The results indicated successful propagation.

Figure 5, E and H field for wave guide

The magnitude of E-field and H-field is the strength (amplitude) of the electric and magnetic fields in each waveguide. The vectors are the direction and orientation of the electric and magnetic fields. Field value results shown in Table 1.

| Max and min results | Magnetic field | Electric field |
| --- | --- | --- |
| Max | 20.688 A/m | 7771.745 V/m |
| Min | 0.018 A/m | 0.000 V/m |

Table 1, Field values

The results from HFSS show that the simulated electric and magnetic field distributions align with the expected. TE10 mode behaviour. The electric field reaches a maximum of 7771.745 V/m at the centre of the waveguide, with a minimum of 0 V/m at the boundaries. The magnetic field has a peak value of 20.688 A/m, with a minimum of 0.018 A/m, indicating proper field propagation. The observed field patterns confirm that the waveguide is operating in the dominant. TE10  mode, as expected. The theoretical analysis of the TE10  mode in a rectangular waveguide confirms that the electric field exhibits a sinusoidal variation across the width, while the magnetic field follows a cosine variation.

# Task 1b:

In this section, the rectangular waveguide is redesigned with Perfect Electric Conductor (PEC) walls and filled with Teflon. The waveguide maintains the same dimensions and simulation parameters.

Using the same equation as before with a slight change, instead of 2, ɛr is used meaning a 2.1 relative permittivity of Teflon.

fcmn= cɛrma2+nb2

Figure 6, TEmn mode equation

Using the equation, the results have been plotted in Table 2 for all three modes.

| Mode | Results |
| --- | --- |
| 1 (TE10 ) | fc = 4.5200GHz |
| 2 (TE20 ) | fc = 9.00GHz |
| 3 (TE01 ) | fc = 13.48GHz |

Table 2, Teflon results

The simulated results are shown in Figure 4.

Figure 7, Simulated Teflon results

The results matched the estimated values confirming the accuracy of the waveguide. The decrease in cut off frequencies due to the Teflon filling aligns with expectations, as the permittivity increase lowers the wave velocity inside the waveguide. The simulated results from the Teflon simulation are shown in Figure 5.

Figure 8, Teflon final results

### Comparing cut off frequency results

Comparing the results in Table 3, vacuum filled results in a higher cut off frequency. Teflon filled shows a drop in cut off frequencies in all modes. The dominant mode TE10  shifts from 6.52GHz (vacuum) to 4.52 GHz (Teflon) a 31% reduction.

| Mode | Cut off (vacuum) | Cut off (Teflon) | Difference (GHz) |
| --- | --- | --- | --- |
| 1 (TE10 ) | 6.52 | 4.52 | 2.00 |
| 2 (TE20 ) | 12.99 | 9.00 | 3.99 |
| 3 (TE01 ) | 19.50 | 13.48 | 6.02 |

Table 3, comparison of cut off frequencies

The reason for the change in permittivity (ɛr) is due to the speed of light inside the waveguide. Since Teflon increases (2.1) permittivity the speed of light decreases, lowering the cut off frequency. Using a Teflon-filled waveguide enables wave propagation at lower frequencies, which can be beneficial for applications that require lower-frequency operation, such as reduced signal loss and controlled wave propagation.

# Task 2:

## Half wave dipole antenna

The following dipole antenna was designed with a radius of 1mm intended to radiate electromagnetic waves of:

Operating Wavelength (λ): 230 mm, Dipole Length (L) 115mm and Radius (r) 1 mm:

The antenna was constructed with PEC and the applied excitation through a 50Ω lumped port. To design the dipole antenna, I used the ANSYS [1] programme to execute simulations. The design was within a rectangular radiating surface shown in Figure 6.

Figure 9, radiating surface

The length of the dipole is 57.5mm

L=λ2=2302=115mm

Figure 10, length equation

The operating frequency is 1.304GHz

f=cλ=3x1030.23      f=1.304 x 109Hz=1.304GHz

Figure 11, operating frequency calculation

S-Parameter (scattering parameter) plot, is used to analyse how much power is reflected from the antenna. The best resonance, meaning the antenna is radiating efficiently is at 7.3125GHz as shown in Figure 7.

Figure 12, S-parameter

Figure 13, Finished dipole design

Figure 8 shows the final dipole design. The expected Radiation Pattern are Omnidirectional radiation pattern in the H-plane (azimuth plane, XY- plane) meaning it radiates equally in all directions perpendicular to the dipole. The pattern in the E-plane (elevation plane, XZ-plane)—meaning the strongest radiation is broadside to the antenna, with nulls at the ends.

Maximum Radiation: Perpendicular to the dipole (broadside).
Minimum Radiation: Along the dipole axis (nulls at the ends).
Polarization: Typically, linear aligned with the dipole’s orientation.

Actual results shown in Figure 9, minimum gain of -34.32 and a maximum of 5.01.

Figure 14, Gain results

## Results

The expected theoretical resonance was 1.304GHz, the simulated result was 7.3125GHz. The reasons for this change may have been a scaling issue or simulation boundary conditions. If the same test was carried out again, changing the dimensions of the antenna, material settings or performing parameter sweeps could improve accuracy.

### Conclusion

This study explored the design and simulation of a rectangular waveguide and half pole antenna, looking in to their electromagnetic properties. Two rectangle waveguides where made one with vacuum and one with Teflon the results showed that higher permittivity in Teflon lowered the cut off frequency. With the dominant TE10 mode shifting from 6.52GHz to 4.52GHz. However, the halfwave dipole antenna’s simulated results deviated from theoretical expectations, indicating potential discrepancies due to modelling assumptions, material losses, or boundary conditions. These findings highlight the impact of dielectric materials on waveguide performance and the challenges in achieving theoretical resonance in practical antenna designs.

# Task 3: RADAR

‘Listening ears’ are 3 concrete half domes found in Kent, Figure 15, provided early warning for enemy aircraft, they worked by reflecting sound from a focal point of an acoustic mirror to an operator. These where used between 1916 – 1930’s.

Figure 15, Listening ears,

### The birth of the radar – 1930’s.

Experiments where conducted by Heinrich Hertz showing radio waves were reflected by metallic objects [2]. The theory behind it was that signals could ‘bounce’ off enemy aircraft highlighting them. On February 26, 1935, there was a successful demonstration of the system using a BBC transmitter while focusing on a test aircraft. After the successful demonstration, tests were conducted via towers, two wooden towers transmitting and four steel towers receiving. This became the first radar station based in Bawdsey, as World War two was starting.

In 1941 the first secret radar system was installed, starting off as temporary systems that soon moved to fixed systems. The Type 7 search radar and Type 13 height finding radar.

## What is a radar

Radar (Radio Detection and Ranging) Is a system used to detect objects using waves. Electromagnetic energy travels through the air at a constant speed - approximately the speed of light (186,000 miles per second). The distance between the reflecting object and the radar site is determined by measuring the time between transmission and reception of the energy pulses [3]. These electromagnetic waves are used for recognising, tracking, and locating objects at a distance. They work of sending a wave out and observing echo’s, Figure 16. With the results you can measure size, direction of movement and speed. Radars can be used where cameras fall short, radars can “see through” the elements like fog and rain where a camera can’t, while also decreasing privacy concerns and covering a larger area then a camera can reach.

Figure 16, Radar echo waves

## Traditional radar use

Traditionally radars are used for military purposes, being used to detect the enemy, in the air, land and sea. While radars are commonly thought to be used in a military context, commercially they are used in the archaeology, mining, meteorology and the automotive industries.

## Technological advances

- 1916 – Listening ears invented

Look back at section “History of the radar”

- 1935 - 1st military application

A chain of early warning systems where erected along the south coast of England, with the ability of picking up enemy aircraft at 80miles away.

- 1940 – World war two meant significant improvements in radar technology

A device called the cavity magnetron, produced microwaves meaning improved accuracy and achieved greater distance.

- 1951 – Phased array

Powered by groups of antennas to create a high-gain beam, meaning more flexible scanning

- 1960 – Doppler radar

Bouncing microwave signals of a desired target and analysing the objects motion [4]

- 1978 – Colour radar, visual data

Used to determine if something is moving away or towards the radar, typically use red (moving away) and green (coming towards).

- 1980 – Transition from analogue to digital

Adding ADC (analogue to digital converters) to the front end, then processed by computers. Improving performance.

- 1995 – Synthetic aperture enhancing imaging capabilities

Used to track oil spills and icebergs as it reacts with pulse energy on physical structures.

- 2020 – Frequency modulated continuous wave radar

Short range used for distance determining. Can be used when there is more than one echo returning. Used in aircraft landing.

## Challenges and failures

- Range issues

- Target type and location accuracy

- Distinguishing targets

- Interfering echoing

- Can’t track multiple objects at once

- Stealth technology

Target sizes picked up by a radar will be determined by its “cross section” measured in square meters meaning what’s seen may not be its actual size. Air-crafts cross section can differ depending if you’re looking at it broadside or not. These particular targets of interest fluctuate rapidly, with cross sections changing by a factor or 10 to 1000 in respect to the radar, after extensive research into radar results we can tell what is being echoed back. Some radars can’t make out what is a bird or drone as cross sections can be similar, so may disregard them. Drones also have the capability of flying in swarms, with Israel being the first country to use a drone swarm in combat and 8,100, being the Guinness world record as of September 2024 [5] Figure 17.

Stealth vehicles will have a “spectral signature” which varies its reflectance of material with respect to wavelengths [6].

Nuisance echo’s such as rain, meteors or birds can limit Radar capabilities. The atmosphere can form “ducts” that trap and guide radar energy around the curvature of the Earth and allow detection at ranges beyond the normal horizon [7].

Figure 17, Guinness world record

## Societal and environmental impacts

### potential hazards

Radiation risk

High doses of radiation can affect the body by effecting the functionality of tissues and organs, cause vomiting or nausea or in extreme cases it can cause death. [8]

### Positives

Safety navigation systems

Primarily used at sea for navigating and manoeuvring ships.

Weather forecasting

Radars are used to observe and understand weather as they can “see” inside clouds. 15-18 radars cover the whole British Isles.

Air traffic control

Ensuring safe operations of both private and commercial flights, coordinating movements and directing them for landing.

Law and enforcement

Most commonly known systems are the ones used by the police to detect speeding drivers. People also use radar to detect wither they are being picked up from a police camera. While, not illegal to own, they are illegal to use to spoof the police cameras.

Radars most known use is in defence, identifying threats such as missiles, aircraft or drones.

## Recent developments

With drones becoming an increasing threat technology needs improving.

- Doppler radar

High-resolution doppler radars help distinguish birds from drones, reading signatures produced from wingbeat frequencies and blade frequency. AI analysis is being implemented along with improved frequency resolution.

- AI for radar processing

Clutter causes issues with traditional radars machine learning can help learn patterns that drones produce compared to other objects (movement patterns, size, speed). Real time processing adapts to new drone types and behaviours.

- Multi static and passive radars

Multi static - Multiple transmitters and receivers improve accuracy.

Passive radar – analyse existing signals instead of emitting its own.

Higher sensitivity capable of detecting stealth drones, lower power consumption and increased range.

- 3D radar

Provide height and range for precise tracking. Detects in lower altitudes and can track swarm attacks.

- Frequency modulated continuous radar

Continuously detects and measures frequency’s shifts to detect objects, used with slow-moving, low-altitude drones. Higher bandwidths improve detection in close ranges and can be used with thermal sensors.

- Cognitive radar systems

AI driven with adaptive waveforms by learning its environments to adjust transmission parameters. It’s self-learning, reduces false alarms and can adjust frequencies to counter stealth drones.

## Radars and drones

Traditional radars use echoing waves and are used to pick up larger objects then drones. Issues become apparent when the drone’s cross section are too small, they move at a slow speed or low altitude.

σ=lim4πR2|Es|²Eo|²

Figure 18, Radar cross section

E0  = Electronic field strength of incident wave impinging on target

Es = Electronic field strength of scattered wave at radar (in range R)

## Example of RCs

| Target | RCs in M² | RCs in dBsm |
| --- | --- | --- |
| F-117A Nighthawk | 0.001 – 0.01 | -30 - -20 |
| Nano-UAV | <0.01 | <-20 |
| (Bird) Sparrow | 0.017 – 0.018 | -17.7 - -17.4 |
| (Bird) Eagle | 10 - 25 | 10 -14 |
| Walking man | 0.5 – 1.3 | -3 -0.8 |
| Commercial quad copter | 0.032 | -15 |

Table 4, Example RCs [9]

Speed and altitude challenges

Drones blend into the environment and move slower than a commercial aircraft, sometimes hovering. A lot of radars filter out slow moving objects thinking they are noise clutter.

## Proposed improvements

From looking at Table 4 [9]the best proposed improvements would consist of the following built into one system.

- Micro-doppler radar

Birds and drones have similar cross references, movement signatures will differ, wing flaps are irregular movements while drones have stable high frequency blade motions. Doppler analysis extract motion figures, with AI classification will distinguish the difference.

- Higher frequency bands

A higher frequency radar has better resolution and sensitivity for detecting more objects like drones. A combination of the two higher frequency radars would detect most size drones.

| Radar Band | Frequency | Usage |
| --- | --- | --- |
| 1-2 GHz | Long range detection | Large aircraft |
| 8-12 GHz | Medium range | Best for drones |
| 12-18 GHz | Short range | Ideal for small drones |

Table 5, Radar bands

- Multi-Static & Passive Radar for Stealthy & Low-Altitude Drones

Stealth drones can have RCs of <0.01m² and can escape traditional radars, others blend with ground clutter. Multiple receivers in multiple locations can detect small RC objects. Passive radar, to detect drones that absorb radar waves.

- 3D Radar for Precise Drone Tracking

3D radars provide altitude, range and azimuth which is much more detailed then the 2D radars that only do range and azimuth. This technology can track drone altitude and movement.

- AI & Machine Learning for Smart Target Classification

A mix of radar, acoustic and optical sensors provide multi-layer detection. Deep learning models can train on a vast amount of data. Eliminating false alarms caused by birds and clutter.

# Conclusion

As drones continue to evolve in both capability and accessibility, the need for more advanced radar detection systems becomes increasingly important. Traditional radar systems, while effective for detecting larger aircraft, struggle with the unique challenges posed by drones, such as their small cross-section, low-altitude flight, and ability to manoeuvre unpredictably. Recent advancements in radar technology, including AI-driven cognitive radar, micro-Doppler analysis, and 3D radar, have significantly improved detection capabilities. These innovations enhance accuracy, reduce false alarms, and enable the tracking of stealth and swarm drones. Additionally, integrating multi-static and passive radar systems provides better coverage and sensitivity, even for low-observable targets. Despite these improvements, challenges remain, such as mitigating nuisance echoes, improving target classification, and addressing potential radiation risks. However, by combining advanced radar technologies with AI and machine learning, future systems can adapt to emerging threats more effectively. Ultimately, the continuous development of radar-based drone detection is crucial for enhancing security, protecting critical infrastructure, and ensuring safe airspace operations. With ongoing research and technological integration, radar systems will continue to evolve, offering greater precision, reliability, and adaptability in an increasingly drone-populated world.

# References

| [1] | “https://www.ansys.com/academic,” ANSYS, [Online]. Available: https://www.ansys.com/academic. [Accessed 18 02 2025]. |
| --- | --- |
| [2] | A. d. r. m. RAF, “Birth of the Radar,” RAF, Air defence radar museum, [Online]. Available: https://www.radarmuseum.co.uk/history/world-war-two/#:~:text=In%20May%201935%20Watson%2DWatt,world's%20first%20working%20radar%20system.. [Accessed 05 02 2025]. |
| [3] | C. Bloom, “Introductiont to radar,” Solutions Architect for Arrow Electronics, [Online]. Available: https://www.arrow.com/en/research-and-events/articles/introduction-to-radar. [Accessed 05 02 2025]. |
| [4] | D. W. Wragg, “Doppler radar (Wikipedia),” A Dictionary of Aviation , 1973. [Online]. Available: https://en.wikipedia.org/wiki/Doppler_radar#cite_note-1. [Accessed 05 02 2025]. |
| [5] | A. magazine, “Record-breaking 8,100 drones take to sky for show in China,” AV magazine , 16 09 2024. [Online]. Available: https://www.avinteractive.com/news/drones/record-breaking-8100-drones-take-to-sky-for-show-in-china-16-09-2024/#:~:text=Two%20Guinness%20World%20Records%20were,have%20broken%20Guinness%20World%20Records.. [Accessed 05 02 2025]. |
| [6] | wikipedia, “Spectral signature,” wikipedia, 10 2023. [Online]. Available: https://en.wikipedia.org/wiki/Spectral_signature. [Accessed 05 02 2025]. |
| [7] | M. I. Skolnik, “Factors affecting radar performance,” Britannica, 09 01 2025. [Online]. Available: https://www.britannica.com/technology/microwave-oven. [Accessed 05 02 2025]. |
| [8] | “Radiation and health,” World health organisatoin , 07 07 2023. [Online]. Available: https://www.who.int/news-room/questions-and-answers/item/radiation-and-health#:~:text=What%20are%20the%20acute%20health,burns)%2C%20or%20even%20death.. [Accessed 06 02 2025]. |
| [9] | C. B. W. Wolff, “Radar and electronics warfare,” in Pocket guide , Germany, Rohde and schwarz, pp. 104-105. |