| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| MAY 2018 | MAI 2018 |

MODULE CODE: M5X02064

CAE & Stress Analysis

MOCK EXAM

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator

Adnoddau neu ddefnyddiau sydd eu hangen - Calculator

FORMULAE BOOKLET

Simple Bending Equation:

MI=σy=ER

Second Moment of Area of common shape:

- Rectangular cross-section:

b

d

Neutral Axis

I=b.d312

- Circular cross-section:

r

Neutral Axis

I=π.r44

- section:

d

b

d1

d1

b1

I=b1.d+2.d1312-b1-b.d312

Neutral Axis

Slope and Deflection of Beams:

deflection=y                    slope=dydx                        Bending Moment Mx=EId2ydx2

- QUESTION ONE (30 marks)

A beam 10 m long has a circular cross section of diameter 10 cm and is simply supported as shown in Fig. 1. If the maximum direct stress in the beam is limited to 200 N/mm2 and the maximum permissible uniformly distributed load that may be applied is 70 kN/m, determine the location of the reactions R1 and R2 assuming that the reactions are equidistant from mid-span.

x1

x2

x1

70 kN/m

Fig. 1: Beam’s applied load and supports

- QUESTION TWO (35 marks)

Consider the beam loaded as shown in Fig. 2.

- Draw the Shear Force and Bending Moment Diagrams. Values of shear force and bending moment at points A, B, C, D, E, F and G should be provided.

- Determine the position and magnitude of the maximum Bending Moment.

- Determine the position of any point of contraflexure.

40kN

10kN

20kN

20kN

20kN/m

10kN/m

D

2 m

1 m

2 m

3 m

1m

1m

E

D

A

G

C

RF

RB

F

B

Fig. 2: Beam’s applied loads and supports

- QUESTION THREE (35 marks)

The beam shown in Fig. 3 is 13 m long with an E value of 250 GN/m2 and second moment of area value of 8×10-4 m4.

- Draw the Shear Force and Bending Moment Diagrams.

- Determine slope and defection in the middle of the beam.

5 kN

10 kN

5 kN

20 kN

1 m kN

3 m

2 m

2m

3m

2 m

RB

RA

Fig. 3: Beam’s applied loads and supports

Diwedd/End