60%

APPENDIX GA36b

#### LEVEL 5 ASSESSMENT SPECIFICATION

| Student name: | Matthew Permintella | Student ID number: | PER18841948 |
| --- | --- | --- | --- |
| Programme: | HND LEVEL 5 |  |  |
| Module: | MECHANICAL PRINCIPLES |  |  |
| Module code: | M5X00926 | ## Contribution to Overall Module Assessment (%): | 25% |
| Lecturer: | EMILY NYATHI | Internal Verifier: | EDWARD HORNAGOLD |
| Assignment Title: | COMPLEX LOADING II | Word count (or equivalent): |  |
| Submission deadline: | 21/03/2020 | Return date of provisional marks & written feedback: | 28/03/2020 |
| Submission method: | All written assessments, where practical and possible, must be submitted via Turnitin unless otherwise instructed by the Lecturer. (Please DO NOT put this assessment specification into Turnitin or it will match many similarities with other students’ submissions.) Alternative submission method (if applicable): Late submission of the assessment will result in a late penalty mark. Penalties for late submission: Up to one-week late, maximum mark of 40%. Over one week late, 0%. Only the Extenuating Circumstances Panel may approve a change to submission dates. |  |  |
| Academic honesty / referencing: | Academic honesty is required. In the main body of your submission you must give credit to authors on whose research and ideas your work is based. Append to your submission a reference list that indicates the books, articles, etc. that you have used, cited or quoted in order to complete this assessment. |  |  |

| Module Learning Outcomes (From module syllabus) |  |
| --- | --- |
| LO1. Determine the behavioural characteristics of materials subjected to complex loading systems. |

Pressure vessels complex loading

HND MECHANICAL ENGINEERING 
6/3/2020

# Matthew Permintella

Task 2 total 4175

Task 2

Problem solving

Design 1

Fig (1)

You are given a cylindrical pressure vessel with an internal pressure of 620KPa, an inner diameter of 560mm and a wall thickness of 6.4mm

Step 1 determine if the cylinder is thin or thick walled:

rt ≥10 thin walled                   rt ≤10 (thick walled)

560mm6.4mm= 87.5    (Vessel is thin walled)

- Determine the stresses at point A.

δhoop= p*rt = 620*103*2806.4=27.125*106 Pa

δhoop= 27.125MPa

δlongitudinal= p*r2*t = 620*103*2802*6.4=13.563*106 Pa

δlongitudinal = 13.563 MP

- Draw the stresses on a stress element.

Fig (2) shows stress element for point (a)

Design 2

- Given a spherical vessel with the same dimensions as those in figure 1 above, that is: an inner diameter of 560mm and a wall thickness of 6.4mm, determine the maximum internal pressure that the spherical vessel can withstand.

determine if the cylinder is thin or thick walled:

rt ≥10 thin walled                   rt ≤10 (thick walled)

280mm6.4mm= 43.75    (Vessel is thin walled)

δcircumference= p*r2t

13.563 MPa=p(280)2*6.4
p=27.125*106*(2*6.4)280 = 1.24 *106Pa

1.24MPa

- Emmerging techs company needs to replace one of their pressure vessels. A new vessel is to be designed with an inner diameter of 1.2m and thickness 12mm. determine the maximum internal pressure it can sustain so that neither its circumferential nor its longitudinal stress component exceeds 140MPa.

determine if the cylinder is thin or thick walled:

rt ≥10 thin walled                   rt ≤10 (thick walled)

600mm12mm= 50    (Vessel is thin walled)

Diameter = 1200

1200/2= 600mm

r = 600mm

δcircumference= p*rt  
140*106=p60012

Solve for p

p=140*106*12600 = 2.8 *106Pa

2.8MPa

δlongitudinal= 0.5(140*106) =70*106 Pa

70MPa

- From the results you obtained in the above questions for the cylindrical and spherical vessels which design would you recommend and why?

Cylindrical has a Lower operation pressure and generally easier to produce non-spherical vessel, as these are very complex and expensive to make.  Unless the higher operating pressure are needed for a specific purpose.

Task 3

Problem solving

Fig (3)

You have been given a barrel with an internal diameter of 610mm and a required to fit an A-36 ring with an internal diameter of 609.5mm.

Convert ℉ > ℃

65℉=18.33℃        ℉-32* 59= ℃

Determine if the cylinder is thin or thick walled:

rt ≥10 thin walled                   rt ≤10 (thick walled)

610mm6.35mm= 96.06    (Vessel is thin walled)

Wall thickness, t = 6.35mm

- If the temperature outside is 65 degrees F, determine how hot the ring would have to be in order for it to slip over the 610mm diameter barrel?

C1=barrel circumference (D*π)
 610* π = 1.9163 * 103mm

C2=ring circumference (D*π)
 609.5* π = 1.9148 * 103m

The amount of stretch needed = 1.5*10^-3 ( σ ) (1.5mm)

σ= ∝∆TL

σ=stress
∝ =  co-effienct of thermal expansion
∆T=Chance in temperture
L=Lenght

δ= α(Tf-Ti)(D*π)

σ=stress
∝ =  co-efficient of thermal expansion
Ti=temperature initial
Tf=temperature final
Db=Diamenter of barrel
π=Pi

1.5*103=11.7*10-6*(Tf-18.33)(610*π)

Solve for Tf:

1.511.7*10-6*610*π+18.33=85.22℃

Tf- Ti=Temperature needed to strench ring

85.22-18.33=66.89℃

- Find the stress in the ring and the pressure that it exerts on the barrel once it has cooled off?

Find young modulus for steel A-36 = 200GPa

ϵ=200 GPa Steel A-36(Bird, J., 2015).

P*LA* ϵ= ∝∆T*L = (elastic equation due to thermal expansion)

P = Pressure

L = Length

A = Cross-sectional area

ϵ=Young's Modulus

α=Co-effienct of thermal expansion

∆T=Change in temperture

Solve for A:

A=25.4*10-3*6.4*10-3
A=0.1625*10-3mm³
A=0.163*10-3mm³ (2 d.p)

Using:

P*LA* ϵ= ∝∆T*L

L cancel out so it becomes (P =A*ϵ* ∝∆T)

P = 0.163*10-3*200*109*11.7*10-6*66.89
P =25513.183Pa
P = 25.513 KPa

Using :

δ1= PA

δ1=25.51*1030.163*10-3=156.503*106

156.50MPa

δ1=p*r (12 Diameter of barrel)t

t = 6.35mm wall thickness
156.50*106= p*3056.35

Solve for P= pressure

p= 156.50*106*6.35305

p=3.258*106
p=3.26 MPa (2 d.p)

Task 4

Given a sphere made of AISI 501 OQT1100 with the following dimensions:

D˳=500mm
t=40mm
N design factor=4

- Determine the maximum stress that the vessel can withstand?

Step 1 determine if the cylinder is thin or thick walled:

rt ≥10 thin walled                   rt ≤10 (thick walled)

250mm40mm= 6.25    (Vessel is thick walled)

Step 2 Using Lamis equation (find maximum pressure) (2):

δ1max= P(ro3+2ri3)2(ro3+2ri3)

Finding yield strength of AISI 501 OQT1100 = 931*106  Pa (931MPa) δy

(Bird, J., 2015).

δy4= P(ro3+2ri3)2(ro3-2ri3)

Determine ro & ri :

ro= Do2 =250mm
ri= ro-t=210mm

ri=Inner radius

ro=Outer radius

Solve for P:

δy* 2(ro3-2ri³)4ro3+2ri3= P  
P = Internal pressure can be Ve+ o0r Ve-

∴  931* 106* 2(250o³+210i³)4250o3+2(210i3= P

P=  86.75*106 Pa
P (stress)=86.75 MPa

- What is the failure criterion for that type of vessel mentioned above?

Using  δy (yield strenght)N (safety factor)

δy= 931 MPa4=232.75 MPa

Bibliography

There were three books that I used for my research:

Bird, J., 2015. Science for engineering. 5th ed. 2 Park Square, Milton Park, Abingdon, Oxon OX14 4RN: Routledge.

Groover, M, P., 2013. Principles of modern manufacturing. 5th ed. 111 River Street, Hoboken, NJ, 07030: John Wiley & Sons

Callister, D, W., 2010. Materials Science and Engineering. 8th ed. 111 River Street, Hoboken, NJ, 07030: John Wiley & Sons