Faculty of Architecture, Computing and Engineering

School of Applied Computing

HN/BSc (Hons) Computer Networks

Network Principles (M4X01126)

2015-2016

Lab based Practical Exam

Student Name	: ______________________

Student ID		: ______________________

Date			: ______________________

This is an individual practical lab-test, held under examination conditions. You will be assigned a Router, a Switch and two PCs; you should configure these devices based on the following specifications.

This paper is marked out of 100.

You will have 2 hours to complete this task.

TOPOLOGY DIAGRAM

IP addressing scheme

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 172.16.10.1 | 255.255.255.0 | N/A |
|  | G0/1 | 172.16.11.1 | 255.255.255.0 | N/A |
| S1 | VLAN 1 | 172.16.11.10 | 255.255.255.0 | 172.16.11.1 |
| PC-A | NIC | 172.16.11.254 | 255.255.255.0 | 172.16.11.1 |
| PC-B | NIC | 172.16.10.254 | 255.255.255.0 | 172.16.10.1 |

Task 1: Network Connection (15 Marks)

Physically cable the network as shown in the topology diagram.

Task 2: Configure the Router (25 Marks)

      - Assign a device name R1 to the router.

      - Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

      - Assign class as the privileged EXEC encrypted password.

      - Assign cisco as the console password and enable login.

      - Assign cisco as the VTY password and enable login.

      - Encrypt the clear text passwords.

      - Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

      - Configure and activate both interfaces on the router.

      - Configure an interface description for each interface indicating which device is connected to it.

      - Save the running configuration to the startup configuration file.

Task 3: Configure the Switch (20 Marks)

      - Enter global configuration mode and assign the switch hostname.

      - Assign class as the privileged EXEC encrypted password.

      - Assign cisco as the console password and enable login.

      - Encrypt the clear text passwords.

Prevent unwanted Domain Name System (DNS) lookups.

Configure a login message-of-the-day (MOTD) banner.

Enter global configuration mode to set the SVI (VLAN 1) IP address to allow remote switch management.

Configure IP default gateway.

Configure the virtual terminal (VTY) line for the switch to allow Telnet access. If you do not configure a VTY password, you will not be able to Telnet to the switch.

Save the running configuration to the startup configuration file.

Task 4: Configure host PCs (20 marks)

After configuring each host computer, record the host network settings with the ipconfig /all command.

| Host PCA Network Configuration |  |
| --- | --- |
| Description |  |
| Physical Address |  |
| IP Address |  |
| Subnet Mask |  |
| Default Gateway |  |

| Host PCB Network Configuration |  |
| --- | --- |
| Description |  |
| Physical Address |  |
| IP Address |  |
| Subnet Mask |  |
| Default Gateway |  |

Task 5: Test Connectivity (10 Marks)

| Task | Result |
| --- | --- |
| PCA to PCB and vice versa |  |
| PCA to R1 |  |
| PCA to S1 |  |
| PCB to S1 |  |
| PCB to R1 |  |

Task 6: Answer all questions (10 Marks)

- What shortcut key is used to complete the remainder of a partially typed command or keyword? (2 marks)

_______________________________________________________________________________________

- Which access method allows you to access a device even if no networking services have been configured? (2 marks)

________________________________________________________________________________________

- What version of the Cisco IOS is used on the Router and Switch? (2 marks)

_________________________________________________________________________________________

- In addition to the ping command, what other Windows command is useful in displaying network delay and breaks in the path to the destination? (2 marks)

_________ ______________________________________________________________________________

- Write the command to set date and time on a Router? (2 marks)

- ______________________________________ _________________________________________________

Submission Requirement

All documentation for your solution should be submitted. Your name should be clearly printed on the documents, and signed.

At the end of the test a flash memory will be given to you, the configuration files for all routers should be copied to the media and handed in to your instructor.

The file name should be studentname_ID_devicename.txt

Show running-configuration

Show ip interface brief