| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 7 | LEFEL 7 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| MAY 2021 | MAI 2021 |

MODULE CODE: ACCA7029

COD Y MODWL: ACCA7029

ENTERPRISE CORE NETWORKING

TIME:  3 HOURS

Name			:

Student ID		:

Date & Time		:

This is an individual practical test, held under examination conditions. You will be assigned three Switches and three Routers; you should configure these devices based on the given specifications.

This paper is marked out of 100.

Topology

Addressing Table

| Device | Interface | IPv4 Address | IPv6 Address | IPv6 Link-Local |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 209.165.200.225/27 | 2001:db8:200::1/64 | fe80::1:1 |
| R1 | G0/0/1 | 10.0.10.1/24 | 2001:db8:100:1010::1/64 | fe80::1:2 |
| R1 | S0/1/0 | 10.0.13.1/24 | 2001:db8:100:1013::1/64 | fe80::1:3 |
| R2 | G0/0/0 | 209.165.200.226/27 | 2001:db8:200::2/64 | fe80::2:1 |
| R2 | Loopback0 | 2.2.2.2/32 | 2001:db8:2222::1/128 | fe80::2:3 |
| R3 | G0/0/1 | 10.0.11.1/24 | 2001:db8:100:1011::1/64 | fe80::3:2 |
| R3 | S0/1/0 | 10.0.13.3/24 | 2001:db8:100:1013::3/64 | fe80::3:3 |
| D1 | G1/0/11 | 10.0.10.2/24 | 2001:db8:100:1010::2/64 | fe80::d1:1 |
| D1 | VLAN 100 | 10.0.100.1/24 | 2001:db8:100:100::1/64 | fe80::d1:2 |
| D1 | VLAN 101 | 10.0.101.1/24 | 2001:db8:100:101::1/64 | fe80::d1:3 |
| D1 | VLAN 102 | 10.0.102.1/24 | 2001:db8:100:102::1/64 | fe80::d1:4 |
| D2 | G1/0/11 | 10.0.11.2/24 | 2001:db8:100:1011::2/64 | fe80::d2:1 |
| D2 | VLAN 100 | 10.0.100.2/24 | 2001:db8:100:100::2/64 | fe80::d2:2 |
| D2 | VLAN 101 | 10.0.101.2/24 | 2001:db8:100:101::2/64 | fe80::d2:3 |
| D2 | VLAN 102 | 10.0.102.2/24 | 2001:db8:100:102::2/64 | fe80::d2:4 |
| A1 | VLAN 100 | 10.0.100.3/23 | 2001:db8:100:100::3/64 | fe80::a1:1 |
| PC1 | NIC | 10.0.100.5/24 | 2001:db8:100:100::5/64 | EUI-64 |
| PC2 | NIC | DHCP | SLAAC | EUI-64 |
| PC3 | NIC | DHCP | SLAAC | EUI-64 |
| PC4 | NIC | 10.0.100.6/24 | 2001:db8:100:100::6/64 | EUI-64 |

Task 1: - Topology Creation and Basic Configuration (10 marks)

Using Packet Tracer software create the topology shown above and complete all tasks below.

- Configure hostnames on each device.

- Disable DNS lookup on all routers

- Enable ipv6 unicast routing

- Configure all devices with IP address shown in the above tables and network topology.

Task 2: - Device Configuration (80 marks)

- On the “Company Network” (i.e., R1, R3, D1, and D2), configure single-area OSPFv2 in area 0. Use OSPF Process ID 4.

  - Assign the following router-IDs. R1: 0.0.4.1, R3: 0.0.4.3, D1: 0.0.4.131, D2: 0.0.4.132

  - On R1, R3, D1, and D2, advertise all directly connected networks / VLANs in Area 0. On R1, do not advertise the R1 – R2 network.

  - On R1, propagate a default route. Note that the default route will be provided by BGP.

  - Disable OSPFv2 advertisements on: D1: All interfaces except G1/0/11. D2: All interfaces except G1/0/11.

- On the “Company Network” (i.e., R1, R3, D1, and D2), configure classic single-area OSPFv3 in area 0. Use OSPF Process ID 6

  - Assign the following router-IDs. R1: 0.0.6.1, R3: 0.0.6.3, D1: 0.0.6.131, D2: 0.0.6.132.

  - On R1, R3, D1, and D2, advertise all directly connected networks / VLANs in Area 0. On R1, do not advertise the R1 – R2 network.

  - On R1, propagate a default route. Note that the default route will be provided by BGP.

  - Disable OSPFv3 advertisements on: D1: All interfaces except G1/0/11. D2: All interfaces except G1/0/11.

- On R1 in the “ISP Network”, configure MP-BGP.

  - Configure R1 in BGP ASN 300 and use the router-id 1.1.1.1.

  - Configure an IPv4 and IPv6 neighbor relationship with R2 in ASN 500.

  - In IPv4 address family:

    - Disable the IPv6 neighbor relationship.

    - Enable the IPv4 neighbor relationship.

    - Advertise the 10.0.0.0/8 network.

  - In IPv6 address family

    - Disable the IPv4 neighbor relationship.

    - Enable the IPv6 neighbor relationship.

    - Advertise the 2001:db8:100::/48 network.

  - Configure two static summary routes to interface Null 0:

    - A summary IPv4 route for 10.0.0.0/8.

    - A summary IPv6 route for 2001:db8:100::/48.

- R2 in the “ISP Network”, configure MP-BGP.

  - Configure R2 in BGP ASN 500 and use the router-id 2.2.2.2.

  - Configure and enable an IPv4 and IPv6 neighbor relationship with R1 in ASN 300

  - In IPv4 address family:

    - The Loopback 0 IPv4 network (/32).

    - The default route 0.0.0.0/0.

  - In IPv6 address family

    - The Loopback 0 IPv0 network (/128).

    - The default route (::/0).

  - Configure two static summary routes to interface Loopback 0:

    - An IPv4 default static route.

    - An IPv6 default static route.

- On all switches configure VLANs as shown in the table:

| VLAN number | Name | Network |
| --- | --- | --- |
| 100 | Management | 10.0.100.0/24 |
| 101 | UserGroupA | 10.0.101.0/24 |
| 102 | UserGroupB | 10.0.102.0/24 |
| 999 | NATIVE |  |

- Set D1 and D2 as DHCP server for VLANs 101 and 102. Exclude 10.0.x.1 to 109 and 141 to 254 on both LANs.

- On all switches, configure IEEE 802.1Q trunk interfaces on interconnecting switch links.

- On all switches, change the Native VLAN on trunk links.

- On all switches, enable the Rapid Spanning-Tree Protocol.

- On D1 and D2, configure the appropriate RSTP root bridges based on the information in the topology diagram. D1 and D2 must provide backup in case of root bridge failure.

- On all switches, configure an LACP EtherChannel as shown in the topology.

- On D1, configure HSRP version 2.

- D1 is the primary router for VLANs 100 and 102; therefore, their priority will also be changed to 150.

  - Configure IPv4 HSRP group 104 for VLAN 100, assign virtual IP address 10.0.100.254, group priority 150 and enable preemption

  - Configure IPv4 HSRP group 114 for VLAN 101, assign virtual IP address 10.0.101.254 and enable preemption

  - Configure IPv4 HSRP group 124 for VLAN 102, assign virtual IP address 10.0.102.254, group priority 150 and enable preemption

  - Configure IPv6 HSRP group 106 for VLAN 100, assign virtual IP address ipv6 autoconfig, group priority 150 and enable preemption

  - Configure IPv6 HSRP group 116 for VLAN 101, assign virtual IP address ipv6 autoconfig and enable preemption

  - Configure IPv6 HSRP group 126 for VLAN 102, assign virtual IP address ipv6 autoconfig, group priority 150 and enable preemption

- On D2, configure HSRP version 2.

- D2 is the primary router for VLANs 101; therefore, their priority will also be changed to 150.

  - Configure IPv4 HSRP group 104 for VLAN 100, assign virtual IP address 10.0.100.254 and enable preemption

  - Configure IPv4 HSRP group 114 for VLAN 101, assign virtual IP address 10.0.101.254, group priority 150 and enable preemption

  - Configure IPv4 HSRP group 124 for VLAN 102, assign virtual IP address 10.0.102.254 and enable preemption

  - Configure IPv6 HSRP group 106 for VLAN 100, assign virtual IP address ipv6 autoconfig and enable preemption

  - Configure IPv6 HSRP group 116 for VLAN 101, assign virtual IP address ipv6 autoconfig, group priority 150 and enable preemption

  - Configure IPv6 HSRP group 126 for VLAN 102, assign virtual IP address ipv6 autoconfig and enable preemption

- On all devices, set the local clock to the current UTC time.

- Configure R2 as NTP master at stratum level 3.

- Configure NTP as follows: R1 must synchronise with R2. R3, D1, and A1 to synchronize time with R1. D2 to synchronize time with R3.

- On all switches, configure host access ports connecting to PC1, PC2, PC3, and PC4.

- Configure PortFast and BPDU Guard on the interfaces connected to PCs. Shutdown all unused ports.

- Enable port security on ports connected to end devices. Set the maximum so that only one device can access the Fast Ethernet ports.

- Secure the ports so that the MAC address of a device is dynamically learned and added to the running configuration.   Set the violation mode so that the Fast Ethernet ports are not disabled when a violation occurs, but a notification of the security violation is generated and packets from the unknown source are dropped.

Task3: Network connectivity

Using ping command, verify end-to-end connectivity:

From PC-A, ping PC-B & PC-C. Is it successful? ___yes/no_____

From PC-A, ping default gateway. Is it successful? ___yes/no_____

From PC-B, ping 172.27.0.1. Is it successful? ___yes/no_____

From PC-A, ping 209.165.200.225. Is it successful? ___yes/no_____

Verify HSRP is working as expected

Issue the shutdown command on R1 G0/1, and then re-issue the following commands to verify that HSRP is working as expected:

From PC-B, ping 172.27.0.1. Ping should not be successful ___yes/no___

From PC-B, ping default gateway. Is it successful? ___yes/no_____

From PC-B, ping 209.165.200.225. Is it successful? ___yes/no_____

Submission Requirement

The packet tracer file should be saved as “studentname_id” and uploaded onto Moodle. In addition to packet tracer file, you must also submit

- This document with Task 3 completed

All documentation for your solution should be submitted electronically on Moodle.

It is your responsibility that you submit all the outputs and files correctly.

Final Configuration:

Router R1

R1# show run

Building configuration...

Current configuration : 3406 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R1

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$0C3pnVdgrnhnY9$uzGA.WZfcLg5IhuyJu22mIf.YyZ/83VgqbO3rXBDuwo

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

ipv6 unicast-routing

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

username sadmin privilege 15 secret 9 $9$XCO4pzqbRT.3EP$ymouLOQI5/o0FOkYDtA1ztejFra67MnkJJ5Y3bhyQe6

!

redundancy

mode none

!

interface GigabitEthernet0/0/0

ip address 209.165.200.225 255.255.255.224

negotiation auto

ipv6 address FE80::1:1 link-local

ipv6 address 2001:DB8:200::1/64

!

interface GigabitEthernet0/0/1

ip address 10.0.10.1 255.255.255.0

negotiation auto

ipv6 address FE80::1:2 link-local

ipv6 address 2001:DB8:100:1010::1/64

ipv6 ospf 6 area 0

!

interface Serial0/1/0

ip address 10.0.13.1 255.255.255.0

ipv6 address FE80::1:3 link-local

ipv6 address 2001:DB8:100:1013::1/64

ipv6 ospf 6 area 0

!

interface Serial0/1/1

no ip address

!

router ospf 4

router-id 0.0.4.1

network 10.0.10.0 0.0.0.255 area 0

network 10.0.13.0 0.0.0.255 area 0

default-information originate

!

router bgp 300

bgp router-id 1.1.1.1

bgp log-neighbor-changes

neighbor 2001:DB8:200::2 remote-as 500

neighbor 209.165.200.226 remote-as 500

!

address-family ipv4

network 10.0.0.0

no neighbor 2001:DB8:200::2 activate

neighbor 209.165.200.226 activate

exit-address-family

!

address-family ipv6

network 2001:DB8:100::/48

neighbor 2001:DB8:200::2 activate

exit-address-family

!

ip forward-protocol nd

no ip http server

ip http secure-server

ip route 10.0.0.0 255.0.0.0 Null0

!

ip access-list standard SNMP-NMS

permit 10.0.100.5

logging trap warnings

logging host 10.0.100.5

ipv6 route 2001:DB8:100::/48 Null0

ipv6 router ospf 6

router-id 0.0.6.1

default-information originate

!

!

control-plane

!

banner motd ^C R1, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

!

ntp server 2.2.2.2

!

end

Router R2

R2# show run

Building configuration...

Current configuration : 2029 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R2

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$kWM5eeaWgcjgDk$klw0rmhA2j9zzPN13oTIYc/.yk9aczrrDxNq4rUNf5c

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

ipv6 unicast-routing

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

username sadmin privilege 15 secret 9 $9$xfCWZaD1xuZ5Q.$rje2SE7dafmrTg87ls/vn.PNtMXbaL3kfmN3Jr08yNU

!

redundancy

mode none

!

interface Loopback0

ip address 2.2.2.2 255.255.255.255

ipv6 address FE80::2:3 link-local

ipv6 address 2001:DB8:2222::1/128

!

interface GigabitEthernet0/0/0

ip address 209.165.200.226 255.255.255.224

negotiation auto

ipv6 address FE80::2:1 link-local

ipv6 address 2001:DB8:200::2/64

!

interface GigabitEthernet0/0/1

no ip address

negotiation auto

!

router bgp 500

bgp router-id 2.2.2.2

bgp log-neighbor-changes

neighbor 2001:DB8:200::1 remote-as 300

neighbor 209.165.200.225 remote-as 300

!

address-family ipv4

network 0.0.0.0

network 2.2.2.2 mask 255.255.255.255

no neighbor 2001:DB8:200::1 activate

neighbor 209.165.200.225 activate

exit-address-family

!

address-family ipv6

network ::/0

network 2001:DB8:2222::/128

neighbor 2001:DB8:200::1 activate

exit-address-family

!

ip forward-protocol nd

no ip http server

ip http secure-server

ip route 0.0.0.0 0.0.0.0 Loopback0

!

ipv6 route ::/0 Loopback0

!

control-plane

!

banner motd ^C R2, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

!

ntp master 3

!

end

Router R3

R3# show run

Building configuration...

Current configuration : 2765 bytes

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

no platform punt-keepalive disable-kernel-core

!

hostname R3

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$X1WR7NQHvbYXHY$HevkjyeTexlsUxwhnwaZWeh/VEB3CIoGxlPSJ9O.F6o

!

no ip domain lookup

!

login on-success log

!

subscriber templating

!

ipv6 unicast-routing

multilink bundle-name authenticated

!

spanning-tree extend system-id

!

username sadmin privilege 15 secret 9 $9$y02cJ/kvRKO7DI$eYITN996n5QFlG2zu7OoHu2RLPwbw/8v8lO4nv/n8Aw

!

redundancy

mode none

!

interface GigabitEthernet0/0/0

no ip address

negotiation auto

!

interface GigabitEthernet0/0/1

ip address 10.0.11.1 255.255.255.0

negotiation auto

ipv6 address FE80::3:2 link-local

ipv6 address 2001:DB8:100:1011::1/64

ipv6 ospf 6 area 0

!

interface Serial0/1/0

ip address 10.0.13.3 255.255.255.0

ipv6 address FE80::3:3 link-local

ipv6 address 2001:DB8:100:1010::2/64

ipv6 ospf 6 area 0

!

interface Serial0/1/1

no ip address

!

router ospf 4

router-id 0.0.4.3

network 10.0.11.0 0.0.0.255 area 0

network 10.0.13.0 0.0.0.255 area 0

!

ip forward-protocol nd

no ip http server

ip http secure-server

!

ip access-list standard SNMP-NMS

permit 10.0.100.5

logging trap warnings

logging host 10.0.100.5

ipv6 router ospf 6

router-id 0.0.6.3

!

control-plane

!

banner motd ^C R3, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

!

ntp server 10.0.10.1

!

end

Switch D1

D1# show run

Building configuration...

Current configuration : 8260 bytes

!

version 16.9

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

!

hostname D1

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

enable secret 9 $9$RWOFeoZQQ/zqJk$rEnKpZ9Dx6asfA/16o3cPHR3hYQvn2gFiZuybdaFo82

!

aaa new-model

!

aaa authentication login default group radius local

!

aaa session-id common

switch 1 provision ws-c3650-24ps

!

ip routing

!

no ip domain lookup

ip dhcp excluded-address 10.0.101.1 10.0.101.109

ip dhcp excluded-address 10.0.101.141 10.0.101.254

ip dhcp excluded-address 10.0.102.1 10.0.102.109

ip dhcp excluded-address 10.0.102.141 10.0.102.254

!

ip dhcp pool VLAN-101

network 10.0.101.0 255.255.255.0

default-router 10.0.101.254

!

ip dhcp pool VLAN-102

network 10.0.102.0 255.255.255.0

default-router 10.0.102.254

!

login on-success log

ipv6 unicast-routing

!

license boot level ipservicesk9

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

spanning-tree vlan 100,102 priority 24576

spanning-tree vlan 101 priority 28672

!

username sadmin privilege 15 secret 9 $9$yBNV4PYk3Zdpak$N2uvIju4cfG5jQsynRkIv0EHas6ivCZRAtkztAnLiVo

!

redundancy

mode sso

!

transceiver type all

!

!

interface Port-channel1

switchport trunk native vlan 999

switchport mode trunk

!

interface Port-channel12

switchport trunk native vlan 999

switchport mode trunk

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

negotiation auto

!

interface GigabitEthernet1/0/1

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/2

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/3

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/4

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/5

switchport trunk native vlan 999

switchport mode trunk

channel-group 1 mode active

!

interface GigabitEthernet1/0/6

switchport trunk native vlan 999

switchport mode trunk

channel-group 1 mode active

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

no switchport

ip address 10.0.10.2 255.255.255.0

ipv6 address FE80::D1:1 link-local

ipv6 address 2001:DB8:100:1010::2/64

ipv6 ospf 6 area 0

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

switchport access vlan 100

switchport mode access

spanning-tree portfast

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/1/1

shutdown

!

interface GigabitEthernet1/1/2

shutdown

!

interface GigabitEthernet1/1/3

shutdown

!

interface GigabitEthernet1/1/4

shutdown

!

interface Vlan1

no ip address

!

interface Vlan100

ip address 10.0.100.1 255.255.255.0

standby version 2

standby 104 ip 10.0.100.254

standby 104 priority 150

standby 104 preempt

standby 104 track 4 decrement 60

standby 106 ipv6 autoconfig

standby 106 priority 150

standby 106 preempt

standby 106 track 6 decrement 60

ipv6 address FE80::D1:2 link-local

ipv6 address 2001:DB8:100:100::1/64

ipv6 ospf 6 area 0

!

interface Vlan101

ip address 10.0.101.1 255.255.255.0

standby version 2

standby 114 ip 10.0.101.254

standby 114 preempt

standby 114 track 4 decrement 60

standby 116 ipv6 autoconfig

standby 116 preempt

standby 116 track 6 decrement 60

ipv6 address FE80::D1:3 link-local

ipv6 address 2001:DB8:100:101::1/64

ipv6 ospf 6 area 0

!

interface Vlan102

ip address 10.0.102.1 255.255.255.0

standby version 2

standby 124 ip 10.0.102.254

standby 124 priority 150

standby 124 preempt

standby 124 track 4 decrement 60

standby 126 ipv6 autoconfig

standby 126 priority 150

standby 126 preempt

standby 126 track 6 decrement 60

ipv6 address FE80::D1:4 link-local

ipv6 address 2001:DB8:100:102::1/64

ipv6 ospf 6 area 0

!

router ospf 4

router-id 0.0.4.131

passive-interface default

no passive-interface GigabitEthernet1/0/11

network 10.0.10.0 0.0.0.255 area 0

network 10.0.100.0 0.0.0.255 area 0

network 10.0.101.0 0.0.0.255 area 0

network 10.0.102.0 0.0.0.255 area 0

!

ip forward-protocol nd

ip http server

ip http secure-server

!

ip access-list standard SNMP-NMS

permit 10.0.100.5

!

ipv6 router ospf 6

router-id 0.0.6.131

passive-interface default

no passive-interface GigabitEthernet1/0/11

!

!

banner motd ^C D1, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

stopbits 1

line vty 5 15

!

ntp server 10.0.10.1

!

end

Switch D2

D2# show run

Building configuration...

Current configuration : 8208 bytes

!

version 16.9

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no platform punt-keepalive disable-kernel-core

!

hostname D2

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

enable secret 9 $9$CQubYNwHPhsPpE$QWfTfAlfzmWD3ELHkcFNzlDlp24FkpjLnGBRMPbUNow

!

aaa new-model

!

aaa authentication login default group radius local

!

aaa session-id common

switch 1 provision ws-c3650-24ps

!

ip routing

!

no ip domain lookup

ip dhcp excluded-address 10.0.101.1 10.0.101.209

ip dhcp excluded-address 10.0.101.241 10.0.101.254

ip dhcp excluded-address 10.0.102.1 10.0.102.209

ip dhcp excluded-address 10.0.102.241 10.0.102.254

!

ip dhcp pool VLAN-101

network 10.0.101.0 255.255.255.0

default-router 10.0.101.254

!

ip dhcp pool VLAN-102

network 10.0.102.0 255.255.255.0

default-router 10.0.102.254

!

login on-success log

ipv6 unicast-routing

!

license boot level ipservicesk9

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

spanning-tree vlan 100,102 priority 28672

spanning-tree vlan 101 priority 24576

!

username sadmin privilege 15 secret 9 $9$0bnG9yhbASQv9k$geQoMT2qxu1ItBXC5pl/SOR2YeWhqDOW0lsMIsicQDw

!

interface Port-channel2

switchport trunk native vlan 999

switchport mode trunk

!

interface Port-channel12

switchport trunk native vlan 999

switchport mode trunk

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

negotiation auto

!

interface GigabitEthernet1/0/1

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/2

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/3

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/4

switchport trunk native vlan 999

switchport mode trunk

channel-group 12 mode active

!

interface GigabitEthernet1/0/5

switchport trunk native vlan 999

switchport mode trunk

channel-group 2 mode active

!

interface GigabitEthernet1/0/6

switchport trunk native vlan 999

switchport mode trunk

channel-group 2 mode active

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

no switchport

ip address 10.0.11.2 255.255.255.0

ipv6 address FE80::D1:1 link-local

ipv6 address 2001:DB8:100:1011::2/64

ipv6 ospf 6 area 0

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

switchport access vlan 102

switchport mode access

spanning-tree portfast

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/1/1

shutdown

!

interface GigabitEthernet1/1/2

shutdown

!

interface GigabitEthernet1/1/3

shutdown

!

interface GigabitEthernet1/1/4

shutdown

!

interface Vlan1

no ip address

!

interface Vlan100

ip address 10.0.100.2 255.255.255.0

standby version 2

standby 104 ip 10.0.100.254

standby 104 preempt

standby 104 track 4 decrement 60

standby 106 ipv6 autoconfig

standby 106 preempt

standby 106 track 6 decrement 60

ipv6 address FE80::D2:2 link-local

ipv6 address 2001:DB8:100:100::2/64

ipv6 ospf 6 area 0

!

interface Vlan101

ip address 10.0.101.2 255.255.255.0

standby version 2

standby 114 ip 10.0.101.254

standby 114 priority 150

standby 114 preempt

standby 114 track 4 decrement 60

standby 116 ipv6 autoconfig

standby 116 priority 150

standby 116 preempt

standby 116 track 6 decrement 60

ipv6 address FE80::D2:3 link-local

ipv6 address 2001:DB8:100:101::2/64

ipv6 ospf 6 area 0

!

interface Vlan102

ip address 10.0.102.2 255.255.255.0

standby version 2

standby 124 ip 10.0.102.254

standby 124 preempt

standby 124 track 4 decrement 60

standby 126 ipv6 autoconfig

standby 126 preempt

standby 126 track 6 decrement 60

ipv6 address FE80::D2:4 link-local

ipv6 address 2001:DB8:100:102::2/64

ipv6 ospf 6 area 0

!

router ospf 4

router-id 0.0.4.132

passive-interface default

no passive-interface GigabitEthernet1/0/11

network 10.0.11.0 0.0.0.255 area 0

network 10.0.100.0 0.0.0.255 area 0

network 10.0.101.0 0.0.0.255 area 0

network 10.0.102.0 0.0.0.255 area 0

!

ip forward-protocol nd

ip http server

ip http secure-server

!

ip access-list standard SNMP-NMS

permit 10.0.100.5

!

ipv6 router ospf 6

router-id 0.0.6.132

passive-interface default

no passive-interface GigabitEthernet1/0/11

!

banner motd ^C D2, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

stopbits 1

line aux 0

stopbits 1

line vty 5 15

!

ntp server 10.0.10.1

!

end

Switch A1

A1# show run

Building configuration...

Current configuration : 3102 bytes

!

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname A1

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$W4yJyY0jfUFGt3$hgWzRhouqq81DGKiSw3oN3ICGIRFKI1TF9C4Qo2BoGk

!

username sadmin privilege 15 secret 9 $9$rlz/oiC6xETwLL$4MFl7ezehKgosutkpnwabhdf83xQOcDXYyW.dvyoneY

aaa new-model

!

no ip domain-lookup

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

interface Port-channel1

switchport trunk native vlan 999

switchport mode trunk

!

interface Port-channel2

switchport trunk native vlan 999

switchport mode trunk

!

interface FastEthernet0/1

switchport trunk native vlan 999

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/2

switchport trunk native vlan 999

switchport mode trunk

channel-group 1 mode active

!

interface FastEthernet0/3

switchport trunk native vlan 999

switchport mode trunk

channel-group 2 mode active

!

interface FastEthernet0/4

switchport trunk native vlan 999

switchport mode trunk

channel-group 2 mode active

!

interface FastEthernet0/5

shutdown

!

interface FastEthernet0/6

shutdown

!

interface FastEthernet0/7

shutdown

!

interface FastEthernet0/8

shutdown

!

interface FastEthernet0/9

shutdown

!

interface FastEthernet0/10

shutdown

!

interface FastEthernet0/11

shutdown

!

interface FastEthernet0/12

shutdown

!

interface FastEthernet0/13

shutdown

!

interface FastEthernet0/14

shutdown

!

interface FastEthernet0/15

shutdown

!

interface FastEthernet0/16

shutdown

!

interface FastEthernet0/17

shutdown

!

interface FastEthernet0/18

shutdown

!

interface FastEthernet0/19

shutdown

!

interface FastEthernet0/20

shutdown

!

interface FastEthernet0/21

shutdown

!

interface FastEthernet0/22

shutdown

!

interface FastEthernet0/23

switchport access vlan 101

switchport mode access

spanning-tree portfast edge

!

interface FastEthernet0/24

switchport access vlan 100

switchport mode access

spanning-tree portfast edge

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Vlan1

no ip address

shutdown

!

interface Vlan100

ip address 10.0.100.3 255.255.255.0

ipv6 address FE80::A1:1 link-local

ipv6 address 2001:DB8:100:100::3/64

!

ip default-gateway 10.0.100.254

ip http server

ip http secure-server

!

ip access-list standard SNMP-NMS

permit 10.0.100.5

!

logging trap warnings

logging host 10.0.100.5

!

!

banner motd ^C A1, ENCOR Skills Assessment, Scenario 1 ^C

!

line con 0

exec-timeout 0 0

logging synchronous

line vty 5 15

!

ntp server 10.0.10.1

end