#include <main.h>

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#include <lcd.c>

// define function/subroutines

void analog(void);

void SW_A(void);

void SW_B(void);

void SW_C(void);

void SW_D(void);

// declare global variables

unsigned int adc;

unsigned int switches;

int disp = 0;

unsigned int last_button_state;

unsigned int lightsA[16]={128,64,32,16,8,4,2,1,1,2,4,8,16,32,64,128};

//start of the main loop

void main()

{

setup_adc_ports(sAN0);							// initialise adc channel

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);	// setup adc operation

lcd_init();										// initialise the LCD

while(TRUE)

{

analog();					//call read the analog function

switches=input_a();			//read value of port A

switches=switches & 0x3c;	//mask of unwanted bits 00111100

switches/=2;				//dived by 2

switch(switches)

{

case 2: 				//switch in first position

SW_A();					//call function SW_A

//SW_B();

disp=0;					//reset disp variable

break;

case 4:					//switch in secound position

SW_B();					//call function SW_B

disp=0;					//reset disp variable

break;

case 8:					//switch in third position

SW_C();					//call function SW_C

disp=0;					//reset disp variable

break;

case 16:				//switch in fourth position

disp = disp+1; 			//add 1 to the disp variable

SW_D();					//call function SW_D

break;

default:  break;

}

}

}

//read the adc

void analog(void)

{

set_adc_channel(0);		// read adc channel 0

delay_us(30);			// wait 30uS

adc = read_adc();		// store value in variable ADC

}

void SW_A(void)

{

output_b(adc & 0xFF);

printf("ADC= %3u \r\n", adc);

//delay_ms(100);

printf(lcd_putc,"\f"); 				//clear the LCD display

lcd_gotoxy(7,1);					//goto line 1 position 7 on the LCD display

printf(lcd_putc,"ADC= %3u", adc);	//print the ADC value to the LCD display

if (adc <= 75)

{

output_low(PIN_C0);							//set pin C0 low

output_high(PIN_C1);						//set pin C1 high

lcd_gotoxy(2,2);							//got to line 2 position 2

printf(lcd_putc,"Spin Anticlockwise");		//print "Spin Anticlockwise" to the LCD display

delay_ms(100);								//100ms delay

}

if (adc >= 175)

{

output_low(PIN_C1);						//set pin C1 low

output_high(PIN_C0);					//set pin C0 high

lcd_gotoxy(4,2);						//go to line 2 position 4

printf(lcd_putc,"Spin Clockwise");		//print "spin clockwise" to the LCD display

delay_ms(100);							//100ms delay

}

if((adc >=76) & (adc <= 174))

{

output_low(PIN_C0);					//set pin C0 low

output_low(PIN_C1);					//set pin C1 low

lcd_gotoxy(6,2); 					//centre on line 2

printf(lcd_putc,"Don't Spin"); 		//print "Don't Spin" to the LCD display

delay_ms(100);						//100ms delay

}

//used to stop motors when the switch changes positions

output_low(PIN_C0);						//set pin C0 low

output_low(PIN_C1);						//set pin C1 low

printf(lcd_putc,"\f");					//clear the LCD display

}

void SW_B(void)

{

output_b(adc & 0xFF);  				//displays adc in bitwise on the 8 leds

lcd_gotoxy(7,1); 					//go to line 1 postion 7 on the LCD display

printf(lcd_putc,"ADC= %3u", adc); 	//print the ADC value to the LCD display

printf("ADC= %3u \r\n", adc);		//print the ADC value to the virtual terminal

delay_ms(500);						//500ms delay

printf(lcd_putc,"\f");				//clear the LCD display

output_b(0x00);						//set all leds to off

}

void SW_C(void)

{

int current_button_state = input(pin_a1);		//read the curret button state

if(current_button_state != last_button_state)	//cheks if the current button state is not the same as its previous state

{

if (current_button_state)

{

printf("Button pressed \r\n");			//print to the virutal terminal

output_low(PIN_C1);						//set pin C1 low for clockwise motor movement

output_high(PIN_C0);					//set pin C0 high for clockwise motor movement

lcd_gotoxy(24,1);						//go to center of line 3 on the LCD display

printf(lcd_putc,"Button pressed ");		//print to the LCD display

}

else

{

printf("Button released \r\n");			//print to the virtual terminal

output_low(PIN_C1);						//set pin C1 low to turn the motor off

output_low(PIN_C0);						//set pin C1 low to turn the motor off

lcd_gotoxy(24,1);						//go to center of line 3 on the LCD display

printf(lcd_putc,"Button released ");	//print to the LCD display

}

last_button_state = current_button_state;	//update last button state

}

}

//kinght rider led function

void SW_D()

{

printf(lcd_putc,"\f"); 						//clear the LCD display

lcd_gotoxy(27,2);							//got to center of line 4 on the LCD display

printf(lcd_putc,"Kitt mode");				//print to LCD display

if(disp == 1) printf("Kitt mode \r\n");		//if disp is 1 print to virtual termial

unsigned int q; 							//loop counter variable

//sequence for knight rider led's

for(q=0;q<16;q++)

{

output_b(lightsA[q]);	//turn the speficed led on

delay_ms(200);			//delay of 200ms

}

output_b(0x00);				//set all leds to off

printf(lcd_putc,"\f");		//clear the LCD display

}

Comments

2.. 20   3.. 30    4.. 40   5.. 10

****************************************

2.. Your code has a number of structures and all work. Your statements indicate what each line does 75%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented. 75%