Faculty of Architecture, Computing and Engineering/ Cyfadran Pensaernïaeth, Cyfrifiadura a Pheirianneg

School of Applied Computing/Ysgol Cyfrifiadureg Gymhwysol

Local Area Networks

BSc Computer Networks (M2X8625)

Practical Lab Assessment

Year 2/Level 5

May 2015

Name			:

Student ID		:

Date & Time		:

This is an individual practical lab-test, held under examination conditions. You will be assigned three Switches and a Router; you should configure these devices based on the given specifications. Router ISP is preconfigured.

This paper is marked out of 100.

You will have 2 hours to complete this task.

## TOPOLOGY DIAGRAM

VLAN 10 – Students: 172.16.10.0/24		VLAN 20 – Staff: 172.16.20.0/24

VLAN 30 – Guest: 172.16.30.0/24			VLAN 99 – Management: 172.16.99.0/24

## Addressing Table (9 marks)

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| ISP | S0/0/0 | 209.165.24.225 | 255.255.255.252 | N/A |
|  | Lo0 | 199.199.199.1 | 255.255.255.0 | N/A |
| R1 | S0/0/0 |  |  |  |
|  | Ge0/0 |  |  |  |
|  | Ge0/0.10 |  |  |  |
|  | Ge0/0.20 |  |  |  |
|  | Ge0/0.30 |  |  |  |
|  | Ge0/0.99 |  |  |  |
| DS1 | VLAN99 |  |  |  |
| AS1 | VLAN99 |  |  |  |
| AS2 | VLAN99 |  |  |  |
| PC1 | NIC |  |  |  |
| PC2 | NIC |  |  |  |
| PC3 | NIC |  |  |  |

## Port Assignments

## DS1

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/1 – 0/5 | 802.1q Trunks (Native VLAN 99) | 172.16.99.0 |

## AS1

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/1 – 0/4 | 802.1q Trunks (Native VLAN 99) | 172.16.99.0 |
| Fa0/5 – 0/10 | VLAN 10 (Students) | 172.16.10.0 |
| Fa0/11 – 0/18 | VLAN 20 (Staff) | 172.16.20.0 |
| FA0/19 – 0/24 | VLAN 30 (Guest) | 172.16.30.0 |

## AS2

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/1 – 0/4 | 802.1q Trunks (Native VLAN 99) | 172.16.99.0 |
| Fa0/5 – 0/10 | VLAN 10 (Students) | 172.16.10.0 |
| Fa0/11 – 0/18 | VLAN 20 (Staff) | 172.16.20.0 |
| FA0/19 – 0/24 | VLAN 30 (Guest) | 172.16.30.0 |

Task 1: Prepare the Network (10 marks)

Step 1: Cable a network that is similar to the one in the topology diagram.

Step 2: Clear any existing configurations on the devices.

Before proceeding, have your cabling verified by the examiner.    Instructor Sign-off: ______________

Task 2: Perform Basic Device Configurations (15 marks)

Configure the DS1, AS1, and AS2 switches and R1 router according to the following guidelines:

- Configure the hostname.

- Disable DNS lookup.

- Configure an EXEC mode password.

- Configure a message-of-the-day banner.

- Configure a password for console connections.

- Configure synchronous logging.

- Configure a password for vty connections.

- Enable service password encryption.

Task 3: Router and Switch configuration instructions (60 marks)

- Configure DS1 as a VTP server, and AS1 & AS2 as VTP clients in the mh505.com domain. Set the VTP password to ‘cisco’.

- Configure EtherChannel between the switches as per the topology diagram.

- Create VLANs 10, 20 & 30, and name as required. Populate the VLANs with the appropriate port as per the diagram.

- Assign ports to appropriate VLANs as shown in the Port Assignments table.

- Configure RSTP PVST+ on all switches.

- Configure DS1 to be the primary root for all VLANs and AS2 to be secondary.

- Configure PortFast and BPDU Guard on the interface connected to PC1, PC2, and PC3.

- Configure DS1 as a DHCP server to provide IP addresses and default gateways to users on VLAN 10, 20 & 30. Verify all PCs receive IP address details.

- Configure management IP addresses for all network switches in VLAN 99, and configure them for Telnet access.

- Configure SSH access on DS1. Configure port security on AS1 and AS2.

- Configure router-on-a-stick inter-VLAN routing.

- Configure subinterfaces on R1 to operate on VLAN 10, 20, 30 and 99 with the address from the Address table.

- Configure static routing on R1 and ISP.

- Configure dynamic NAT on R1.

Task 4: After configuring each host computer, record the host network settings with the ipconfig /all command.  (6 marks)

| Host PC1 Network Configuration |  |
| --- | --- |
| Description |  |
| Physical Address |  |
| IP Address |  |
| Subnet Mask |  |
| Default Gateway |  |

| Host PC2 Network Configuration |  |
| --- | --- |
| Description |  |
| Physical Address |  |
| IP Address |  |
| Subnet Mask |  |
| Default Gateway |  |

| Host PC3 Network Configuration |  |
| --- | --- |
| Description |  |
| Physical Address |  |
| IP Address |  |
| Subnet Mask |  |
| Default Gateway |  |

Task 5: Document the Configurations

All documentation for your solution should be submitted. Your name should be clearly printed on the documents.

At the end of the test a flash memory will be given to you, the configuration files for all routers should be copied to the media and handed in to your instructor.  It is you responsibility that you copy the files correctly.

On each device, issue the following commands and capture the configurations. Save the file as devicename_surname_Pno.txt.

show run

sh ip int brief

sh ip route

sh nat statistics

sh vlan

sh int trunk

sh spanning-tree

sh etherchannel summary

sh vtp status