Vehicle  Dynamics Motorsport Vehicle Dynamics

Good practice in Assignment 1

- Included allowance for lateral acceleration when rear braking.

- Discusses average vs peak deceleration decision.

- Demonstrated method of extracting deceleration from data .

- Found wheel displacement with zeroed pots or difference  to static.

- Correctly determined wheel load from wheel displacement.

- Demonstrated that  4 wheel braking µ is simply a/g

- Created summary table / graph of pressure to grip  relationship.

- Included summary results page to clarify all findings.

- Discussed characteristics of braking deceleration profile.

- Included screen dumps of all data sets.

- Averaged results for each braking event.

- Discussed change of deceleration with each subsequent braking  event in terms of increasing temp.

- Showed all calcs used for WT.

- Plotted  deceleration vs braking events for different pressure – compares like with like as test progresses.

- Wheel deflection better at latter part of each event when steady state has been achieved

- Related  Pi dataset number to test.