ABAQUS Exercise:

- Execute ABAQUS CAE

- Run with standard/explicit solver

- Set Working Folder:

- In PART module: Create a simple cantilever with rectangular cross section :

- Remember: Hold down Ctrl and Alt then Left Mouse button is Rotate and Middle Button is Pan. Middle Wheel is zoom at any time.

- Create a rectangular cross section:

- Draw rectangle then DONE DONE  (Middle button is DONE ). Then enter extrusion length of  1m

- In PROPERTY module create a new material for Steel. You need density, Youngs modulus and Poisson’s ratio. (Don’t click on OK until all values entered.)Eg to  enter E and ν:

- Create a steel section:

- Assign this steel section to  the part on the screen: Follow instructions at bottom of window

In ASSEMBLY Module: create an instance of the part :

- In the STEP Module create a step that defines the solution type:

- In the LOAD Module: create a surface traction of 8000N applied perpendicular to  the top surface of the cantilever:

- Make sure you select general traction as shown below. Enter 8000 as magnitude and select two  points to  define perpendicular direction.

Now clamp one end face with an ENCASTER Boundary Condition:

In the MESH Module:

SEED  the Part with a  global mesh size:

ASSIGN MESH CONTROLS :

ASSIGN ELEMENT TYPE:

MESH the PART:

- In JOB module: create a new JOB:

- Select JOB Manager to  SUBMIT job and then MONITOR  then RESULTS

In VIZUALIZER module ( or select RESULTS from JOB MANAGER in JOB module) plot stress contours: