UWTSD

Ordnance Munitions & Explosives

BEng

Academic Year 2024 - 2025

James Cawte

Electromagnetic Radiation

© UWTSD 2025. All rights reserved. No part of this publication may be reproduced without the written permission of the copyright owner.

Academic integrity declaration

I declare that:

- the thesis submitted has been written by me alone.

- the thesis submitted has not been previously submitted to this university or any other.

- that all content, including primary and/or secondary data, is true to the best of my knowledge.

- that all quotations and references have been duly acknowledged according to the requirements of academic research.

I understand that to knowingly submit work in violation of the above statement will be considered by examiners as academic misconduct.

Contents

Academic integrity declaration	2

Table of Figures	5

Table of Equations	7

Abstract	8

Introduction	9

Background	9

Section 1 – Waveguide	9

1.1 – Method	9

1.2 – Results	10

1.3 – Calculations	23

1.4 – Conclusions	23

1.5 – Teflon Method	24

1.6 – Teflon Results	24

1.7 – Teflon Calculations	36

1.8 Teflon Conclusions	37

Section 2 – Dipole Antenna	38

2.1 – Method	38

2.2 – Results	38

2.3 – Calculations	40

2.4 – Conclusions	41

References	42

# Table of Figures

Figure 1 - Waveguide Simulated Cutoff Frequency	10

Figure 2 - Port 1, Mode 1 (m=1 n=0)	11

Figure 3 - Port 1, Mode 2 (m=2 n=0)	12

Figure 4 - Port 1, Mode 2 (m=3 n=0)	13

Figure 5 - Vacuum, Mode 1 E-Field Heat Map	13

Figure 6 - Vacuum, Mode 1 E-Field Direction	14

Figure 7 - Vacuum, Mode 1 H-Field Heat Map	15

Figure 8 - Vacuum, Mode 1 H-Field Direction	16

Figure 9 - Vacuum, Mode 2 E-Field Heat Map	17

Figure 10 - Vacuum, Mode 2 E-Field Direction	18

Figure 11 - Vacuum, Mode 2 H-Field Heat Map	19

Figure 12 - Vacuum, Mode 2 H-Field Direction	20

Figure 13 - Vacuum, Mode 3 E-Field Heat Map	20

Figure 14 - Vacuum, Mode 3 E-Field Direction	21

Figure 15 - Vacuum, Mode 3 H-Field Heat Map	22

Figure 16 - Vacuum, Mode 3 H-Field Direction	22

Figure 17 - Teflon Gamma Plot	24

Figure 18 - Teflon, Mode 1, E-Field	25

Figure 19 - Teflon, Mode 1, E-Field Direction	26

Figure 20 - Teflon, Mode 1, H-Field	27

Figure 21 - Teflon, Mode 1, H-Field Direction	28

Figure 22 - Teflon, Mode 2, E-Field	29

Figure 23 - Teflon, Mode 2, E-Field Direction	30

Figure 24 - Teflon, Mode 2, H-Field	31

Figure 25 - Teflon, Mode 2, H-Field Direction	32

Figure 26 - Teflon, Mode 3, E-Field	33

Figure 27 - Teflon, Mode 3, E-Field Direction	34

Figure 28 - Teflon, Mode 3, H-Field	35

Figure 29 - Teflon, Mode 3, H-Field Direction	36

Figure 30 - Dipole Antenna being Constructed	38

Figure 31 – Excitation being added	39

Figure 32 - Scope of the simulation	39

Figure 33 - S Parameter Plot	40

Figure 34 - Graphical representation of emission	40

# Table of Equations

Equation 1 - Cutoff Frequency	23

Equation 2 - Length of a Dipole	38

Equation 3 - Frequency, Speed, Wavelength	40

# Abstract

A rectangular waveguide was constructed in a simulation program and then 3 modes were simulated passing through it. These were then visualized providing evidence that some instances of simulation the wave guide functioned and clearly transported the wave along its length. The cutoff frequencies were theoretically calculated and then generated from the simulation to get very similar values for all 3 modes. Once this had been completed the waveguide was altered to contain a Teflon fill instead of vacuum to investigate how this would alter the cutoff frequencies. The result of this was significantly reduced cutoff frequencies due to the slower speed of light through the medium. However, there were larger discrepancies between the theoretical values and simulated values for cutoff frequencies.

A dipole antenna was constructed in the same software and set to emit in a controlled region of the simulation. This provided a visual aid to the emission pattern of a dipole antenna. Using data extracted from the simulation the operating frequency was compared to the theoretically calculated one but there was significant difference between the values. Reasons for this could include the simulation not being accurate to real world scenarios.

# Introduction

Section 1 of this document will cover the process and results of creating a virtual rectangular waveguide with perfect electric conductor walls and a vacuum fill. This was simulated to produce results used to visualize cutoff frequencies. This was then repeated with a Teflon fill to compare against. Section 2 covers the design of a dipole antenna simulation used to obtain a radiation pattern.

# Background

Waveguides are pieces of hardware designed to guide waves into a specific place and direction [1]. These come in many forms such as rectangular or cylindrical which act similarly to pipes for signals. There are optical waveguides which rely on reflecting signals along line of sight to where they need to be. Similarly, fiber optic cables are a type of waveguide relying on total internal reflection.

Antennae receive and transmit signals. There are a wide range of antennae such as dipoles which are discussed in this document, Yagi-Uda arrays, parabolic dishes and helical antenna which are used in satellites [2].

Antennae and waveguides both interact with waves on the electromagnetic spectrum and utilize them as a means of communication. Most common use in communication are radio and microwaves although most other types of EM waves can be used for specific communication purposes [3].

# Section 1 – Waveguide

## 1.1 – Method

The first task was to create a rectangular waveguide in Ansys Electronics with measurments 23mm by 6.5mm by 50mm. The 4 long faces were then assigned as perfect electric conductors as per the assignment breif. The solution type was edited to High Frequency Structure Simulator to allow for a high frequency wave to be simulated along the waveguide. The transparency of the model was increased to allow for easier viewing of the internals. The 2 end faces were then asigned exitations with 3 modes. This allowed for 3 modes of vibration to be tested simulatniously. The fullport impedance was set to 50 ohms. These exitations were then verified visually on the model. Next, a solution was set up to run the simulation and gather results. A suitable number of passes for the simulation were selected and the frequency band it would sweep was set from 2 GHz to 30 GHz to ensure the cutoff frequency would be included. The solution was validated and run to produce results.

## 1.2 – Results

Figure 1 - Waveguide Simulated Cutoff Frequency

This graph shows the exact values for the cutoff frequency fc. According to the simulation data, mode 1 has a cut of frequency of 6.48 GHz. Mode 2 has a cutoff frequency of 12.99 GHz. Mode 3 has a cutoff frequency of 19.50 GHz.

Figure 2 - Port 1, Mode 1 (m=1 n=0)

This figure shows mode 1 for the vacuum filled waveguide with half a wavelength horizontally and no wavelength vertically. This means that for later use in equations m=1 and n=0.

Figure 3 - Port 1, Mode 2 (m=2 n=0)

This figure shows mode 2 for the vacuum filled waveguide. There is one full wavelength visible due to the directional arrows meaning that m=2 and n=0 as there are no wavelengths vertically.

Figure 4 - Port 1, Mode 2 (m=3 n=0)

This figure shows mode 3 for the waveguide. Visible horizontally are three half wavelengths and no vertical wavelengths meaning m=3 and n=0 for the calculations.

Figure 5 - Vacuum, Mode 1 E-Field Heat Map

Figure 5 shows the electric field strength along the waveguide. The areas of higher electric field strength will be points of major energy transfer due to the geometry of the waveguide. If this was a real-world waveguide and not a simulation then these points could be expected to heat up. At the end of the waveguide there is the approximate shape of the mode being propagated through the waveguide showing it functions to some degree.

Figure 6 - Vacuum, Mode 1 E-Field Direction

This visualization shows the directions of the electric field strength throughout the waveguide.

Figure 7 - Vacuum, Mode 1 H-Field Heat Map

This figure shows the waveguide with mode 1 being passed through it. This is a visual representation of the magnetic field intensity. An increased magnetic field intensity would induce larger current due to faradays law [4]. This needs considering in real world applications where unknown induction of current could cause issues for equipment.

Figure 8 - Vacuum, Mode 1 H-Field Direction

This figure represents the direction of the magnetic field at all points along the waveguide.

Figure 9 - Vacuum, Mode 2 E-Field Heat Map

This image show the electric field strength for the waveguide being simulated for mode 2. This mode is clearly visible at the port of the waveguide with the areas of high field strength corresponding to the areas of maximum amplitude in Figure 3. This is a more typical waveguide field however there are still unexpected areas of low electric field strength. These are likely due to the dimensions of the waveguide.

Figure 10 - Vacuum, Mode 2 E-Field Direction

This figure shows the direction of the electric field for mode 2. Noteworthy is the end of the waveguide where the directions of the E-field correspond to the directions of the mode in Figure 3.

Figure 11 - Vacuum, Mode 2 H-Field Heat Map

This image shows the magnetic field intensity for mode 2 with obvious focus points at both ends. These are likely to induce current. The end of the waveguide almost corresponds to the mode being passed through it.

Figure 12 - Vacuum, Mode 2 H-Field Direction

This shows the direction of the magnetic field intensity for mode 2.

Figure 13 - Vacuum, Mode 3 E-Field Heat Map

This figure displays the electric field strength for mode 3. The pattern on the end of the waveguide matches what would be expected based on the mode in Figure 4.

Figure 14 - Vacuum, Mode 3 E-Field Direction

This figure shows the direction of the electronic field for mode 3.

Figure 15 - Vacuum, Mode 3 H-Field Heat Map

This figure shows the magnetic field intensity for mode 3 in the waveguide.

Figure 16 - Vacuum, Mode 3 H-Field Direction

This figure shows the direction of the magnetic field.

## 1.3 – Calculations

fc=cεrμr(m2a)2+(n2b)2

Equation 1 - Cutoff Frequency

c=3*108

εr= permittivity of free space

μr= permeability of free space

a=23*10-3

b=6.5*10-3

Mode 1

m=1

n=0

fc=6521739130 Hz = 6.52 GHz (2.d.p)

Mode 2

m=2

n=0

fc=13043478260 Hz = 13.04 GHz (2.d.p)

Mode 3

m=3

n=0

fc=19565217390 Hz = 19.57 GHz (2.d.p)

## 1.4 – Conclusions

The conclusions which can be drawn are that the waveguide does not always function as expected. If the waveguide was achieving its purpose of transfering a wave from one end to the other then the same modal pattern as seen in Figure 2, Figure 3 and Figure 4 would be seen at the exit port. This is not the case in every one of the graphical representations of the E-field and H-field. Figure 13 and Figure 9 provide an accurate recreation of the mode that was fed in from the other end suggesting that the waveguide works as intended.

The theoretically calculated values for the cutoff frequency are very close to the simulated values (6.48 GHz vs 6.52 GHz for mode 1, 12.99 GHz vs 13.04 GHz for mode 2 and 19.50 GHz vs 19.57 GHz for mode 3). This suggests that the simulation is accurate.

## 1.5 – Teflon Method

The method to create the Teflon filled waveguide was identical to the method outlined in 1.1 - Method. The only difference is the inside being Teflon instead of vacuum.

## 1.6 – Teflon Results

Figure 17 - Teflon Gamma Plot

This graph shows the simulated cutoff frequencies for all 3 modes in a Teflon waveguide. Mode 1 is 4.47 GHz, mode 2 is 8.86 GHz and mode 3 is 13.34 GHz.

Figure 18 - Teflon, Mode 1, E-Field

This figure shows the electric field strength on the Teflon waveguide. The pattern lines up with what would be expected from mode 1.

Figure 19 - Teflon, Mode 1, E-Field Direction

This figure shows the direction of the electric field for mode 1.

Figure 20 - Teflon, Mode 1, H-Field

This figure shows the magnetic field intensity. The pattern of high and low intensity matches the mode.

Figure 21 - Teflon, Mode 1, H-Field Direction

This figure shows the direction of the magnetic field for mode 1.

Figure 22 - Teflon, Mode 2, E-Field

This figure shows the electric field strength for mode 2 and the pattern does not match what would be expected.

Figure 23 - Teflon, Mode 2, E-Field Direction

This figure shows the direction of the electric field for mode 2.

Figure 24 - Teflon, Mode 2, H-Field

This figure shows the magnetic field intensity for mode 2. This does not match the pattern of mode 2 and is likely caused by the geometry of the waveguide.

Figure 25 - Teflon, Mode 2, H-Field Direction

This figure shows the direction of the magnetic field for mode 2.

Figure 26 - Teflon, Mode 3, E-Field

This figure shows the electric field strength for mode 3. The pattern produced by the simulation matches what would be expected from mode 3.

Figure 27 - Teflon, Mode 3, E-Field Direction

This graphic shows the direction of the electronic field for mode 3.

Figure 28 - Teflon, Mode 3, H-Field

This figure shows the magnetic field intensity for mode 3. The pattern produced partially matches what would be expected but does not fully match the pattern of mode 3 meaning the wave is not traveling along the waveguide properly.

Figure 29 - Teflon, Mode 3, H-Field Direction

This figure shows the direction of the magnetic field for mode 3.

## 1.7 – Teflon Calculations

fc=cεrμr(m2a)2+(n2b)2

Equation 1 - Cutoff Frequency

c=3*108

εr= permittivity of Teflon = 2.1 [5]

μr= permeability of Teflon = 1

a=23*10-3

b=6.5*10-3

Mode 1

m=1

n=0

fc=4500427561 Hz = 4.50 GHz

Mode 2

m=2

n=0

fc=9000855122 Hz = 9.00 GHz

Mode 3

m=3

n=0

fc=13501282680 Hz = 13.50 GHz

## 1.8 Teflon Conclusions

In conclusion, having a Teflon filling instead of a vacuum reduced the cutoff frequencies for all modes. This is because the waves travel more slowly through a medium which reduces the wavelength so a lower frequency wave can enter the waveguide and be compressed to the required frequency to propagate through the waveguide [6].

# Section 2 – Dipole Antenna

## 2.1 – Method

To create the dipole antenna, a 1mm radius that was the required length to transmit 231mm waves.

L= λ / 2

Equation 2 - Length of a Dipole

Using Equation 2, the length of the dipole antenna was 115.5mm. Once this was extruded, an excitement was placed at the centre point of the length. Once the antenna was constructed, a solution was created and validated and a simulation run. Once the simulation was completed, the results were extracted into various formats.

## 2.2 – Results

Figure 30 - Dipole Antenna being Constructed

Figure 31 – Excitation being added

Figure 32 - Scope of the simulation

Figure 33 - S Parameter Plot

This S parameter plot suggests that operating frequency will be 1.18 GHz (2.d.p).

Figure 34 - Graphical representation of emission

## 2.3 – Calculations

As the length of the dipole is one half wavelength, the operating frequency can be calculated as follows:

f=c/λ

Equation 3 - Frequency, Speed, Wavelength

λ=231*10-3 m

c=3*108 ms-1

f = 1298701299 Hz = 1.30 GHz (2.d.p)

## 2.4 – Conclusions

In conclusion, the dipole antenna will operate at a frequency of 1.18-1.30 GHz and will extend 57.75mm either side of the emitter. The discrepancy in the simulated value and the theoretically worked out value for operating frequency may be attributed to an insufficient simulation. The emission it produces will be cylindrical with uniform emission in a ring around the length with a null at either end.

# References

| [1] | N. Marcuvitz, in Waveguide Handbook, McGraw Hill, 1951. |
| --- | --- |
| [2] | M. R. a. A. B. Abdul Qadir Khan, “Various Types of Antenna with Respect to their Applications: A Review,” INTERNATIONAL JOURNAL OF MULTIDISCIPLINARY SCIENCES AND ENGINEERING, vol. 7, no. 3, 2016. |
| [3] | A. Ishimaru, in Electromagnetic Wave Propagation, Radiation, and Scattering: From Fundamentals to Applications, John Wiley & Sons, 2017. |
| [4] | S. C. R. a. S. P. Robert Kingman, An experimental observation of Faraday’s law of induction, Berrien Springs, Michigan : American, 2000. |
| [5] | “The Engineering Tool Box,” [Online]. Available: https://www.engineeringtoolbox.com/relative-permittivity-d_1660.html. [Accessed 25 04 2025]. |
| [6] | M. Steer, “LibreTexts Engineering,” NICE CXone Expert, [Online]. Available: https://eng.libretexts.org/Bookshelves/Electrical_Engineering/Electronics/Microwave_and_RF_Design_II_-_Transmission_Lines_%28Steer%29/06%3A_Waveguides/6.04%3A_Rectangular_Waveguide?utm_source=chatgpt.com. [Accessed 25 04 2025]. |