Course Title:			BEng Electrical/Electronic Engineering

Module:				ACDA6002

Assignment Number	:	2

Assignment Title:		Marine Weather Monitor / Logger

Student Name:		Carys Williams

Student Number:		1708572

Hand in Date:			30/12/2021

Lecturer Name:		Dr. Gary Jones

Contents

Contents	2

1 Introduction	1

2 Marine Weather Monitor / Logger	2

2.1 Type 1	2

2.2 Type 2	2

2.3 Type 3	3

2.4 Type 4	3

3 Comparison of Marine weather monitors	4

4 Sensors	5

4.1 Temperature Comparison	5

4.2 Humidity Comparison	5

4.3 Temperature and Humidity Selection	6

4.4 Wave Hight	6

4.4.1 Comparison	6

4.4.2 Selection	7

4.5 Wind speed Comparison	7

4.6 Wind Direction Comparison	8

4.7 Wind Speed and Direction Selection	8

4.6 Sunlight Brightness	9

4.6.1 Comparison	9

4.6.2 Selection	9

4.7 GPS Position	10

4.7.1 Comparison	10

4.7.2 Selection	10

5 LED Selection	11

6 Data	11

6.1 RS232	11

6.2 Non-Volatile memory	11

6.3 SD card Selection	12

6.4 Logging Rates	12

7 Microcontroller	12

7.1 Resolution Calculations	12

7.2 Specification	12

7.3 Selection	13

8 Battery	13

8.1 Specification	13

8.2 Comparison	14

8.3 Selection	14

9 Connectors	15

9.1 Comparison and selection	15

10 Casing	15

11 Pricing Table	16

12 Proteus Design	17

12.1 Full Design	17

12.2 Signal Conditioning	17

12.3 Temperature and Humidity	18

12.4 Wave Hight	18

12.5 Wind Speed and Direction	18

12.6 Sunlight Brightness	19

12.7 GPS Position	19

12.8 USB	19

12.9 SD Card	20

12.10 LED	20

12.11 Battery	20

12.12 Microcontroller	21

13 Easy-PC Design	22

13.1 Full Design Schematic	22

13.2 Full design PCB	23

13.3 Front Facing 3D view	24

13.4 3D view of layers and bottom	24

14 PCB Quote	25

15 Flow Chart	26

16 Block Diagram	27

17 Improvements	28

18 Reference	29

19 Bibliography	31

# 1 Introduction

Marine weather-based data logger, that takes measurements and logs data over a 7-day period, is what this report is based on. It must be portable and adaptable for any number of potential vessels.

The following data will be acquired during the 7day cycle: -

- Temperature

- Humidity

- Wave height

- Wind speed

- Wind direction

- Sunlight brightness

- GPS position

This design does not require a display except for a LED which flashes once every 10 secs for 1 sec.

# 2 Marine Weather Monitor / Logger

Weather monitor/ logger are data acquisition systems. They take the different conditions and translate it into data that can easily be compared to other data. There are many varieties of weather monitor that can be purchased on the market. However, they all have differences. Below will show a selection that are available to buy. Table 1 will show the comparisons of the selected weather monitors/ loggers.

## 2.1 Type 1

This is a ‘Pro2’ which offers an accurate and trustworthy data with self-contained sensor suite, which is simple to install. Large Robust console with an easy-to-read display. It displays: Time and date, moon phase, weather forecast, indoor and outdoor temperature, outdoor humidity, atmosphere pressure, rainfall, wind speed, wind direction, dew pint, sunrise/sunset, weather alarm/trends. [1]

## 2.2 Type 2

This is a Dyacon MS-120. It is a cable-connect weather station. Wind, temperature, pressure, humidity is a few of the sensors that come with it however, an upgrade is possible. The options include rain gauge, solar sensor, or soil moisture sensor. A wireless connection can be possible; however, it will need to be upgraded again.[2]

## 2.3 Type 3

The MWS-M625, can weight up to three and a half pounds and adds a micro lidar components for cloud hight measurements. It becomes the most ruggedized, lightweight, and low-cost solution for weather sensing with cloud height measurements. [3]

## 2.4 Type 4

This is a Bresser 7-in-1 solar 6-day 4 CAST PRO WI-FI Weather centre. It is a solar-powered weather station that measures temperature, humidity, precipitation, wind speed, wind direction etc. it shows a 6-day weather forecast via WI-FI on a display.  [4]

# 3 Comparison of Marine weather monitors

Here is a table that shows the comparison of the 4 types of weather monitors seleted.

| Name | Wireless Vantage Pro2 Bundle | MS-120 | MWS-M625 | Bresser 4CAST PRO WLAN |
| --- | --- | --- | --- | --- |
| Images |  |  |  |  |
| Product Code | Vantage-pro2-bundle | MS-120 | MWS-M625 | 7003210 |
| Voltage Supply | Solar Powered, 12VDC | 12-24V | Solar power Nickel cadmium batteries | 5V Solar panel |
| Wind Speed | Yes | Yes | Yes | Yes |
| Wind Direction | Yes | Yes | Yes | Yes |
| Temperature | Yes | Yes | Yes | Yes |
| Humidity | Yes | Yes | Yes | Yes |
| Pressure | No | Yes | Yes | Yes |
| Other Features? | - Rainfall - Windchill - Dew point - heat calculated. | - Real-Time Gust Detection | - Precipitation - Cloud Ceiling - Angular tilt - Dust Accumulation - Lightning Distance - 360 Imaging sensor | - Moon Phase - Precipitation - Time and Date - Rainfall - UV level and light |
| Upgrades available? | N0 | Yes | Yes | Yes |
| Data | Wireless, anemometer (12m away max), app | Cable, on bored Modbus RTU (RS-485) port. | USB cable Transmitted data satellite | Wi-Fi, USB |
| Dimension | 350 x 220 x 360 mm | 420mm x 120mm | 19 x 14 x 14 cm | Sensor: 390 x 174 x 465mm Base: 215 x 27 x 176.5mm |
| Price | £865.00 | +$2,493.00 | $7,300 - $27,000 | £264.00 |
| Reference | [1] | [2] | [3] | [4] |

# 4 Sensors

Selecting the correct sensors are essential to develop a Weather data logger. The following section will consist of comparing and selecting sensors.

## 4.1 Temperature Comparison

| Type | Thermistor | Thermocouple | Temperature and Humidity |
| --- | --- | --- | --- |
| Image |  |  |  |
| Robustness | Good | Good | Very good |
| Connection | Simple | Simple | Simple |
| Price | Cheap | Cheap | Expensive |
| IP rating | Low | High | High |
| Dual reading | n/a | n/a | Yes |
| Reference | [5] | [6] | [7] |

## 4.2 Humidity Comparison

| Type | Thermistor | Thermocouple | Temperature and Humidity |
| --- | --- | --- | --- |
| Image |  |  |  |
| Robustness | Good | Good | Very good |
| Connection | Simple | Simple | Simple |
| Price | Cheap | Cheap | Expensive |
| IP rating | Low | Mid-range | High |
| Dual reading | n/a | n/a | Yes |
| Reference | [8] | [9] | [7] |

## 4.3 Temperature and Humidity Selection

| Name | HTM2500LF | T9602 – 5 - A | HX94B |
| --- | --- | --- | --- |
| Images |  |  |  |
| Mfr. Number | HPP809A031 | T9602-5-A | HX94B |
| Voltage Supply | 4.75V – 5.25V | 5Vdc | 9 Vdc – 30Vdc |
| Output | 1 – 4Vdc | 0 – 5V (750uA) | 0 – 10Vdc |
| Range (temperature °C) | -40°C – 85°C | -20°C – 70°C | 0°C – 100 °C |
| Range (Humidity%) | 1 – 99 % | 0 – 95% | 0% - 100% |
| Connection | 5 wires | 4 Wires | 4 wires |
| Analogue or Digital | A | A | D |
| IP rating | IP65 | IP67 | IP68 |
| Dimension | 11.6mm x 76.3mm | 41.5mm x 10.5mm | 258mm x 19mm |
| Price | £69.38 | £52.14 | £304.80 |
| Reference | [10] | [11] | [12] |

## 4.4 Wave Hight

## 4.4.1 Comparison

| Name | Pressure | Retro - Reflective | Ultrasonic |
| --- | --- | --- | --- |
| Images |  |  |  |
| IP rating | Low - Mid | Low - Mid | Low - Mid |
| Price | Mid-range | Mid-range | Cheap |
| Accurate | Yes | Weather dependant | Yes |
| connection | Complex | Complex | Simple |
| Reference | [13] | [14] | [15] |

### 4.4.2 Selection

| Name | 28015 | 166-9496 or 166-9497 | Sick M30 x 1.5 |
| --- | --- | --- | --- |
| Images |  |  |  |
| Mfr. Number | 28015 | 166-9496 166-9497 | UM30-215113 |
| Receiver or transmitter? | Both | Order 1 Transmitter Order 1 Receiver | n/a |
| Supply | +5VDC | 20V | 15V – 30V |
| Output | 0V – 5V (35mA max) | 0V – 20V |  |
| Range | 0.02m – 3M | 0.2 – 4M | 600 – 6000mm (0.6 – 6M) |
| Frequency | 40KHz | 40KHz | 80KHz |
| Connection | 3 pins | 2 pins | 5 pin M12 Connector |
| Analogue or Digital | A | A | A |
| Dimension | 22mm x 46mmm x 16mm | 7.1mm x 9.9mm | 105mm x 65mm |
| Price | £25.18 | £13.02 each | £580.09 |
| Reference | [15] | [16] | [17] |

For the wave height it was decided that it would consist of 2 plastic pipes that are in the water. One will have a large whole at the bottom, as for the other will have littles holes to allow a small flow of water to enter. This it will be able to show the height of the wave without connected another object to the seabed.

## 4.5 Wind speed Comparison

| Name | Doppler Radar | Laser-Based LIDAR | Anemometer Ultrasonic (Wind Speed/Direction) |
| --- | --- | --- | --- |
| Images |  |  |  |
| Accuracy | Good | Good | Very Good |
| Price | Mid-range | Cheap (large scale) | Expensive |
| Connection | Simple | Simple | Simple |
| Dual reading | N/A | N/A | Yes |
| IP rating | High | High | High |
| Reference | [18] | [19] | [20] |

## 4.6 Wind Direction Comparison

| Name | Weathervane | Anemometer Ultrasonic (Wind Speed/Direction) |
| --- | --- | --- |
| Images |  |  |
| Accuracy | Bad | Very Good |
| Price | Cheap | Expensive |
| Connection | Simple | Simple |
| Durable | Bad (moving part) | Good |
| Dual reading | N/A | Yes |
| Reference | [21] | [20] |

## 4.7 Wind Speed and Direction Selection

| Name | Wind Sonic | Wind Observer | WMS-23 |
| --- | --- | --- | --- |
| Images |  |  |  |
| Mfr. Number | 1405-PK-021 | 1390-70-B-142 | WMS-23 |
| Voltage | 5VDC – 30VDC | 9VDC – 30VDC | 8VDC – 30VDC |
| Output | 12V | 0-5V | 4 – 20 mA |
| Accuracy | +/- 2% | +/- 2% | +/- 1% |
| Response Time | 0.25 s | n/a | 0.8 mph |
| Wind Direction | 0° - 359° | 0° - 359° | 0°–360° (every 2 – 5 = 355) |
| Wind Speed | 0 – 60 m/s | 0 – 70 m/s | 0 – 100mph |
| Pins | 4 wires | 4 wires | 5 wires |
| Analogue or Digital | RS232 | RS422/RS485 | Both |
| Dimension | 142mm x 163mm | 389mm x 213mm | 114mm x 216mm |
| Reference | [22] | [23] | [24] |

## 4.6 Sunlight Brightness

### 4.6.1 Comparison

| Name | Phototransistor | LDR | Photodiode |
| --- | --- | --- | --- |
| Images |  |  |  |
| Price | Cheap | Cheap | Cheap |
| Durability | Fragile | Fragile | Very Fragile |
| Connection | Simple | Simple | Simple |
| Temperature range | limited | Very high | Sesnitive |
| Response time | Very Fast | Fast | Fast |
| Contaminated lens | Yes | Yes | Yes |
| Negative | - surges - spikes - EM energy |  | - High Voltage |
| Reference | [25] | [25] | [25] |

### 4.6.2 Selection

| Name | NORPS-12 |  |
| --- | --- | --- |
| Images |  |  |
| Mfr. Number | NORPS-12 | NSL-19M51 |
| Voltage | +250V | +100V |
| No Pins | 2 pins | 2 pins |
| Resistance range | 5.4KΩ – 12.6KΩ | 20kΩ -100KΩ |
| Temperature range | -60°C - +75°C | -60°C - +75°C |
| Mounting | Through Hole | Through Hole |
| Dimension | 14.35mm x 38mm | 4.1mm x 37.8mm |
| Price | £2.91 (pack of 4) | £0.78 |
| Reference | [26] | [27] |

## 4.7 GPS Position

### 4.7.1 Comparison

| Name | GPS | N/A |
| --- | --- | --- |
| Images |  |  |
| Positive | - Worldwide - Accurate - Cheap - Works in all weathers |  |
| Negative | - May need backup map direction |  |
| Reference | [28] |  |

## 4.7.2 Selection

| Name | NEO6M | ADAFRUIT INDUSTRIES | Tiny GPS Module |
| --- | --- | --- | --- |
| Images |  |  |  |
| Mfr. Number | GYGPSV5-NEO | 1059 | KQP_Nameless_GPS |
| Voltage | 3V – 5V | 3V – 4.3V | 5V |
| Support rate | 9600dps | 9600dps | 9600dps |
| Connection | 4 Pins | 4 pins | 4 pins |
| Analogue or Digital | RS232 | RS232 | RS232 |
| Dimension | 23mm x 30mm / 12mm x 12mm | 15mm x 15mm x 4mm | 12mm x 16mm x 6mm |
| Price | £3.60 | £38.48 | £8.99 |
| Reference | [29] | [30] | [31] |

# 5 LED Selection

An LED is required to be fitted that will flash every 10 seconds for 1 second. A correct sized resistor will need to be in series with the LED. It will be connected as normal to the PIC. For the timer it is possible to add it to the program. [32]

An LED Pipe will be added to prevent water contamination [33]

The calculation is as followed:

R=Battery V-LED VDesired LED Current= R= VresIf

R= (5V-1.9v)20mA= 3.1V0.02

R=155Ω

# 6 Data

## 6.1 RS232

RS232 ports are required for this system. There is one available on the PIC. Running a full RS232 will use 15V, this is not advised. A TTL would be better as it runs on 0V – 5V. This can then be connected to a PC via a USB.

## 6.2 Non-Volatile memory

For this design a non-volatile memory will be required. Below shows the calculations taken for the required memory storage for the duration needed.

| Measurements | Logging | Calculation | Total bytes |
| --- | --- | --- | --- |
| Temperature and Humidity | 1s | 60*60*24*7 | 604,800 |
| Wave hight | 0.5s | 60*60*24*7*2 | 1,209,600 |
| Wind Speed and Direction | 0.1s | 60*60*24*7*10 | 6,048,000 |
| Sunlight brightness | 1s | 60*60*24*7 | 604,800 |
| GPS | 1s | 60*60*24*7 | 604,800 |
| Total memory |  |  | 9,072,000 |

MB= 9,072,0001,000,000=      =   9.072 MB

For a 7 day test a total of 10MB memory is required.

## 6.3 SD card Selection

1GB SD memory card was selected. This is cheaper than a lower memory at £7.29. Also, there is room if any alterations are needed. [34]

## 6.4 Logging Rates

| Time | Wind Speed & Direction | Wave Height | Temperature & Humidity | Sunlight | GPS |
| --- | --- | --- | --- | --- | --- |
| 0s | Wind Speed & Direction | Wave Height | Temperature & Humidity | Sunlight | GPS |
| 0.1s | Wind Speed & Direction |  |  |  |  |
| 0.2s | Wind Speed & Direction |  |  |  |  |
|  | Wind Speed & Direction |  |  |  |  |
| 0.5s | Wind Speed & Direction | Wave Height |  |  |  |
| 1s | Wind Speed & Direction | Wave Height | Temperature & Humidity | Sunlight | GPS |
| 1s | Wind Speed & Direction | Wave Height | Temperature & Humidity | Sunlight | GPS |

# 7 Microcontroller

### 7.1 Resolution Calculations

#### Temperature

-20°C – 70°C range = 90 steps

Measure every 0.5

90/0.5 = 180 = 8 bits

#### Humidity

0% – 95% range = 95 steps

Measure every 0.5

95/0.5 = 190 = 8 bits

#### Wave Hight

20mm – 3000mm = 2980 steps

Measure every 20mm

2980/20 = 149 = 8 bits

### 7.2 Specification

| Name | Analogue or Digital | Pins | Resolution | Sample Time |
| --- | --- | --- | --- | --- |
| Temperature & Humidity | Analogue | 2 | 8 Bits | 1s |
| Wave height | Digital | 2 | 8 Bits | 0.5s |
| Wind speed & Direction | Digital | 2 | RS232 | 0.1s |
| Sun Brightness | Analogue | 1 | 8 Bits | 1s |
| GPS | Digital | 2 | RS232 | 1s |
| SD Card | Digital | 3 |
| USB | D+/D- | 2 |
| RS232 | Digital | 2 |
| LED | Digital | 1 |
| Switch | Digital | 1 |

With the resolution all components need to run on the same. It has been decided to run at 8 bits.

|  | Pins |
| --- | --- |
| Data Storage | 3 (SPI) |
| Power | 2 or 4 (depending on chip) |
| Digital Pins (plus interrupt) | 6 |
| Analogue Pins | 4 (8 bits) |
| USB | 2 |
| RS232 Debug | 2 |
| LED | 1 |
| Total | 22 |

For this design a 40-pin chip will be selected. This is to ensure Improvement can be added and there are enough pins available.

The following needs to be considered when selecting a Microcontroller

| Pins Required | 22 |
| --- | --- |
| Memory | 9.072MB |

## 7.3 Selection

| Mfr. number | PIC18F45K50-IP | PIC18F45K50-I/MV | PIC18LF45K50-I/P |
| --- | --- | --- | --- |
| Images |  |  |  |
| CPU Speed | 48MHz | 48MHz | 48MHz |
| SPI | Yes | Yes | Yes |
| Program Memory | 32KB | 32KB | 32KB |
| Max Voltage | 5.5V | 5.5V | 3.6V |
| Min Voltage | 2.3V | 2.3V | 1.8V |
| RAM Memory | 2KB | 2KB | 2KB |
| Price (1 ex VAT) | £3.88 | £3.45 | £3.06 |
| Reference | [35] | [36] | [37] |

# 8 Battery

## 8.1 Specification

To calculate the size of the battery required. The maximum Current for each component must be taken.

| Components | Max Current |
| --- | --- |
| Temperature and Humidity | 750uA |
| Wind Speed and Direction | 44mA |
| Wave Height | 35mA (x2) |
| Sunlight | 10mA |
| GPS | 40mA |
| LED | 30mA |
| SD Card | 10uA |
| Total | 194.76mA |

Capacity Amp Hourly=Total Current Amps× Time hour

Capacity Amps in a day=0.185Ah ×24 Hours

Capacity Amps a week=4.68Ah ×7 days

Total Capacity needed for a week=32.76Ah

To ensure the battery can last long enough the total capacity will be  rounded to 50Ah.

## 8.2 Comparison

| Battery | Nickel Metal Hydride (NiMH) | Lithium-ion | Lead Acid |
| --- | --- | --- | --- |
| Images |  |  |  |
| Pros | - High power density - ‘Standard’ sizing - Rechargeable | - High energy density - Self-discharge - Low maintenance - Variety | - Cheap - powerful - High power capacity - Rechargeable |
| Cons | - Low Density to LiPo - Self-discharges quickly - Limit discharge current - Heat loss when charging | - Protection - Ageing - Cost | - Heavy - Low power density |
| Reference | [38] | [39] | [40] |

## 8.3 Selection

| Manufacturer | QTshine | TOOSPON | MAXOAK |
| --- | --- | --- | --- |
| Images |  |  |  |
| Mfr. Number | HX200Q6 | APD101 | K”-50000mAh |
| Voltage | 5V | 5V | 3.7V |
| Amp Hourly | 26800 (2 required) | 50000mAh | 50000mAh |
| Dimension | 155mm x 79mm x 22mm | 178mm x 84mm x 39mm | 206mm x 135mm x 33mm |
| Price | £23.95 | £49.99 | £169.99 |
| Reference | [41] | [42] | [43] |

# 9 Connectors

## 9.1 Comparison and selection

| Manufacture | LEMO | LEMO | XLR |
| --- | --- | --- | --- |
| Images |  |  |  |
| Mfr. Number male | EGG.TT.304.CLL | FMP.0M.305.LLCJ | 457-908 |
| Mfr. Number female | FGG.TT.304.CLAC31Z | PMP.0M.305.LLMJ | 457-885 |
| Plating | Gold | Gold | Silver |
| Price (Male) | £35.46 | £73.47 | £2.88 |
| Price (Female) | £40.91 | £88.92 | £2.91 |
| Quality | Medium | Medium | Medium |
| Protection | Good | Good | Low |
| Positive | - Medium quality - Good protection | - Medium quality - Multicolour - Good Protection | - Medium quality - cheap |
| IP | IP68 | IP68 | Not stated |
| Reference | [44], [45] | [46], [47] | [48], [49] |

# 10 Casing

| Name | MAX300 | Hurricane | Storm case |
| --- | --- | --- | --- |
| Images |  |  |  |
| Mfr. Number | MAX300 | EN-EC-37-1-OR | iM2275 |
| Positive | - Foam interior - Good protection | - Foam interior - Very good protection | - Foam interior - Good Protection |
| Negative | - No Wheels | - No Wheels | - No wheels |
| Dimension | 300mm x 225mm x 132mm | 275mm x 203mm x 95.4mm | 359mm x 335mm x 241mm |
| IP rating | IP67 | IP67 / IK08 | IP67 |
| Price | £48.42 | £23.44 | £232.80 |
| Reference | [50] | [51] | [52] |

# 11 Pricing Table

| Items | Mfr. Number | Supplier | Qty. | Cost | Total Cost | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Temp & humidity sensor | T9602-5-A | Farnell | 1 | £52.14 | £52.14 | [11] |
| Wave height Sensor | 28015 | Farnell | 2 | £25.18 | £50.36 | [15] |
| Sunlight Sensor | NORPS | RS-Online | 1 | £2.91 (4) | £2.91 | [26] |
| Wind speed & Direction sensor | 1405-PK-021 | Gill Instruments | 1 | £640.46 | £640.46 | [22] |
| GPS sensor | GYGPSV5-NEO | AliExpress | 1 | £3.60 | £3.60 | [29] |
| LED | HLMP-1301 | Farnell | 1 | £0.51 (+5) | £0.102 | [32] |
| LED Pipe | 1316.2001 | Farnell | 1 | £2.89 | £2.89 | [33] |
| SD Card | TS1GSDC | PicStop | 1 | £7.29 | £7.29 | [34] |
| Battery | APD101 | Amazon | 1 | £49.99 | £49.99 | [42] |
| Case | EN-EC-37-1-OR | Cases-and-enclosures | 1 | £23.44 | £23.44 | [51] |
| PCB Bored | Weather Logger 2 | JLCPCB | 1 | $9.23 (roughly £6.30) | $9.23 (roughly £6.30) | [53] |
| Microcontroller | PIC18F25K50-I/SO | Farnell | 1 | £3.12 | £3.12 | [36] |
| Connector Male | FMP.0M.305.LLCJ | Lemo | 2 | £73.47 | £146.94 | [46] |
| Connector Female | PMP.0M.305.LLMJ | Lemo | 2 | £88.92 | £177.84 | [47] |
| Total |  |  |  |  | £1,167.38 |  |

# 12 Proteus Design

## 12.1 Full Design

## 12.2 Signal Conditioning

Here is a design of a clamp. This must be present on some sensors to limit the voltage to +5V for the microcontroller

## 12.3 Temperature and Humidity

## 12.4 Wave Hight

## 12.5 Wind Speed and Direction

## 12.6 Sunlight Brightness

## 12.7 GPS Position

## 12.8 USB

## 12.9 SD Card

## 12.10 LED

## 12.11 Battery

## 12.12 Microcontroller

# 13 Easy-PC Design

### 13.1 Full Design Schematic

### 13.2 Full design PCB

### 13.3 Front Facing 3D view

### 13.4 3D view of layers and bottom

# 14 PCB Quote

Here is an image showing the price of the PCB when using reference [53].

# 15 Flow Chart

# 16 Block Diagram

# 17 Improvements

On the Completion of the marine weather monitor, a few improvements could be done. They are as followed: -

Firstly, for the battery, solar panels could be used instead. This would be more environmentally friendly as well as improve the power efficiency of the system. This would make it possible to record for a longer period.

Secondly, adding other sensors to measure other circumstances would improve the weather monitor. Such as rain fall, or wind strength. These would be more helpful to calculate the conditions for sailors.

Connectors could have been chosen differently if there was a price cap. However, there wasn’t so the connectors are of high quality and good protection. This would help with the overall price of the device.

Another improvement that should be considered is sensors. Some sensors need better protection or a higher IP rating like the ultrasonic sensors. However, this is down to personal preference of how to measure wave height.

# 18 Reference

[1] “www.weather-station-products.co.uk”, 2019. [Online]. Available:  Wireless Vantage Pro2 Bundle (weather-station-products.co.uk) . [Accessed 1/12/2021].

[2] “www.weathershop.co.uk”, 2021. [Online]. Available: Davis Vantage Pro2 Wireless Weather Station & WeatherLink Live Bundle 6122 UK | Weather Shop ® UK . [Accessed 1/12/2021].

[3] “www.dycon.com”, 2019. [Online]. Available: MS-120 - Dyacon . [Accessed 1/12/2021].

[4] “intellisenseinc.com”, 2021. [Online]. Available: The All-in-One Weather Station with Cloud Height - The MWS-M625 (intellisenseinc.com) . [Accessed 1/12/2021].

[5] “electricalterminology.com”, 2021. [Online]. Available: Advantages and disadvantages of thermistor | 27 benefits and drawbacks (electricalterminology.com) . [Accessed 5/12/2021].

[6] “Sciencing.com”, 2019. [Online]. Available: Advantages and Disadvantages of Using Thermocouples (sciencing.com). [Accessed 5/12/2021].

[7] “rs-online.com”, 2021. [Online]. Available: HPC169_C HTM2500LF data sheet (rs-online.com). [Accessed 5/12/2021].

[8] “rotronic.com”, 2019. [Online]. Available: Webinar 2 v4 - Pros & Cons of Humidity Measurement Technologies - 9-17 (rotronic.com). [Accessed 5/12/2021].

[9] “rotronic.com”, 2019. [Online]. Available: Webinar 2 v4 - Pros & Cons of Humidity Measurement Technologies - 9-17 (rotronic.com). [Accessed 5/12/2021].

[10] “rs-online.com”, 2021. [Online]. Available: HPC169_C HTM2500LF data sheet (rs-online.com). [Accessed 5/12/2021].

[11] “uk.arnell.com”, 2021. [Online]. Available: Telaire T9602 Humidity & Temperature Sensor (farnell.com). [Accessed 5/12/2021].

[12] “omega.co.uk”, 2021. [Online]. Available: Humidity & Temperature Transmitters:Relative Humidity/Ambient Temperature Transmitters - HX94BV2W (omega.co.uk). [Accessed 5/12/2021].

[13] “rs-online.com”, 2021. [Online]. Available: RS PRO Relative Pressure Sensor for Air, Gas, Hydraulic Fluid, Liquid, Water, 10bar Max Pressure Reading, Voltage | RS Components (rs-online.com). [Accessed 7/12/2021].

[14] “rs-online”, 2021. [Online]. Available: RS PRO Retroreflective Photoelectric Sensor Block Sensor, 5 m Detection Range | RS Components (rs-online.com). [Accessed 7/12/2021].

[15] “uk.farnell.com”, 2021. [Online]. Available: 28015 Parallax, Sensing Range Min:2cm | Farnell. [Accessed 7/12/2021].

[16] “rs-online.com”, 2021. [Online]. Available: RS Pro Ultrasonic Sensor Barrel, 0.2 → 4 m, PCB | RS Components (rs-online.com). [Accessed 8/12/2021].

[17] “rs-online.com”, 2021. [Online]. Available: UM30-215113 | Sick M30 x 1.5 Ultrasonic Proximity Sensor - Barrel, Analogue Output, 600 → 6000 mm Detection, IP67 | RS Components (rs-online.com). [Accessed 8/12/2021].

[18] “lidarradar.com”, 2021. [Online]. Available: Advantages and Disadvantages of Doppler Radar – LiDAR and RADAR Information (lidarradar.com) . [Accessed 10/12/2021].

[19] “lidarradar.com”, 2021. [Online]. Available: Advantages and Disadvantages of LiDAR – LiDAR and RADAR Information (lidarradar.com). [Accessed 10/12/2021].

[20] “windpowerengineering.com”, 2021. [Online]. Available: Ultrasonic wind sensors or cup anemometers? (windpowerengineering.com). [Accessed 10/12/2021].

[21] “weatherspares.co.uk-”, 2021. [Online]. Available: https://weatherspares.co.uk/products/garni-940-wind-vane-940v?variant=32255807127648&currency=GBP&utm_medium=product_sync&utm_source=google&utm_content=sag_organic&utm_campaign=sag_organic. [Accessed 11/12/2021].

[22] “gillinstruments.com”, 2021. [Online]. Available: windsonic-1405-027-iss7.pdf (gillinstruments.com).[Accessed 11/12/2021].

[23] “gillinstruments.com”, 2021. [Online]. Available: 1390-0033 Iss 6 WindObserver70 plus OPTIONS.pdf (gillinstruments.com). [Accessed 11/12/2021].

[24] “uk.farnell.com”, 2019. [Online]. Available: WMS-23 Omega, ENV MONITORS, WEATHER STATION | Farnell. [Accessed 11/12/2021].

[25] “ecstuff4u.com”, 2020. [Online]. Available: ECSTUFF4U for Electronics Engineer: light sensor advantages and disadvantages. [Accessed 11/12/2021].

[26] “rs-online.com”, 2021. [Online]. Available: Luna Optoelectronics, 250 V Through Hole LDR (Light Dependent Resistor) 5.4 kΩ Light, 1MΩ Dark, 2-Pin TO-18 NORPS-12 | RS Components (rs-online.com). [Accessed 13/12/2021].

[27] “rs-online.com”, 2021. [Online]. Available: Luna Optoelectronics, 100 V Through Hole LDR (Light Dependent Resistor) 20 kΩ Light, 20MΩ Dark, 2-Pin TO-18 NSL-19M51 | RS Components (rs-online.com). [Accessed 13/12/2021].

[28] “geeksforgeeks.org”, 2020. [Online]. Available: Advantages and disadvantages of GPS - GeeksforGeeks .[Accessed 16/12/2021].

[29] “www.aliexpress.com”, 2021. [Online]. Available: WAVGAT APM2.5 NEO 6M NEO M8N NEO M7N 0 000 GPS Module GYGPSV1 8M 3 5V GYGPSV5 NEO for Pixhawk APM|Integrated Circuits| - AliExpress. [Accessed 16/12/2021].

[30] “rs-online.com”, 2021. [Online]. Available: ADAFRUIT INDUSTRIES 1059 GPS Module | RS Components (rs-online.com). [Accessed 16/12/2021].

[31] “kingquad.shop”, 2021. [Online]. Available: NameLessRC Tiny GPS Module 5V 9600dps UBLOX (kingquad.shop) .[Accessed 16/12/2021].

[32] “uk.farnell.com”, 2021. [Online]. Available: HLMP-1301 Broadcom, LED, Red, Through Hole | Farnell. [Accessed 17/12/2021].

[33] “uk.farnell.com”, 2021. [Online]. Available: LL_11_1_Products_03_42_v4_cs3.indd (farnell.com). [Accessed 17/12/2021].

[34] “picstop.co.uk”, 2021. [Online]. Available: Transcend Secure Digital Card 1GB (picstop.co.uk). [Accessed 17/12/2021].

[35] “uk.farnell.com”, 2021. [Online]. Available: PIC18F45K50-I/P Microchip, 8 Bit MCU, Flash, PIC18 Family PIC18F K5x Series Microcontrollers | Farnell [Accessed 18/12/2021].

[36] “uk.farnell.com”, 2021. [Online]. Available: PIC18F45K50-I/MV Microchip, 8 Bit MCU, Flash, PIC18 Family PIC18F K5x Series Microcontrollers | Farnell. [Accessed 18/12/2021].

[37] “rs-online.com”, 2021. [Online]. Available: PIC18LF45K50-I/P Microchip, 8 Bit MCU, Flash, PIC18 Family PIC18F K5x Series Microcontrollers | Farnell [Accessed 18/12/2021].

[38] “www.large.net”, 2021. [Online]. Available: NiMH or Lithium Batteries-Pros and Cons-battery-knowledge | Large Power [Accessed 20/12/2021].

[39] “www.electronics-notes.com”, 2021. [Online]. Available: Li-Ion Battery Advantages / Disadvantages: Lithium Ion » Electronics Notes (electronics-notes.com) . [Accessed 20/12/2021].

[40] “www.rfwireless-world.com”, 2020. [Online]. Available: Advantages of Lead Acid Battery,disadvantages of Lead Acid Battery (rfwireless-world.com) . [Accessed 20/12/2021].

[41] “Amazon.co.uk”, 2021. [Online]. Available: QTshine Power Bank,【26800 mAh High Capacity】Portable Charger Fast Charge with 3 Outputs & Type-C Input External Battery Pack with 4 LED Lights Ultra Compact for Smart Phone, Tablet - Classic Black : Amazon.co.uk: Electronics & Photo. [Accessed 21/12/2021].

[42] “Amazon.co.uk”, 2021. [Online]. Available: Portable Charger 50000mAh Portable Charger Simple Stylish and Compact Design Quick Charge Smart Phones Tablet Camera etc (Black): Amazon.co.uk: Electronics & Photo. [Accessed 21/12/2021].

[43] “Amazon.co.uk”, 2021. [Online]. Available: MAXOAK 50000mAh 6 Port (5V/12V/ 20V) Portable Battery Charger External Battery Power Bank for Laptop & Notebook (not for Apple Laptop) : Amazon.co.uk: Computers & Accessories. [Accessed 21/12/2021].

[44] “rs-online.com”, 2021. [Online]. Available: EGG.TT.304.CLL | Lemo Panel Mount Circular Connector, 4 Contacts, M7 Connector | RS Components (rs-online.com) [Accessed 22/12/2021].

[45] “rs-online.com”, 2021. [Online]. Available: FGG.TT.304.CLAC31Z | Lemo Cable Mount Circular Connector, 4 Contacts | RS Components (rs-online.com) [Accessed 22/12/2021].

[46] “www.mouser.co.uk”, 2021. [Online]. Available: FMN.0M.304.ILCT LEMO | Mouser United Kingdom [Accessed 22/12/2021].

[47] “www.mouser.co.uk”, 2021. [Online]. Available: HEN.0M.304.ILNP LEMO | Mouser United Kingdom  [Accessed 22/12/2021].

[48] “rs-online.com”, 2021. [Online]. Available: RS PRO Cable Mount XLR Connector, Male, 250 V ac, 4 Way, Silver Plating | RS Components (rs-online.com) [Accessed 22/12/2021].

[49] “rs-online.com”, 2021. [Online]. Available: RS PRO Cable Mount XLR Connector, Female, 250 V ac, 4 Way, Silver Plating | RS Components (rs-online.com) [Accessed 22/12/2021].

[50] “www.thecaseshop.co.uk”, 2021. [Online]. Available: Max Case MAX300 - Case Empty - Black | The Case Shop [Accessed 26/12/2021].

[51] “www.cases-and-enclosures.co.uk”, 2021. [Online]. Available: Hurricane Waterproof and Shockproof Hard Equipment Case - Orange (L275xW203xH95.4mm) | cases-and-enclosures.co.uk Suppliers of Aluminium Flight Cases, Sealed Plastic Enclosures, Hurricane Equipment Cases, Custom Foam [Accessed 26/12/2021].

[52] “www.peliproducts.co.uk”, 2021. [Online]. Available: Peli iM2275 Storm Case - Equipment Hard Case - Peli UK (peliproducts.co.uk). [Accessed 26/12/2021].

[53] “cart.jlcpcb.com”, 2022. [Online]. Available: PCB Order & Online PCB Quote & SMT Assembly Quote - JLCPCB. [Accessed 1/01/2022].

# 19 Bibliography

Horowitz.P and Hill.W(1990) ‘The Art of Electronics’,2nd edition. Cambridge: Press Syndicate.

Microprossesor and interfacing programming and hardware by douglas V. Hall 1986

[1] “www.weather-station-products.co.uk”, 2019. [Online]. Available:  Wireless Vantage Pro2 Bundle (weather-station-products.co.uk) . [Accessed 1/12/2021].

[2] “www.weathershop.co.uk”, 2021. [Online]. Available: Davis Vantage Pro2 Wireless Weather Station & WeatherLink Live Bundle 6122 UK | Weather Shop ® UK . [Accessed 1/12/2021].

[3] “www.dycon.com”, 2019. [Online]. Available: MS-120 - Dyacon . [Accessed 1/12/2021].

[4] “intellisenseinc.com”, 2021. [Online]. Available: The All-in-One Weather Station with Cloud Height - The MWS-M625 (intellisenseinc.com) . [Accessed 1/12/2021].

[5] “electricalterminology.com”, 2021. [Online]. Available: Advantages and disadvantages of thermistor | 27 benefits and drawbacks (electricalterminology.com) . [Accessed 5/12/2021].

[6] “Sciencing.com”, 2019. [Online]. Available: Advantages and Disadvantages of Using Thermocouples (sciencing.com). [Accessed 5/12/2021].

[7] “rs-online.com”, 2021. [Online]. Available: HPC169_C HTM2500LF data sheet (rs-online.com). [Accessed 5/12/2021].

[8] “rotronic.com”, 2019. [Online]. Available: Webinar 2 v4 - Pros & Cons of Humidity Measurement Technologies - 9-17 (rotronic.com). [Accessed 5/12/2021].

[9] “rotronic.com”, 2019. [Online]. Available: Webinar 2 v4 - Pros & Cons of Humidity Measurement Technologies - 9-17 (rotronic.com). [Accessed 5/12/2021].

[10] “rs-online.com”, 2021. [Online]. Available: HPC169_C HTM2500LF data sheet (rs-online.com). [Accessed 5/12/2021].

[11] “uk.arnell.com”, 2021. [Online]. Available: Telaire T9602 Humidity & Temperature Sensor (farnell.com). [Accessed 5/12/2021].

[12] “omega.co.uk”, 2021. [Online]. Available: Humidity & Temperature Transmitters:Relative Humidity/Ambient Temperature Transmitters - HX94BV2W (omega.co.uk). [Accessed 5/12/2021].

[13] “rs-online.com”, 2021. [Online]. Available: RS PRO Relative Pressure Sensor for Air, Gas, Hydraulic Fluid, Liquid, Water, 10bar Max Pressure Reading, Voltage | RS Components (rs-online.com). [Accessed 7/12/2021].

[14] “rs-online”, 2021. [Online]. Available: RS PRO Retroreflective Photoelectric Sensor Block Sensor, 5 m Detection Range | RS Components (rs-online.com). [Accessed 7/12/2021].

[15] “uk.farnell.com”, 2021. [Online]. Available: 28015 Parallax, Sensing Range Min:2cm | Farnell. [Accessed 7/12/2021].

[16] “rs-online.com”, 2021. [Online]. Available: RS Pro Ultrasonic Sensor Barrel, 0.2 → 4 m, PCB | RS Components (rs-online.com). [Accessed 8/12/2021].

[17] “rs-online.com”, 2021. [Online]. Available: UM30-215113 | Sick M30 x 1.5 Ultrasonic Proximity Sensor - Barrel, Analogue Output, 600 → 6000 mm Detection, IP67 | RS Components (rs-online.com). [Accessed 8/12/2021].

[18] “lidarradar.com”, 2021. [Online]. Available: Advantages and Disadvantages of Doppler Radar – LiDAR and RADAR Information (lidarradar.com) . [Accessed 10/12/2021].

[19] “lidarradar.com”, 2021. [Online]. Available: Advantages and Disadvantages of LiDAR – LiDAR and RADAR Information (lidarradar.com). [Accessed 10/12/2021].

[20] “windpowerengineering.com”, 2021. [Online]. Available: Ultrasonic wind sensors or cup anemometers? (windpowerengineering.com). [Accessed 10/12/2021].

[21] “weatherspares.co.uk-”, 2021. [Online]. Available: https://weatherspares.co.uk/products/garni-940-wind-vane-940v?variant=32255807127648&currency=GBP&utm_medium=product_sync&utm_source=google&utm_content=sag_organic&utm_campaign=sag_organic. [Accessed 11/12/2021].

[22] “gillinstruments.com”, 2021. [Online]. Available: windsonic-1405-027-iss7.pdf (gillinstruments.com).[Accessed 11/12/2021].

[23] “gillinstruments.com”, 2021. [Online]. Available: 1390-0033 Iss 6 WindObserver70 plus OPTIONS.pdf (gillinstruments.com). [Accessed 11/12/2021].

[24] “uk.farnell.com”, 2019. [Online]. Available: WMS-23 Omega, ENV MONITORS, WEATHER STATION | Farnell. [Accessed 11/12/2021].

[25] “ecstuff4u.com”, 2020. [Online]. Available: ECSTUFF4U for Electronics Engineer: light sensor advantages and disadvantages. [Accessed 11/12/2021].

[26] “rs-online.com”, 2021. [Online]. Available: Luna Optoelectronics, 250 V Through Hole LDR (Light Dependent Resistor) 5.4 kΩ Light, 1MΩ Dark, 2-Pin TO-18 NORPS-12 | RS Components (rs-online.com). [Accessed 13/12/2021].

[27] “rs-online.com”, 2021. [Online]. Available: Luna Optoelectronics, 100 V Through Hole LDR (Light Dependent Resistor) 20 kΩ Light, 20MΩ Dark, 2-Pin TO-18 NSL-19M51 | RS Components (rs-online.com). [Accessed 13/12/2021].

[28] “geeksforgeeks.org”, 2020. [Online]. Available: Advantages and disadvantages of GPS - GeeksforGeeks .[Accessed 16/12/2021].

[29] “www.aliexpress.com”, 2021. [Online]. Available: WAVGAT APM2.5 NEO 6M NEO M8N NEO M7N 0 000 GPS Module GYGPSV1 8M 3 5V GYGPSV5 NEO for Pixhawk APM|Integrated Circuits| - AliExpress. [Accessed 16/12/2021].

[30] “rs-online.com”, 2021. [Online]. Available: ADAFRUIT INDUSTRIES 1059 GPS Module | RS Components (rs-online.com). [Accessed 16/12/2021].

[31] “kingquad.shop”, 2021. [Online]. Available: NameLessRC Tiny GPS Module 5V 9600dps UBLOX (kingquad.shop) .[Accessed 16/12/2021].

[32] “uk.farnell.com”, 2021. [Online]. Available: HLMP-1301 Broadcom, LED, Red, Through Hole | Farnell. [Accessed 17/12/2021].

[33] “uk.farnell.com”, 2021. [Online]. Available: LL_11_1_Products_03_42_v4_cs3.indd (farnell.com). [Accessed 17/12/2021].

[34] “picstop.co.uk”, 2021. [Online]. Available: Transcend Secure Digital Card 1GB (picstop.co.uk). [Accessed 17/12/2021].

[35] “uk.farnell.com”, 2021. [Online]. Available: PIC18F25K50-I/SO Microchip, 8 Bit MCU, Flash, PIC18 Family PIC18F K5x Series Microcontrollers | Farnell. [Accessed 18/12/2021].

[36] “uk.farnell.com”, 2021. [Online]. Available: PIC18F25K42-I/SP Microchip, 8 Bit MCU, PIC18 Family PIC18F K4x Series Microcontrollers, 64 MHz | Farnell. [Accessed 18/12/2021].

[37] “rs-online.com”, 2021. [Online]. Available: Microchip PIC18F252-I/SO, 8bit PIC Microcontroller, PIC18F, 40MHz, 32 kB Flash, 28-Pin SOIC | RS Components (rs-online.com). [Accessed 18/12/2021].

[38] “www.large.net”, 2021. [Online]. Available: NiMH or Lithium Batteries-Pros and Cons-battery-knowledge | Large Power [Accessed 20/12/2021].

[39] “www.electronics-notes.com”, 2021. [Online]. Available: Li-Ion Battery Advantages / Disadvantages: Lithium Ion » Electronics Notes (electronics-notes.com) . [Accessed 20/12/2021].

[40] “www.rfwireless-world.com”, 2020. [Online]. Available: Advantages of Lead Acid Battery,disadvantages of Lead Acid Battery (rfwireless-world.com) . [Accessed 20/12/2021].

[41] “Amazon.co.uk”, 2021. [Online]. Available: QTshine Power Bank,【26800 mAh High Capacity】Portable Charger Fast Charge with 3 Outputs & Type-C Input External Battery Pack with 4 LED Lights Ultra Compact for Smart Phone, Tablet - Classic Black : Amazon.co.uk: Electronics & Photo. [Accessed 21/12/2021].

[42] “Amazon.co.uk”, 2021. [Online]. Available: Portable Charger 50000mAh Portable Charger Simple Stylish and Compact Design Quick Charge Smart Phones Tablet Camera etc (Black): Amazon.co.uk: Electronics & Photo. [Accessed 21/12/2021].

[43] “Amazon.co.uk”, 2021. [Online]. Available: MAXOAK 50000mAh 6 Port (5V/12V/ 20V) Portable Battery Charger External Battery Power Bank for Laptop & Notebook (not for Apple Laptop) : Amazon.co.uk: Computers & Accessories. [Accessed 21/12/2021].

[44] “rs-online.com”, 2021. [Online]. Available: EGG.TT.304.CLL | Lemo Panel Mount Circular Connector, 4 Contacts, M7 Connector | RS Components (rs-online.com) [Accessed 22/12/2021].

[45] “rs-online.com”, 2021. [Online]. Available: FGG.TT.304.CLAC31Z | Lemo Cable Mount Circular Connector, 4 Contacts | RS Components (rs-online.com) [Accessed 22/12/2021].

[46] “www.mouser.co.uk”, 2021. [Online]. Available: FMN.0M.304.ILCT LEMO | Mouser United Kingdom [Accessed 22/12/2021].

[47] “www.mouser.co.uk”, 2021. [Online]. Available: HEN.0M.304.ILNP LEMO | Mouser United Kingdom  [Accessed 22/12/2021].

[48] “rs-online.com”, 2021. [Online]. Available: RS PRO Cable Mount XLR Connector, Male, 250 V ac, 4 Way, Silver Plating | RS Components (rs-online.com) [Accessed 22/12/2021].

[49] “rs-online.com”, 2021. [Online]. Available: RS PRO Cable Mount XLR Connector, Female, 250 V ac, 4 Way, Silver Plating | RS Components (rs-online.com) [Accessed 22/12/2021].

[50] “www.thecaseshop.co.uk”, 2021. [Online]. Available: Max Case MAX300 - Case Empty - Black | The Case Shop [Accessed 26/12/2021].

Wireless Vantage Pro2 Bundle (weather-station-products.co.uk)

MS-120 - Dyacon

The All-in-One Weather Station with Cloud Height - The MWS-M625 (intellisenseinc.com)

Bresser UK | BRESSER 4CAST PRO WLAN | Call 01342 837 098 for advice and sales.

Davis Vantage Pro2 Wireless Weather Station & WeatherLink Live Bundle 6122 UK | Weather Shop ® UK

MS-120 - Dyacon

The All-in-One Weather Station with Cloud Height - The MWS-M625 (intellisenseinc.com)

Bresser UK | BRESSER 4CAST PRO WLAN | Call 01342 837 098 for advice and sales.

RS PRO Thermistor 5Ω, 15 x 6mm | RS Components (rs-online.com)

RS PRO Type K Thermocouple 1m Length, 1mm Diameter → +800°C | RS Components (rs-online.com)

HPC169_C HTM2500LF data sheet (rs-online.com)

RS PRO Thermistor 5Ω, 15 x 6mm | RS Components (rs-online.com)

RS PRO Type K Thermocouple 1m Length, 1mm Diameter → +800°C | RS Components (rs-online.com)

HPC169_C HTM2500LF data sheet (rs-online.com)

HPC169_C HTM2500LF data sheet (rs-online.com)

T9602-5-A. Amphenol Advanced Sensors, HUMIDITY/TEMP SENSOR, 0-100% RH, IP67 | Farnell

Humidity & Temperature Transmitters:Relative Humidity/Ambient Temperature Transmitters - HX94BV2W (omega.co.uk)

RS PRO Relative Pressure Sensor for Air, Gas, Hydraulic Fluid, Liquid, Water, 10bar Max Pressure Reading, Voltage | RS Components (rs-online.com)

RS PRO Retroreflective Photoelectric Sensor Block Sensor, 5 m Detection Range | RS Components (rs-online.com)

28015 Parallax, Sensing Range Min:2cm | Farnell

28015 Parallax, Sensing Range Min:2cm | Farnell

PowerPoint Presentation (rs-online.com)

UM30-215113 | Sick M30 x 1.5 Ultrasonic Proximity Sensor - Barrel, Analogue Output, 600 → 6000 mm Detection, IP67 | RS Components (rs-online.com)

https://www.pinterest.co.uk/pin/655344183250909634/

LIDAR-Lite v3 - SEN-14032 - SparkFun Electronics

What are the Advantages of Ultrasonic Wind Sensors? | Lufft Blog

https://weatherspares.co.uk/products/garni-940-wind-vane-940v?variant=32255807127648&currency=GBP&utm_medium=product_sync&utm_source=google&utm_content=sag_organic&utm_campaign=sag_organic

What are the Advantages of Ultrasonic Wind Sensors? | Lufft Blog

windsonic-1405-027-iss7.pdf (gillinstruments.com)

1390-0033 Iss 6 WindObserver70 plus OPTIONS.pdf (gillinstruments.com)

WMS-23 Omega, ENV MONITORS, WEATHER STATION | Farnell

Photo Transistor Light Sensor– The Pi Hut

Light Sensor including Photocell and LDR Sensor (electronics-tutorials.ws)

Light Sensor including Photocell and LDR Sensor (electronics-tutorials.ws)

Luna Optoelectronics, 250 V Through Hole LDR (Light Dependent Resistor) 5.4 kΩ Light, 1MΩ Dark, 2-Pin TO-18 NORPS-12 | RS Components (rs-online.com)

Luna Optoelectronics, 100 V Through Hole LDR (Light Dependent Resistor) 20 kΩ Light, 20MΩ Dark, 2-Pin TO-18 NSL-19M51 | RS Components (rs-online.com)

Molex 206560-0050 Square Omnidirectional GSM & GPRS Antenna, GPS | RS Components (rs-online.com)

WAVGAT APM2.5 NEO 6M NEO M8N NEO M7N 0 000 GPS Module GYGPSV1 8M 3 5V GYGPSV5 NEO for Pixhawk APM|Integrated Circuits| - AliExpress

ADAFRUIT INDUSTRIES 1059 GPS Module | RS Components (rs-online.com)

NameLessRC Tiny GPS Module 5V 9600dps UBLOX (kingquad.shop)

HLMP-1301 Broadcom, LED, Red, Through Hole | Farnell

Transcend Secure Digital Card 1GB (picstop.co.uk)

PIC18F45K50-I/P Microchip, 8 Bit MCU, Flash, PIC18 Family PIC18F K5x Series Microcontrollers | Farnell

PIC18F45K50-I/MV Microchip, 8 Bit MCU, Flash, PIC18 Family PIC18F K5x Series Microcontrollers | Farnell

PIC18LF45K50-I/P Microchip, 8 Bit MCU, Flash, PIC18 Family PIC18F K5x Series Microcontrollers | Farnell

NiMH or Lithium Batteries-Pros and Cons-battery-knowledge | Large Power

Li-Ion Battery Advantages / Disadvantages: Lithium Ion » Electronics Notes (electronics-notes.com)

Advantages of Lead Acid Battery,disadvantages of Lead Acid Battery (rfwireless-world.com)

QTshine Power Bank,【26800 mAh High Capacity】Portable Charger Fast Charge with 3 Outputs & Type-C Input External Battery Pack with 4 LED Lights Ultra Compact for Smart Phone, Tablet - Classic Black : Amazon.co.uk: Electronics & Photo

Portable Charger 50000mAh Portable Charger Simple Stylish and Compact Design Quick Charge Smart Phones Tablet Camera etc (Black): Amazon.co.uk: Electronics & Photo

MAXOAK 50000mAh 6 Port (5V/12V/ 20V) Portable Battery Charger External Battery Power Bank for Laptop & Notebook (not for Apple Laptop) : Amazon.co.uk: Computers & Accessories

EGG.TT.304.CLL | Lemo Panel Mount Circular Connector, 4 Contacts, M7 Connector | RS Components (rs-online.com)

M Connector | LEMO Connectors | Push-Pull, Circular Connectors | Cables

RS PRO Cable Mount XLR Connector, Male, 250 V ac, 4 Way, Silver Plating | RS Components (rs-online.com)

⁣Max Case MAX300 - Case Empty - Black | The Case Shop

Hurricane Waterproof and Shockproof Hard Equipment Case - Orange (L275xW203xH95.4mm) | cases-and-enclosures.co.uk Suppliers of Aluminium Flight Cases, Sealed Plastic Enclosures, Hurricane Equipment Cases, Custom Foam

Peli iM2275 Storm Case - Equipment Hard Case - Peli UK (peliproducts.co.uk)

PCB Order & Online PCB Quote & SMT Assembly Quote - JLCPCB