| LOCATION: london | LLEOLIAD: llundain |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2016 | mai 2016 |

MODULE CODE: accc5002

LOCAL AREA NETWORKS

TIME:  2 HOURS

Local Area Networks

BSc Computer Networks (ACCC5002)

Practical Lab Assessment

Year 2/Level 5

May 2016

Name			:

Student ID		:

Date & Time		:

This is an individual practical lab-test, held under examination conditions. You will be assigned three Switches and a Router; you should configure these devices based on the given specifications.

This paper is marked out of 100.

You will have 2 hours to complete this task.

## TOPOLOGY DIAGRAM

VLAN 10 – Students: 	172.16.10.0/24

VLAN 20 – Staff: 		172.16.20.0/24

VLAN 99 – Management: 	172.16.99.0/24

## Addressing Table 									(10 marks)

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | Ge0/0.10 | 172.16.10.254 | 255.255.255.0 | N/A |
|  | Ge0/0.20 | 172.16.20.254 | 255.255.255.0 | N/A |
|  | Ge0/0.99 | 172.16.99.254 | 255.255.255.0 | N/A |
| S1 | VLAN99 | 172.16.99.1 | 255.255.255.0 | 172.16.99.254 |
| S2 | VLAN99 | 172.16.99.2 | 255.255.255.0 | 172.16.99.254 |
| S3 | VLAN99 | 172.16.99.3 | 255.255.255.0 | 172.16.99.254 |
| PC1 | NIC | DHCP | DHCP | DHCP |
| PC2 | NIC | DHCP | DHCP | DHCP |

## Port Assignments

## S1

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/2 – 0/5 | 802.1q Trunks (Native VLAN 99) | 172.16.99.0 |
| Fa0/1 | VLAN 10 (Students) | 172.16.10.0 |

## S2

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/2 – 0/5 | 802.1q Trunks (Native VLAN 99) | 172.16.99.0 |
| Fa0/1 | VLAN 20 (Staff) | 172.16.20.0 |

## S3

| Ports | Assignment | Network |
| --- | --- | --- |
| Fa0/1 – 0/5 | 802.1q Trunks (Native VLAN 99) | 172.16.99.0 |

Task 1: Prepare the Network (5 marks)

Step 1: Cable a network that is similar to the one in the topology diagram.

Step 2: Clear any existing configurations on the devices.

Task 2: Perform Basic Device Configurations (10 marks)

Configure the S1, S2, and S3 switches and R1 router according to the following guidelines:

- Configure the hostname.

- Disable DNS lookup.

- Configure an EXEC mode password.

- Configure a message-of-the-day banner.

- Configure a password for console connections.

- Configure synchronous logging.

- Configure a password for vty connections.

- Enable service password encryption.

Task 3: Router and Switch configuration instructions (50 marks)

- Configure S3 as a VTP server, and S1 & S2 as VTP clients in the example.com domain. Set the VTP password to ‘cisco’.

- Configure EtherChannel between the switches as per the topology diagram.

- Create VLANs 10, 20 & 99, and name as required.

- Assign ports to appropriate VLANs as shown in the Port Assignments table.

- Configure RSTP PVST+ on all switches.

- Configure S3 to be the primary root for all VLANs and S1 to be secondary.

- Configure PortFast and BPDU Guard on the interface connected to PC1 and PC2.

- Configure S1 as a DHCP server to provide IP addresses and default gateways to users on VLAN 10 & 20. Verify all PCs receive IP address details.

- Configure management IP addresses for all network switches in VLAN 99, and configure them for Telnet access.

- Configure router-on-a-stick inter-VLAN routing.

- Configure subinterfaces on R1 to operate on VLAN 10, 20, and 99 with the address from the Address table.

Task 4: After configuring each host computer, record the host network settings with the ipconfig /all command.  											(15 marks)

| Host PC1 Network Configuration |  |
| --- | --- |
| Description | DHCP |
| Physical Address | DHCP |
| IP Address | DHCP |
| Subnet Mask | DHCP |
| Default Gateway | DHCP |

| Host PC2 Network Configuration |  |
| --- | --- |
| Description | DHCP |
| Physical Address | DHCP |
| IP Address | DHCP |
| Subnet Mask | DHCP |
| Default Gateway | DHCP |

| Host PC3 Network Configuration |  |
| --- | --- |
| Description | DHCP |
| Physical Address | DHCP |
| IP Address | DHCP |
| Subnet Mask | DHCP |
| Default Gateway | DHCP |

Task 5: Document the Configurations (10 marks)

All documentation for your solution should be submitted. Your name should be clearly printed on the documents.

At the end of the test a pen drive will be given to you, the configuration files for all routers should be copied to the media and handed in to your instructor.  It is you responsibility that you copy the files correctly.

On each device, issue the following commands and capture the configurations. Save the file as devicename_surname_Pno.txt

show run

sh ip int brief

sh ip route

sh vlan

sh int trunk

sh spanning-tree

sh etherchannel summary

sh vtp status

Router R1:

R1#sh run

Building configuration...

Current configuration : 1117 bytes

!

version 12.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R1

!

!

!

enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

no ip domain-lookup

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface FastEthernet0/0

no ip address

ip helper-address 172.16.99.1

duplex auto

speed auto

!

interface FastEthernet0/0.10

encapsulation dot1Q 10

ip address 172.16.10.254 255.255.255.0

ip helper-address 172.16.99.1

!

interface FastEthernet0/0.20

encapsulation dot1Q 20

ip address 172.16.20.254 255.255.255.0

ip helper-address 172.16.99.1

!

interface FastEthernet0/0.99

encapsulation dot1Q 99

ip address 172.16.99.254 255.255.255.0

!

interface FastEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

!

ip flow-export version 9

!

!

!

banner motd ^CNO UNAUTHORIZED ACCESS^C

!

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

logging synchronous

login

!

!

!

end

R1#

Switch 1 – S1

S1#sh run

Building configuration...

Current configuration : 2166 bytes

!

version 12.2

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname S1

!

enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0

!

!

!

ip dhcp pool staff

network 172.16.20.0 255.255.255.0

default-router 172.16.20.254

ip dhcp pool students

network 172.16.10.0 255.255.255.0

default-router 172.16.10.254

no ip domain-lookup

!

!

spanning-tree mode rapid-pvst

spanning-tree vlan 10,20,99 priority 28672

!

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/2

switchport trunk native vlan 99

channel-group 1 mode active

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk native vlan 99

channel-group 1 mode active

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 99

channel-group 3 mode active

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk native vlan 99

channel-group 3 mode active

switchport mode trunk

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Port-channel 1

switchport trunk native vlan 99

switchport mode trunk

!

interface Port-channel 3

switchport trunk native vlan 99

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.99.1 255.255.255.0

!

ip default-gateway 172.16.99.254

!

banner motd ^CNO UNAUTHORIZED ACCESS^C

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line vty 0 4

password 7 0822455D0A16

logging synchronous

login

line vty 5 15

login

!

!

end

S1#

#

Switch 2 – S2

S2#sh run

Building configuration...

Current configuration : 1952 bytes

!

version 12.2

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname S2

!

enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0

!

!

!

no ip domain-lookup

!

!

spanning-tree mode rapid-pvst

!

interface FastEthernet0/1

switchport access vlan 20

switchport mode access

spanning-tree portfast

spanning-tree bpduguard enable

!

interface FastEthernet0/2

switchport trunk native vlan 99

channel-group 3 mode active

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk native vlan 99

channel-group 3 mode active

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 99

channel-group 2 mode active

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk native vlan 99

channel-group 2 mode active

switchport mode trunk

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Port-channel 2

switchport trunk native vlan 99

switchport mode trunk

!

interface Port-channel 3

switchport trunk native vlan 99

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.99.2 255.255.255.0

!

ip default-gateway 172.16.99.254

!

banner motd ^CNO UNAUTHORIZED ACCESS^C

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line vty 0 4

password 7 0822455D0A16

logging synchronous

login

line vty 5 15

login

!

!

end

S2#

Switch 3 – S3

S3#

S3#sh run

Building configuration...

Current configuration : 1942 bytes

!

version 12.2

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname S3

!

enable secret 5 $1$mERr$hx5rVt7rPNoS4wqbXKX7m0

!

!

!

no ip domain-lookup

!

!

spanning-tree mode rapid-pvst

spanning-tree vlan 10,20,99 priority 24576

!

interface FastEthernet0/1

switchport trunk native vlan 99

switchport mode trunk

!

interface FastEthernet0/2

switchport trunk native vlan 99

channel-group 1 mode active

switchport mode trunk

!

interface FastEthernet0/3

switchport trunk native vlan 99

channel-group 1 mode active

switchport mode trunk

!

interface FastEthernet0/4

switchport trunk native vlan 99

channel-group 2 mode active

switchport mode trunk

!

interface FastEthernet0/5

switchport trunk native vlan 99

channel-group 2 mode active

switchport mode trunk

!

interface FastEthernet0/6

!

interface FastEthernet0/7

!

interface FastEthernet0/8

!

interface FastEthernet0/9

!

interface FastEthernet0/10

!

interface FastEthernet0/11

!

interface FastEthernet0/12

!

interface FastEthernet0/13

!

interface FastEthernet0/14

!

interface FastEthernet0/15

!

interface FastEthernet0/16

!

interface FastEthernet0/17

!

interface FastEthernet0/18

!

interface FastEthernet0/19

!

interface FastEthernet0/20

!

interface FastEthernet0/21

!

interface FastEthernet0/22

!

interface FastEthernet0/23

!

interface FastEthernet0/24

!

interface GigabitEthernet0/1

!

interface GigabitEthernet0/2

!

interface Port-channel 1

switchport trunk native vlan 99

switchport mode trunk

!

interface Port-channel 2

switchport trunk native vlan 99

switchport mode trunk

!

interface Vlan1

no ip address

shutdown

!

interface Vlan99

ip address 172.16.99.3 255.255.255.0

!

ip default-gateway 172.16.99.1

!

banner motd ^CNO UNAUTHORIZED ACCESS^C

!

!

!

line con 0

password 7 0822455D0A16

logging synchronous

login

!

line vty 0 4

password 7 0822455D0A16

logging synchronous

login

line vty 5 15

login

!

!

end

S3#

S3#

S3#