| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2022 | mai 2022 |

Solutions

MODULE CODE: GOEE4003

Electrical and electronics principles

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator, Formula Sheet

Question One (25 marks)

For the circuit shown in Figure 1 below:-

- Calculate the resistance of the circuit within the dashed lines designated

the LOAD (i.e. R3 to R7 inclusive)	                                    [10 marks]

- Calculate the voltage at point A 				        	  [5 marks]

- Calculate the total power dissipated.  In the LOAD	          		  [3 marks]

- Calculate the value of I1 and I2					  [7 marks]

Figure 1

solution

R34=R3×R4R3+R4=8×128+12= 9620

R34=4.8 Ω                   [3 marks]

R346=R34+R6=  4.8+10=14.8  Ω      2 marks

R3456=R346×R5R346+R5=14.8×1014.8+10= 14824.8=5.97 Ω        [3 marks]

RL=R3456+R7=5.97+2

RT=7.97 Ω          [2 marks]

IT=8A-5A=3A

IT=3A

RL=7.97  Ω

VA=IT×RL=3A ×7.97 Ω

VA=23.91 V            [5 marks]

PL=IT2×RL=3 2×7.97

PL=71.7 W           [3 marks]

I1=R346R346+R5IT

I1=14.814.8+103= 14.824.83

I1=1.79 A

I2=R5R346+R5IT

I2= 1010+14.83= 1024.83

I2=1.209 A         [7 marks]

Question Two (15 marks)

An AC circuit is composed of a serial connection of a resistor with resistance 50 Ω, a coil with inductance 0.3 H and a capacitor with capacitance 15 μF. The circuit is connected to an AC voltage source with amplitude 25 V and frequency 50 Hz (see Figure 2). Determine:

- The inductive reactance XL  							 [3 marks]

- The capacitive reactance XC  							 [3 marks]

- The total impedance Z.							 [3 marks]

- The electric current in the circuit. 						 [3 marks]

- The phase difference between the voltage and the current. 			 [3 marks]

Figure 2

Solutions

XL=2πfL

XL=2π50(0.3)

XL=94.2 Ω                   [3 marks]

XC=12πfC

XC=12π50(15×10-6)

XC=212.2 Ω           [3 marks]

Z=   R2 +(XL-XC)2

Z=   502 +(94.2-212.2 )2

Z=128 Ω            [3 marks]

I=VZ

I=25128

I=0.2 A              [3 marks]

φ= cos-1  RZ

φ= cos-1  50128

φ= 67 °          [3 marks]

Question Three (30 Marks)

- For the circuit in Figure 3, find the branch current I1, I2 and I3 using mesh analysis.

[15 marks]

Figure 3

Solutions:

Obtain the mesh currents using KVL. For mesh 1.

-15+5I1+10I1-I2+10=0

-15+5I1+10I1-10I2+10=0

15I1-10I2=5

3I1-2I2=1       [5 marks]

For mesh 2:

-10+10I2-I1+6I2+4I2=0

-10+10I2-10I1+6I2+4I2=0

-10I1+20I2=10

-I1+2I2=1         [5 marks]

3I1-2I2=1                (1)

-I1+2I2=1              (2)

Add equation1 to equation2 gives:

3I1-2I2=1                (1)

-I1+2I2=1              (2)

2I1=2

I1=1 A

3(1)-2I2=1

2I2=2

I2=1  A

I3= I1-I2

I3= 0 A

Solution      I1=1 A,     I2=1 A,      I3=0 A          [5 marks]

- For the circuit in Figure 4 below.

- Determine the “Thevenin” equivalent circuit, between points “A” and “B”.

- Calculate the load power if a 4Ω load is attached to the terminals A and B.

[15 marks]

Figure 4

Solution

VTH=R2R1+R2VS

VTH=66+410

VTH=61010

VTH=6 V              [5marks]

RTH= R1×R2R1+R2+R3

RTH= 6×46+4+3

RTH= 2410+3

RTH= 5.4 Ω       5marks

VTH

IT= 65.4+4

IT= 0.638 A

IL=IT

PL=IL2 ×RL

PL=0.6382 ×4

PL=1.628 W        [5marks]

VTH=6 volts        RTH=5.4Ω      PL=1.63 W

Question Foure (30 Marks)

- For the circuit of Figure 5, calculate:

- The base-emitter bias voltage VBE

- The collector-emitter voltage (the voltage across the transistor) VCE

[10 marks]

Figure 5

VE= IE RE

IE=1 mA

RE=1 KΩ

VE=1 mA × 1 KΩ

VE=1 V

VB=10KΩ10KΩ+100KΩ×VCC

VB=10KΩ110KΩ×9V

VB=0.81V

VBE=VB-VE

VBE=0.81V-1V

VBE=0.19 V     [5marks]

Consider      IC≈ IE    so IC ≈1 mA

RL=3.9 kΩ

VL=IC RL=1 mA × 3.9 kΩ

VL=3.9 V

The voltage across the transistor VCE is therefore given by:

VCE= VCC-VL-VE

VCE= 9V-3.9V-1V

VCE= 4.1 V    [5marks]

- Draw the logic circuit design for the following Boolean equations below.

- output=(A.B+C)D

- output=(A.B+(B+C)

[8 marks]

Solution

- Write Boolean algebraic expressions for the circuit below.

[6 marks]

Solutions

- Z=A.B   +C

- Z=A .(B+ B.C)

- Simplify the output function Y using DeMorgan’s theorem.

Y=(A.B )(A+C)

[6 marks]

Solution

Y=(A.B )(A+C) = A.B +(A+C)

Y=(A.B)+(A .C)

Y=(A.B)+(A .C)

Y=AB+AC        [6marks]