| Activity being assessed: |  |  |  |  |  | General site hazards |  |  |  |  |  |  |  |  | Date: |  | April 2024 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  |  |  |  | Prepared by: |  |  |  |  | Elliott Lewis |  |  |  |  |  |  |
| People at risk: |  |  | Employees ☒ Other contractors ☒ Site visitors ☒ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PPE Required: |  | ☒ ☒ ☒ ☐ ☐ ☐ ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  |  | Potential Harm |  |  |  | P | S | RR |  | How to make it safer….. |  |  |  | P | S | RRR |
| - Slips - Trips - Falls |  |  |  |  | Injury to self and others if not suitably trained and unfamiliar with the site and control measures in place |  |  |  | 4 | 4 | 16 |  | - All staff will be receiving a site specific induction by the site manager and follow all project manager safety rules. - All work will be carried out by trained and competent staff, holding CSCS cards. - All staff will be appropriately dressed for with PPE, including safety boots, long trouser, hard hats and high visibility tabards, which must be worn at all times. - Staff will liaise with site manager and other contractors on a daily basis to ensure works across site are on schedule to avoid activities clashing, increasing risk on site - Apprentices will be supervised by an experienced member of staff. - Hand and power tools will not be left unattended. - The site will be made safe at the end of each shift with tools and materials securely stored to deter theft and accidents to others. |  |  |  | 2 | 2 | 4 |
| Probability (P) |  |  |  | Severity (S) |  |  | Risk Rating (RR): Probability X Severity (PXS) |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. Improbable |  |  |  | 1. No risk of injury or disease |  |  | Residual Risk |  |  |  |  | Action |  |  |  |  |  |  |  |
| 2. Remote |  |  |  | 2. Minor injury |  |  | 1-6 LOW |  |  |  |  | Reduce risk wherever possible. |  |  |  |  |  |  |  |
| 3. Possible |  |  |  | 3. RIDDOR > 3 day off work |  |  | 7-14 MEDIUM |  |  |  |  | Only proceed with management authorisation |  |  |  |  |  |  |  |
| 4. Probable |  |  |  | 4. RIDDOR major injury |  |  |  |  |  |  |  | Consult with assessment personnel, re-define and reduce risk. |  |  |  |  |  |  |  |
| 5. Likely |  |  |  | 5. One or more fatalities |  |  | 15–25 HIGH |  |  |  |  | Do NOT proceed. Re-define, re-assess and reduce risk. |  |  |  |  |  |  |  |

| Activity being assessed: |  |  |  |  | Use of cartridge operated tools |  |  |  |  |  |  |  |  | Date: | April 2024 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  |  |  | Prepared by: |  |  |  |  | Elliott Lewis |  |  |  |  |  |
| People at risk: |  | Employees ☒ Other contractors ☒ Site visitors ☐ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  | Potential Harm |  |  |  | P | S | RR |  | How to make it safer….. |  |  | P | S | RRR |
| Wrong power setting for type of fixing and type of material. Soft material is punctured by nail/fastener when misfired. Use of incorrect cartridge/misfire. Ricochet of fastener or fixing device. Negligent discharge of projectile. Fragmentation of brittle materials during firing. Recoil throwing operative off balance. Exposure to noise. |  |  |  | Operatives and others sustaining fatal or severe puncture injuries Lacerations to eyes and exposed skin Serious injury or even fatal injury could occur if a worker falls. Hearing loss Tinnitus |  |  |  | 4 | 4 | 16 |  | - Operatives must be trained and competent, certificated and authorised to use the cartridge tool - a record will be kept of those authorised. - Operatives shall wear impact eye protection and head protection during loading, use and unloading. - As cartridges are colour coded no person with defective colour vision may operate the tool. - Prior to work, test fixings shall be carried out especially in concrete and masonry. - Cartridge operated tools will be held firmly and flush with the surface and fired at an angle of 90°. - Brittle surfaces will be covered. - Any new fixing must be 50mm away from an existing hole. - Only fixing devices designed for use with the tool shall be used. - Guards to be in place and of sufficient size to check flying pins and debris - Other persons shall be cleared from the danger area. - Tool to be unloaded and made safe when not in use, and stored in a coal dry place. - Tools and cartridges shall be held under lock and key in secure containers when not in use and issues suitably controlled. - Any unfired or misfired cartridges should be handed to the site supervisor for safe disposal - Cartridge operated tools will only be used from stable position on secure and safe platforms. |  |  | 2 | 3 | 6 |
| Probability (P) |  |  | Severity (S) |  |  | Risk Rating (RR): Probability X Severity (PXS) |  |  |  |  |  |  |  |  |  |  |  |
| 1. Improbable |  |  | 1. No risk of injury or disease |  |  | Residual Risk |  |  |  |  | Action |  |  |  |  |  |  |
| 2. Remote |  |  | 2. Minor injury |  |  | 1-6 LOW |  |  |  |  | Reduce risk wherever possible. |  |  |  |  |  |  |
| 3. Possible |  |  | 3. RIDDOR > 3 day off work |  |  | 7-14 MEDIUM |  |  |  |  | Only proceed with management authorisation |  |  |  |  |  |  |
| 4. Probable |  |  | 4. RIDDOR major injury |  |  |  |  |  |  |  | Consult with assessment personnel, re-define and reduce risk. |  |  |  |  |  |  |
| 5. Likely |  |  | 5. One or more fatalities |  |  | 15–25 HIGH |  |  |  |  | Do NOT proceed. Re-define, re-assess and reduce risk. |  |  |  |  |  |  |

| Activity being assessed: |  |  |  |  | Use of nailing tool – Pneumatic/Gas |  |  |  |  |  |  |  |  | Date: | April 2024 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  |  |  | Prepared by: |  |  |  |  | Elliott Lewis |  |  |  |  |  |
| People at risk: |  | Employees ☒ Other contractors ☒ Site visitors ☒ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  | Potential Harm |  |  |  | P | S | RR |  | How to make it safer….. |  |  | P | S | RRR |
| Puncture/piercing injury Misuse Injury to eyes Falls from height Malfunction Exposure to noise |  |  |  | Severe lacerations injuries to exposed eyes and skin Loss of sight Hearing loss/tinnitus Serious injury or death if worker falls. |  |  |  | 4 | 4 | 16 |  | - The operator shall be competent, with certification to ensure they have been fully instructed, in accordance with the maker's recommendations, in the safe operation of the tool and the safety precautions to be observed. - Operators shall be authorised. - Only to be used in accordance with makers instructions - Tool shall be unloaded and made safe when not in use. - The operator shall wear suitable eye protection during loading, use and unloading. - Operator and other persons affected shall wear suitable ear protection. - Nailing tools will only be used from stable position on secure and safe platforms. - Before driving fasteners, check that no one is present immediately behind or below the working surface. - Spent gas canisters shall be disposed of in a safe manner, i.e. given to the site supervisor for safe disposal. |  |  | 2 | 3 | 6 |
| Probability (P) |  |  | Severity (S) |  |  | Risk Rating (RR): Probability X Severity (PXS) |  |  |  |  |  |  |  |  |  |  |  |
| 1. Improbable |  |  | 1. No risk of injury or disease |  |  | Residual Risk |  |  |  |  | Action |  |  |  |  |  |  |
| 2. Remote |  |  | 2. Minor injury |  |  | 1-6 LOW |  |  |  |  | Reduce risk wherever possible. |  |  |  |  |  |  |
| 3. Possible |  |  | 3. RIDDOR > 3 day off work |  |  | 7-14 MEDIUM |  |  |  |  | Only proceed with management authorisation |  |  |  |  |  |  |
| 4. Probable |  |  | 4. RIDDOR major injury |  |  |  |  |  |  |  | Consult with assessment personnel, re-define and reduce risk. |  |  |  |  |  |  |
| Activity being assessed: |  |  |  |  | Manual Handling |  |  |  |  |  |  | Assessment No: |  | Date: | April 2024 |  |  |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  |  |  | Prepared by: |  |  |  |  | Elliott Lewis |  | Review Date: |  |  |  |
| People at risk: |  | Employees ☒ Other contractors ☐ Site visitors ☐ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  | Potential Harm |  |  |  | P | S | RR |  | How to make it safer….. |  |  | P | S | RRR |
| - Heavy loads - Awkward shaped loads - Unstable contents - Loads at low level - Loads at high level - Sharp edges - Excessive distance - Obstructions on route |  |  |  | -Muscular skeletal injury -Strains -Sprains -Cuts/abrasions -Dropping of load injuring self and/or others |  |  |  | 3 | 3 | 9 |  | - Loads will be mechanically positioned as close as possible to its final position. - Lifting aids will be employed where practical. - Staff have received training in safe lifting methods. Implement TILE assessment principal: Task - what to where Individual(s) - Suitably fit, willing and able Load - Heavy, hot awkward or sharp Environment - free from obstruction and changes in height, level. - Team lifting will be carried out under the direction of a competent coordinator. - Team members will be fully fit and of similar abilities. - Area of lift and route along which it is to be carried must be cleared of any slip, trip or fall hazards. |  |  | 2 | 2 | 4 |
| Probability (P) |  |  | Severity (S) |  |  | Risk Rating (RR): Probability X Severity (PXS) |  |  |  |  |  |  |  |  |  |  |  |
| 1. Improbable |  |  | 1. No risk of injury or disease |  |  | Residual Risk |  |  |  |  | Action |  |  |  |  |  |  |
| 2. Remote |  |  | 2. Minor injury |  |  | 1-6 LOW |  |  |  |  | Reduce risk wherever possible. |  |  |  |  |  |  |
| 3. Possible |  |  | 3. RIDDOR > 3 day off work |  |  | 7-14 MEDIUM |  |  |  |  | Only proceed with management authorisation |  |  |  |  |  |  |
| 4. Probable |  |  | 4. RIDDOR major injury |  |  |  |  |  |  |  | Consult with assessment personnel, re-define and reduce risk. |  |  |  |  |  |  |
| 5. Likely |  |  | 5. One or more fatalities |  |  | 15–25 HIGH |  |  |  |  | Do NOT proceed. Re-define, re-assess and reduce risk. |  |  |  |  |  |  |

| Activity being assessed: |  |  |  | Exposure to noise |  |  |  |  | Assessment No: |  | Date: | 09/02/2024 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  | Prepared by: |  |  |  | Elliott Lewis |  | Review Date: | 08/02/2025 |  |  |
| People at risk: |  | Employees ☒ Other contractors ☒ Site visitors ☐ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  | Potential Harm |  |  | P | S | RR | How to make it safer….. |  |  | P | S | RRR |
| - Noise levels between 80 – 85dB - Noise levels between 85 - 90dB |  |  | Hearing loss, tinnitus. |  |  | 3 | 3 | 9 | - Low noise tools have been selected where possible and appropriate - Consideration will be given to the selection of appropriate tools, and quieter methods of working selected where appropriate (residential/built up areas) - If noise exposure is between 80 - 85dB, personnel will be provided with hearing protectors if requested. - For noise exposure between 85 - 90dB, provide personnel with hearing protectors and ensure their proper use. - Areas where the use of hearing protection is compulsory, will be identified and mark them with suitable signage. - Provide personnel with training and information on how to use and care for the hearing protectors. - Make sure the protectors give enough protection - Select a protector with an SNR of: - 85-90: 20 or less - 90-95: 20-30 - 95-100: 25-35 - 100-105: 30 or more. Aim at least to get below 85 dB at the ear. - Make sure the protectors are compatible when worn with other protective equipment (e.g. hard hats, dust masks and eye protection). |  |  | 1 | 1 | 2 |

| Probability (P) | Severity (S) | Risk Rating (RR): Probability X Severity (PXS) |  |
| --- | --- | --- | --- |
| 1. Improbable | 1. No risk of injury or disease | Residual Risk | Action |
| 2. Remote | 2. Minor injury | 1-6 LOW | Reduce risk wherever possible. |
| 3. Possible | 3. RIDDOR > 3 day off work | 7-14 MEDIUM | Only proceed with management authorisation |
| 4. Probable | 4. RIDDOR major injury |  | Consult with assessment personnel, re-define and reduce risk. |
| 5. Likely | 5. One or more fatalities | 15–25 HIGH | Do NOT proceed. Re-define, re-assess and reduce risk. |

| Activity being assessed: |  |  |  |  | Use of hand tools |  |  |  |  |  |  |  |  | Date: | April 2024 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  |  |  | Prepared by: |  |  |  |  | Elliott Lewis |  |  |  |  |  |
| People at risk: |  | Employees ☒ Other contractors ☒ Site visitors ☐ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  | Potential Harm |  |  |  | P | S | RR |  | How to make it safer….. |  |  | P | S | RRR |
| Inappropriate application causing injury to self or others Damage to tools and/or works being undertaken |  |  |  | - Eye injuries caused by flying Particles - Injury from contact with tool or workpiece (bruising, crushing, cuts, etc) - Strains and sprains from incorrect use - Injury caused by tripping over tools (due to poor housekeeping) |  |  |  |  |  |  |  | - Tools must not be used for anything other than intended purpose. do not use a screwdriver as a chisel. - Staff to carry out visual checks each day before use, and remove and report any damaged tools. - Eye protection must be worn when using tools (i.e. chisels, hand saw) where there is a risk of flying particles. - Open-bladed knives, screwdrivers and other sharp tools must not be carried in pockets or in a manner which could cause injury to self for other. - When using Knife: cut away from the body, secure the piece and wear approved gloves to present lacerations. - Sharpening is a skilled task and must only be caried out by trained and authorised persons. - Tools must not be left lying around, but returned to storage after use. - Hearing protection must be worn if carrying out sustained hammering or other noisy tasks. |  |  |  |  |  |
| Probability (P) |  |  | Severity (S) |  |  | Risk Rating (RR): Probability X Severity (PXS) |  |  |  |  |  |  |  |  |  |  |  |
| 1. Improbable |  |  | 1. No risk of injury or disease |  |  | Residual Risk |  |  |  |  | Action |  |  |  |  |  |  |
| 2. Remote |  |  | 2. Minor injury |  |  | 1-6 LOW |  |  |  |  | Reduce risk wherever possible. |  |  |  |  |  |  |
| 3. Possible |  |  | 3. RIDDOR > 3 day off work |  |  | 7-14 MEDIUM |  |  |  |  | Only proceed with management authorisation |  |  |  |  |  |  |
| 4. Probable |  |  | 4. RIDDOR major injury |  |  |  |  |  |  |  | Consult with assessment personnel, re-define and reduce risk. |  |  |  |  |  |  |
| 5. Likely |  |  | 5. One or more fatalities |  |  | 15–25 HIGH |  |  |  |  | Do NOT proceed. Re-define, re-assess and reduce risk. |  |  |  |  |  |  |

| Activity being assessed: |  |  |  |  | Circular Saw 110v |  |  |  |  |  |  | Assessment No: |  | Date: | 09/02/2024 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  |  |  | Prepared by: |  |  |  |  | Elliott Lewis |  | Review Date: | 08/02/2025 |  |  |
| People at risk: |  | Employees ☒ Other contractors ☒ Site visitors ☐ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  | Potential Harm |  |  |  | P | S | RR |  | How to make it safer….. |  |  | P | S | RRR |
| Wood Dust Noise Poorly maintained equipment |  |  |  | Eye injury from wood dust or splinter Personal injury - laceration/amputation from saw blade Electric shock (110 volt), personal injury, burns II-health from inhalation of wood dust (soft wood) Noise induced hearing loss II-health from effect of vibration (Hand/Arm Vibration (HAY) |  |  |  |  |  |  |  | - Eye protection recommended to be worn. - Equipment to be maintained in good working order, with all fitted guards being operational (spring loaded and self returning). - Hands/arms and loose clothing to be kept clear of working area. - Daily inspection required by user prior to use. - Where possible, equipment should be used externally to allow dust to disperse. - Disposable respirator should be worn if used in an enclosed space or if dust is excessive. - Hearing protection must be worn if noise levels are in excess of85 dB(A). - Timber being cut should be adequately supported. |  |  |  |  |  |
| Probability (P) |  |  | Severity (S) |  |  | Risk Rating (RR): Probability X Severity (PXS) |  |  |  |  |  |  |  |  |  |  |  |
| 1. Improbable |  |  | 1. No risk of injury or disease |  |  | Residual Risk |  |  |  |  | Action |  |  |  |  |  |  |
| 2. Remote |  |  | 2. Minor injury |  |  | 1-6 LOW |  |  |  |  | Reduce risk wherever possible. |  |  |  |  |  |  |
| 3. Possible |  |  | 3. RIDDOR > 3 day off work |  |  | 7-14 MEDIUM |  |  |  |  | Only proceed with management authorisation |  |  |  |  |  |  |
| 4. Probable |  |  | 4. RIDDOR major injury |  |  |  |  |  |  |  | Consult with assessment personnel, re-define and reduce risk. |  |  |  |  |  |  |
| 5. Likely |  |  | 5. One or more fatalities |  |  | 15–25 HIGH |  |  |  |  | Do NOT proceed. Re-define, re-assess and reduce risk. |  |  |  |  |  |  |

| Activity being assessed: |  |  |  |  | Young Persons |  |  |  |  |  |  |  |  | Date: | 09/02/2024 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  |  |  | Prepared by: |  |  |  |  | Elliott Lewis |  | Review Date: | 08/02/2025 |  |  |  |
| People at risk: |  | Employees ☒ Other contractors ☒ Site visitors ☐ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  | Potential Harm |  |  |  | P | S | RR |  | How to make it safer….. |  |  | P | S | RRR |  |
| Inexperienced Overly confident |  |  |  | Minor, major or fatal injuries from lack of experience and understanding of the workplace. |  |  |  |  |  |  |  | - Young persons shall receive specific site induction training. - Consideration of the work to be carried out, and the subject's capability to do so, will cover the following points: - - Their inexperience, lack of awareness of risks, and maturity. - Their place of work will be assessed to ensure it is safe and the young person shall be advised to leave area if they feel it has become unsafe and report this to their supervisor. - The use of work equipment will be controlled - they will be trained to ensure they are aware of the safe working practices and the dangers posed by the equipment. - Young persons shall be prevented from using work equipment or plant which is high risk, i.e. equipment producing high levels of noise and/or vibration; (assess all plant and work equipment prior to work). - Young person's work activities shall be organised to ensure they are not put in unsafe or unfamiliar situations. - Young persons shall be prevented from using, or exposed to substances hazardous to health. - A competent person shall supervise young persons. - Supervisors shall monitor regularly the extent of the health and safe training provided. |  |  |  |  |  |  |
| Probability (P) |  |  | Severity (S) |  |  | Risk Rating (RR): Probability X Severity (PXS) |  |  |  |  |  |  |  |  |  |  |  |
| 1. Improbable |  |  | 1. No risk of injury or disease |  |  | Residual Risk |  |  |  |  | Action |  |  |  |  |  |  |
| 2. Remote |  |  | 2. Minor injury |  |  | 1-6 LOW |  |  |  |  | Reduce risk wherever possible. |  |  |  |  |  |  |
| 3. Possible |  |  | 3. RIDDOR > 3 day off work |  |  | 7-14 MEDIUM |  |  |  |  | Only proceed with management authorisation |  |  |  |  |  |  |
| 4. Probable |  |  | 4. RIDDOR major injury |  |  |  |  |  |  |  | Consult with assessment personnel, re-define and reduce risk. |  |  |  |  |  |  |
| 5. Likely |  |  | 5. One or more fatalities |  |  | 15–25 HIGH |  |  |  |  | Do NOT proceed. Re-define, re-assess and reduce risk. |  |  |  |  |  |  |

| Activity being assessed: |  |  |  |  | Crane lifting operations |  |  |  |  |  |  | Assessment No: |  | Date: | 09/02/2024 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  |  |  | Prepared by: |  |  |  |  | Elliott Lewis |  | Review Date: | 08/02/2025 |  |  |
| People at risk: |  | Employees ☒ Other contractors ☐ Site visitors ☐ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  | Potential Harm |  |  |  | P | S | RR |  | How to make it safer….. |  |  | P | S | RRR |
| Machine over-turning Materials or equipment during lifting operation Failure of lifting gear or equipment |  |  |  | Operatives and others sustaining fatal or severe crush injuries Death or loss of limbs Operatives and persons nearby suffering serious or fatal burns Severe injury or death by electric shock |  |  |  |  | 4 | 16 |  | - Craning operations will be organised and controlled by an 'Appointed Person', and will be carried out in accordance with the Lifting Operations and Lifting Equipment Regulations (LOLER) and BS 7121- a Lifting Plan will be prepared. - The Appointed Person will be responsible to ensure that inspections and maintenance of all lifting gear has been carried out. - The crane operator will be trained and competent, and be responsible for safe operation, daily maintenance and completion of records relating to the crane and lifting gear. - Slingers/signallers will be adequately trained and competent, able to communicate clearly and be aware of their duties – certificates of training will be retained on site. They will be required to know the weight of materials to be raised before commencing a lift, and to ensure stability and security of loads. - The surface from which mobile crane is to operate will be prepared and checked to ensure stability. - Machinery and accessories used for lifting loads must be clearly marked with its safe working load (SWL)- this must not be exceeded. - If the crane operator cannot see the load during the whole of the lifting operation, an additional slinger/signaller or radio communication will be used. Where overhead power lines are likely to cause danger, the electricity distributor who owns the lines should always be contacted to calculate the safe working distances - these will depend on voltage, maximum sag and lateral swing (also see HSEGuidanceNote GS6). |  |  | 2 | 2 | 4 |
|  |  |  |  |  |  |  |  |  |  |  |  | - A 600mm wide clearance should be provided whenever practicable belween travelling and slewing cranes and any fixture (e.g. adjacent buildings, guardrails, access, etc.). If the clearance is not practicable, then access should be barred off. - Where reasonably practicable, the area covered by the crane operation should be kept clear of other work operations, pedestrian access etc. - When working over pavements or access ways, the operator must ensure that no loads are lifted over persons at any time. |  |  |  |  |  |
| Probability (P) |  |  | Severity (S) |  |  | Risk Rating (RR): Probability X Severity (PXS) |  |  |  |  |  |  |  |  |  |  |  |
| 1. Improbable |  |  | 1. No risk of injury or disease |  |  | Residual Risk |  |  |  |  | Action |  |  |  |  |  |  |
| 2. Remote |  |  | 2. Minor injury |  |  | 1-6 LOW |  |  |  |  | Reduce risk wherever possible. |  |  |  |  |  |  |
| 3. Possible |  |  | 3. RIDDOR > 3 day off work |  |  | 7-14 MEDIUM |  |  |  |  | Only proceed with management authorisation |  |  |  |  |  |  |
| 4. Probable |  |  | 4. RIDDOR major injury |  |  |  |  |  |  |  | Consult with assessment personnel, re-define and reduce risk. |  |  |  |  |  |  |
| 5. Likely |  |  | 5. One or more fatalities |  |  | 15–25 HIGH |  |  |  |  | Do NOT proceed. Re-define, re-assess and reduce risk. |  |  |  |  |  |  |

| Activity being assessed: |  |  |  |  | Extreme Weather conditions |  |  |  |  |  |  | Assessment No: |  | Date: | 09/02/2024 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  |  |  | Prepared by: |  |  |  |  | Elliott Lewis |  | Review Date: | 08/02/2025 |  |  |
| People at risk: |  | Employees ☒ Other contractors ☒ Site visitors ☐ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  | Potential Harm |  |  |  | P | S | RR |  | How to make it safer….. |  |  | P | S | RRR |
| Exposure to hot temperatures and sunlight Exposure to excessively cold temperatures Slippery surfaces |  |  |  | - Heat exhaustion/sun stroke - Sun burn - Hypothermia - Increased fatigue - Slips and strains due to icy conditions |  |  |  | 3 | 3 | 9 |  | - Consideration will be taken when working outdoors/in exposed areas for the current weather conditions. - Where weather warnings have been issued, Alpha Construction will assess the risk to employees in terms of travel to site and working conditions, making alternative arrangements where possible, to ensure staff safety is not compromised. - Alpha Construction staff are provided with uniform including warm high vis trousers and Jackets, cap and hat and waterproofs to help protect against the elements. Staff must select the correct clothing for the conditions. - Welfare facilities providing shade or warmth must be provided by the main contractor. - Frequent breaks may be required in extreme hot or cold conditions. This will be agreed with Alpha Construction team leader on the day. |  |  | 2 | 1 | 2 |
| Probability (P) |  |  | Severity (S) |  |  | Risk Rating (RR): Probability X Severity (PXS) |  |  |  |  |  |  |  |  |  |  |  |
| 1. Improbable |  |  | 1. No risk of injury or disease |  |  | Residual Risk |  |  |  |  | Action |  |  |  |  |  |  |
| 2. Remote |  |  | 2. Minor injury |  |  | 1-6 LOW |  |  |  |  | Reduce risk wherever possible. |  |  |  |  |  |  |
| 3. Possible |  |  | 3. RIDDOR > 3 day off work |  |  | 7-14 MEDIUM |  |  |  |  | Only proceed with management authorisation |  |  |  |  |  |  |
| 4. Probable |  |  | 4. RIDDOR major injury |  |  |  |  |  |  |  | Consult with assessment personnel, re-define and reduce risk. |  |  |  |  |  |  |
| 5. Likely |  |  | 5. One or more fatalities |  |  | 15–25 HIGH |  |  |  |  | Do NOT proceed. Re-define, re-assess and reduce risk. |  |  |  |  |  |  |

| Activity being assessed: |  |  |  |  | Slinging of loads |  |  |  |  |  |  | Assessment No: |  | Date: | 09/02/2024 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location: | Bethany Square Car Park, Bethany Square, 87 Station Rd, Port Talbot SA13 1PQ |  |  |  |  |  | Prepared by: |  |  |  |  | Elliott Lewis |  | Review Date: | 08/02/2025 |  |  |
| People at risk: |  | Employees ☒ Other contractors ☒ Site visitors ☐ Others ☐ _______________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  | Potential Harm |  |  |  | P | S | RR |  | How to make it safer….. |  |  | P | S | RRR |
| Unintentional release of load Unplanned movement of load Damage to equipment and load |  |  |  | Crush injuries to personnel Trap injuries - fingers being caught between sling/chain and load |  |  |  | 3 | 4 | 12 |  | - Check certification or lifting equipment and slingers before allowing work to commence. - When selecting lifting equipment, consideration must be given to the weight, size, shape and center or gravity of the load. - All lifting gear should be individually identifiable and stored so as to prevent damage or deterioration. - The safe working load of the lifting equipment and all lifting gear must be clearly marked on the item and never exceeded. - All lifting gear must be visually inspected before each use. - Lifting gear (chains, slings etc.) must be protected from damage during lifting by the use of packing on sharp edges. - Before fully raising the load, checks must be made to ensure that it is being lifted evenly and is not likely to swing or sway when raised. - Tail ropes must be used for larger loads to assist in maneuvering operations. - No persons are allowed to stand or work within the operating radius of the lifting appliance without the operators express permission. - Loads must not be transported over persons. - A banksman must be used at all times when the operator's vision is impeded or whilst operating in a congested area. |  |  | 2 | 2 | 4 |
|  |  |  |  |  |  |  |  |  |  |  |  | l) Work must cease if weather conditions are likely to prevent safe operations. - The area within the arc of the operation must be cleared of personnel before the load is lifted. - No person may be permitted to stand below a suspended load. |  |  |  |  |  |
| Probability (P) |  |  | Severity (S) |  |  | Risk Rating (RR): Probability X Severity (PXS) |  |  |  |  |  |  |  |  |  |  |  |
| 1. Improbable |  |  | 1. No risk of injury or disease |  |  | Residual Risk |  |  |  |  | Action |  |  |  |  |  |  |
| 2. Remote |  |  | 2. Minor injury |  |  | 1-6 LOW |  |  |  |  | Reduce risk wherever possible. |  |  |  |  |  |  |
| 3. Possible |  |  | 3. RIDDOR > 3 day off work |  |  | 7-14 MEDIUM |  |  |  |  | Only proceed with management authorisation |  |  |  |  |  |  |
| 4. Probable |  |  | 4. RIDDOR major injury |  |  |  |  |  |  |  | Consult with assessment personnel, re-define and reduce risk. |  |  |  |  |  |  |
| 5. Likely |  |  | 5. One or more fatalities |  |  | 15–25 HIGH |  |  |  |  | Do NOT proceed. Re-define, re-assess and reduce risk. |  |  |  |  |  |  |