Lab: IPv4, Subnetting, VLSM and IPv6 Addressing

Understanding IPv4 and IPv6 addressing (word count 1000)

This part of lab worksheet require you research and write a report on comparing IPv4 and IPv6 addressing. You may discuss but not limited to structure, address types, features and security.

Subnetting and VLSM

NOTE: To complete this part you would need full understanding of Lab 10 to 14.

Using the Network address 172.16.2.0/24, VLSM the network to support the following requirements:

Subnet A: 12 hosts

Subnet B: 60 hosts

Subnet C: 120 hosts

Subnet D: 45 hosts

Subnet E: 6 hosts

Subnet F: 180 hosts and Serial links between routers

Table 1

| Subnet | Network Address | Subnet mask | First Address | Last Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- |
| F | 172.16.2.0 | 255.255.255.0 | 172.16.2.1 | 172.16.2.254 | 172.16.2.255 |
| C | 172.16.3.0 | 255.255.255.128 | 172.16.3.1 | 172.16.3.126 | 172.16.3.127 |
| B | 172.16.3.128 | 255.255.255.192 | 172.16.3.129 | 172.16.3.190 | 172.16.3.191 |
| D | 172.16.3.192 | 255.255.255.192 | 172.16.3.193 | 172.16.3.254 | 172.16.3.255 |
| A | 172.16.4.0 | 255.255.255.240 | 172.16.4.1 | 172.16.4.14 | 172.16.4.15 |
| E | 172.16.4.16 | 255.255.255.248 | 172.16.4.17 | 172.16.4.22 | 172.16.4.23 |
| R1R2 | 172.16.4.24 | 255.255.255.252 | 172.16.4.25 | 172.16.4.26 | 172.16.4.27 |
| R2R3 | 172.16.4.28 | 255.255.255.252 | 172.16.4.29 | 172.16.4.30 | 172.16.4.31 |