| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 6 | LEFEL 6 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| Mock 2026 | MoCK 2026 |

MODULE CODE: ACDA6001

Electronics II (Mock Exam)

TIME:  2 HOURS

answer ALL THE questions

Equipment or materials required – Calculator

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

QUESTION 1

- Determine the reduced state diagram for the state diagram given below.

[6 Marks]

- For the Obtained reduced state diagram

  - Show its state Assignment.

  - Obtain the State Table.

  - Obtain Excitation Table using any Flip- flop.

  - Output Expressions

  - Logic Circuit

QUESTION 2

- Design a 3- bit Johnson Counter using D- Flipflop for a Synchronous sequential circuit. This counter will be used to monitor operational stages in a digital control system-based appliance. Your Solution Should include the following.

- State Diagram to illustrate state transitions.				[5 Marks]

- State transition table 							[4 Marks]

- Excitation Table for determining input requirements.			[5 Marks]

- Simplified K-Map Expressions for logic minimization.			[6 Marks]

- Logic Diagram to represent the final gate-level design.			[5 Marks]

QUESTION 3

- For a  3- input LUT of an FPGA Logic block and  determine the bits to be stored in its storage cell by using   the function  f= ABC+ABC+ABC+ABC  to realize by answering the following

  - Truth Table								[3 Marks]

  - With the help of the above truth table, design a  3- input LUT.    [8 Marks]

  - Also, explain its operation in detail by assuming the values of inputs to be 001.									[9 Marks]

- Program FPGA for the functions  f= f1+ f2  by considering  f1= x2x3 andf2= x1x3							[5 Marks]

QUESTION 4

- For the functions the w (A, B, C)=∑m  (4,5,6,7)   and z (A, B, C)=∑m  (1, 2, 3).

Answer the following.

  - Illustrate the implementation of functions 𝑤 and 𝑧 using a single decoder.

[6 Marks]

  - Present the truth tables and the fundamental Boolean expressions for both w and z.								[5 Marks]

  - Derive and show the simplified Boolean logic expressions for w and z.

[4 Marks]

  - Design w and z using Programmable Logic Array (PLA).	           [10 Marks]