| PROGRAMME BEng Electrical & Electronic Engineering | Applied programmable logic controllers Assignment 1 – Factory control Thomas, Scott 2024 |
| --- | --- |

Table of Contents

Introduction	0

Example of PLCs	1

Rack Mounted	1

Applications	1

Advantages and disadvantages	2

Modular	2

Applications	2

Advantages and disadvantages	3

Unitary	3

Applications	4

Advantages and disadvantages	4

Summary	4

Allen Bradley - L23E-QB1	5

Factory I/O	8

I/O addresses	10

Inputs	11

Outputs	13

Components & Equipment layout	16

Alarm strategy	21

Sequence of events & flow chart	22

Sequence of events	22

Flow Chart	24

PLC Program in Siemens S7	25

Ladder	27

HMI Screens	40

HMI Tags	41

Root Screen	42

Conclusion	51

List of tables and figures	54

References	61

# Introduction

The task, detailed in this report, is to complete a design using Factory I/O and controlled by S7 Siemens PLC. The purpose of the design is to sort different packages by weight and size. The following elements must be included in the design:

- Four conveyor belts

- Two pushers

- One conveyor scale

- Two diffuse sensors

- One emergency stop

- Two buttons

- One siren

- One warning light

- Two stack lights

Considerations will be made for the current draw when starting up the conveyors in sequence. The system, designed using the above elements, also needed a HMI for operator interaction on a root screen, warning screen and engineering screen. Below are the requirements for the HMI:

- Buttons that are capable of start and stopping the system

- Counters that will have a warning if exceeded

- Reset button for counters

- One emergency stop button

- Warning and fault indication

- A Replication of the stack lights in the system

# Example of PLCs

## Rack Mounted

A rack mounted PLC (Programmable Logic Controller) is the most flexible out of the different types of PLCs. This is because the PLC is built from multiple modules that slot into a backplate. This allows easy expansion and customisation when removing or installing modules into the rack. A rack mounted PLC is made up of a rack, CPU, Power supply, inputs modules, output modules, communication modules and speciality modules. Figure 1 shows what this might look like. [1]

Figure 1 - Rack mounted PLC

### Applications

Table 1 - Rack mounted applications

### Advantages and disadvantages

Table 2 - Rack mounted advantages & disadvantages

## Modular

Figure 2 - Modular PLC

A modular PLC, unlike the rack mounted PLC, is a PLC system that consists of separate and interchangeable modules that can be designed or configured in different ways depending on the specific application. This means that the make of the module can be different. The structure of the modular is similar to the rack mounted system, but the footprint of the modular PLC is smaller which can be useful on the shop floor next to the equipment or in limited space. Figure 2 shows what this might look like. [2]

### Applications

Table 3 - Modular applications

### Advantages and disadvantages

Table 4 - Modular advantages & disadvantages

## Unitary

Figure 3 - Unitary PLC

A unitary PLC is also known as a compact PLC or an all-in-one PLC, unlike the previous types. This means that all the previous structure of the PLC is built in to the unitary PLC, such as the CPU, input/output I/O ports and power supply. Due to the smaller size of the unitary PLC, it has a greater ability to fit in tight spaces. The design of the system is simple, enabling easier installation and wiring. However there aren’t as many complex applications that it can carry out, especially when compared with the other types of PLCs that can be upgraded or have their components changed. If a unitary PLC is damaged or faulty then the whole unit will need to be changed. Figure 3 shows what this might look like.  [3]

### Applications

Table 5 - Unitary applications

### Advantages and disadvantages

Table 6 - Unitary advantages & disadvantages

## Summary

In summary, each type of PLC have their own advantages and disadvantages depending on the application required. The key factors to consider when designing a system are space availability, budget constraints, scalability requirements and how complex the application is.

When cost and simplicity is a concern then unitary PLCs would be the best choice. For medium scale applications with some flexibility or scalability, then modular PLCs would be ideal. For anything bigger, and more complex requirements a rack- mounted PLC is used due to it being the most versatility.

For this report a modular Siemens S7 PLC is used. This can consist of various modules that can be configured and connected together based on the application on system design. This is a suitable chose for Factory I/O.

Example of PLCs in industry

Within Tata steel, there are a number of different PLCs used in the industry. Below are some working examples.

## Allen Bradley - L23E-QB1

Figure 4 - Main Pumphouse Substation Ifix

The Allen Bradley is one of the primary programmable logic controllers (PLCs) employed within the energy department to ensure standardisation across all PLCs utilised within the department, in regards to spare parts management. In this application, the PLC will monitor the main pumphouse substation 11Kv where it oversees the control of high voltage circuit switching and receives signals from the signal telecommunication (STC) system. The signals are 50Vdc signal over significant distances. By using a PLC and the implementation of fiber optic cable, there is a much higher signal speed and reliability which mitigates many potential issues, such as voltage drop, contact instability from relays and contactors ‘chattering’. The Ifix (HMI) screen seen in figure 4, allows remote operation of individual high voltage breakers; this mitigates any physical interaction with the circuit breakers themselves. This makes all switching operations in this substation safer.

When a high voltage circuit breaker trips without being operated by personnel, an alarm will be triggered which alerts the energy control room engineers. Due to the sensitive nature of production, this will need to be investigated as soon as possible. The PLC gives the energy controllers a fast response time, instantly telling the engineers something is wrong e.g.  a fault on the cable, transformer or circuit breaker.

Figure 5 - Main pumphouse control cubicle

In figure 5, the PLC itself has discrete inputs and outputs into it, expansion cards are inputs and analog inputs. The PLC provides telemetry of the high voltage switchgear e.g. circuit breaker open or closed. The PLC provides local control via a HMI of all circuit breakers, also providing remote control through SCADA / Ifix.

Figure 6 - PLC software

Shown in figure 6, this is the view of the embedded and expansion I/O mentioned above. This can also be trended, meaning the energy controllers can see when the last time the circuit breaker was operated and if any faults have previously happened.

# Factory I/O

Factory I/O is a simulation software that is used to learn and help train engineers in the setting of industrial automation. It simulates real-world industrial environments and allows the users to create a system that can be based on a real production line or sections of it. Factory I/O is mainly used for educational purposes, developing skills with PLC and industrial control.

Some advantages and disadvantages of Factory I/O are stated in table 7:

Table 7 - Factory I/O advantages & disadvantages

From Factory I/O a design has been made, adhering to the outlined task criteria. Figure 7 shows the final design:

Figure 7 - Factory I/O layout 1

Figure 8 - Factory I/O layout 2

As can be seen from Figure 7, there are two pushers which each push a different sized box down the conveyor belt. The first is a large box, and the second is the medium sized box(es). The medium boxes come in two different weights, which are then sent down different conveyors. The small boxes have no pusher and carries down the conveyor. Within the compound there are three safety doors, each having access to different parts of the compound. There is also a walkway going over the compound to help with visual fault finding instead of going into the compound.

## I/O addresses

Figure 9 - I/O addresses

From figure 9 below, on the left are the input sensors and on the right are the output sensors. The inputs are the sensors, buttons, weight and safety doors. The outputs are lights, conveyors and pushers. The lights will flash when that piece of equipment is activated. Using the tag name, it is then used in the PLC ladder logic.

## Inputs

Following on from the I/O figure 10 shows the inputs, table 8 details the equipment used on the inputs:

Figure 10 - I/O addresses inputs

Table 8 - I/O addresses inputs

## Outputs

Table 9 shows the breakdown of the equipment used for the outputs. These have different functions compared to the inputs. Figure 11 shows the outputs only.

Figure 11 - I/O addresses outputs

Table 9 - I/O addresses outputs

## Components & Equipment layout

Figure 12-16 and table 10 shows the breakdown of the components and equipment used in Factory I/O to create the design:

Figure 12 - Factory I/O layout 1-14

Figure 13 - Factory I/O 15

Figure 14 - Factory I/O 16

Figure 15 - Factory I/O 16

Figure 16 - Factory I/O 17-23

Table 10 - Components & equipment

# Alarm strategy

For the alarm strategy, there are some key points that needed to be filled. Below are the alarms that the system will stop when activated:

- Emergency stop on the HMI

- Local emergency stop in the compound

- Overweight limit on the weight conveyor

- If any box is misses e.g. fallen off the conveyor, this will be counted when the box has entered the system

- Three safety doors on compound, these will trigger like an emergency stop

- Master alarm, this will latch if any of the alarms are tiggered (the master alarm can only be reset by an engineer not an operator)

The points above are the minimum requirement for the alarms, and there can be many more placed in the system if deemed necessary. Other systems might need the use of a safety PLC controller. This is a system that has redundancy installed. These measures are set out in the machinery regulations, machinery directive 2006/42/EC and provision and use of work equipment (PUWR) as examples. [4] There is also the use of alarm management, used for ongoing processes outlined in standards like ISA-18.2 and IEC 62682, involving distinct stages of management. This continues on through the life of the process, evaluating and adjusting depending on the operators/engineers handling of alarms. The performance evaluation and monitoring are crucial aspects of effective alarm management. This will continue until the line is no longer operational. [5]

# Sequence of events & flow chart

## Sequence of events

The sequence of events (SSOE) is designed to give the order of the events with details of what occurs in what order. This helps the design of the system, and helps when the system does not follow the correct sequency, making this useful when operating the system on a daily bases. Table 11 is the order of the system that has been designed, starting with the start button and ending when the box has left the influence of the system. [6]

Table 11 - Sequence of events

## Flow Chart

Figure 17 - Flow chart

The flow chart (figure 17)  shows a picture format of the separate steps in the process. If the flow chart and sequence of events are compared, it will show the same process that has occurred. The only difference is that the flow chart will show the decisions made along the sequence of events.

# PLC Program in Siemens S7

A Siemens S7 is a series of programmable logic controller or PLC, manufactured by Siemens. There are a varied range of models such as the S7-1200 or the s7-400, and many more. The software used in the Siemens S7 can be STEP 7 or TIA portal; this is to create the logic and control algorithms. This example uses ladder logic and function block diagrams. When the logic is complete then it is uploaded to the PLC to control industrial processes, machinery or in this case Factory I/O. Below are the tags and ladder logic used in this design. [7] [8]

Tags

Tags are used in ladder logic to have unique variables such as inputs, outputs and counters etc. Tags are used a placeholders that store a memory location that have physical inputs and outputs, such as when a pusher is used to push a box down a conveyor belt. By doing this, the ladder logic is easier to understand and fault find on. The more accurate the information the better that tag is. As can be seen in figure 18-19, the most important information is the name, data type and address. [9]

Figure 18 - PLC tags 1

Figure 19 - PLC tags 3

Figure 20 - PLC tags 2

## Ladder

From the images below, each network is broken down into functions e.g. the tags for each network with have a name, explaining and what it is carrying out. This makes it easier to fault find. especially on large programs.

Figure 21 - Network 1

Network 1 (figure 21) is to enable the network/Siemens PLC and with Factory IO to communicate.

Figure 22 - Network 2

Network 2 (figure 22) has all start, stop, emergency stop and safety door emergency stop. The network consists of normally open contacts, normally closed contacts and the assignments. When operating either the start button on the control panel or the HMI the operate will be triggered. When the stop button is not active and no alarm. Safety doors, when open, will trigger an emergency stop and trigger alarm 5. When the emergency stop on the control panel is triggered, alarm 4 will be activated.

Figure 23 - Network 3

Network 3 (figure 23) is for the scale conveyor, this confirms the amount of time that the conveyor will stop for a measurement to be taken. The only alarm is when there is an overweight item on the conveyor. This will activate alarm 6, when the weight exceeds 7Kg. These can be reset by the general / master reset.

Network 4 is the siren and all conveyors. The siren will sound for 2s before any conveyors start moving. This is to alert all personnel in the area that the line is about to start. The conveyors will start 500ms after each other, this is to make sure the current draw on the supply would not trip.

TP – designed to send a pulse, in this case a 2s pulse to sound the siren.

Figure 24 - Network 4a

TON – designed to wait for a set amount of time before energising the assignment. This is done through a preset timer.

Figure 25 - Network 4b

Network 4 (figure 24 & 25) is all the factories conveyors and siren before start up. When the siren sounds, there is a 2 second delay before the conveyors start. This is done by a TP, when energised a pulse is sent for 2 seconds. Each conveyor will start in sequence 500ms apart. This is done using a TON timer. The maximum time for all conveyors to be energised is 8.250 seconds.  With the longest delay time being the furthest conveyor away from the starting point.

Figure 26 - Network 5

Figure 27 - Network 6

Network 5 (figure 26) is the indication of the panel lights. The indication has two different colours. red and green. Depending on the operation of the line, the NO or NC contactors will be activated. when in operation the green indicator will be green. When on stop the indicator will be red.

Network 6 (figure 27) is for the pushers in the system. these will be activated by a diffuse sensor located next to the pusher. There is a 200mS preset timer for pusher 0 and a 700ms preset timer for pusher 1. When there is no power doing to the pusher the arm will retract automatically.

Figure 28 - Network 7

Network 7 (figure 27) is for the counters in the system. There are 11 counters in this network, each counting something that passes a diffuse sensor. Every counter can be reset with the counter reset button on the HMI.

Counter 0 – This counts the large boxes entering the system.

Counter 1 – This counts the medium boxes entering the system.

Counter 2 – This counts the palletised boxes exiting the system.

Counter 3 – This counts the large boxes exiting the system.

Counter 9 – This counts the large medium box exiting the system.

Counter 10 – This counts the small medium box exiting the system.

Counter 11 – This counts the total boxes entering the system.

Figure 29 - Network 8

Network 8 (figure 29) is for the stop and start of the weighing conveyor, this is so that it will get an accurate reading of the box. This is for 1s (TON), so that other boxes don’t have time to reach the conveyor before the weight is recorded. After the weight is taken, the TP will give a 1s pulse to unbreak the sensor, this means that even if the sensor is broken within that 1s the conveyor will not stop.

Figure 30 - Network 9

Network 9 (figure 30) is for the digital display on the control panel. This is to have a visual of the weight so it can be monitored.

Figure 31 - Network 10

Network 10 (figure 31) is the pop-up sorter direction, depending on the weight. This works by having a minimum and maximum value (weight). When the weight is in between 3.5Kg and 4.5Kg, the sorter will roll left. If the boxes weight is in between 4.75Kg and 5.75Kg the sorter will roll right.

Figure 32 – Network 11

Network 11 (Figure 32) is for the creation of an error count for large boxes. If there are any faults within the line, such as falling off the conveyor or getting stuck, this will notice that. Counter 4 will count up (CU) and down (CD) when the pusher 0 and diffuse sensors 5 are activated. The error will then be live updated on the HMI screen. Due to the need for warning and alarms if the errors are too high, warning 3 and alarm 3 will be used. If the warning error limit is reached, a indication will be activated on the alarm and home screen on the HMI. When the alarm limit is reached the system will stop.

Figure 33 - Network 11

Network 12 (figure 33) is for the creation of an error for medium boxes. This is the same set up for large boxes. The main difference is that the medium boxes are sorted into L-medium and S-medium boxes, this means that there are two exits along the conveyor. This means that there are two diffuse sensors 7 and 6 at the two exits. Counter 5 will count up when the box enters the system, and count down when either sensor 7 or 6 are activated. There are also warning (2) and alarm (2) that will be activated with the limit is reached.

Figure 34 - Network 13

Network 13 (figure 34) is for the warning and stack lights. The warning lights are activated whenever the system is in operation. This is to ensure there is a visual cue when the plant is in operation. Stack lights are slightly different, the lights have different uses, red means the line is not in operation. Yellow means the line is about to start up or in alarm. And green means the system is in operation. The stack lights will operate on when conveyor 0 is in operation. This means that it is the first bit of plant to either start or finish.

Figure 35 - Network 14

Network 14 (figure 35) is for error counting, warning and alarm of palletised boxes, the same principle of the large and medium boxes. The palletised boxes has no pusher in its system. this comes down to two diffuse sensors, 8 and 4. Counter 6 will count up and down when the diffuse sensors are activated. Warning 1 and alarm 1 are used for the error upper limit.

Figure 36 - Network 15

Network 15 (figure 36) is used for the total count. Counter 8 is used to count up the boxes entering the system when sensor 1 bream is broken, and then when sensors 7, 6, 5 or 4 beam is broken count down. This is to ensure, any box leaving the system is counted correctly. Alarm 7 and warning 0 are used when the limits are reached. Warning is triggered first and the alarm has a higher limit that will shut the system off.

Figure 37 - Network 16

Network 16 &17 (figure 37) is the check for the warning and alarm flags, these are the assignments. The “warnings” cover the eight warning assignments that are in place. The “alarms” cover the eight alarm assignments. When any of the warning or alarms are activated, the alarm / warning flags will be activated.

# HMI Screens

In this design, multiple HMI screens are required. HMI stands for Human-Machine Interface, a device or software application that allows an interaction between human and machinery. HMI has multiple uses: alarms, control or engineering fault finding. The screens can provide graphical representation of the systems status, and allow real time information such as counters, input commands or make adjustments. This can be advantages for operators and engineers for different applications. In the examples below, the HMI are touch screen interfaces. However, this can also be physical buttons or a mixture of both. HMI can also connect to other systems e.g. ERP, SCADA and MES systems. HMI is an excellent way to improve efficiency, safety, usability (not much training required), and can be used in a varied amount of applications such as energy management, automotive and manufacturing. [10] [11]

Figure 38 - HMI screens

In this application, there is a requirement for three screens. These are called a root, warning and engineering screen. These are shown in Figure 37.

[12]

## HMI Tags

The HMI tags are used for the same purpose as the ladder tags, however this time they just relate to the picture form on the screen themselves. This can be a button, counter etc. The tags also give the option of adding comments, so in future if another engineer is going through the program, they can read the comments to better understand the program. Figures 39 and 40 shown below.

Figure 39 - HMI tags 1a

Figure 40 - HMI tags 1b

## Root Screen

A root screen or a homepage is a screen that the operators would use the most. This give you an overall view of operations. This is designed in TIA Portal, and by using tags assigned to their own properties enables a connection to the main logic program. This means that all live data is shown on this screen from the Factory I/O simulator. All the screens are linked through the interface with home, left and right keys. Figure 41 shows the HMI in use in a live system. Figure 42 shows the root screen in maintenance mode where alterations to the layout, button selection, modification etc can be done. Table 12 is the breakdown of the uses of each button, light and counter.

Figure 41 - Root screen HMI 1a

Figure 42 - Root screen HMI 1b

Table 12 - Root screen breakdown

Warning Screen

Warning or alarm screen is designed to show when the system is healthy or has a fault. The icon will show green for healthy or red for a fault. Depending on the application when a warning has gone into fault the machinery will not stop. This is just to notify the operator that something is wrong. When an alarm goes from healthy to fault, the machine will stop and the master alarm will be active. When this happens, only by using the engineer screen can the fault be cleared. This is to make sure that a competent person can fix the issue correctly without putting people in unknown danger. Figure 43 shows the warning screen in maintenance mode, where medication can be made. Figure 44 shows the HMI in operation. Tables 13-15 have the breakdown of the components on the HMI.

Figure 43 - Warning screen HMI 1a

Figure 44 - Warning screen HMI 1b

The buttons below are used to navigate the system.

Table 13 - Warning screen navigate

Warnings

Each of the lights will light up when the one of the warnings in the program is triggered (listed in table 14). This is to ensure the operator can see what warning is active, so the operator can fix the problem before it becomes an alarm and stop the system.

Table 14 - Warning screen warnings

Alarms

Like the warnings, in table 15 below shows which one trigger for each problem. This will stop the system, meaning the prosses will stop. When an alarm is triggered, the master alarm will stay active until it has been reset on the engineering screen.

Table 15 - Warning screen alarms

Engineer Screen

The engineer screen is only for engineers or competent personnel. This screen contains the master reset, which must be used before any alarms can be reset. This screen also has a count of the amount of errors and alarms that have happened during normal operation. This will be reset when the machine is back in working order. In the field this screen would normally have a password, making it secure. Figure 45 shows the HMI in normal operation. In figure 46, this shows how you would edit the values for the alarms or warnings. This can be used in fault finding. Table 16 shows the breakdown of each button and counters.

Figure 45 - Engineer screen 1a

Figure 46 - Engineer screen 1b

Table 16 - Engineer screen breakdown

# Conclusion

From the criteria that was set, the project has performed very well. The HMI screen also works well. There are some improvements that can be done visually on the HMI screens to make it more user friendly and a better layout with a password for the engineering screen. The control panel would also be moved outside the compound. The initial thought was that the HMI would be mounted on a panel outside the compound with the local panel inside. After looking through regulation such as PUWR (Provision and use of work equipment), the control panel must be in reach at all times with an emergency stop. In the initial design this is not the case due to moving machinery. [13]

S7 Siemens ladder has been easy to use with many ways of using the program, the only improvement that can be done is reducing the number of networks / number of commands used. This would streamline the program making it easier to fault find if an issue would arise.

The program can be more complex, adding in additional warnings and alarms. This giving more safety measures and prevent damaging equipment or assets unnecessarily. A example of this would be adding in an alarm when one conveyor fails to start in the startup sequence. This would prevent a clog with boxes, and the adjacent conveyor pushing boxes off the conveyor damaging the boxes.

The Factory I/O system that was designed could be designed smaller with less devices being used. This would mean the footprint would decrease overall and the expense of using less equipment would also decrease. This would also increase productivity as there would be less time taken for boxes being sorted. Below is an example of this design. This would be able to meet the purpose of the initial design.

Figure 47 - Factory I/O example 1a

Figure 48 - Factory I/O example 1b

Figure 49 - Factory I/O example 1c

Using 3 diffuse sensors at the start of the system would give a read of what shape of box enters palletised, medium or large. Then the pusher would push the large and small boxes off the conveyor, while the medium sized boxes would be weighed and sent down one of two destinations.

# List of tables and figures

Table 1 - Rack mounted applications	3

Table 2 - Rack mounted advantages & disadvantages	4

Table 3 - Modular applications	5

Table 4 - Modular advantages & disadvantages	5

Table 5 - Unitary applications	6

Table 6 - Unitary advantages & disadvantages	6

Table 7 - Factory I/O advantages & disadvantages	11

Table 8 - I/O addresses inputs	14

Table 9 - I/O addresses outputs	17

Table 10 - Components & equipment	21

Table 11 - Sequence of events	25

Table 12 - Root screen breakdown	49

Table 13 - Warning screen navigate	51

Table 14 - Warning screen warnings	52

Table 15 - Warning screen alarms	53

Table 16 - Engineer screen breakdown	55

Figure 1 - Rack mounted PLC	7

Figure 1 - Rack mounted PLC	7

Figure 1 - Rack mounted PLC	7

Figure 1 - Rack mounted PLC	7

Figure 2 - Modular PLC	8

Figure 2 - Modular PLC	8

Figure 2 - Modular PLC	8

Figure 2 - Modular PLC	8

Figure 3 - Unitary PLC	9

Figure 3 - Unitary PLC	9

Figure 3 - Unitary PLC	9

Figure 3 - Unitary PLC	9

Figure 4 - Main Pumphouse Substation Ifix	12

Figure 4 - Main Pumphouse Substation Ifix	12

Figure 4 - Main Pumphouse Substation Ifix	12

Figure 4 - Main Pumphouse Substation Ifix	12

Figure 5 - Main pumphouse control cubicle	13

Figure 5 - Main pumphouse control cubicle	13

Figure 5 - Main pumphouse control cubicle	13

Figure 5 - Main pumphouse control cubicle	13

Figure 6 - PLC software	14

Figure 6 - PLC software	14

Figure 6 - PLC software	14

Figure 6 - PLC software	14

Figure 7 - Factory I/O layout 1	15

Figure 7 - Factory I/O layout 1	15

Figure 7 - Factory I/O layout 1	15

Figure 7 - Factory I/O layout 1	15

Figure 8 - Factory I/O layout 2	16

Figure 8 - Factory I/O layout 2	16

Figure 8 - Factory I/O layout 2	16

Figure 8 - Factory I/O layout 2	16

Figure 9 - I/O addresses	17

Figure 9 - I/O addresses	17

Figure 9 - I/O addresses	17

Figure 9 - I/O addresses	17

Figure 10 - I/O addresses inputs	18

Figure 10 - I/O addresses inputs	18

Figure 10 - I/O addresses inputs	18

Figure 10 - I/O addresses inputs	18

Figure 11 - I/O addresses outputs	20

Figure 11 - I/O addresses outputs	20

Figure 11 - I/O addresses outputs	20

Figure 11 - I/O addresses outputs	20

Figure 12 - Factory I/O layout 1-14	23

Figure 12 - Factory I/O layout 1-14	23

Figure 12 - Factory I/O layout 1-14	23

Figure 12 - Factory I/O layout 1-14	23

Figure 13 - Factory I/O 15	24

Figure 13 - Factory I/O 15	24

Figure 13 - Factory I/O 15	24

Figure 13 - Factory I/O 15	24

Figure 14 - Factory I/O 16	24

Figure 14 - Factory I/O 16	24

Figure 14 - Factory I/O 16	24

Figure 14 - Factory I/O 16	24

Figure 15 - Factory I/O 16	24

Figure 15 - Factory I/O 16	24

Figure 15 - Factory I/O 16	24

Figure 15 - Factory I/O 16	24

Figure 16 - Factory I/O 17-23	25

Figure 16 - Factory I/O 17-23	25

Figure 16 - Factory I/O 17-23	25

Figure 16 - Factory I/O 17-23	25

Figure 17 - Flow chart	31

Figure 17 - Flow chart	31

Figure 17 - Flow chart	31

Figure 17 - Flow chart	31

Figure 18 - PLC tags 1	33

Figure 18 - PLC tags 1	33

Figure 18 - PLC tags 1	33

Figure 18 - PLC tags 1	33

Figure 19 - PLC tags 3	34

Figure 19 - PLC tags 3	34

Figure 19 - PLC tags 3	34

Figure 19 - PLC tags 3	34

Figure 20 - PLC tags 2	34

Figure 20 - PLC tags 2	34

Figure 20 - PLC tags 2	34

Figure 20 - PLC tags 2	34

Figure 21 - Network 1	35

Figure 21 - Network 1	35

Figure 21 - Network 1	35

Figure 21 - Network 1	35

Figure 22 - Network 2	35

Figure 22 - Network 2	35

Figure 22 - Network 2	35

Figure 22 - Network 2	35

Figure 23 - Network 3	36

Figure 23 - Network 3	36

Figure 23 - Network 3	36

Figure 23 - Network 3	36

Figure 24 - Network 4a	37

Figure 24 - Network 4a	37

Figure 24 - Network 4a	37

Figure 24 - Network 4a	37

Figure 25 - Network 4b	39

Figure 25 - Network 4b	39

Figure 25 - Network 4b	39

Figure 25 - Network 4b	39

Figure 26 - Network 5	39

Figure 26 - Network 5	39

Figure 26 - Network 5	39

Figure 26 - Network 5	39

Figure 27 - Network 6	39

Figure 27 - Network 6	39

Figure 27 - Network 6	39

Figure 27 - Network 6	39

Figure 28 - Network 7	40

Figure 28 - Network 7	40

Figure 28 - Network 7	40

Figure 28 - Network 7	40

Figure 29 - Network 8	41

Figure 29 - Network 8	41

Figure 29 - Network 8	41

Figure 29 - Network 8	41

Figure 30 - Network 9	42

Figure 30 - Network 9	42

Figure 30 - Network 9	42

Figure 30 - Network 9	42

Figure 31 - Network 10	42

Figure 31 - Network 10	42

Figure 31 - Network 10	42

Figure 31 - Network 10	42

Figure 32 - Network 11	43

Figure 32 - Network 11	43

Figure 32 - Network 11	43

Figure 32 - Network 11	43

Figure 33 - Network 13	44

Figure 33 - Network 13	44

Figure 33 - Network 13	44

Figure 33 - Network 13	44

Figure 34 - Network 14	45

Figure 34 - Network 14	45

Figure 34 - Network 14	45

Figure 34 - Network 14	45

Figure 35 - Network 15	46

Figure 35 - Network 15	46

Figure 35 - Network 15	46

Figure 35 - Network 15	46

Figure 36 - Network 16	47

Figure 36 - Network 16	47

Figure 36 - Network 16	47

Figure 36 - Network 16	47

Figure 37 - HMI screens	49

Figure 37 - HMI screens	49

Figure 37 - HMI screens	49

Figure 37 - HMI screens	49

Figure 38 - HMI tags 1a	50

Figure 38 - HMI tags 1a	50

Figure 38 - HMI tags 1a	50

Figure 38 - HMI tags 1a	50

Figure 39 - HMI tags 1b	51

Figure 39 - HMI tags 1b	51

Figure 39 - HMI tags 1b	51

Figure 39 - HMI tags 1b	51

Figure 40 - Root screen HMI 1a	51

Figure 40 - Root screen HMI 1a	51

Figure 40 - Root screen HMI 1a	51

Figure 40 - Root screen HMI 1a	51

Figure 41 - Root screen HMI 1b	52

Figure 41 - Root screen HMI 1b	52

Figure 41 - Root screen HMI 1b	52

Figure 41 - Root screen HMI 1b	52

Figure 42 - Warning screen HMI 1a	54

Figure 42 - Warning screen HMI 1a	54

Figure 42 - Warning screen HMI 1a	54

Figure 42 - Warning screen HMI 1a	54

Figure 43 - Warning screen HMI 1b	55

Figure 43 - Warning screen HMI 1b	55

Figure 43 - Warning screen HMI 1b	55

Figure 43 - Warning screen HMI 1b	55

Figure 44 - Engineer screen 1a	58

Figure 44 - Engineer screen 1a	58

Figure 44 - Engineer screen 1a	58

Figure 44 - Engineer screen 1a	58

Figure 45 - Engineer screen 1b	58

Figure 45 - Engineer screen 1b	58

Figure 45 - Engineer screen 1b	58

Figure 45 - Engineer screen 1b	58

Figure 46 - Factory I/O example 1a	61

Figure 46 - Factory I/O example 1a	61

Figure 46 - Factory I/O example 1a	61

Figure 46 - Factory I/O example 1a	61

Figure 47 - Factory I/O example 1b	62

Figure 47 - Factory I/O example 1b	62

Figure 47 - Factory I/O example 1b	62

Figure 47 - Factory I/O example 1b	62

Figure 48 - Factory I/O example 1c	63

# References

| [1] | ladderlogicworld, “ladderlogicworld.com,” 03 03 2024. [Online]. Available: https://ladderlogicworld.com/plc-hardware/. [Accessed 06 03 2024]. |
| --- | --- |
| [2] | C. Gilbert, 19 10 2014. [Online]. Available: https://blog.cetrain.isu.edu/blog/bid/353287/advantages-of-a-modular-plc-over-a-fixed-plc. [Accessed 06 03 2024]. |
| [3] | unicourse, “unicourse.org,” 06 03 2024. [Online]. Available: https://unicourse.org/wp-content/uploads/2020/04/U15-WB-1.pdf. [Accessed 06 03 2024]. |
| [4] | pilz, “https://www.pilz.com,” 19 04 2024. [Online]. Available: https://www.pilz.com/en-GB/support/law-standards-norms/manufacturer-machine-operators/machinery-regulation. [Accessed 19 04 2024]. |
| [5] | isa, “https://www.isa.org,” 19 04 2024. [Online]. Available: https://www.isa.org/intech-home/2020/march-april/features/alarm-management-questions-that-everyone-asks#:~:text=Alarm%20management%20is%20a%20process,systems%20in%20a%20process%20plant.. [Accessed 19 04 2024]. |
| [6] | docs.oracle, “https://docs.oracle.com,” 19 04 2024. [Online]. Available: https://docs.oracle.com/en/applications/jd-edwards/supply-chain-manufacturing/9.2/eoadf/understanding-sequence-of-events.html. [Accessed 19 04 2024]. |
| [7] | Evghnnii, “isd-soft.com,” 17 01 2017. [Online]. Available: https://isd-soft.com/tech_blog/basics-siemens-plcs-programming-simatic-step7/. [Accessed 13 03 2024]. |
| [8] | siemens, “www.siemens.com,” 13 03 2024. [Online]. Available: https://www.siemens.com/global/en/company/sustainability/sce/learning-training-documents/basics-of-plc-programming.html. [Accessed 13 03 2024]. |
| [9] | S. Dietrich, “www.control.com,” 31 05 2023. [Online]. Available: https://control.com/technical-articles/understanding-plc-tags-controller-scope-vs-program-scope/. [Accessed 13 03 2024]. |
| [10] | aveva, “www.aveva.com,” 10 03 2024. [Online]. Available: https://www.aveva.com/en/solutions/operations/hmi/#:~:text=HMI%20stands%20for%20Human%20Machine,processes%20in%20factories%20and%20plants.. [Accessed 10 03 2024]. |
| [11] | inductiveautomation, “inductiveautomation.com,” 10 08 2018. [Online]. Available: https://inductiveautomation.com/resources/article/what-is-hmi. [Accessed 10 03 2024]. |
| [12] | siemens, “www.siemens.com,” 10 03 2024. [Online]. Available: https://www.siemens.com/global/en/products/automation/simatic-hmi/panels.html. [Accessed 10 03 2024]. |
| [13] | hse.gov, “https://www.hse.gov.uk,” HSE, 19 04 2024. [Online]. Available: https://www.hse.gov.uk/work-equipment-machinery/puwer.htm. [Accessed 19 04 2024]. |
| [14] | J. Kyrnin, “www.thoughtco.com,” 04 03 2020. [Online]. Available: https://www.thoughtco.com/tagging-advantages-3469879#:~:text=Tags%20are%20simple%20pieces%20of,that%20have%20the%20same%20tag.. [Accessed 11 03 2024]. |