EXAMINATIONS 2013/2014		Semester 2-Resit

AHOLIADAU 2013/2014

FACULTY OF: Applied Design and Engineering School of Applied Computing

| NAME OF COURSE/PROGRAMME: | BSc (Hons) Computer Networks HND Computer Networks |
| --- | --- |
| YEAR OR LEVEL: | 4 |
| NAME OF MODULE/UNIT/EXAM PAPER: | IP Internetworking |
| MODULE NO: | M1X8609 M5X8914 |
| DATE OF EXAMINATION: |  |
| EXAMINATION TIMES: |  |
| START TIME: |  |
| FINISH TIME: |  |
| TIME ALLOWED: |  |

INSTRUCTIONS TO CANDIDATES:

Answer three out of four questions. Each question carries equal marks.

Only basic calculators are permitted. Scientific calculators especially with internal memories are not allowed.
QUESTION  ONE

- Explain interior gateway routing protocols and exterior gateway routing protocols and give some examples. 										 (6 marks)

Figure 1

Refer to the partial output shown above in Figure 1 answer the following:

- Which routing protocol is configured on router R1?

- What are the remote networks learned by R1 using the protocol?

- What is the cost used by the protocol to reach each remote network? 	(6 marks)

- In reference with OSPF, explain hello intervals and dead intervals. 		(3 marks)

- Explain EIGRP packet types.								(10 marks)

QUESTION TWO

- List the metrics used in IP routing protocols.					(6 marks)

- What is the purpose of Administrative Distance? Write the AD value for protocols listed below.

- Connected interface

- Static

- RIP

- EIGRP

- OSPF

- Unknown									(8 marks)

- Refer to figure 1, answer the following:

How many subnets are there?

How many collision domains and broadcast domains?

Identify the devices and describe their functions. 			(11 marks)

Figure 2

QUESTION THREE

- With reference to Figure 2, design a addressing scheme using VLSM. The network address is 200.120.10.0/24. Write subnet address, subnet mask, first IP address, last IP address and broadcast address for each segments: LANs on Fastethernet and Serial links between the routers.										(25 marks)

Figure 3

QUESTION FOUR

- For the network shown in Figure 3, explain static route and when to use static route.

(9 marks)

Figure 4

- Compare Distance Vector and Link State routing protocols.				(6 marks)

- Compare classful and classless routing protocols 					(10 marks)