| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2021 | mai 2021 |

HND Electrical and Electronic Engineering

MODULE CODE: M4X01405

Dynamics Control and Instrumentation

TIME:  2 HOURS

answer all Six questions

Equipment or materials required – Calculators are allowed

Adnoddau neu ddefnyddiau sydd eu hangen -

Question One (10 marks)

A particle is projected up a plane which is set at 30o  elevation to the horizontal (Figure 1 below). The particle leaves point A with a velocity of 70m/s at an angle of 60o  to the plane.  If the effects of drag are ignored, determine:-

- The time to impact at point B to 3 decimal places

(6 marks)

- The total distance along the plane to B, the point of impact on the plane.

(4 marks)

(Assume gravity is 9.81m/s2)

Figure 1

Question Two (10 marks)

The motion of three particles A, B and C have a motion which can be described by the following expressions:-

For particle A   S = 3t2 + 10t - 8

For particle B    S = 7t3 + 4t2 + 4

For particle C    S = 10t3 - 30t + 2

Where S = displacement in m, t = time in seconds.

Determine:

- The time when the velocity of particle B  is three times that of particle A to 3 decimal places

(5 marks)

- The time when the acceleration  of particle B  is equal to that of particle C to 3 decimal places

(2 marks)

- The distance travelled by each particle in the period from t= 0 to t= 4 seconds

(3 marks)

Question Three (10 marks)

For a unity feedback system with an open loop transfer function Gs= (s+2)s(s+1)(s+3)

- Find it’s value at the point s = -3 +2j using vector techniques.

(7 marks)

- From your knowledge of root locus determine if the point -3 +2j is on the locus

(3 marks)

Question Four (10 marks)

A mass spring damper system as shown in figure 2 below comprises of a mass of mass (m), with 3 identical springs with stiffness coefficient (k), and 2 viscous dampers with damping coefficient (c).

- Determine an expression for the transfer function for this system (assume Zero for all initial conditions).

(6 marks)

Figure 2

- If  m = 6, c = 2 and k = 2, determine the natural frequency (in Hz) for the system.

(2marks)

- Determine the damping coefficient for the system

(1 marks)

- Show if the system is Under, Over or Critically damped

(1 mark)

Question five (10 marks)

Given the Laplace transforms

unit step= 1s

e-at     = 1(s+a)

For a system having the transfer function

Gs= 4(s+1)(s+2)(s+3)

Determine :-

- The system’s Poles and Zeros and hence determine its stability .

(3 marks)

- The system output after 0.8 seconds if a step signal of magnitude 1 is applied to its input (to 3 decimal places)?

(7 marks)

Question six (10 marks)

- Convert the diagram of the system shown in figure 3 below into a signal

flow graph.

( assume the following flow directions

G1,G2,G3 G4  left to right

H1, H2  and unity feedback right to left)

(4 marks)

- Using the Mason gain formula Derive a simplified expression for the transfer function of the system.

(6 marks)

Figure 3

Diwedd/End					Total (60 Marks)