Applied Computing

Wales Institute of Science and Art (WISA)

Enterprise Networking, Security, and Automation (ENSA)

(ACCA6023)

Lab 7: Optimize, Monitor and Troubleshoot Network

Research and record your observations

- Understand Network Monitoring and the tools used:

  - Describe network monitoring as you understand it. Give an example of how it might be used in a production network.

  - Research and list out network monitoring tools and describe them covering the following aspects.

    - Vendor, Name of the Tool, Purpose of the tool, Performance Metrics

    - Explain your reasons for choosing the tool, including specific features that you consider important

- Password Recovery Procedures

Background / Scenario

The purpose of this lab is to research the procedure for recovering or resetting the enable password on a specific Cisco router. The enable password protects access to privileged EXEC and configuration mode on Cisco devices. The enable password can be recovered, but the enable secret password is encrypted and would need to be replaced with a new password.

In order to bypass a password, a user must be familiar with the ROM monitor (ROMMON) mode, as well as the configuration register setting for Cisco routers. ROMMON is basic CLI software stored in ROM that can be used to troubleshoot boot errors and recover a router when an IOS is not found.

In this lab, you will begin by researching the purpose and settings of the configuration register for Cisco devices. You will then research and detail the exact procedure for password recovery for a specific Cisco router.

2.1 Research the Configuration Register

To recover or reset an enable password, a user will utilize the ROMMON interface to instruct the router to ignore the startup configuration when booting. When booted, the user will access privilege EXEC mode, overwrite the running configuration with the saved startup configuration, recover or reset the password, and restore the router’s boot process to include the startup configuration.

The router’s configuration register plays a vital role in the process of password recovery. In the first part of this lab, you will research the purpose of a router’s configuration register and the meaning of certain configuration register values.

Describe the purpose of the configuration register.

    - What is the purpose of the configuration register?

    - What command changes the configuration register in global configuration mode?

    - What command changes the configuration register in ROMMON mode?

Document the Password Recovery Procedure for a Specific Cisco Router

Describe the exact procedure for recovering or resetting a password from a 4321 Cisco router and answer questions based on your research.

    - Detail the process to recover a password on a 4321 Cisco router

    - Research and list the steps and commands that you need to recover or reset the enable or enable secret password from your Cisco router. Summarize the steps in your own words.

    - Describe how to find the current setting for your configuration register.

    - Describe the process for entering ROMMON mode.

    - What commands do you need to enter the ROMMON interface?

    - What message would you expect to see when the router boots?

    - Why is it important to load the startup configuration into the running configuration?

    - Why is it important to change the configuration register back to the original value after recovering password?