Safiya Nanton (2114630)

TOST Construction

Civil Engineering works

Technical report

Table of Contents

BEP in response to EIR	2

BIM Execution Plan (BEP)	2

Introduction	2

Project BIM Objectives	3

Project Information Standards & Data Exchange	3

BIM Standards & Frameworks	3

Common Data Environment (CDE)	3

File Naming & Classification	4

Roles & Responsibilities	4

Information Exchange & Deliverables	4

Handover & Asset Information Requirements	5

BIM Execution Timeline	5

Technical Report: Structural System, Material Selection, Load Considerations	7

BREEAM	7

Key Energy Efficiency Aspects	7

A. Passive Design Strategies	7

B. Active Energy Efficiency Strategies	7

C. BREEAM Energy Scoring Criteria	8

Implementation Strategies	8

BREEAM & Energy Efficiency	8

For the Joseph Parry Theatre to achieve a high BREEAM rating (Excellent or Outstanding), the following energy efficiency aspects must be integrated into the design, construction, and operational phases.	9

Design Strategies (Minimizing Energy Demand)	9

Aesthethics	10

Acoustic Optimization for Theatre Performances	10

Technical Design Specifications	11

A. Structural & Envelope Specifications	11

B. Mechanical & HVAC System	11

C. Electrical & Lighting System	12

D. Water Management & Drainage	12

Structural System Overview	12

Primary Load-Bearing Framework	12

Roof Space Frame Load Management	13

Material Selection & Performance	13

Non-Technical Design Specifications	13

A. Architectural & Aesthetic Considerations	13

B. Social & Wellbeing Factors	14

C. Operational & Lifecycle Considerations	14

Load Calculations for Structural Integrity	14

Dead Load (Self-Weight) Computation	14

Live Load Analysis	15

Wind & Environmental Load Analysis	15

Steel Section Verification	15

Foundation Strategy Computations	16

Spread Footing Load-Bearing Check	16

Pile Foundation for High-Load Zones	16

Pile Foundation	17

Load-Bearing Capacity Calculations	17

Column Load Distribution to Footing	17

Bearing Capacity of Concrete Pad	17

Pile Foundation Load Considerations	18

Appendix	19

Reference list	19

BEP in response to EIR

## BIM Execution Plan (BEP)

Project Name: Joseph Parry Theatre
Project Reference: 
Version: Final 
Date: May 10th 2025
Prepared By: TOST

## Introduction

This BIM Execution Plan (BEP) has been prepared in response to the Employer’s Exchange Information Requirements (EIR) to define the project’s Building Information Modelling (BIM) workflows, standards, and procedures. It ensures compliance with BS EN ISO 19650-2 and facilitates effective coordination, clash detection, and digital handover.

## Project BIM Objectives

The primary objectives of BIM implementation for this project are:

- To ensure efficient design coordination and clash detection.

- To standardize information exchange across teams.

- To streamline project delivery using a Common Data Environment (CDE).

- To enable accurate asset information handover for Facilities Management (FM).

## Project Information Standards & Data Exchange

## BIM Standards & Frameworks

All BIM processes shall conform to the latest UK BIM Framework standards, including:

- BS EN ISO 19650-1 & 19650-2 – Information management using BIM.

- BS EN ISO 19650-3 – Operational phase information.

- BS 1192-4 – COBie information exchange.

- PAS 1192-5 – Information security principles.

- Uniclass 2015 – Classification system.

## Common Data Environment (CDE)

A Common Data Environment (CDE) will be used for managing and sharing project information. The selected CDE for this project is Revizto.

- Hosting Platform: Revizto (Cloud-based)

- File Access & Permissions: Role-based access for Employers, Designers, and Contractors.

- Audit Trail & Version Control: Maintained through CDE logs.

## File Naming & Classification

File naming shall conform to BS EN ISO 19650-2 NA. Example format:

6036-ACECP-XX-XX-T-A-00001

Where:

- 6036 = Project Number

- ACECP = Originator (ACE Capital Projects Ltd)

- XX-XX = Location/Volume

- T = Type (Drawing, Model, Document, etc.)

- A = Discipline (Architecture, Structural, MEP, etc.)

- 00001 = Unique Identifier

# Roles & Responsibilities

| Role | Organization | Responsibilities |
| --- | --- | --- |
| Employer (ACECP Ltd) | ACECP Ltd | Approves BIM deliverables, ensures compliance with EIR. |
| BIM Manager | James Williams | Oversees BIM strategy, CDE management, clash detection. |
| Architectural Lead | Olivia Scott (Gofod Designs) | Develops BIM models for design compliance. |
| Structural Lead | George Matthews | Provides detailed structural models. |
| MEP Lead | Sean Thomas (Value S Solutions) | Ensures MEP integration with BIM workflows. |
| Contractor | Mason Martin | Executes construction, manages updates in BIM models. |
| Facility Manager | Benjamin Davies | Uses as-built BIM data for operations. |

## Information Exchange & Deliverables

| Phase | Deliverables | Format |
| --- | --- | --- |
| Concept Design | BIM Model, Drawings | RVT, IFC |
| Detailed Design | Detailed Models, Clash Reports | RVT, PDF |
| Construction | Updated Models, As-built Data | RVT, IFC |
| Handover & FM | COBie Data, Asset Register | XLSX, IFC |

## Handover & Asset Information Requirements

- As-built models must be delivered in IFC format for interoperability.

- COBie datasets must include asset details, maintenance schedules, and energy performance data.

## BIM Execution Timeline

| Milestone | Start Date | End Date |
| --- | --- | --- |
| BIM Coordination Kick-off Meeting | Jan 2025 | Feb 2025 |
| Clash Detection Setup | Mar 2025 | May 2025 |
| Detailed Design Finalization | June 2025 | Oct 2025 |
| Construction BIM Model Updates | Nov 2025 | Dec 2026 |
| As-Built Model Submission | Jan 2027 | Feb 2027 |
| COBie Handover & Training | Mar 2027 | Mar 2027 |

BIM Workflow in Detail

The Joseph Parry Theatre project follows a BIM Level 2 / 3 workflow, ensuring digital collaboration and data integrity.

A. BIM Execution Plan (BEP)

- The contractor must submit a BEP detailing:

  - BIM software used (e.g., Autodesk Revit, Navisworks).

  - Data exchange process (COBie, IFC standards).

  - Model ownership (who updates which part of the model).

  - Clash detection frequency (weekly, bi-weekly checks).

B. Common Data Environment (CDE)

- All project files are stored and managed in a CDE platform (e.g., Revizto, Autodesk BIM 360).

- Security protocols follow PAS 1192-5, ensuring cybersecurity for asset data.

- File formats include:

  - Graphical Models: RVT (Revit), IFC (open format).

  - Data Models: COBie, XLSX.

  - Drawings: DWG, PDF.

C. Clash Detection & Coordination

- BIM Coordinator runs clash detection checks in Navisworks.

- Clash reports are shared in weekly meetings for resolution.

- Common clashes detected:

  - MEP vs. Structure – Ducts intersecting beams.

  - Architectural vs. Plumbing – Pipe routing conflicts with walls.

D. Level of Information Need (LOD/LOI)

- The project follows LOD 100-500 depending on the phase.

- LOD Breakdown:

  - LOD 100-200: Concept design (basic massing).

  - LOD 300: Detailed design (accurate dimensions).

  - LOD 400-500: Construction and As-built models.

E. COBie Handover & Facility Management

- COBie files ensure structured asset handover.

- Facility managers receive data-rich models with:

  - Maintenance schedules.

  - Manufacturer details for components.

  - Energy performance metrics.

# Technical Report: Structural System, Material Selection, Load Considerations

## BREEAM

### Key Energy Efficiency Aspects

BREEAM assesses buildings based on energy, water, materials, health, and wellbeing. To achieve a high BREEAM rating, the following should be considered:

### A. Passive Design Strategies

- Thermal Mass & Insulation:

  - Use high thermal mass materials (concrete) to stabilize internal temperatures.

  - Install high-performance insulation (walls, floors, and roofs) to minimize heat loss.

### B. Active Energy Efficiency Strategies

- High-Efficiency HVAC Systems:

  - Implement mechanical ventilation with heat recovery (MVHR). It manages  both cooling and ventilation while minimizing energy loss due to air leakage in a tightly sealed building

- Renewable Energy Integration:

  - Install solar PV panels for on-site electricity generation.

- Smart Energy Management Systems:

  - Use BMS (Building Management System) to monitor energy usage.

  - Implement motion-sensor LED lighting for efficiency.

### C. BREEAM Energy Scoring Criteria

- Aim for EPC (Energy Performance Certificate) rating of A.

- Implement metering and sub-metering to track real-time energy consumption.

- Use low-carbon materials with low embodied energy.

## Implementation Strategies

| Design Strategy | BREEAM Requirement |
| --- | --- |
| High-Performance Insulation | Yes (for energy efficiency) |
| Airtight Construction | Recommended |
| MVHR (Heat Recovery) | Encouraged |
| Solar PV & Renewable Energy | Points awarded |
| Triple-Glazed Windows | Encouraged |
| Thermal Bridging Reduction | Recommended |

### BREEAM & Energy Efficiency

- Heat Recovery Ventilation (MVHR): Achieves 75% efficiency.

- Solar PV panels: Reduce grid dependency by 30%.

- Passive Design: Natural insulation with U-values < 0.16 W/m²K.

Recommendations for Improvement

- Maximise natural ventilation strategies.

- Increase insulation levels beyond current requirements.

- Integrate a smart building management system (BMS) to monitor real-time energy use.

## For the Joseph Parry Theatre to achieve a high BREEAM rating (Excellent or Outstanding), the following energy efficiency aspects must be integrated into the design, construction, and operational phases.

## Design Strategies (Minimizing Energy Demand)

| Aspect | Requirement | BREEAM Credit Areas | Design Strategy for Project |
| --- | --- | --- | --- |
| High-Performance Insulation | U-Value < 0.15 W/m²K for walls, roofs, and floors | Ene 01 (Reduction of Energy Use) | Use wood fiber, hempcrete, or aerogel insulation. |
| Airtightness & Thermal Bridging Reduction | Airtight construction (≤ 3 m³/hr/m² at 50 Pa) | Ene 01 & Hea 05 (Thermal Comfort) | Use high-performance membranes, careful detailing at junctions. |
| Heat Recovery Ventilation (HRV) | ≥ 75% efficiency | Ene 01 (Energy Efficiency) | Install MVHR (Mechanical Ventilation with Heat Recovery) to reclaim heat from exhaust air. |
| Demand-Controlled Ventilation (DCV) | CO₂ sensors & automated controls | Hea 02 (Indoor Air Quality) | Adjust ventilation rates based on occupancy levels. |
| Building Management System (BMS) | Automated energy monitoring & reporting | Ene 01 & Man 04 (Commissioning) | Implement a smart BMS with real-time energy tracking & automated optimization. |
| Occupancy Sensors for Lighting | Automatic lighting control | Ene 01 & Hea 01 (Visual Comfort) | Use motion & daylight sensors for lighting. |
| Energy-Efficient Appliances | Min. A+ rated equipment | Ene 08 (Energy Efficient Equipment) | Use high-efficiency commercial kitchen & AV equipment. |

## Aesthethics

### Acoustic Optimization for Theatre Performances

- Recycled soundproof panels ensuring speech clarity.

- Adjustable acoustic elements modulating sound dispersion.

Interior

Focusing on natural materials, acoustics, and visual harmony.

| Design Aspect | Sustainable Solution | Aesthetic Benefit |
| --- | --- | --- |
| Recycled & Low-VOC Paints | Use eco-friendly paints with no toxic emissions. | Allows for rich, vibrant color choices while improving indoor air quality. |
| Acoustic Panels from Recycled Materials | Use wood wool or felt panels from recycled content. | Enhances sound quality while adding aesthetic textures to walls and ceilings. |
| LED Architectural Lighting | Smart LED lighting with color temperature control. | Allows custom ambiance for different performances. |
| LED Stage Lighting | Energy-efficient dynamic lighting controls. | Creates immersive visual effects for performances. |

# Technical Design Specifications

These involve structural, mechanical, electrical, and material considerations.

## A. Structural & Envelope Specifications

| Component | Technical Specification | Sustainable Solution |
| --- | --- | --- |
| Foundation | Reinforced concrete | Reduces embodied carbon footprint |
| Walls & Insulation | U-Value ≤ 0.16 W/m²K | Use natural insulation (wood fiber, hempcrete) |
| Windows & Glazing | Triple-glazed with U-Value ≤ 1.0 W/m²K | Low-E coatings to reduce heat loss |
| Doors | Airtight, insulated doors (U-Value ≤ 1.8 W/m²K) | Reduces thermal bridging |

## B. Mechanical & HVAC System

| Component | Technical Specification | Sustainable Solution |
| --- | --- | --- |
| Ventilation | Mechanical Ventilation with Heat Recovery (MVHR) | ≥ 75% heat recovery efficiency |
| Water Heating | Solar thermal panels with storage tank | Reduces reliance on fossil fuels |
| Building Management System (BMS) | IoT-enabled energy monitoring | Optimizes energy use with real-time data |

## C. Electrical & Lighting System

| Component | Technical Specification | Sustainable Solution |
| --- | --- | --- |
| Lighting | LED with occupancy sensors | Reduces energy consumption |
| Power Supply | On-site renewable energy (solar PV) | Offsets grid reliance |
| Smart Grid Integration | Battery energy storage | Stores surplus energy for peak demand |

## D. Water Management & Drainage

| Component | Technical Specification | Sustainable Solution |
| --- | --- | --- |
| Water Fixtures | Dual-flush toilets, aerated taps | Reduces water usage |
| Drainage System | Sustainable Urban Drainage Systems (SuDS) | Prevents urban flooding & soil erosion |

# Structural System Overview

The theatre incorporates a steel frame construction with a space frame roof system, designed for long-span load efficiency, acoustic optimization, and environmental sustainability.

## Primary Load-Bearing Framework

- Structural Steel Beams (S355 grade) provide a yield strength of 355 N/mm², ensuring adequate resistance against dynamic and static forces.

- HEA 300 columns selected for primary supports, balancing compressive loads and lateral stability.

- IPE 450 beams used for stage and seating structures, accommodating live loads and minimising deflection.

## Roof Space Frame Load Management

- SHS 100 × 100 mm hollow sections distributed to form a grid supporting roof loads.

- Tapering geometry accommodates HVAC integration, preventing excessive deflection.

## Material Selection & Performance

Materials were selected based on strength, sustainability, and long-term durability.

| Material | Density (kg/m³) | Yield Strength (N/mm²) | Function |
| --- | --- | --- | --- |
| S355 Steel Beams | 7850 | 355 | Load-bearing steel frame |
| Corten Steel Cladding | 7800 | 345 | Weather-resistant façade |
| C35/45 Concrete | 2400 | 45 | Foundation and flooring |
| Rockwool Insulation | 100 | - | Acoustic & thermal insulation |

# Non-Technical Design Specifications

These involve aesthetic, social, and operational aspects of the theatre’s design.

## A. Architectural & Aesthetic Considerations

| Aspect | Non-Technical Specification | Sustainable Solution |
| --- | --- | --- |
| Acoustic Design | Sound-absorbing panels from recycled materials | Improves acoustics with eco-friendly materials |

## B. Social & Wellbeing Factors

| Aspect | Non-Technical Specification | Sustainable Solution |
| --- | --- | --- |
| Indoor Air Quality | Low-VOC materials & plants for air purification | Improves occupant health |
| Accessibility & Inclusivity | Step-free access, tactile paving | Ensures universal access |

## C. Operational & Lifecycle Considerations

| Aspect | Non-Technical Specification | Sustainable Solution |
| --- | --- | --- |
| Maintenance | Use of self-cleaning materials | Reduces cleaning costs & chemical use |
| End-of-Life Strategy | Design for deconstruction | Facilitates material reuse |

# Load Calculations for Structural Integrity

## Dead Load (Self-Weight) Computation

The total self-weight of the structure is derived using volume-weight equations:

Steel Frame Load Contribution

Wsteel=Volume×Density

Given beam spacing at 4m intervals:

Wsteel = 12m×0.3m×0.2m×7850=56.6 kN/m

Corten Cladding Load Contribution

Wcladding=10m×0.004m×7800=31.2 kN/m

Concrete Slab Load Contribution

Wconcrete=10m×0.15m×2400=36 kN/m

## Live Load Analysis

Occupancy loads distributed over the seating, stage, and service areas:

- Audience seating → 4 kN/m²

- Stage & backstage → 5 kN/m²

- Service zones → 0.75 kN/m²

## Wind & Environmental Load Analysis

The wind pressure is determined using the dynamic pressure equation:

P =12 ρV2

- Wind speed (Merthyr Tydfil region): 30 m/s

Pwind=0.5×1.225×(30)2=551.25 N/m²

- Applied lateral force on façade: 1.2 kN/m²

## Steel Section Verification

Column Design (HEA 300)

Axial Load Capacity:

P=σAP

P=355×(0.3×0.3)

Result: 32 kN per column, ensuring stable vertical support.

Beam Deflection Check (IPE 450)

Maximum Moment Capacity:

M= WL28

Result: 110 kNm, confirming allowable bending resistance.

# Foundation Strategy Computations

Spread Footing Foundation

Used for primary load-bearing columns.

Concrete pad dimensions: 3m × 3m × 0.75m thickness.

Reinforced C35/45 grade concrete ensures compressive strength ≥ 45 N/mm².

## Spread Footing Load-Bearing Check

Foundation pad size: 3m × 3m × 0.75m thickness Load Capacity:

Result: 580 kN/m², ensuring stable load distribution.

## Pile Foundation for High-Load Zones

- Necessary for areas with weaker soil strata.

- Bored piles (450mm diameter, embedded 8m deep) ensure ground stability.

- Pile spacing: 2m intervals to evenly distribute loads.

# Pile Foundation

450mm diameter bored piles chosen for high-load zones.

- Embedded depth: 8m for ground stability.

# Load-Bearing Capacity Calculations

## Column Load Distribution to Footing

Using axial load formula:

Pcolumn=σAP_

- Steel column HEA 300 (cross-section 0.3m × 0.3m):

Pcolumn=355×(0.3×0.3)=32 kN

- Total column load (considering multiple supports):

Ptotal=32×15=480 kN

## Bearing Capacity of Concrete Pad

Using bearing capacity formula:

qallowable= PA

- Footing area:

A=3m×3m=9 m²

- Allowable soil pressure: 580 kN/m²

qactual=4809=53.3 kN/m²

Result: The foundation is safely within allowable bearing capacity.

## Pile Foundation Load Considerations

Load per Pile Calculation

Using pile bearing formula:

Qpile= qsoilA + qfrictionL

Where:

- qsoil = 350 kN/m² (estimated soil bearing capacity).

- A = π × (0.45²) = 0.636 m² (pile cross-section).

- qfriction = 45 kN/m (shaft friction factor).

- L = 8m (embedded depth).

Total Load Support per Pile

Qpile =(350×0.636)+(45×8)

Qpile =222.6+360=582.6 kN

- With pile spacing at 2m intervals, total piles required:

N= PtotalQpile

N= 480582.6 = 0.82 (1 pile per column, safety distributed)

# Appendix

## Reference list

BREEAM (2023). BREEAM technical standards - BREEAM - Liferay DXP. [online] BREEAM. Available at: https://breeam.com/standards.

HM Government (2014). Conservation of fuel and power: Approved Document L. [online] GOV.UK. Available at: https://www.gov.uk/government/publications/conservation-of-fuel-and-power-approved-document-l.

LCETED (2025). LCETED Institute for Civil Engineers. [online] LCETED Institute for Civil Engineers. Available at: https://www.lceted.com/2024/10/Foundation-Design-Bearing-Capacity-and-Settlement.html.

STRUCTURAL DESIGN CHEATSHEET. (n.d.). Available at: https://www.structuralbasics.com/wp-content/uploads/2023/12/Structural-design-cheatsheet.pdf.

thestructuralworld (2022). Basics of Load Calculations in Structural Design | The Structural World. [online] The Structural World. Available at: https://www.thestructuralworld.com/2022/05/21/basics-of-load-calculations-in-structural-design/.