Marking Scheme

SENARIO

In an industrial factory, a conveyor belt transports products through different processing stages.

To maintain safety, prevent collisions, and detect jams, the system must continuously monitor the distance between an object on the conveyor and a fixed reference point.

An ultrasonic HC-SR04 sensor is mounted above the conveyor. When an object approaches, the system evaluates its distance and activates different LED indicators to represent the operational status.

A PIC16F877A microcontroller reads the distance from the ultrasonic sensor, displays the measured value and system status on a 16×2 LCD, and controls three LEDs that represent SAFE, WARNING, and DANGER conditions.

SYSTEM SPECIFACTIONS

- A 12V DC source must be regulated to 5V using a 7805-voltage regulator. This regulated 5V powers the PIC16F877A, HC-SR04 ultrasonic sensor, LEDs, and LCD.

- An Ultrasonic Distance Sensor (HC-SR04) measures the distance to an object on the conveyor and outputs the time-of-flight pulse.

  - Trigger pin connected to RB0.

  - Echo pin connected to RB4.

- Three LEDs connected to PORTC are used to communicate operational states:

Heater (RB0):

  - SAFE (Green LED – RC0)

    - Turns ON when the measured distance is greater than 50 cm.

    - Indicates the conveyor is clear and operating normally.

  - WARNING (Yellow LED – RC1)

    - Turns ON when distance is between 20 cm and 50 cm.

    - Indicates that an object is approaching the inspection zone.

  - DANGER (Red LED – RC2)

    - Turns ON when distance is less than 20 cm.

    - Indicates a jam, collision risk, or object too close to the sensor.

- A 16x2 LCD must display the operational status based on distance:

- “CONVEYOR NORMAL!” when distance > 50 cm

- “MOVEMENT NEARBY!” when 20–50 cm

- “INTRUDER ALERT!” when distance < 20 cm

Answer the Following

- Design the following circuits with proper components and wiring:

- 12V to 5V Power Supply using 7805.                                                	 [10 Marks]

SOLUTION

    - Correct use of 7805 voltage regulator (input, output, ground pins properly connected)								[2 Marks]

    - Input capacitor (typically 0.1µF or 10µF) connected between input and ground									[1 Mark]

    - Output capacitor (typically 0.1µF or 10µF) connected between output and ground								[1 Mark]

    - Input voltage source correctly set to 12V DC			[1 Mark]

    - Regulated 5V output successfully provided			[2 Marks]

    - Correct grounding for all components				[1 Mark]

    - Circuit layout clarity and completeness (no floating pins, labelled components)

[2 Marks]

- Ultrasonic distance measurement circuit using HC-SR04 interfaced with PIC16F877A. 										[10 Marks]

SOLUTION

- Correct 4-pin wiring: VCC, GND, TRIG (RB0), ECHO (RB4) 		[2 Marks]

- TRIG pin connected to PIC output (RB0) for generating 10 µs pulse 	[2 Marks]

- ECHO pin connected to PIC input (RB4) with proper signal routing 	[2 Marks]

- Stable 5V supply provided to VCC and proper grounding to GND 	[2 Marks]

- Clean layout and simulation-ready setup in Proteus.			[2 Marks]

- Simulate and validate the above (i) said designed circuits:

- Show regulated 5V output from the power supply.			[5 Marks]

SOLUTION

[5 Marks]

(b)	HC-SR04 ultrasonic sensor simulation showing varying distance outputs. [5 Marks]

SOLUTION

[1.5 Marks]

[1.5 Marks]

[2 Marks]

- Develop a single CCS C program to:

- Generate trigger pulse on RB0 for the ultrasonic sensor.

- Measure echo pulse on RB4 using Timer1 and calculate distance.

- Control three outputs (RC0 = SAFE, RC1 = WARNING, RC2 = DANGER) based on distance range.

- Display “CONVEYOR NORMAL!”, “MOVEMENT NEARBY!”, or “INTRUDER ALERT!” on LCD.

[30 Marks]

SOLUTIONS

LCD Connections, Ultrasonic pins, LED Status				[2 Marks]

Threshold Values								[4 Marks]

Sending Triger Pulse								[2 Marks]

Measuring Echo pulse								[4 Marks]

Time ticks to Distance								[4 Marks]

Conditions for Distance and display status					[4 Marks]

  - Including Adequate comments						[2 Marks]

  - Variable Naming and Declaration: Considering the Ports from the question and declaring them.								[ 2 Marks]

  - Code Optimization							[2 Marks]

  - Code Structure and Readability: Marks for logical Flow and easy understandability

[2 Marks]

  - Correctness and Functionality.						[2 Marks]

- Simulate the complete system and demonstrate functionality by submitting clear screenshots showing the correct outputs for the responses on relays and LCD under cold and hot conditions.												[20 Marks]

- Functional output for SAFE condition (distance > 50 cm):
- Green LED (SAFE_LED on RC0) ON					[2 Marks]
- LCD displays: "CONVEYOR NORMAL!"				[2 Marks]

- Functional output for WARNING condition (20 cm ≤ distance ≤ 50 cm):

- Yellow LED (WARN_LED on RC1) ON 				[2 Marks]

- LCD displays: "MOVEMENT NEARBY!" 				[2 Marks]

- Functional output for DANGER condition (distance < 20 cm):

- Red LED (DANG_LED on RC2) ON 				[2 Marks]

- LCD displays: "INTRUDER ALERT!" 				[2 Marks]

- Clear and labelled screenshots showing all three states  		[4 Marks]

- Screenshot 1: ≈60 cm with Green LED ON and “CONVEYOR NORMAL!”

- Screenshot 2: ≈35 cm with Yellow LED ON and “MOVEMENT NEARBY!”

- Screenshot 3: ≈15 cm with Red LED ON and “INTRUDER ALERT!”

- Labels for sensor pins (TRIG/ECHO), LEDs, LCD, PIC pins, and supply

- Accuracy and presentation of the overall system (clear layout, correct logic, no floating connections, realistic behaviour)					[4 Marks]

- Neat wiring
- No floating pins
- Consistent logic
- Proper component placement (power, crystal, regulators)

- Answer the following theoretically

- Explain the working principle of the HC-SR04 ultrasonic sensor with a neat diagram.										[10 Marks]

SOLUTION

[3 Marks]

- MCU sends a 10 µs HIGH pulse on TRIG 					[1 Mark]

- sensor emits an 8-cycle burst of ultrasound at ~ 40 kHz.		[1 Mark]

- Sound travels through air, hits an object, and reflects.			[1 Mark]

- Sensor sets ECHO pin HIGH immediately after the burst; when echo returns, ECHO goes LOW. 										[1 Mark]

- The duration ECHO stayed HIGH = round-trip time of sound.		[1 Mark]

- Because the sound travels to the object and back, the /2 divides the round trip.

[1 Mark]

- Distance is calculated using formula:

Distance = (Time of Flight × Speed of Sound) / 2

[1 Mark]

- Describe the internal operation and need for a 7805-voltage regulator with a neat diagram. 										[10 Marks]

SOLUITION

- The voltage regulator has an unregulated dc input voltage Vi applied to one input terminal, a regulated output dc voltage Vo from a second terminal, and the third terminal connected to ground.						[2 Marks]

[3 Marks]

- The 7812 is a fixed +12 V linear voltage regulator used to obtain a constant DC output.

An unregulated DC input voltage (Vi) higher than 12 V is applied to Pin 1 (IN).

- Capacitor C1 at the input filters ripples and noise from the incoming supply. Inside the 7812, an internal reference voltage and control circuit compare the output with the reference.										[1 Mark]

- Based on this comparison, the internal pass transistor adjusts automatically to maintain a steady 12 V output.

- Pin 3 (GND) provides a reference ground for proper regulation.			[1 Mark]

- The regulated +12 V output (Vo) is taken from Pin 2 (OUT).				[1 Mark]

- Capacitor C2 at the output improves stability and reduces output voltage fluctuations.

[1 Mark]

- The 7812 includes built-in protection against overcurrent, overheating, and short circuits.

[1 Mark]

- Explain the functions of RS, RW, E, D0–D7, VSS, VDD and VEE in a 16x2 LCD module.													[10 Marks]

SOLUTION

- RS (Register Select):

  - Selects between instruction register (RS = 0) and data register (RS = 1).

  - Controls whether the input sent is a command or display data.

[2 Marks]

- RW (Read/Write):

  - Controls the direction of data.

  - RW = 0 for write operations (from MCU to LCD),

  - RW = 1 for read operations (from LCD to MCU).

[2 Marks]

- E (Enable):

  - Acts as a latch.

  - On a high-to-low pulse, the LCD reads data from the data lines.

  - Must be toggled each time data/command is sent.

[2 Marks]

  - D0–D7 data pins – State their role in sending data or instructions; mention 4-bit/8-bit modes.							[2 Marks]

- VSS and VDD pins – Identify ground and positive supply terminals.

[1 Mark]

- VEE pin – Mention its use for adjusting display contrast.

[1 Mark]