# Design Portfolio Activity

ACDB 4005 Principles of Systems Engineering

Level 4 Ordnance, Munitions and Explosive Engineering

School of Applied Computing

Elias James

Table of Contents

1.   Introduction	2

2.   Stakeholders and Operational Requirements	3

3.   Customer Requirements and Use Case	4

4.   Functional Analysis	6

5.   Rationalising Customer Requirements through Pairwise comparison	10

6.   Quality Function Deployment 1	13

7.   Functional Metrics	16

8.   Concept Generation and Functional Means Analysis	18

9.   Concept Selection	20

10. Quality Function Deployment 2	22

11. Conclusion	25

12. Bibliography	26

Table of Figures and Tables

Figure 1 Functional Flow Diagram for the kettle.	10

Figure 2 Results of the PWC matrix.	13

Figure 3 QFD 1 functional importances derived by the QFD 1 matrix.	16

Figure 4 Design feature importance derived from QFD 2 matrix.	25

Table 1 Stakeholder requirements.	4

Table 2 Generic functions of a domestic kettle derived from the use case and stakeholder requirements.	8

Table 3 Pairwise Comparison matrix weighing customer requirements against each other.	12

Table 4 QFD 1 matrix.	15

Table 5 Functional Metrics matrix using Heat contents function.	18

Table 6 FMA table.	20

Table 7 Concept solutions derived from the FMA table.	21

Table 8 Pugh Matrix using three concept solutions.	22

Table 9 QFD 2 Matrix.	24

# Introduction

Systems engineering is a multi-disciplinary approach to designing, integrating and managing complex systems throughout their lifecycle [1]. Unlike other engineering disciplines, it focuses on the system as a whole and on how it interacts with other systems, both externally and internally [2].  A system is defined as a set of components working towards a common goal. Therefore, rather than looking at individual components, systems engineering looks at the interactions between the system’s elements and ensures the final system can implement its purpose reliably and efficiently. Systems engineering’s primary focus is the system's success in meeting its requirements and development aims [2]. Examples of systems that require systems engineering include complex systems such as aeroplanes, weather satellites, and power plants.

The benefits of using system engineering can be:

- Complex systems often have dependencies and interactions which are not always obvious at the individual component level. Using methods such as requirement analysis enables you to highlight and understand these interactions.

- In systems engineering, stakeholders’ needs are translated into measurable requirements, and these can be traced throughout the entire process. Resulting in every design decision being justified and the final system addressing the stakeholders’ needs [3].

In this assignment, the systems engineering approach will be used to design an improved household device. By doing this, this assignment can translate stakeholder needs into a fully justifiable design solution. The system engineering methods that will be utilised are requirement analysis, Quality Function Deployment, functional means analysis, and concept selection tools such as Pugh matrices.

# Stakeholders and Operational Requirements

Identifying stakeholders at the start of the systems engineering process ensures their needs are recognised and implemented throughout the design process. A household kettle, although a simple system, has a wide range of users, safety constraints, environments and manufacturing considerations to consider when designing

For this assignment, the primary stakeholders were the domestic users of the kettle, and their requirements are shown in Table 1.

Table 1 Stakeholder requirements.

| Operator | The operator wants a fast boil time |
| --- | --- |
|  | The operator wants low energy consumption |
|  | The operator wants easy to clean |
|  | The operator wants it to have a clear water level with cup amounts and not just litres. |
|  | The operator wants it to have minimal noise when boiling. |
| Partner | The partner requires a large enough capacity for making drinks for a larger party if hosting |

In addition to these users, other external stakeholders can be identified. Manufacturers and the supply chain have an interest in production costs. Retailers would be interested in product appeal. Safety and standards organisations require the kettle to comply with electrical safety regulations and standards. Identifying all stakeholders in the preliminary stages of systems engineering is crucial for considering all needs and constraints to develop the best solution. In a more in-depth systems engineering approach, the many stakeholders can be identified by constructing a stakeholder influence map and then capturing, organising and validating their requirements using affinity and tree diagrams.

To also prevent unrestrained expansion, a system boundary has also been defined. This system comprises only of kettle components, i.e. base, lid, control and sensing, and does not include external items such as the electrical grid and the user's mug, for example.

From these stakeholder requirements, you can then perform functional analysis showing the required functional behaviours and the key functions of the product. By analysing requirements, you can have a fuller understanding of the problem and find any unspoken requirements. The Holistic Requirements Model outlines the complete and consistent set of requirements for a system [4].

- Operational requirements (OR)

- Functional requirements (FR)

- Non-functional requirements (NF)

  - Non-functional performance requirements (NFP)

  - Non-functional system requirements (NFS)

  - Non-functional Implementation requirements (NFI)

Operational Requirements (OP) define the system's purpose and constraints. This is not often stated by the customers/stakeholders and must be derived. From the stakeholder analysis done above, the OP for the kettle was defined as:

To provide a safe, user-friendly and reliable means to heat water to boiling point for hot drinks, using domestic mains power.

This is derived from the stakeholder requirements and design features and provides this assignment with a constant reference point, ensuring all development work remains aligned with the same goal.

# Customer Requirements and Use Case

After identifying stakeholders and defining the OP, the next stage is to structure user needs. This is where you take the customer needs and convert them into functional and design requirements. The customer needs are rationalised to clarify meaning and remove any overlap. By doing this, it results in concise, meaningful and non-duplicated requirements which can offer traceability into functional and design characteristics. The rationalised requirements for this assignment are:

- Fast boil time

- Low energy consumption

- Ease of cleaning

- Water level visibility

- Minimal operating noise

- Sufficient water capacity

- Aesthetically pleasing design

Alongside this, a use case can be derived to determine how a user will interact with a system to achieve the goal or, in this case, the OP. It can understand user behaviour and identify functional requirements. It shows pre- and post-conditions (what success and failure look like in this system), the basic and alternative flows for using a kettle, and any exception flows which outline any undesirable effects that could occur. Below is the use case for the domestic kettle system.

| Use Case for boiling water in a domestic kettle |  |  |  |
| --- | --- | --- | --- |
| Author E James | Issue -1 |  | Date 02/1/26 |
| Actors: Operator Partner Secondary stakeholder: Visitors |  |  |  |
| Pre-conditions: - - The kettle is plugged in. - The kettle is in a safe working condition. - Access to a clean water source I.e. Tap. |  |  |  |
| Post-Conditions: - Success: - Water is heated to boiling point (100C). - Water is safe to pour. - Kettle shuts off automatically. Failure: - Kettle fails and does not heat water - Kettle shuts off due to safety condition |  |  |  |
| Basic Flow:- - Lift kettle from base. - Open lid. - Fill kettle with water to the desired level using indicators. - Close lid. - Place kettle back on base ensuring proper contact. - Switch the kettle on using the power switch. - Kettle light turns on indicating operation. - Heating element activates and heats water. - Thermostat monitors temperature. - Water reaches boiling point. - Thermostat triggers automatic shut-off. - Lift kettle. - Pour water safely into appropriate container. |  |  |  |
| Alternative Flow:- - Step 1 skipped and kettle is filled without removing from base. - Fill through spout instead of opening lid. - Operator turns off kettle manually before boiling point. |  |  |  |
| Exception Flow :- - No water in kettle so dry boils. - Lid not closed properly or not put back on if it comes off completely. - Overfilled beyond the max line. - Kettle not seated correctly on base. - Fails to heat when switched on. - Operator lifts kettle during heating. |  |  |  |

# Functional Analysis

Following the systems engineering practice, these customer requirements and the use case can be translated into a set of generic system functions. By completing this functional analysis, you can show what the system must do without defining how it will do it. This step allows you to separate system behaviour from the technical solutions and therefore supports better design flexibility further down the process.

A verb-noun format was used to define the kettle's functions. This is so the functional requirements do not specify a solution and only what the system must do. For example, secure lid rather than screw on lid [4]. The functions for this assignment are compiled into the generic functions table below (Table 2), where a logical breakdown of the system's behaviour is specified using the use case and stakeholders' needs.

Table 2 Generic functions of a domestic kettle derived from the use case and stakeholder requirements.

| Operational Requirement |  | Generic Function | Definition |
| --- | --- | --- | --- |
| To provide a safe, user-friendly and reliable means to heat water to boiling point for hot drinks, using domestic mains power. | PREPARE WATER | Receive water | Allows the kettle to be filled with water. |
|  |  | Verify water level | Detects if the water volume is between the minimum and maximum safe levels. |
|  |  | Secure lid | Ensures the lid is closed to prevent spills. |
|  | PREPARE HEATING SYSTEM | Seat kettle | Aligns the kettle correctly onto the base establishing an electrical contact. |
|  |  | Supply power | Delivers electrical energy to the heating element. |
|  |  | Indicate Heating | Signal when the kettle is operating. |
|  | HEAT WATER | Heat contents | Converts energy into heat through the heating element. |
|  |  | Monitor temperature | Senses water temperature. |
|  |  | Control heat | Regulates heating and ensures efficient boiling. |
|  | ENSURE SAFETY | Detect boil | Identifies when water has reached boiling point. |
|  |  | Shut off heat | Automatically disables the heating element when boiling is detected. |
|  |  | Indicate Ready | Signal when kettle is finished |
|  | DELIVER WATER | Enable pouring | Allows safe pouring of hot water. |

A functional flow diagram can also be constructed to understand the system's narrative, based on its component functions and how they interact to achieve the goal (OP). This often reveals unidentified functions. It can also outline the system's interfaces, as each flow is an interaction between these generic functions, and display any problem areas [5].

The functional flow diagram in Figure 1 shows the logical sequencing of the kettle components and their interactions with one another from receiving water to delivering water. Exception paths are also included, such as scenarios like dry boiling that are safety-critical behaviours.

Figure 1 Functional Flow Diagram for the kettle.

# Rationalising Customer Requirements through Pairwise comparison

Before making design decisions based on stakeholder requirements, requirements need to be rationalised and therefore weighted against each other. Often, stakeholder requirements are qualitative and vary in significance. After these requirements are assessed and any duplication removed, a pairwise comparison (PWC) can be completed. This is where every requirement is evaluated against each other, and a ranking system of them is produced. This allows for effective decision-making, determines relative importance, and avoids any biased prioritisation.

Table 3 below is the pairwise comparison of the kettle customer requirements. Everything was scored from no preference to strong preference and then normalised, resulting in the bar chart shown in Figure 2. The chart clearly indicates that fast boil time and low energy consumption are the most critical requirements, with the kettle’s aesthetic appeal being the least important.

Table 3 Pairwise Comparison matrix weighing customer requirements against each other.

| Pairwise comparison of requirements | The operator wants a fast boil time | The operator wants low energy consumption | The operator wants easy to clean | The operator wants it to have a clear water level with cup amounts and not just litres. | The operator wants it to have minimal noise when boiling. | The partner requires a large enough capacity for making drinks for a larger party if hosting | The partner wants it to look attractive and fit into the kitchen colours. | The partner wants it to be easy to fill | Row Product | nth root of row product | Total Normalised |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The operator wants a fast boil time | 1 | 1 | 2 | 5 | 5 | 2 | 9 | 2 | 1800.00 | 2.30 | 23.73 |
| The operator wants low energy consumption | 1 | 1 | 2 | 5 | 5 | 2 | 9 | 2 | 1800.00 | 2.30 | 23.73 |
| The operator wants easy to clean | 0.5 | 0.5 | 1 | 2 | 2 | 1 | 5 | 1 | 5.00 | 1.20 | 12.34 |
| The operator wants it to have a clear water level with cup amounts and not just litres. | 0.2 | 0.2 | 0.5 | 1 | 1 | 0.5 | 2 | 0.5 | 0.01 | 0.60 | 6.18 |
| The operator wants it to have minimal noise when boiling. | 0.2 | 0.2 | 0.5 | 1 | 1 | 0.5 | 2 | 0.5 | 0.01 | 0.60 | 6.18 |
| The partner requires a large enough capacity for making drinks for a larger party if hosting | 0.5 | 0.5 | 1 | 2 | 2 | 1 | 5 | 1 | 5.00 | 1.20 | 12.34 |
| The partner wants it to look attractive and fit into the kitchen colours. | 0.11 | 0.11 | 0.2 | 0.5 | 0.5 | 0.2 | 1 | 0.2 | 0.00 | 0.31 | 3.17 |
| The partner wants it to be easy to fill | 0.5 | 0.5 | 1 | 2 | 2 | 1 | 5 | 1 | 5.00 | 1.20 | 12.34 |

Figure 2 Results of the PWC matrix.

# Quality Function Deployment 1

Quality Function Deployment (QFD) is a systems engineering tool used to translate customer needs into valued functions. This process ensures that customer needs are linked to what the system must do and avoids focusing on designing features that do not meaningfully contribute to customer requirements. Burge, Hughes and Walsh emphasise that you need to start from the functions rather than the components, and you should focus on what the system must achieve rather than the specifics of how it is achieved [6].

In this assignment, a QFD 1 matrix was used to weigh the rationalised customer requirements from the PW to the functional requirements identified in the functional analysis. As shown in Table 4, each customer requirement was assigned an importance rating taken directly from the PWC, which was then multiplied by the strength of its relationship to the functional requirements. This was scored by 9, 3 or 1, where 9 is a strong relationship and 1 is weak. Boxes were also left blank where no relationship existed. This ultimately results in a way of identifying which functions matter most to the users.

Figure 3 shows the results from this matrix and shows clearly important functions. By a significant margin, the highest scoring function is heat contents, which reflects the most important customer requirements of fast boiling and low energy consumption. Supplying power and controlling heat came out as the next most important functions, showing that the control and management of heating are strongly valued. Conversely, the functions of system ready and heating indication scored very low on importance. This indicates that these functions have very little importance in the customer requirements and will have little effect on designing a kettle that meets these needs. This has consequently highlighted that these indication features add limited stakeholder benefit and that any design effort put into them could be wasted.

Table 4 QFD 1 matrix.

| How | Importance | Receive water | Verify water level | Secure lid | Seat kettle | Supply power | Indicate Heating | Heat contents | Monitor temperature | Control heat | Detect boil | Shut off heat | Indicate Ready | Enable pouring |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The operator wants a fast boil time | 23.73 |  | 1 | 1 | 1 | 9 |  | 9 | 1 | 3 |  |  |  |  |
| The operator wants low energy consumption | 23.73 | 1 | 3 | 3 | 1 | 3 |  | 9 | 3 | 9 | 3 | 3 |  |  |
| The operator wants easy to clean | 12.34 | 3 | 1 | 3 |  |  |  | 1 |  |  |  |  |  | 3 |
| The operator wants it to have a clear water level with cup amounts and not just litres. | 6.18 | 1 | 9 |  |  |  |  |  |  |  |  |  |  |  |
| The operator wants it to have minimal noise when boiling. | 6.18 |  |  | 3 | 1 |  | 1 | 9 |  | 3 |  |  | 3 |  |
| The partner requires a large enough capacity for making drinks for a larger party if hosting | 12.34 | 9 | 3 | 3 | 3 | 3 |  | 3 |  | 1 | 1 | 1 |  | 3 |
| The partner wants it to look attractive and fit into the kitchen colours. | 3.17 | 3 | 3 | 3 | 3 | 1 | 3 |  |  |  |  |  | 3 | 3 |
| The partner wants it to be easy to fill | 12.34 | 9 | 3 | 9 | 1 |  |  |  |  |  |  |  |  | 3 |
|  |  | 298 | 246 | 308 | 112 | 325 | 16 | 532 | 95 | 316 | 84 | 84 | 28 | 121 |

Figure 3 QFD 1 functional importances derived by the QFD 1 matrix.

# Functional Metrics

Following QFD1, where the highest valued functions were identified, you can then utilise functional metrics to understand how well a function performs. This is a good way to maintain traceability from stakeholder needs to engineering verification.

The metrics used in this matrix are again functional rather than individual component level to avoid bias and ensure solution independence. The highest-ranking functions from QFD 1 are used and weighted against the customer requirements. To complete this fully, you would usually complete a matrix for each highest-ranking function, for example heat contents, supply power and control heat, etc. However, in this assignment, I will demonstrate the functional metrics benefit by completing only the top-ranked function. The key metrics included in Table 5 below are: time to boil, energy used per boil, final water temperature, dry boil cut-off time, noise level during boiling and external surface temperature. These metrics reflect the customer requirements and are directly correlated with the heat contents function, i.e. energy used per boil reflects the customer requirement of low energy consumption. The relationships between the customer requirements and metrics were scored using the same 9, 3 and 1 weighting as QFD 1. This allows us to rank metrics that will strongly influence customer satisfaction and identify which should be priorities during testing and later stages.

Table 5 Functional Metrics matrix using Heat contents function.

| Function:- Heat contents Test method and relationship to requirements | Importance | Relationship | Time to boil | Energy used per boil | Final water temperature | Dry boil cut off time | Noise Level | External Surface Temperature |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Requirements |  |  |  |  |  |  |  |  |
| The operator wants a fast boil time | 23.73 |  | 9 | 3 | 1 |  | 1 |  |
| The operator wants low energy consumption | 23.73 |  | 3 | 9 | 1 | 3 |  | 1 |
| The operator wants easy to clean | 12.34 |  |  |  |  |  |  |  |
| The operator wants it to have a clear water level with cup amounts and not just litres. | 6.18 |  |  |  |  |  |  |  |
| The operator wants it to have minimal noise when boiling. | 6.18 |  | 1 | 1 |  |  | 9 |  |
| The partner requires a large enough capacity for making drinks for a larger party if hosting | 12.34 |  | 3 | 3 | 1 | 1 | 1 | 1 |
| The partner wants it to look attractive and fit into the kitchen colours. | 3.17 |  |  |  |  |  |  | 1 |
| The partner wants it to be easy to fill | 12.34 |  |  |  |  |  |  |  |
|  |  | 0 | 328 | 328 | 60 | 84 | 92 | 39 |

# Concept Generation and Functional Means Analysis

After identifying the system’s functions in functional analysis and QFD 1, Functional Means Analysis (FMA) was used to generate potential design solutions. FMA supports concept generation by listing each function and identifying multiple alternative means of achieving that function, ensuring that the design space is explored entirely before selecting [7].

An FMA table was produced for the kettle in Table 6, with the functional requirements in one column and several means generated for each, ranging from conventional kettle designs to more innovative solutions. It is key that no options are rejected at this point, and there is also the possibility of not doing that function.

Burge, Hughes, and Walsh then suggest the next stage of concept generation in which whole-system concepts are formed by selecting one for each function [7]. From these, three system concepts were produced: Smart Kettle, Outdoor Gas Kettle and a Built-in Kettle. As seen in Table 7.

FMA is valued as it enables structured creativity and functional thinking. It ensures that all three final concepts align with the customer needs and moves the design process to justifiable solutions rather than functions.

Table 6 FMA table.

| Function | Means |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Receive water | Fill through spout | Fill through open lid | Reservoir that attaches | Plumbed into mains water | Funnel through port |  |  |
| Verify water level | Sight glass | float | Electronic Level sensor | Led bar that rises with level | don’t |  |  |
| Secure lid | thread | click/lock | magnetic | Twist lock | Gravity | hinge | don’t |
| Seat kettle | magnetic alignment | twist/lock | central point can be used in any orientation | slide and lock | thread | don’t |  |
| Supply power | mains to base | mains to kettle | Battery | induction pad | solar | wind | gas |
| Indicate Heating | LED | pop up flag | whole kettle lights up | Noise (beep) | don’t |  |  |
| Heat contents | Heating element | Gas flame | induction | nuclear | fire |  |  |
| Monitor temperature | digital probe | Analogue probe | thermistor | infrared | thermal camera | don’t |  |
| Control heat | manual off/on | thermostat controller | boil detection cut off |  |  |  |  |
| Detect boil | steam sensor | temperature sensor | bubble sensor |  |  |  |  |
| Shut off heat | switch cut off | manual switch | time based switch | Electronic Relay | don’t |  |  |
| Indicate Ready | LED | Noise (Beep) | pop up flag | phone notification | whole kettle changes colour | don’t |  |
| Enable pouring | Automatic Dispenser | handle | 2 handles | pouring trigger |  |  |  |

Table 7 Concept solutions derived from the FMA table.

| Function | Smart Kettle | Outdoor Gas Kettle | Built in Kettle |
| --- | --- | --- | --- |
| Receive water | Fill through spout | Reservoir that attaches | plumbed into mains |
| Verify water level | Electronic level sensor | float | sight glass |
| Secure lid | Magnetic | thread | don’t |
| Seat kettle | Magnetic | thread | don’t |
| Supply power | Induction pad | gas | mains to kettle |
| Indicate Heating | Whole kettle lights up | pop up flag | LED |
| Heat contents | Induction | gas flame | Heating element |
| Monitor temperature | Digital probe | analogue probe | infrared |
| Control heat | boil detection cut off | manual off/on | thermostat controller |
| Detect boil | temperature sensor | bubble sensor | steam sensor |
| Shut off heat | electronic relay | manual switch | switch cut off |
| Indicate Ready | phone notification | pop up flag | LED |
| Enable pouring | pouring trigger | 2 handles | automatic dispenser |

# Concept Selection

To select the ultimate kettle system concept, a Pugh Matrix was used. This continues the objective decision-making throughout this process by comparing each function concept against the customer requirements. It is scored from -2 (much worse) to +2 (much better) relative to a standard, in this case, a standard household kettle. By doing this, it supports rational decision-making and avoids bias.

In this assignment, the three system concepts of smart kettle, outdoor gas kettle and built-in kettle were evaluated against the standard household kettle in Table 8. Extra requirements, such as availability and purchase cost, were also factored in. These scores were then weighted against the customer requirement importance found in the PWC to give an overall ranking. This showed that the smart kettle was the superior choice, and the gas kettle was the worst due to its suitability and performance in a domestic capacity. Due to this, the Smart kettle was chosen as the preferred concept.

Table 8 Pugh Matrix using three concept solutions.

|  | Relative Importance Equal | Relative importance Weighted | Standard household kettle | Smart Kettle | Outdoor Gas Kettle | Built in Kettle |
| --- | --- | --- | --- | --- | --- | --- |
| WHATs - Customer Requirements |  |  |  |  |  |  |
| The operator wants a fast boil time | 5 | 23.73 | 0 | 1 | 0 | 2 |
| The operator wants low energy consumption | 5 | 23.73 | 0 | 1 | -1 | -2 |
| The operator wants easy to clean | 5 | 12.34 | 0 | 0 | -1 | -2 |
| The operator wants it to have a clear water level with cup amounts and not just litres. | 5 | 6.18 | 0 | 2 | -1 | -2 |
| The operator wants it to have minimal noise when boiling. | 5 | 6.18 | 0 | 1 | 1 | 0 |
| The partner requires a large enough capacity for making drinks for a larger party if hosting |  | 12.34 | 0 | 0 | 1 | 2 |
| The partner wants it to look attractive and fit into the kitchen colours. | 5 | 3.17 | 0 | 1 | -1 | 2 |
| The partner wants it to be easy to fill | 5 | 12.34 | 0 | 0 | -1 | 2 |
| Development Cost | 5 | 10.00 | 0 | -2 | -1 | -2 |
| Purchase Cost | 5 | 10.00 | 0 | -1 | -1 | -2 |
| Availability | 5 | 10.00 | 0 | -1 | -2 | -1 |
|  |  |  | 0 | 10 | -40 | -25 |
|  |  |  | 0 | 29.1742 | -79.2303 | -31.362 |
| Ranking | 100 |  | 0 | -19.1742 | 39.2303 | 6.362 |

# Quality Function Deployment 2

QFD 2’s purpose is to translate the system design requirements into solution requirements. Unlike QFD 1 which ensured system functions aligned with customer needs, QFD 2 ensures the generated solution is compliant with the design requirements. QFD 2 utilises the prioritised functional requirements from QFD 1 and links these to the selected design solutions. This ensures traceability from stakeholders’ needs to the system design, highlighting and featuring those that will contribute little value.

In this assignment, the smart kettle solutions were mapped against the functions from QFD 1, and the same relationship scores of 9, 3 and 1 were used (Table 9). These values were multiplied by the functional importance rating from QFD 1 to obtain the total feature importance, as shown in Figure 4.

Figure 4 shows that induction heating is by far the most critical design feature as it supports the most important functions found in QFD 1, such as heat contents, control heat and supply power. Other top-ranked design features, such as induction pad and boil detection cut off, rank highly due to also contributing towards critical functions that drive safety and control and therefore align with top stakeholders’ requirements and important functions. The lower-scoring features, such as phone notification and whole kettle lighting up, agree with the previously identified indication functions, which ranked as of lower importance. This ensures the engineering focus is aligned with the most important design solutions.

This process ultimately moves the user-driven functional priorities into design-level priorities and allows decisions to be made on potential design trade-offs and cost allocations. It shows that the smart kettles' performance in delivering the customer requirements will depend on controlling and delivering heat safely and efficiently, and not, for example, on whether the kettle has an indicating light on it.

Table 9 QFD 2 Matrix.

| Design feature | Importance |  | Importance | Fill through spout | Electronic level sensor | Magnetic | Magnetic | Induction pad | Whole kettle lights up | Induction | Digitial probe | Boil detection cut off | Temperature sensor | Electronic relay | Phone notification | Pouring trigger |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Functionality |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Receive water | 298.5 | 10 | 30 | 9 |  |  |  |  |  |  |  |  |  |  |  |  |
| Verify water level | 246.4 | 10 | 25 |  | 9 |  |  |  |  |  |  |  |  |  |  |  |
| Secure lid | 308.0 | 10 | 31 |  |  | 9 |  |  |  |  |  |  |  |  |  |  |
| Seat kettle | 112.5 | 10 | 11 |  |  |  | 9 | 3 |  |  |  |  |  |  |  |  |
| Supply power | 324.9 | 10 | 32 |  |  |  | 3 | 9 |  | 9 |  |  |  |  |  |  |
| Indicate Heating | 15.7 | 10 | 2 |  |  |  |  |  | 9 |  |  |  |  |  |  |  |
| Heat contents | 532.1 | 10 | 53 |  |  |  |  | 3 |  | 9 |  | 1 |  |  |  |  |
| Monitor temperature | 94.9 | 10 | 9 |  |  |  |  |  |  |  | 9 | 3 | 3 |  |  |  |
| Control heat | 315.6 | 10 | 32 |  |  |  |  |  |  | 1 | 3 | 9 | 3 | 3 |  |  |
| Detect boil | 83.5 | 10 | 8 |  |  |  |  |  |  |  | 3 | 3 | 9 | 1 |  |  |
| Shut off heat | 83.5 | 10 | 8 |  |  |  |  |  |  |  |  | 3 | 3 | 9 |  |  |
| Indicate Ready | 28.1 | 10 | 3 |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| Enable pouring | 120.5 | 10 | 12 |  |  | 1 |  |  |  |  |  |  |  |  |  | 9 |
|  |  |  |  | 269 | 222 | 289 | 199 | 486 | 14 | 803 | 205 | 416 | 223 | 178 | 25 | 108 |

Figure 4 Design feature importance derived from QFD 2 matrix.

# Conclusion

This assignment used a structured systems engineering approach to find a design solution for a domestic kettle. It progressed from stakeholders’ needs to a fully rationalised design solution.

Stakeholder and operational requirements were identified and translated into functional requirements rusing a use case and a functional flow diagram. These functions were then prioritised using a PWC and QFD 1 matrix, and the most critical functions were identified. Different function concepts were derived by using FMA, and the top three design solutions were then evaluated against a standard kettle using a Pugh Matrix. From this, the smart kettle concept emerged as the better solution, offering improvements in most areas. This final concept was then evaluated in a QFD 2 matrix, ensuring that each design function solution was critical to the customer’s requirements. This, therefore, highlighted the key areas for further engineering development.

Systems engineering can go on further from this to develop subsystems, components, and a manufacturing design system. This can allow for hazard analysis, cost modelling, verification planning, and ultimately seeing the system through its lifecycle.

Overall, this assignment demonstrates how systems engineering supports stakeholder-driven design decisions and ensures that the final concept is functionally appropriate and aligned with user needs.

# Bibliography

[1]	NASA, NASA Systems Engineering Handbook, 2nd edn. in NASA SP-2016-6105 Rev2. NASA, 2016.

[2]	Seymour Samuel J, Flanigan David A, Biemer Steven M, Wiley InterScience, and Kossiakoff Alexander, Systems engineering: principles and practice, Third edition. in Wiley series in systems engineering and management. Hoboken, NJ: John Wiley & Sons, Inc., 2020.

[3]	B. S. Blanchard and W. J. Fabrycky, Instructors Guide to Problem Solutions For Systems Engineering And Analysis, 13th edn, vol. 4. in Blanchard Solutions Manual, vol. 4. Englewood Cliffs, NJ: Pretince Hall, 1990.

[4]	Burge, Hughes and Walsh Ltd, ‘Holistic Requirements Model Tool’. [Online]. Available: https://www.burgehugheswalsh.co.uk/Uploaded/1/Documents/Holistic-Requirements-Model-Tool-v2.pdf

[5]	Burge, Hughes and Walsh Ltd, ‘Functional Modelling’. [Online]. Available: https://www.burgehugheswalsh.co.uk/Uploaded/1/Documents/Functional-Modelling-Tool-Draft-2.pdf

[6]	Burge, Hughes and Walsh Ltd, ‘A Functional Approach to Quality Function Deployment’, 2007.

[7]	Burge, Hughes and Walsh Ltd, ‘Functional Means Analysis’.