BENDING BEAMS

SOLUTIONS

Exercise 1:

Calculate the maximum stress that will be produced in a steel beam of rectangular cross-section when bent into the arc of a circle of radius 5 m. The depth of the beam is 6 mm, the modulus of Elasticity 210 kN/mm2. Give your answer in kN/mm2.

Solution:

σmax=ymaxER=3×2105×103=0.126 kNmm2

Exercise 2:

Calculate the radius to which a steel strip of thickness 4 mm can be bent if the maximum stress acting on the material is not to exceed 100 N/mm2. The material has a modulus of elasticity of 210 kN/mm2.

Solution:

R=ymaxEσmax=2×210×103100=4200 mm=4.2 m

Example 3:

A beam of rectangular cross-section, 40 mm  160 mm deep, is subjected to a bending of 35 kN.m. Determine the maximum bending stress in the beam material.

Solution:

σmax=MymaxI=35×80×10-340×10-3×80×10-3312=1.64 GPa

Example 4:

The cross-section of the idealised beam shown in the following figure has overall dimension of 100 mm  150 mm deep with a flange and web thickness of 20 mm. Calculate the second moment of area of the section about the neutral axis. If the bending stress in the beam material is not to exceed 180 MN/m2, determine the maximum bending moment in the beam.

150 mm

20

mm

20 mm

20 mm

100 mm

Solution:

M=Iσmaxymax

Where,

I=0.10.11+2×0.02312-0.1-0.02×0.11312=1.925×10-5 m4

Therefore,

M=1.925×10-5×180×1060.075=46204 Nm

Example 5:

A steel bar, 100 mm diameter, is used to support a pulley block. If the bar is simply supported over a span of 1.2 m, determine the maximum bending stress in the material when the pulley block is at mid-span supporting a load of 20 kN.

20 kN

10 kN

10 kN

0.6 m

0.6 m

Solution:

σmax=MmaxymaxI

With,

I=πr44=4.91×10-6 m4

Mmax=moments to left of mid span

∴   Mmax=10×0.6=6kNm

Therefore,

σmax=6×103×0.054.91×10-6=61.12 MNm2

Example 6:

The maximum bending moment and bending stress in a beam of rectangular cross-section must not exceed 25 kN.m and 130 MN/m2 respectively. What will be the minimum dimensions of the beam cross-section if the width of the beam is 2/3 of its depth?

Solution:

I=bd312  and   b=23d

∴   I=23dd312=d418

From simple bending equation,

σ=MyI

Where,

σ=130 MNm2M=25 kNy=0.5d

Therefore,

130×106=25×103×0.5dd418

∴   d3=25×103×0.5×18130×106=1.713×10-3

∴   d=31.713×10-3=120mm

And,

b=23×120=80mm

That is, the beam dimensions are 80 mm x 120 mm deep.