| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| 5th March (MOCK) | 5th March (MOCK) |

MODULE CODE: ACDB5002

Electronics I

TIME:  2 HOURS

ANSWER ALL QUESTIONS

Each Question Carries 25 Marks

Total Marks : 100

This exam does not allow the use of reference materials, scraps of paper, online resources, or observing any practice sessions.

- Equipment or materials required – PC running CAD software or Pencil/Ruler/Paper

- Open MS Word Document to capture the simulation  screen dumps .

- Save the name of the file in this pattern ‘Exam_studentname_studentid’ and upload the file in Moodle.

QUESTION 1

Design a circuit using the PIC16F877A microcontroller, where LEDs are connected to pins RC0 through RC7, each with a 100Ω current-limiting resistor. The LEDs should turn ON simultaneously, with a delay of half a second when a switch is pressed.

The project should include the following :

  - Design the circuit using simulation software.				 [10 Marks]

  - Write a C program to accomplish the specified task. 		[10 Marks]

  - Execute the simulation to verify the operation of the circuit and program. [5 Marks]

QUESTION 2

Demonstrate a down counter using 7- Segment Display each with a 100Ω current-limiting resistor by considering the following conditions. Consider Common Anode 7- Segment display using the hex values given below.

| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0XC0 | 0XF9 | 0XA4 | 0XB0 | 0X99 | 0X92 | 0X82 | 0XF8 | 0X80 | 0X90 |

- When a push button is connected to port pin RC7 is pressed once, the 7 segments should count down from 9 to 1 within a delay of 100ms.

- The counter should stop counting when the button is released.

The project should include the following components:

- Design the circuit using simulation software.				[10 Marks]

- Write a C program to accomplish the specified counting task. 		[10 Marks]

- Execute the simulation to verify the operation of the circuit and program.  [5 Marks]

QUESTION 3

- For the circuit given below, Answer the following questions.

- Identify the above circuit and briefly explain the purpose of it.			[3 Marks]

- Explain the working principle of this circuit.						[8 Marks]

- Design and simulate the above circuit diagram using Simulation Software.	[14 Marks]

QUESTION 4

      - Explain the working principle of LM393 Amplifier with a neat diagram.	 [10 Marks]

      - Design and simulate the Humidity sensor circuit using appropriate simulation software, such as Proteus.									[14 Marks]

Diwedd/End