HEAT TRANSFER

CONVECTION

SOLUTION

Extra exercise:

An electrically heated thin foil of length L = 25 mm and width W = 8 mm is to be used as a wind speed metre. Wind with a temperature Tf and velocity uf blows parallel to the longest side. The foil is internally heated by an electric heater dissipating Q (Watts) from both sides and is to be operated in air with:

Tf=20℃,    cp=1.005kJkg.K,   =1.522×10-5 m2s,     ρ=1.19 kgm3 and   Pr=0.72.

The surface temperature Ts of the foil is to be measured at the trailing edge – but can be assumed to be constant. Estimate the wind speed when Ts=32℃ and Q=0.5 W.

Q=0.5 W from both sides

SOLUTION:

AIR WIND

Ts=32℃

8 mm

uf=?

Tf=20℃

cp=1.005kJkg.K

=1.522×10-5m2s

ρ=1.19kgm3

Pr=0.72

25 mm

Q=h.A.Ts-Tf       and       Pr=μ∙cpk        and     Nu=h∙lk        and      Re=u∙l∙ρμ=u∙l

We can calculate h using

Q=h.A.Ts-Tf

∴    h=QA.Ts-Tf

∴    h=0.52×8×10-3×25×10-3.32-20

∴    h=104.16  Wm2.K

We can calculate k using

Pr=μ∙cpk

∴    k=μ∙cpPr

∴    k=ρ..cpPr   as  μ=ρ.

∴    k=1.19×1.522×10-5×1.005×1030.72 =0.025 Wm.K

According to Table 1 we have the following relationship,

Nu=0.036∙Re0.8∙Pr0.3 if  Re>50,000  (Turbulent)

Nu=0.664∙Re0.5∙Pr0.3  if  Re<50,000  (Laminar)

Assuming a critical (transition) Reynolds number of Re=50,000, the velocity required would be,

Re=u∙l

∴    50,000=u×25×10-31.522×10-5

∴    u=50,000×1.522×10-525×10-3=30.44 ms       (∴109.6kmhr)

Wind speed is unlikely to reach that velocity, so the flow can be assumed to be laminar. Therefore,

Nu=0.664∙Re0.5∙Pr0.3=h∙lk

∴    Re0.5=h.lk×0.664×Pr0.3

∴    Re=h.lk×0.664×Pr0.32

∴    Re=104.16×25×10-30.025×0.664×0.720.32

∴    Re=3.0×104

Therefore,

Re=u∙l

∴    u=Re×l

∴    u=3.0×104×1.522×10-5 25×10-3 =18.25 m/s