(Φ)

## Peak Amplitude = peak (pk) voltage ( approx 1.5 divisions)

Peak – peak amplitude = approx. 3 divisions.

Time period between A and B is called the PERIOD “T”

## The FREQUENCY of the sinusoid “f” = 1/T Unit is the Hertz “Hz”

e.g. 10 Hz = 10 cycles per second.

Curve Y is phase shifted from curve X by +90 or -270  (same thing)

Curve X may be expressed as X = A. Sin Ө , where A represents peak amplitude of waveform and Ө represents the x axis quantity of degrees.

Alternatively X may be described as X = A. Sin (2.π.f.t) if the x axis is time and f = frequency.

Curve Y may be expressed as Y = A. Sin (Ө + Φ) or A. Sin (2.π.f.t + Φ) where Φ represent a phase difference , in degrees, between X and Y, taking X as a zero reference phase.

A sinewave is a single frequency waveform.

If you distort a sinewave the new waveform may contain many frequencies.

### SINEWAVE CHARACTERISTICS