Task 2

1a)

[A] The pulses are broader at the positive peak of the sine wave

1b)

[B] Around 40 pulses in one half cycle

1c)

[YES] A small ripple is observed

2a)

LOAD CURRENT=26.3mA AC

Immediately after the short is applied the current rises rapidly and the supply voltage reduces to zero.  With time, the amplitude of the pules from the inverter at the peak voltage points reduces, causing the current to reduce.

2b)

[B] Supply voltage reduces to zero

2c)

[D] Load current rises to a maximum and the reduces

2d)

[D] Supply current and voltage slowly return to normal

2e)

[NO] Power supply does not immediately return to normal

2f)

[YES] Power supply returns to normal over time

Task 2 Fault finding

Power factor correction was applied to the circuit

| Before Fault |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | No Cap | 10uF | 20uF | 15uF |
| Phase Shift | 25o lag | 16o lag | 0o | 16o lead |
| Current | 497mA | 532mA | 572mA | 588mA |

Fault condition 8 was entered into the circuit.

| After Fault |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | No Cap | 10uF | 15uF | 20uF |  |
| Phase Shift | 32o lag | 4o lead | 16o lead | 34o lead |  |
| Current | 480mA | 434mA | 468mA | 502mA |  |

It can be seen from the second table that the current in motor is reduced from the first set of values at all capacitor values.  This implies that there is a higher resistance in the system.

The voltage was measured across each winding and it was discovered that the voltage across winding 3 was 1.3V higher with the fault in the system than without, suggesting that the high resistance was on winding 3.