Commands on S1:
R1#

*Aug 21 10:11:32.540: %SYS-6-PRIVCFG_ENCRYPT_SUCCESS: Successfully encrypted private config file

R1#

R1#show run

Building configuration...

Current configuration : 6622 bytes

!

! Last configuration change at 10:10:02 UTC Wed Aug 21 2024

!

version 17.9

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

! Call-home is enabled by Smart-Licensing.

service call-home

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname R1

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

!

!

!

!

!

!

!

no ip domain lookup

ip domain name swre-lab.com

!

!

!

login on-success log

!

!

!

!

!

!

!

subscriber templating

vtp version 1

multilink bundle-name authenticated

!

!

!

crypto pki trustpoint TP-self-signed-90536024

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-90536024

revocation-check none

rsakeypair TP-self-signed-90536024

!

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

!

!

crypto pki certificate chain TP-self-signed-90536024

certificate self-signed 01

3082032C 30820214 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

2F312D30 2B060355 04031324 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 39303533 36303234 301E170D 32343038 32313038 34373135

5A170D33 34303832 31303834 3731355A 302F312D 302B0603 55040313 24494F53

2D53656C 662D5369 676E6564 2D436572 74696669 63617465 2D393035 33363032

34308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201 0A028201

0100AFA9 A66E81E5 464C140F 96EF0582 0054443E 097D6403 1DE87A7E A9190B14

11573960 101A7B7D B9D1CB5A C17EF704 D205CF27 F029CB86 CBC376E9 F28FBDDC

7C637F8F 376FE4AC 952FA026 53921CD2 4734B4F6 E2F20CAA 67EEAF5C F7070A1A

2AEF859B 2968FF2B 6F9470E9 CC26FF6B B78823B1 5AF9DC11 34D930E4 6E9932B5

DC32E6E2 5FEC7675 9C893E8F 58E2767F DEA76E5C A64DAB6A 20978DF6 739ED864

44EE97D9 1340AD09 6028E85F 880EFB8C 04E22AD2 9280B2D4 38DF8EAB 8436937B

AB3312C8 D5E1DAB4 7C6627BA 6362D4A5 A867D519 0B0B37AC 6D696B12 CC080789

254DF03A FF54148C 396B6F7F 1E04F91B 56E6A662 A5ACD2A9 B6E8DA72 B485A7FD

A2DD0203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF 301F0603

551D2304 18301680 143BB2D2 A22EFA5B F56BA133 08A53E28 DE48CC28 77301D06

03551D0E 04160414 3BB2D2A2 2EFA5BF5 6BA13308 A53E28DE 48CC2877 300D0609

2A864886 F70D0101 05050003 82010100 A8AD36FA C4CF17F4 5040430F 2C4B8C60

0268F484 76F9BB49 46408BE2 1AF7924C 5F263FA0 B7DB747E 4D1B1ABA D59866C1

98A90888 223387E6 304BD1FD 26FBAB33 06BFE9C4 B19AD22D 734FA52A AEB7A626

00BC6DCA 84721B42 E2B5C97E 146FB2F7 4B90D9E0 D752F087 EC6F63CC 688DA18A

90C20B27 A4EC4B79 5D4FD6C1 F8E08722 2042681A BBE8F2C9 B4F790AA A7A3C32F

ACF16427 6538C11D CBD48E07 00EA7E93 C32778B1 7EB0500D 9CB76C6F 95778648

CB3E1BE5 056B6E43 59590021 65174A94 B31F36F7 541E88F2 DFF1D175 481771FD

C47B1EE4 570262CD 16DF309F CD71B7A1 A7369E5B FEB47264 6B13DD41 98CD5BBD

AC6471A9 C63B3E9B F49FDFB1 3041A3BE

quit

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

30820321 30820209 A0030201 02020101 300D0609 2A864886 F70D0101 0B050030

32310E30 0C060355 040A1305 43697363 6F312030 1E060355 04031317 43697363

6F204C69 63656E73 696E6720 526F6F74 20434130 1E170D31 33303533 30313934

3834375A 170D3338 30353330 31393438 34375A30 32310E30 0C060355 040A1305

43697363 6F312030 1E060355 04031317 43697363 6F204C69 63656E73 696E6720

526F6F74 20434130 82012230 0D06092A 864886F7 0D010101 05000382 010F0030

82010A02 82010100 A6BCBD96 131E05F7 145EA72C 2CD686E6 17222EA1 F1EFF64D

CBB4C798 212AA147 C655D8D7 9471380D 8711441E 1AAF071A 9CAE6388 8A38E520

1C394D78 462EF239 C659F715 B98C0A59 5BBB5CBD 0CFEBEA3 700A8BF7 D8F256EE

4AA4E80D DB6FD1C9 60B1FD18 FFC69C96 6FA68957 A2617DE7 104FDC5F EA2956AC

7390A3EB 2B5436AD C847A2C5 DAB553EB 69A9A535 58E9F3E3 C0BD23CF 58BD7188

68E69491 20F320E7 948E71D7 AE3BCC84 F10684C7 4BC8E00F 539BA42B 42C68BB7

C7479096 B4CB2D62 EA2F505D C7B062A4 6811D95B E8250FC4 5D5D5FB8 8F27D191

C55F0D76 61F9A4CD 3D992327 A8BB03BD 4E6D7069 7CBADF8B DF5F4368 95135E44

DFC7C6CF 04DD7FD1 02030100 01A34230 40300E06 03551D0F 0101FF04 04030201

06300F06 03551D13 0101FF04 05300301 01FF301D 0603551D 0E041604 1449DC85

4B3D31E5 1B3E6A17 606AF333 3D3B4C73 E8300D06 092A8648 86F70D01 010B0500

03820101 00507F24 D3932A66 86025D9F E838AE5C 6D4DF6B0 49631C78 240DA905

604EDCDE FF4FED2B 77FC460E CD636FDB DD44681E 3A5673AB 9093D3B1 6C9E3D8B

D98987BF E40CBD9E 1AECA0C2 2189BB5C 8FA85686 CD98B646 5575B146 8DFC66A8

467A3DF4 4D565700 6ADF0F0D CF835015 3C04FF7C 21E878AC 11BA9CD2 55A9232C

7CA7B7E6 C1AF74F6 152E99B7 B1FCF9BB E973DE7F 5BDDEB86 C71E3B49 1765308B

5FB0DA06 B92AFE7F 494E8A9E 07B85737 F3A58BE1 1A48A229 C37C1E69 39F08678

80DDCD16 D6BACECA EEBC7CF9 8428787B 35202CDC 60E4616A B623CDBD 230E3AFB

418616A9 4093E049 4D10AB75 27E86F73 932E35B5 8862FDAE 0275156F 719BB2F0

D697DF7F 28

quit

!

!

license udi pid C8200L-1N-4T sn FCZ2818R0N0

memory free low-watermark processor 62972

!

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

enable secret 9 $9$BBQX.6ZUm0/SyU$CSTCkmK7K8UrGAQVdElScJDGshUYArbv7ywQhhiOqnk

!

username cisco secret 9 $9$Br.nUoILTclkj.$Vey59xFWHPZ6JQEis3mf8vVL4F0Yshthw5haa0mSH9s

!

redundancy

mode none

!

!

!

!

!

!

!

!

interface Loopback0

ip address 209.165.201.1 255.255.255.0

!

interface GigabitEthernet0/0/0

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet0/0/1

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet0/0/1.7

encapsulation dot1Q 7

ip address 192.168.20.1 255.255.255.0

!

interface GigabitEthernet0/0/1.8

encapsulation dot1Q 8

ip address 192.168.30.1 255.255.255.0

!

interface GigabitEthernet0/0/1.9

encapsulation dot1Q 9

ip address 192.168.40.1 255.255.255.0

!

interface GigabitEthernet0/0/1.11

!

interface GigabitEthernet0/0/2

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet0/0/3

no ip address

shutdown

negotiation auto

!

interface Serial0/1/0

!

interface Serial0/1/1

!

ip http server

ip http authentication local

ip http secure-server

ip forward-protocol nd

!

!

!

!

!

!

control-plane

!

!

line con 0

password 7 110A1016141D5A5E57

login

stopbits 1

line aux 0

line vty 0 4

password 7 070C285F4D06485744

login local

transport input ssh

line vty 5 15

password 7 070C285F4D06485744

login local

transport input ssh

!

call-home

! If contact email address in call-home is configured as sch-smart-licensing@cisco.com

! the email address configured in Cisco Smart License Portal will be used as contact email address to send SCH notifications.

contact-email-addr sch-smart-licensing@cisco.com

profile "CiscoTAC-1"

active

destination transport-method http

!

!

!

!

!

!

end

R1#

User Access Verification

Password:

S1>ena

Password:

S1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#

S1(config)#

S1(config)#inter

S1(config)#interface fa0/6

^

% Invalid input detected at '^' marker.

S1(config)#do show interface brief

^

% Invalid input detected at '^' marker.

S1(config)#do show interface

Vlan1 is administratively down, line protocol is down

Hardware is EtherSVI, address is a0bc.6fe8.07c0 (bia a0bc.6fe8.07c0)

MTU 1500 bytes, BW 1000000 Kbit/sec, DLY 10 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation ARPA, loopback not set

Keepalive not supported

ARP type: ARPA, ARP Timeout 04:00:00

Last input never, output never, output hang never

Last clearing of "show interface" counters never

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

0 packets input, 0 bytes, 0 no buffer

Received 0 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 packets output, 0 bytes, 0 underruns

0 output errors, 1 interface resets

0 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

Vlan9 is up, line protocol is down

S1(config)#do show ip brief

^

% Invalid input detected at '^' marker.

S1(config)#do show ip ?

access-lists    List IP access lists

accounting      The active IP accounting database

admission       Network Admission Control information

aliases         IP alias table

arp             IP ARP table

cache           IP fast-switching route cache

cef             Cisco Express Forwarding

device          Show IP Tracking Hosts

dhcp            Show items in the DHCP database

helper-address  helper-address table

host            IP host information

http            HTTP information

igmp            IGMP extension

interface       IP interface status and configuration

local           IP local options

masks           Masks associated with a network

nat             IP NAT information

port-map        Port to Application Mapping (PAM) information

protocols       IP routing protocol process parameters and statistics

redirects       IP redirects

rib             Routing Information Base

rip             IP RIP show commands

route           IP routing table

sockets         Open IP sockets

source          IP source

ssh             Information on SSH

static          Static operation

traffic         IP protocol statistics

verify          Ip verify

S1(config)#end

S1#sh

*Aug 21 11:16:20.149: %SYS-5-CONFIG_I: Configured from console by consoleow

% Type "show ?" for a list of subcommands

S1#dhow

Translating "dhow"

% Unknown command or computer name, or unable to find computer address

S1#show

% Type "show ?" for a list of subcommands

S1#show ?

access-lists       List access lists

aliases            Display alias commands

auto               Show Automation Template

banner             Display banner information

configuration      Contents of Non-Volatile memory

controllers        Interface controller status

debugging          State of each debugging option

exception          exception informations

history            Display the session command history

hosts              IP domain-name, lookup style, nameservers, and host table

interfaces         Interface status and configuration

location           Display the system location

logging            Show the contents of logging buffers

parser             Display parser information

privilege          Show current privilege level

queue              Show queue contents

queueing           Show queueing configuration

running-config     Current operating configuration

startup-config     Contents of startup configuration

trace              Show trace options

hardware

aaa                Show AAA values

adjacency          Adjacent nodes

alignment          Show alignment information

archive            Archive functions

arp                ARP table

authentication     Shows Auth Manager stats, registrations or sessions

beep               Show BEEP information

bluetooth          bluetooth show commands

boot               show boot attributes

buffers            Buffer pool statistics

bulkstat           Bulkstat show commands

caaa               Subscriber Policy Entity Information

cable-diagnostics  Show Cable Diagnostics Results

cache              Shows Device-Sensor Cache Informations

calendar           Display the hardware calendar

call-home          Show command for call home

capability         Capability Information

cca                CCA information

cdp                CDP information

cef                CEF address family independent status

cisp               Shows CISP information

class-map          Show CPL Class Map

clock              Display the system clock

cns                CNS agents

context            Show context information about recent crash(s)

crypto             Encryption module

dampening          Display dampening information

data-corruption    Show data errors

dcm                Data Collection Manager Core Details

derived-config     Derived operating configuration

device             Display device classifier information

device-sensor      Shows Device Sensor Information

dhcp               Dynamic Host Configuration Protocol status

diagnostic         Show command for diagnostic

dot1x              Dot1x information

dtp                DTP information

dying-gasp         Dying Gasp information

eap                Shows EAP registration/session information

eee                Show Energy Efficient Ethernet

env                Environmental facilities

epm                EPM information

errdisable         Error disable

etherchannel       EtherChannel information

ethernet           Ethernet parameters

fallback           Display Fallback configuration

file               Show filesystem information

fips               FIPS information

flash1:            display information about flash1: file system

flash:             display information about flash: file system

flowcontrol        show flow control information

format             Show format information

html               HTML helper commands

id-manager         ID pool manager

idb                List of Interface Descriptor Blocks

identity           Identity profiles and policies

idprom             show IDPROMs for interfaces

if-mgr             if-mgr information

inventory          Show the physical inventory

ip                 IP information

ipc                Interprocess communications commands

ipv6               IPv6 information

kron               Kron Subsystem

l2                 Layer 2

lacp               Port channel information

license            Displays all the RTU licenses.

line               TTY line information

link               Show Link

lldp               LLDP information

login              Display Secure Login Configurations and State

loopdetect         loopdetect information

mab                MAB information

mac                MAC configuration

macro              Show command macros

memory             Memory statistics

mls                mls global commands

monitor            Monitoring different system events

mtm                MTM

network-policy     Network Policy profile information

nmsp               nmsp show commands

ntp                Network time protocol

object-group       List object groups

odm-format         Show the schema used for ODM input file

pagp               Port channel information

parameter-map      Show parameter map of type

persistent         Show persistent information

platform           platform specific show commands

pm                 Show Port Manager commands

pnp                Display PNP information

policy-manager     Policy Manager

policy-map         Show Policy Map

port-security      Show secure port information

post               Show the results of the system POST

power              Switch Power

pre                PRE show commands

processes          Active process statistics

protocols          Active network routing protocols

psp                Show PSP details

radius             Shows radius information

region             Region Manager Status

registry           Function registry information

reload             Scheduled reload information

rep                Resilient Ethernet Protocol

resource           Resource group statistics

rhosts             Remote-host+user equivalences

rib                Routing Information Base

rmon               rmon statistics

route-tag          route-tag information

sasl               show SASL information

scalable-queue     Scalable Queue statistics

sched-event        Scheduler event information

sdm                Switch database management

service-template   Show service-templates/identity policies

sessions           Information about Telnet connections

setup              Show express setup settings

sflow              sFlow information

snmp               snmp statistics

sockets            Socket Details

spanning-tree      Spanning tree topology

ssh                Status of SSH server connections

stacks             Process stack utilization

standby            Hot Standby Router Protocol (HSRP) information

storm-control      Show storm control configuration

subsys             Show subsystem information

switch             show information about the stack ring

system             Show the system configuration

tacacs             Shows tacacs+ server statistics

tcp                Status of TCP connections

tech-support       Show system information for Tech-Support

template           Template information

terminal           Display terminal configuration parameters

test_rib_access    RIB_ACCESS TEST info

time-range         Time range

topology           Topology instance information

traffic-shape      traffic rate shaping configuration

udld               UDLD information

udp                UDP Details

usb                USB Interface

users              Display information about terminal lines

version            System hardware and software status

vlan               VTP VLAN status

vtp                VTP information

wsma               Show Web Services Management Agents information

xos                Cross-OS Library Information and Traces

xsd-format         Show the ODM XSD for the command

S1#show interface

Vlan1 is administratively down, line protocol is down

Hardware is EtherSVI, address is a0bc.6fe8.07c0 (bia a0bc.6fe8.07c0)

MTU 1500 bytes, BW 1000000 Kbit/sec, DLY 10 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation ARPA, loopback not set

Keepalive not supported

ARP type: ARPA, ARP Timeout 04:00:00

Last input never, output never, output hang never

Last clearing of "show interface" counters never

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

0 packets input, 0 bytes, 0 no buffer

Received 0 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 packets output, 0 bytes, 0 underruns

0 output errors, 1 interface resets

0 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

Vlan9 is up, line protocol is down

S1#show interface ?

Async              Async interface

Auto-Template      Auto-Template interface

BVI                Bridge-Group Virtual Interface

Bluetooth          Bluetooth interface

CTunnel            CTunnel interface

Dialer             Dialer interface

Filter             Filter interface

Filtergroup        Filter Group interface

GigabitEthernet    GigabitEthernet IEEE 802.3z

GroupVI            Group Virtual interface

Loopback           Loopback interface

Lspvif             LSP virtual interface

Null               Null interface

Port-channel       Ethernet Channel of interfaces

Portgroup          Portgroup interface

Pos-channel        POS Channel of interfaces

Tunnel             Tunnel interface

Vif                PGM Multicast Host interface

Virtual-Template   Virtual Template interface

Virtual-TokenRing  Virtual TokenRing

Vlan               Catalyst Vlans

accounting         Show interface accounting

S1#show interfaces ?

Async              Async interface

Auto-Template      Auto-Template interface

BVI                Bridge-Group Virtual Interface

Bluetooth          Bluetooth interface

CTunnel            CTunnel interface

Dialer             Dialer interface

Filter             Filter interface

Filtergroup        Filter Group interface

GigabitEthernet    GigabitEthernet IEEE 802.3z

GroupVI            Group Virtual interface

Loopback           Loopback interface

Lspvif             LSP virtual interface

Null               Null interface

Port-channel       Ethernet Channel of interfaces

Portgroup          Portgroup interface

Pos-channel        POS Channel of interfaces

Tunnel             Tunnel interface

Vif                PGM Multicast Host interface

Virtual-Template   Virtual Template interface

Virtual-TokenRing  Virtual TokenRing

Vlan               Catalyst Vlans

accounting         Show interface accounting

S1#show interfaces ig

S1#show interfaces gig

S1#show interfaces gigabitEthernet

% Incomplete command.

S1#show interfaces gigabitEthernet0/6

^

% Invalid input detected at '^' marker.

S1#show interfaces gigabitEthernet0/0/6

^

% Invalid input detected at '^' marker.

S1#show interfaces brief

^

% Invalid input detected at '^' marker.

S1#show interface brief

^

% Invalid input detected at '^' marker.

S1#show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Vlan1                  unassigned      YES NVRAM  administratively down down

Vlan9                  192.168.40.2    YES manual up                    down

GigabitEthernet1/0/1   unassigned      YES unset  up                    up

GigabitEthernet1/0/2   unassigned      YES unset  up                    up

GigabitEthernet1/0/3   unassigned      YES unset  down                  down

GigabitEthernet1/0/4   unassigned      YES unset  down                  down

GigabitEthernet1/0/5   unassigned      YES unset  down                  down

GigabitEthernet1/0/6   unassigned      YES unset  up                    up

GigabitEthernet1/0/7   unassigned      YES unset  down                  down

GigabitEthernet1/0/8   unassigned      YES unset  down                  down

GigabitEthernet1/0/9   unassigned      YES unset  down                  down

GigabitEthernet1/0/10  unassigned      YES unset  down                  down

GigabitEthernet1/0/11  unassigned      YES unset  down                  down

GigabitEthernet1/0/12  unassigned      YES unset  down                  down

GigabitEthernet1/0/13  unassigned      YES unset  down                  down

GigabitEthernet1/0/14  unassigned      YES unset  down                  down

GigabitEthernet1/0/15  unassigned      YES unset  down                  down

GigabitEthernet1/0/16  unassigned      YES unset  down                  down

GigabitEthernet1/0/17  unassigned      YES unset  down                  down

GigabitEthernet1/0/18  unassigned      YES unset  down                  down

GigabitEthernet1/0/19  unassigned      YES unset  down                  down

GigabitEthernet1/0/20  unassigned      YES unset  down                  down

GigabitEthernet1/0/21  unassigned      YES unset  down                  down

GigabitEthernet1/0/22  unassigned      YES unset  down                  down

GigabitEthernet1/0/23  unassigned      YES unset  down                  down

GigabitEthernet1/0/24  unassigned      YES unset  down                  down

GigabitEthernet1/0/25  unassigned      YES unset  down                  down

GigabitEthernet1/0/26  unassigned      YES unset  down                  down

GigabitEthernet1/0/27  unassigned      YES unset  down                  down

GigabitEthernet1/0/28  unassigned      YES unset  down                  down

S1#

S1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#interface gi1/0/6

S1(config-if)#swit

S1(config-if)#switchport mode acc

S1(config-if)#switchport mode access

S1(config-if)#sw

S1(config-if)#switchport access vlan 7

S1(config-if)#port?

% Unrecognized command

S1(config-if)#?

Interface configuration commands:

aaa                 Authentication, Authorization and Accounting.

access-session      Access Session specific Interface Configuration Commands

arp                 Set arp type (arpa, probe, snap) or timeout or log

options

authentication      Auth Manager Interface Configuration Commands

auto                Configure Automation

bandwidth           Set bandwidth informational parameter

carrier-delay       Specify delay for interface transitions

cdp                 CDP interface subcommands

channel-group       Etherchannel/port bundling configuration

channel-protocol    Select the channel protocol (LACP, PAgP)

dampening           Enable event dampening

default             Set a command to its defaults

delay               Specify interface throughput delay

description         Interface specific description

dot1x               Interface Config Commands for IEEE 802.1X

down-when-looped    Force looped interface down

duplex              Configure duplex operation.

ethernet            Ethernet interface parameters

exit                Exit from interface configuration mode

fair-queue          Enable Fair Queuing on an Interface

flowcontrol         Configure flow operation.

help                Description of the interactive help system

history             Interface history histograms - 60 second, 60 minute and

72 hour

hold-queue          Set hold queue depth

ip                  Interface Internet Protocol config commands

ipv6                IPv6 interface subcommands

keepalive           Enable keepalive

lacp                LACP interface subcommands

link                Interface link related commands

lldp                LLDP interface subcommands

load-interval       Specify interval for load calculation for an interface

location            Interface location information

logging             Configure logging for interface

loopdetect          Configure loopdetect feature setting

mab                 MAC Authentication Bypass Interface Config Commands

mac                 MAC interface commands

macro               Command macro

mdix                Set Media Dependent Interface with Crossover

mls                 mls interface commands

neighbor            interface neighbor configuration mode commands

network-policy      Network Policy

nmsp                NMSP interface configuration

no                  Negate a command or set its defaults

pagp                PAgP interface subcommands

power               Power configuration

priority-queue      Priority Queue

random-detect       Enable Weighted Random Early Detection (WRED) on an

Interface

rep                 Resilient Ethernet Protocol characteristics

routing             Per-interface routing configuration

service-policy      Configure CPL Service Policy

sflow               Enable sflow on the interface

shutdown            Shutdown the selected interface

snmp                Modify SNMP interface parameters

source              Get config from another source

spanning-tree       Spanning Tree Subsystem

speed               Configure speed operation.

srr-queue           Configure shaped round-robin transmit queues

storm-control       storm configuration

subscriber          Subscriber inactivity timeout value.

switchport          Set switching mode characteristics

timeout             Define timeout values for this interface

topology            Configure routing topology on the interface

transmit-interface  Assign a transmit interface to a receive-only interface

tx-ring-limit       Configure PA level transmit ring limit

udld                Configure UDLD enabled or disabled and ignore global UDLD

setting

vtp                 Enable VTP on this interface

S1(config-if)#exit

S1(config)#interface gi1/0/6

S1(config-if)#secu

^

% Invalid input detected at '^' marker.

S1(config-if)#secu

S1(config-if)#port

S1(config-if)#port?

% Unrecognized command

S1(config-if)#secu

S1(config-if)#security

S1(config-if)#security-pol

S1(config-if)#security-policy ?

% Unrecognized command

S1(config-if)#security-policy

^

% Invalid input detected at '^' marker.

S1(config-if)#servi

S1(config-if)#service-policy ?

input   Assign policy-map to the input of an interface

output  Assign policy-map to the output of an interface

type    Configure CPL Service Policy

S1(config-if)#exit

S1(config)#interface range gi1/0/1-2

S1(config-if-range)#?

Interface configuration commands:

aaa                 Authentication, Authorization and Accounting.

access-session      Access Session specific Interface Configuration Commands

arp                 Set arp type (arpa, probe, snap) or timeout or log

options

auto                Configure Automation

bandwidth           Set bandwidth informational parameter

carrier-delay       Specify delay for interface transitions

cdp                 CDP interface subcommands

channel-group       Etherchannel/port bundling configuration

channel-protocol    Select the channel protocol (LACP, PAgP)

dampening           Enable event dampening

default             Set a command to its defaults

delay               Specify interface throughput delay

description         Interface specific description

down-when-looped    Force looped interface down

duplex              Configure duplex operation.

ethernet            Ethernet interface parameters

exit                Exit from interface configuration mode

fair-queue          Enable Fair Queuing on an Interface

flowcontrol         Configure flow operation.

help                Description of the interactive help system

history             Interface history histograms - 60 second, 60 minute and

72 hour

hold-queue          Set hold queue depth

ip                  Interface Internet Protocol config commands

ipv6                IPv6 interface subcommands

keepalive           Enable keepalive

lacp                LACP interface subcommands

link                Interface link related commands

lldp                LLDP interface subcommands

load-interval       Specify interval for load calculation for an interface

location            Interface location information

logging             Configure logging for interface

loopdetect          Configure loopdetect feature setting

mac                 MAC interface commands

macro               Command macro

mdix                Set Media Dependent Interface with Crossover

mls                 mls interface commands

neighbor            interface neighbor configuration mode commands

network-policy      Network Policy

nmsp                NMSP interface configuration

no                  Negate a command or set its defaults

pagp                PAgP interface subcommands

power               Power configuration

S1(config-if-range)#cha

S1(config-if-range)#channel-gr

S1(config-if-range)#channel-group ?

<1-6>  Channel group number

auto   Enable LACP auto on this interface

S1(config-if-range)#channel-group 1

% Incomplete command.

S1(config-if-range)#channel-group 1?

<1-6>

S1(config-if-range)#channel-group 2

% Incomplete command.

S1(config-if-range)#channel-group 2?

<1-6>

S1(config-if-range)#channel-group 1-2

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group ?

<1-6>  Channel group number

auto   Enable LACP auto on this interface

S1(config-if-range)#channel-group 1?

<1-6>

S1(config-if-range)#channel-group 1-6

^

% Invalid input detected at '^' marker.

S1(config-if-range)#?

Interface configuration commands:

aaa                 Authentication, Authorization and Accounting.

access-session      Access Session specific Interface Configuration Commands

arp                 Set arp type (arpa, probe, snap) or timeout or log

options

auto                Configure Automation

bandwidth           Set bandwidth informational parameter

carrier-delay       Specify delay for interface transitions

cdp                 CDP interface subcommands

channel-group       Etherchannel/port bundling configuration

channel-protocol    Select the channel protocol (LACP, PAgP)

dampening           Enable event dampening

default             Set a command to its defaults

delay               Specify interface throughput delay

description         Interface specific description

down-when-looped    Force looped interface down

duplex              Configure duplex operation.

ethernet            Ethernet interface parameters

exit                Exit from interface configuration mode

fair-queue          Enable Fair Queuing on an Interface

flowcontrol         Configure flow operation.

help                Description of the interactive help system

S1(config-if-range)#channel-group 1 6

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1 2

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1 0

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 14

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1

% Incomplete command.

S1(config-if-range)#channel-group 1-6

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1

% Incomplete command.

S1(config-if-range)#channel-group 1?

<1-6>

S1(config-if-range)#channel-group 1-6

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1 ?

mode  Etherchannel Mode of the interface

S1(config-if-range)#channel-group 1 L?

% Unrecognized command

S1(config-if-range)#channel-group 1 p?

% Unrecognized command

S1(config-if-range)#channel-group 1 pagp

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1 pagp?

% Unrecognized command

S1(config-if-range)#channel-group 1 p?

% Unrecognized command

S1(config-if-range)#exit

S1(config)#inter

S1(config)#interface range gi1/0/3-4

S1(config-if-range)#shut

S1(config-if-range)#shutdown

S1(config-if-range)#ex

*Aug 21 11:39:32.292: %LINK-5-CHANGED: Interface GigabitEthernet1/0/3, changed state to administratively down

*Aug 21 11:39:32.293: %LINK-5-CHANGED: Interface GigabitEthernet1/0/4, changed state to administratively downit

S1(config)#exit

S1#

*Aug 21 11:39:37.603: %SYS-5-CONFIG_I: Configured from console by console

S1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#interface range gi1/0/6-28

S1(config-if-range)#shutd

S1(config-if-range)#shutdown

S1(config-if-range)#

*Aug 21 11:40:00.385: %LINK-5-CHANGED: Interface GigabitEthernet1/0/6, changed state to administratively down

*Aug 21 11:40:00.446: %LINK-5-CHANGED: Interface GigabitEthernet1/0/7, changed state to administratively down

*Aug 21 11:40:00.448: %LINK-5-CHANGED: Interface GigabitEthernet1/0/8, changed state to administratively down

*Aug 21 11:40:00.450: %LINK-5-CHANGED: Interface GigabitEthernet1/0/9, changed state to administratively down

*Aug 21 11:40:00.451: %LINK-5-CHANGED: Interface GigabitEthernet1/0/10, changed state to administratively down

*Aug 21 11:40:00.453: %LINK-5-CHANGED: Interface GigabitEthernet1/0/11, changed state to administratively down

*Aug 21 11:40:00.454: %LINK-5-CHANGED: Interface GigabitEthernet1/0/12, changed state to administratively down

*Aug 21 11:40:00.456: %LINK-5-CHANGED: Interface GigabitEthernet1/0/13, changed state to administratively down

*Aug 21 11:40:00.458: %LINK-5-CHANGED: Interface GigabitEthernet1/0/14, changed state to administratively down

*Aug 21 11:40:00.459: %LINK-5-CHANGED: Interface GigabitEthernet1/0/15, changed state to administratively down

*Aug 21 11:40:00.461: %LINK-5-CHANGED: Interface GigabitEthernet1/0/16, changed state to administratively down

*Aug 21 11:40:00.462: %LINK-5-CHANGED: Interface GigabitEthernet1/0/17, changed state to administratively down

*Aug 21 11:40:00.464: %LINK-5-CHANGED: Interface GigabitEthernet1/0/18, changed state to administratively down

*Aug 21 11:40:00.466: %LINK-5-CHANGED: Interface GigabitEthernet1/0/19, changed state to administratively down

*Aug 21 11:40:00.467: %LINK-5-CHANGED: Interface GigabitEthernet1/0/20, changed state to administratively down

*Aug 21 11:40:00.469: %LINK-5-CHANGED: Interface GigabitEthernet1/0/21, changed state to administratively down

*Aug 21 11:40:00.470: %LINK-5-CHANGED: Interface GigabitEthernet1/0/22, changed state to administratively down

*Aug 21 11:40:00.472: %LINK-5-CHANGED: Interface GigabitEthernet1/0/23, changed state to administratively down

*Aug 21 11:40:00.474: %LINK-5-CHANGED: Interface GigabitEthernet1/0/24, changed state to administratively down

*Aug 21 11:40:00.475: %LINK-5-CHANGED: Interface GigabitEthernet1/0/25, changed state to administratively down

*Aug 21 11:40:00.477: %LINK-5-CHANGED: Interface GigabitEthernet1/0/26, changed state to administratively down

*Aug 21 11:40:00.478: %LINK-5-CHANGED: Interface GigabitEthernet1/0/27, changed state to administratively down

*Aug 21 11:40:00.479: %LINK-5-CHANGED: Interface GigabitEthernet1/0/28, changed state to administratively down

*Aug 21 11:40:01.385: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/6, changed state to down

S1(config-if-range)#

S1(config-if-range)#exit

S1(config)#do show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Gi1/0/1, Gi1/0/2, Gi1/0/3

Gi1/0/4, Gi1/0/5, Gi1/0/7

Gi1/0/8, Gi1/0/9, Gi1/0/10

Gi1/0/11, Gi1/0/12, Gi1/0/13

Gi1/0/14, Gi1/0/15, Gi1/0/16

Gi1/0/17, Gi1/0/18, Gi1/0/19

Gi1/0/20, Gi1/0/21, Gi1/0/22

Gi1/0/23, Gi1/0/24, Gi1/0/25

Gi1/0/26, Gi1/0/27, Gi1/0/28

7    London                           active    Gi1/0/6

8    Swansea                          active

9    Management                       active

10   Birmingham                       active

11   Native                           active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

S1(config)#

S1(config)#do write

Building configuration...

[OK]

S1(config)#

User Access Verification

Password:

Password:

R1>ena

Password:

R1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#do show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

Primary Secondary Type              Ports

------- --------- ----------------- ------------------------------------------

R1(config)#exit

R1#

*Aug 21 10:46:06.636: %SYS-5-CONFIG_I: Configured from console by console

R1#

S1(config)#

S1(config)#do show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Gi1/0/1, Gi1/0/2, Gi1/0/3

Gi1/0/4, Gi1/0/5, Gi1/0/7

Gi1/0/8, Gi1/0/9, Gi1/0/10

Gi1/0/11, Gi1/0/12, Gi1/0/13

Gi1/0/14, Gi1/0/15, Gi1/0/16

Gi1/0/17, Gi1/0/18, Gi1/0/19

Gi1/0/20, Gi1/0/21, Gi1/0/22

Gi1/0/23, Gi1/0/24, Gi1/0/25

Gi1/0/26, Gi1/0/27, Gi1/0/28

7    London                           active    Gi1/0/6

8    Swansea                          active

9    Management                       active

10   Birmingham                       active

11   Native                           active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

S1(config)#interface gi/0/1

^

% Invalid input detected at '^' marker.

S1(config)#interface gi1/0/1

S1(config-if)#sw

S1(config-if)#switchport tr

S1(config-if)#switchport trunk mode

^

% Invalid input detected at '^' marker.

S1(config-if)#

S1(config-if)#switchport trunk ?

allowed  Set allowed VLAN characteristics when interface is in trunking mode

native   Set trunking native characteristics when interface is in trunking

mode

pruning  Set pruning VLAN characteristics when interface is in trunking mode

S1(config-if)#switchport ?

access         Set access mode characteristics of the interface

autostate      Include or exclude this port from vlan link up calculation

block          Disable forwarding of unknown uni/multi cast addresses

host           Set port host

mode           Set trunking mode of the interface

nonegotiate    Device will not engage in negotiation protocol on this

interface

port-security  Security related command

priority       Set appliance 802.1p priority

private-vlan   Set the private VLAN configuration

protected      Configure an interface to be a protected port

trunk          Set trunking characteristics of the interface

voice          Voice appliance attributes

<cr>

S1(config-if)#switchport mode ?

access        Set trunking mode to ACCESS unconditionally

dynamic       Set trunking mode to dynamically negotiate access or trunk mode

private-vlan  Set private-vlan mode

trunk         Set trunking mode to TRUNK unconditionally

S1(config-if)#switchport mode tr

S1(config-if)#switchport mode trunk

S1(config-if)#swi

*Aug 21 11:52:26.605: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to downt

S1(config-if)#

*Aug 21 11:52:28.740: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan9, changed state to up

*Aug 21 11:52:29.670: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to up

S1(config-if)#

S1(config-if)#swi

S1(config-if)#switchport ?

access         Set access mode characteristics of the interface

autostate      Include or exclude this port from vlan link up calculation

block          Disable forwarding of unknown uni/multi cast addresses

host           Set port host

mode           Set trunking mode of the interface

nonegotiate    Device will not engage in negotiation protocol on this

interface

port-security  Security related command

priority       Set appliance 802.1p priority

private-vlan   Set the private VLAN configuration

protected      Configure an interface to be a protected port

trunk          Set trunking characteristics of the interface

voice          Voice appliance attributes

<cr>

S1(config-if)#switchport trunk native vlan ?

<1-4094>  VLAN ID of the native VLAN when this port is in trunking mode

S1(config-if)#switchport trunk native vlan 11

S1(config-if)#

*Aug 21 11:53:09.131: %SPANTREE-2-RECV_PVID_ERR: Received BPDU with inconsistent peer vlan id 1 on GigabitEthernet1/0/1 VLAN11.

*Aug 21 11:53:09.131: %SPANTREE-2-BLOCK_PVID_PEER: Blocking GigabitEthernet1/0/1 on VLAN0001. Inconsistent peer vlan.

*Aug 21 11:53:09.131: %SPANTREE-2-BLOCK_PVID_LOCAL: Blocking GigabitEthernet1/0/1 on VLAN0011. Inconsistent local vlan.

*Aug 21 11:53:31.253: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 11:54:28.380: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config-if)#exit

S1(config)#do show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Vlan1                  unassigned      YES NVRAM  administratively down down

Vlan9                  192.168.40.2    YES manual up                    up

GigabitEthernet1/0/1   unassigned      YES unset  up                    up

GigabitEthernet1/0/2   unassigned      YES unset  up                    up

GigabitEthernet1/0/3   unassigned      YES unset  administratively down down

GigabitEthernet1/0/4   unassigned      YES unset  administratively down down

GigabitEthernet1/0/5   unassigned      YES unset  down                  down

GigabitEthernet1/0/6   unassigned      YES unset  administratively down down

GigabitEthernet1/0/7   unassigned      YES unset  administratively down down

GigabitEthernet1/0/8   unassigned      YES unset  administratively down down

GigabitEthernet1/0/9   unassigned      YES unset  administratively down down

GigabitEthernet1/0/10  unassigned      YES unset  administratively down down

GigabitEthernet1/0/11  unassigned      YES unset  administratively down down

GigabitEthernet1/0/12  unassigned      YES unset  administratively down down

GigabitEthernet1/0/13  unassigned      YES unset  administratively down down

GigabitEthernet1/0/14  unassigned      YES unset  administratively down down

GigabitEthernet1/0/15  unassigned      YES unset  administratively down down

GigabitEthernet1/0/16  unassigned      YES unset  administratively down down

GigabitEthernet1/0/17  unassigned      YES unset  administratively down down

GigabitEthernet1/0/18  unassigned      YES unset  administratively down down

GigabitEthernet1/0/19  unassigned      YES unset  administratively down down

--More--

*Aug 21 11:55:24.138: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernetGigabitEthernet1/0/20  unassigned      YES unset  administratively down down

GigabitEthernet1/0/21  unassigned      YES unset  administratively down down

S1(config)#

S1(config)#sw

S1(config)#interface gi1/0/1

S1(config-if)#sw

S1(config-if)#switchport

*Aug 21 11:56:14.655: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).native

^

% Invalid input detected at '^' marker.

S1(config-if)#switchport trunk  native ?

vlan  Set native VLAN when interface is in trunking mode

S1(config-if)#switchport trunk  native vlan ?

<1-4094>  VLAN ID of the native VLAN when this port is in trunking mode

S1(config-if)#switchport trunk ?

allowed  Set allowed VLAN characteristics when interface is in trunking mode

native   Set trunking native characteristics when interface is in trunking

mode

pruning  Set pruning VLAN characteristics when interface is in trunking mode

S1(config-if)#switchport trunk native

% Incomplete command.

S1(config-if)#switchport trunk native ?

vlan  Set native VLAN when interface is in trunking mode

S1(config-if)#switchport trunk native 11 ?

% Unrecognized command

S1(config-if)#switchport trunk native 11?

% Unrecognized command

S1(config-if)#switchport trunk ?

allowed  Set allowed VLAN characteristics when interface is in trunking mode

native   Set trunking native characteristics when interface is in trunking

mode

pruning  Set pruning VLAN characteristics when interface is in trunking mode

S1(config-if)#switchport trunk

*Aug 21 11:57:10.488: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEinterface gi1/0/

% Incomplete command.

S1(config)#switchport trunk native ?

% Unrecognized command

S1(config)#switchport trunk native

^

% Invalid input detected at '^' marker.

S1(config)#switchport trunk native ?

% Unrecognized command

S1(config)#interfa gi1/0/1

S1(config-if)#swi

S1(config-if)#switchport trunk ?

allowed  Set allowed VLAN characteristics when interface is in trunking mode

native   Set trunking native characteristics when interface is in trunking

mode

pruning  Set pruning VLAN characteristics when interface is in trunking mode

S1(config-if)#switchport trunk native ?

vlan  Set native VLAN when interface is in trunking mode

S1(config-if)#switchport trunk native vlan ?

<1-4094>  VLAN ID of the native VLAN when this port is in trunking mode

S1(config-if)#switchport trunk native vlan

*Aug 21 11:58:00.608: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.

S1(config-if)#

S1(config-if)#exit

S1(config)#do show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Gi1/0/2, Gi1/0/3, Gi1/0/4

Gi1/0/5, Gi1/0/7, Gi1/0/8

Gi1/0/9, Gi1/0/10, Gi1/0/11

Gi1/0/12, Gi1/0/13, Gi1/0/14

Gi1/0/15, Gi1/0/16, Gi1/0/17

Gi1/0/18, Gi1/0/19, Gi1/0/20

Gi1/0/21, Gi1/0/22, Gi1/0/23

Gi1/0/24, Gi1/0/25, Gi1/0/26

Gi1/0/27, Gi1/0/28

7    London                           active    Gi1/0/6

8    Swansea                          active

9    Management                       active

10   Birmingham                       active

11   Native                           active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

S1(config)#

*Aug 21 11:58:53.524: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 11:59:42.522: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config)#

S1(config)#interfa gi1/0/1

S1(config-if)#switchport mode ?

access        Set trunking mode to ACCESS unconditionally

dynamic       Set trunking mode to dynamically negotiate access or trunk mode

private-vlan  Set private-vlan mode

trunk         Set trunking mode to TRUNK unconditionally

S1(config-if)#switchport mode trunk ?

<cr>

S1(config-if)#switchport mode trunk

S1(config-if)#swit?

switchport

S1(config-if)#swit

S1(config-if)#switchport ?

access         Set access mode characteristics of the interface

autostate      Include or exclude this port from vlan link up calculation

block          Disable forwarding of unknown uni/multi cast addresses

host           Set port host

mode           Set trunking mode of the interface

nonegotiate    Device will not engage in negotiation protocol on this

interface

port-security  Security related command

priority       Set appliance 802.1p priority

private-vlan   Set the private VLAN configuration

protected      Configure an interface to be a protected port

trunk          Set trunking characteristics of the interface

voice          Voice appliance attributes

<cr>

S1(config-if)#switchport

*Aug 21 12:00:31.523: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).access ?

vlan  Set VLAN when interface is in access mode

S1(config-if)#switchport access vlan ?

<1-4094>  VLAN ID of the VLAN when this port is in access mode

name      Set VLAN when interface is in access mode

S1(config-if)#switchport access vlan 11

S1(config-if)#

S1(config-if)#exit

S1(config)#do show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Gi1/0/2, Gi1/0/3, Gi1/0/4

Gi1/0/5, Gi1/0/7, Gi1/0/8

Gi1/0/9, Gi1/0/10, Gi1/0/11

Gi1/0/12, Gi1/0/13, Gi1/0/14

Gi1/0/15, Gi1/0/16, Gi1/0/17

Gi1/0/18, Gi1/0/19, Gi1/0/20

Gi1/0/21, Gi1/0/22, Gi1/0/23

Gi1/0/24, Gi1/0/25, Gi1/0/26

Gi1/0/27, Gi1/0/28

7    London                           active    Gi1/0/6

8    Swansea                          active

9    Management                       active

10   Birmingham                       active

11   Native                           active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

S1(config)#

*Aug 21 12:01:26.395: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config)#

*Aug 21 12:02:17.696: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:03:12.451: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:04:05.631: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:05:01.218: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:05:49.556: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:06:42.795: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:07:39.154: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:08:36.760: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:09:27.303: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:10:20.081: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config)#

S1(config)#

*Aug 21 12:11:10.847: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config)#

S1(config)#interface range gi1/0/1-2

S1(config-if-range)#.

^

% Invalid input detected at '^' marker.

S1(config-if-range)#chaa

S1(config-if-range)#chann

S1(config-if-range)#channel-gro

S1(config-if-range)#channel-group 1 mode active

Creating a port-channel interface Port-channel 1

S1(config-if-range)#

*Aug 21 12:12:04.316: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:12:04.934: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Gi1/0/1 and will be suspended (trunk mode of Gi1/0/2 is access, Gi1/0/1 is trunk)

*Aug 21 12:12:04.940: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan9, changed state to down

*Aug 21 12:12:05.868: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to down

*Aug 21 12:12:05.932: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/2, changed state to downexit

S1(config)#

S1(config)#

*Aug 21 12:12:12.966: %EC-5-L3DONTBNDL2: Gi1/0/1 suspended: LACP currently not enabled on the remote port.

*Aug 21 12:14:00.479: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Gi1/0/1 and will be suspended (trunk mode of Gi1/0/2 is access, Gi1/0/1 is trunk)

*Aug 21 12:14:00.546: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Gi1/0/1 and will be suspended (trunk mode of Gi1/0/2 is access, Gi1/0/1 is trunk)

*Aug 21 12:14:04.540: %SPANTREE-2-RECV_PVID_ERR: Received BPDU with inconsistent peer vlan id 1 on Port-channel1 VLAN11.

*Aug 21 12:14:04.540: %SPANTREE-2-BLOCK_PVID_PEER: Blocking Port-channel1 on VLAN0001. Inconsistent peer vlan.

*Aug 21 12:14:04.540: %SPANTREE-2-BLOCK_PVID_LOCAL: Blocking Port-channel1 on VLAN0011. Inconsistent local vlan.

*Aug 21 12:14:04.543: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan9, changed state to up

*Aug 21 12:14:05.534: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to up

*Aug 21 12:14:05.536: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:06.528: %LINK-3-UPDOWN: Interface Port-channel1, changed state to up

*Aug 21 12:14:06.535: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:07.528: %LINEPROTO-5-UPDOWN: Line protocol on Interface Port-channel1, changed state to up

*Aug 21 12:14:07.536: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:08.534: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:09.536: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:10.539: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:11.536: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:12.534: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:13.536: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:15:10.888: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:16:05.482: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:16:54.873: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config)#

S1(config)#interface gi1/0/1

S1(config-if)#sw

S1(config-if)#switchport trunk mode

^

% Invalid input detected at '^' marker.

S1(config-if)#switchport trunk mo

*Aug 21 12:17:52.486: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1

^

% Invalid input detected at '^' marker.

S1(config-if)#switchport mode trunk

S1(config-if)#switchport trunk encapsulation dot1q

^

% Invalid input detected at '^' marker.

S1(config-if)#

*Aug 21 12:18:51.362: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config-if)#

S1(config-if)#?

Interface configuration commands:

aaa                 Authentication, Authorization and Accounting.

arp                 Set arp type (arpa, probe, snap) or timeout or log

options

auto                Configure Automation

bandwidth           Set bandwidth informational parameter

carrier-delay       Specify delay for interface transitions

cdp                 CDP interface subcommands

channel-group       Etherchannel/port bundling configuration

channel-protocol    Select the channel protocol (LACP, PAgP)

dampening           Enable event dampening

default             Set a command to its defaults

delay               Specify interface throughput delay

description         Interface specific description

down-when-looped    Force looped interface down

duplex              Configure duplex operation.

ethernet            Ethernet interface parameters

exit                Exit from interface configuration mode

fair-queue          Enable Fair Queuing on an Interface

flowcontrol         Configure flow operation.

help                Description of the interactive help system

history             Interface history histograms - 60 second, 60 minute and

72 hour

hold-queue          Set hold queue depth

ip                  Interface Internet Protocol config commands

ipv6                IPv6 interface subcommands

keepalive           Enable keepalive

lacp                LACP interface subcommands

link                Interface link related commands

lldp                LLDP interface subcommands

load-interval       Specify interval for load calculation for an interface

location            Interface location information

logging             Configure logging for interface

loopdetect          Configure loopdetect feature setting

mac                 MAC interface commands

macro               Command macro

mdix                Set Media Dependent Interface with Crossover

mls                 mls interface commands

neighbor            interface neighbor configuration mode commands

network-policy      Network Policy

nmsp                NMSP interface configuration

no                  Negate a command or set its defaults

pagp                PAgP interface subcommands

power               Power configuration

priority-queue      Priority Queue

random-detect       Enable Weighted Random Early Detection (WRED) on an

Interface

rep                 Resilient Ethernet Protocol characteristics

routing             Per-interface routing configuration

service-policy      Configure CPL Service Policy

sflow               Enable sflow on the interface

shutdown            Shutdown the selected interface

snmp                Modify SNMP interface parameters

source              Get config from another source

spanning-tree       Spanning Tree Subsystem

speed               Configure speed operation.

srr-queue           Configure shaped round-robin transmit queues

storm-control       storm configuration

subscriber          Subscriber inactivity timeout value.

switchport          Set switching mode characteristics

timeout             Define timeout values for this interface

topology            Configure routing topology on the interface

transmit-interface  Assign a transmit interface to a receive-only interface

tx-ring-limit       Configure PA level transmit ring limit

udld                Configure UDLD enabled or disabled and ignore global UDLD

setting

vtp                 Enable VTP on this interface

S1(config-if)#swit

S1(config-if)#switchport trunk ?

allowed  Set allowed VLAN characteristics when interface is in trunking mode

native   Set trunking native characteristics when interface is in trunking

mode

pruning  Set pruning VLAN characteristics when interface is in trunking mode

S1(config-if)#switchport trunk native ?

vlan  Set native VLAN when interface is in trunking mode

S1(config-if)#switchport trunk native vlan ?

<1-4094>  VLAN ID of the native VLAN when this port is in trunking mode

S1(config-if)#switchport trunk native vlan 11

S1(config-if)#

S1(config-if)#

*Aug 21 12:19:42.934: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config-if)#switchport trunk al

S1(config-if)#switchport trunk allowed ?

vlan  Set allowed VLANs when interface is in trunking mode

S1(config-if)#switchport trunk allowed vlan ?

WORD    VLAN IDs of the allowed VLANs when this port is in trunking mode

add     add VLANs to the current list

all     all VLANs

except  all VLANs except the following

none    no VLANs

remove  remove VLANs from the current list

S1(config-if)#switchport trunk allowed vlan 7,8,9,10,11

S1(config-if)#

*Aug 21 12:20:18.350: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Po1 and will be suspended (access vlan of Gi1/0/2 is 1, Po1 is 11)

*Aug 21 12:20:18.350: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Po1 and will be suspended (access vlan of Gi1/0/2 is 1, Po1 is 11)

*Aug 21 12:20:18.357: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan9, changed state to down

*Aug 21 12:20:19.345: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to down

*Aug 21

S1(config-if)#12:20:19.346: %LINEPROTO-5-UPDOWN: Line protocol on Interface Port-channel1, changed state to down

*Aug 21 12:20:20.350: %LINK-3-UPDOWN: Interface Port-channel1, changed state to down

*Aug 21 12:20:26.057: %SPANTREE-2-RECV_PVID_ERR: Received BPDU with inconsistent peer vlan id 1 on Port-channel1 VLAN11.

*Aug 21 12:20:26.057: %SPANTREE-2-BLOCK_PVID_LOCAL: Blocking Port-channel1 on VLAN0011. Inconsistent local vlan.

*Aug 21 12:20:26.060: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan9, changed state to up

*Aug 21 12:20:27.047: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to up

*Aug 21 12:20:27.054: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:28.045: %LINK-3-UPDOWN: Interface Port-channel1, changed state to up

*Aug 21 12:20:28.052: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:29.045: %LINEPROTO-5-UPDOWN: Line protocol on Interface Port-channel1, changed state to up

*Aug 21 12:20:29.053: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:30.052: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:31.051: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:32.052: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:33.051: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:34.052: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:35.051: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:21:30.906: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config-if)#

*Aug 21 12:22:19.494: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

User Access Verification

Password:

Password:

Password:

% Bad passwords

R1 con0 is now available

Press RETURN to get started.

User Access Verification

Password:

R1>ena

Password:

Password:

R1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#

R1(config)#

R1(config)#ip dhcp-exclude address

^

% Invalid input detected at '^' marker.

R1(config)#ip dhcp ?

aaa                        Configure aaa attributes

auto-broadcast             Configure auto broadcast feature

binding                    DHCP address bindings

bootp                      BOOTP specific configuration

class                      Configure DHCP classes

compatibility              Compatibility configuration

conflict                   DHCP address conflict parameters

database                   Configure DHCP database agents

drop-inform                Drop inform message if enabled

event-notification         DHCP event notifications

excluded-address           Prevent DHCP from assigning certain addresses

global-options             Configure global DHCP options

limit                      Limit DHCP Lease

limited-broadcast-address  Use all 1's broadcast address

monitor                    Monitor DHCP packets

ping                       Specify ping parameters used by DHCP

pool                       Configure DHCP address pools

relay                      DHCP relay agent parameters

remember                   Remember released bindings

route                      Specify the type of routes for clients on

unnumbered interfaces

smart-relay                Enable Smart Relay feature

snooping                   DHCP Snooping

subscriber-id              Global subscriber-id configuration

support                    Configure support for certain features

update                     Configure dynamic updates

use                        Configure use of certain parameters during

allocation

R1(config)#ip dhcp my_pool

^

% Invalid input detected at '^' marker.

R1(config)#ip dhcp pool my_pool

R1(dhcp-config)#network 192.168.20.1 ?

/nn or A.B.C.D  Network mask or prefix length

<cr>            <cr>

R1(dhcp-config)#network 192.168.20.1

R1(dhcp-config)#network 192.168.30.1

R1(dhcp-config)#network 192.168.40.1

R1(dhcp-config)#ip defa

R1(dhcp-config)#ip default-router ?

% Unrecognized command

R1(dhcp-config)#?

DHCP pool configuration commands:

accounting           Send Accounting Start/Stop messages

address              Configure a reserved address

authorization        Obtain address allocation information from AAA

bootfile             Boot file name

class                Specify a DHCP class

client-identifier    Client identifier

client-name          Client name

default-router       Default routers

dns-server           DNS servers

domain-name          Domain name

exit                 Exit from DHCP pool configuration mode

hardware-address     Client hardware address

host                 Client IP address and mask

import               Programatically importing DHCP option parameters

lease                Address lease time

netbios-name-server  NetBIOS (WINS) name servers

netbios-node-type    NetBIOS node type

network              Network number and mask

next-server          Next server in boot process

no                   Negate a command or set its defaults

option               Raw DHCP options

origin               Configure the origin of the pool

relay                Function as a DHCP relay

remember             Remember released bindings

renew                Configure renewal policy

reserved-only        Only allocate reserved addresses

server               Configure the server ID option value

subnet               Subnet allocation commands

update               Dynamic updates

utilization          Configure various utilization parameters

vrf                  Associate this pool with a VRF

R1(dhcp-config)#defa

R1(dhcp-config)#default-router 192.168.40.1

R1(dhcp-config)#dns

R1(dhcp-config)#dns-server 8.8.8.8 ?

Hostname or A.B.C.D  Server's name or IP address

<cr>                 <cr>

R1(dhcp-config)#dns-server 8.8.8.8

R1(dhcp-config)#doma

R1(dhcp-config)#domain-name Google.com

R1(dhcp-config)#lease 7

R1(dhcp-config)#

R1(dhcp-config)#exit

R1(config)#

*Aug 21 12:41:32.284: %EC-5-L3DONTBNDL2: Gi1/0/1 suspended: LACP currently not enabled on the remote port.

User Access Verification

Password:

S1>ena

Password:

S1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#

S1(config)#inter

S1(config)#interface gi1/0/6

S1(config-if)#swi

S1(config-if)#switchport port

S1(config-if)#switchport port-security max

S1(config-if)#switchport port-security maximum 3

S1(config-if)#exit

S1(config)#do write

Building configuration...

[OK]

S1(config)#inter

S1(config)#interface range  gi1/0/1-2

S1(config-if-range)#sw

S1(config-if-range)#switchport trunk

S1(config-if-range)#switchport trunk native vlan 11

S1(config-if-range)#

*Aug 21 12:48:10.126: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Po1 and will be suspended (access vlan of Gi1/0/2 is 1, Po1 is 11)

*Aug 21 12:48:10.126: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Po1 and will be suspended (access vlan of Gi1/0/2 is 1, Po1 is 11)

*Aug 21 12:48:17.898: %EC-5-L3DONTBNDL2: Gi1/0/1 suspended: LACP currently not enabled on the remote port.switchport trunk allowed ?

vlan  Set allowed VLANs when interface is in trunking mode

S1(config-if-range)#switchport trunk allowed ?

vlan  Set allowed VLANs when interface is in trunking mode

S1(config-if-range)#switchport trunk allowed vlan 7,8,9,10,11

S1(config-if-range)#

*Aug 21 12:48:42.724: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Po1 and will be suspended (access vlan of Gi1/0/2 is 1, Po1 is 11)

*Aug 21 12:48:50.610: %EC-5-L3DONTBNDL2: Gi1/0/1 suspended: LACP currently not enabled on the remote port.

*Aug 21 12:53:03.007: %EC-5-L3DONTBNDL2: Gi1/0/1 suspended: LACP currently not enabled on the remote port.

*Aug 21 12:53:05.994: %EC-5-L3DONTBNDL2: Gi1/0/2 suspended: LACP currently not enabled on the remote port.

S1 con0 is now available

Press RETURN to get started.

*Aug 21 13:00:11.112: %SYS-5-CONFIG_I: Configured from console by console

User Access Verification

Password:

S1>ena

Password:

Password:

S1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#interface vlan 7

S1(config-if)#1

*Aug 21 13:01:00.122: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan7, changed state to dowip add

S1(config-if)#ip address 192.168.20.2 255.255.255.0

S1(config-if)#no shut

S1(config-if)#no shutdown

S1(config-if)#

S1(config-if)#interface vlan 8

S1(config-if)#ip address 192.168.20.2 255.255.255.0

*Aug 21 13:01:22.337: %LINEPROTO-5-UPDOWN: Line protocol on Inp address 192.168.30.2 255.255.255.0

S1(config-if)#no shutdown

S1(config-if)#

S1(config-if)#interface vlan 9

S1(config-if)#ip address 192.168.40.2 255.255.255.0

S1(config-if)#no shutdown

S1(config-if)#

S1(config-if)#exit

S1(config)#

S1(config)#interfa gi1/0/1

S1(config-if)#sw

S1(config-if)#switchport m

S1(config-if)#switchport mode t

S1(config-if)#switchport mode trunk

S1(config-if)#s

S1(config-if)#sw

S1(config-if)#switchport tr

S1(config-if)#switchport trunk native vlan 11

S1(config-if)#

S1(config-if)#switchport trunk allow

S1(config-if)#switchport trunk allowed vlan 7,8,9,10,11

S1(config-if)#

S1(config-if)#exit

S1(config)#do write

Building configuration...

[OK]

S1(config)#

S1(config)#inte

S1(config)#interface gi1/0/2

S1(config-if)#switchport mode trunk

S1(config-if)#switchport trunk native vlan 11

S1(config-if)#switchport trunk allowed vlan 7,8,9,10,11

S1(config-if)#exit

S1(config)#do write

Building configuration...

[OK]

S1(config)#

S1(config)#

S1(config)#

S1(config)#end

S1#

S1#show

*Aug 21 13:16:23.772: %SYS-5-CONFIG_I: Configured from console by con

S1#

S1#show run

S1#show running-config

Building configuration...

Current configuration : 4465 bytes

!

! Last configuration change at 13:16:23 UTC Wed Aug 21 2024

!

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname S1

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$xXvoADXyT.K4eJ$HgBhlbsOgNbxdd/lF9ShWA8QggT8NSUw3MTEhCkfu0w

!

username cisco secret 9 $9$f8OZZv7OS1SmQD$Aka52NM3k758RfB0/3z8hp7BG2gZ5B3fU65ETt83FFc

no aaa new-model

switch 1 provision c1000-24t-4g-l

system mtu routing 1500

!

!

no ip domain-lookup

ip domain-name swre-lab.com

!

!

!

!

!

!

crypto pki trustpoint TP-self-signed-1877477248

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1877477248

revocation-check none

rsakeypair TP-self-signed-1877477248

!

!

crypto pki certificate chain TP-self-signed-1877477248

certificate self-signed 01

3082022B 30820194 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 31383737 34373732 3438301E 170D3234 30383231 31303236

32355A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D31 38373734

37373234 3830819F 300D0609 2A864886 F70D0101 01050003 818D0030 81890281

8100C618 5BB0DAFE D2AA3C79 22B86DED 95A07A9C 694CD83C 52351104 60F655D7

71B201A5 BE2F231C 1B81D4C7 CFBC9C74 F5D5939A 6A4EBC13 727273B7 C2407993

125256D0 EDB5F144 D7C0C541 D1403600 4C9EE0D9 437DC610 AD7DD500 62A62EB9

AB0C451F 0D4D0C0F 37A13A8C DD273945 8A6568CC 902C4E23 0B66B736 F2A6DE4B

31D90203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF 301F0603

551D2304 18301680 14571337 76A72C30 3A6E0D10 5F97B01E EE04FEFB CE301D06

03551D0E 04160414 57133776 A72C303A 6E0D105F 97B01EEE 04FEFBCE 300D0609

2A864886 F70D0101 05050003 818100BD F3CDA9FB 11953CF9 BC925511 8ADA5B31

BFECC7DD 44869044 9BB6C8D6 248B9F21 5893435C C441DCBB 6B6CD3C3 14C4F6ED

559409E2 37B36F87 50159A57 0DF77EF7 3F1FC0B8 06121061 2BEC14FB 64E5731A

036A18DD 048617C5 E47286A0 C33602A9 373F0A58 9FB06A0F C67003C2 47B4A12E

8BAF6BD3 91939A4E 653BC6BC 17C5D4

quit

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

!

!

!

!

!

interface Port-channel1

switchport access vlan 11

switchport trunk allowed vlan 7-11

switchport trunk native vlan 11

switchport mode trunk

!

interface GigabitEthernet1/0/1

switchport access vlan 11

switchport trunk allowed vlan 7-11

switchport trunk native vlan 11

switchport mode trunk

channel-group 1 mode active

!

interface GigabitEthernet1/0/2

switchport trunk allowed vlan 7-11

switchport trunk native vlan 11

switchport mode trunk

channel-group 1 mode active

!

interface GigabitEthernet1/0/3

shutdown

!

interface GigabitEthernet1/0/4

shutdown

!

interface GigabitEthernet1/0/5

!

interface GigabitEthernet1/0/6

switchport access vlan 7

switchport mode access

switchport port-security maximum 3

shutdown

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

shutdown

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

shutdown

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/0/25

shutdown

!

interface GigabitEthernet1/0/26

shutdown

!

interface GigabitEthernet1/0/27

shutdown

!

interface GigabitEthernet1/0/28

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan7

ip address 192.168.20.2 255.255.255.0

!

interface Vlan8

ip address 192.168.30.2 255.255.255.0

!

interface Vlan9

ip address 192.168.40.2 255.255.255.0

!

ip default-gateway 192.168.40.1

ip http server

ip http secure-server

!

!

!

!

line con 0

password 7 045802150C2E1D1C5A

login

line vty 0 4

password 7 045802150C2E1D1C5A

login local

transport input none

line vty 5 15

password 7 104D000A061843595F

login local

transport input none

!

end

S1#show ip inter

S1#show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Vlan1                  unassigned      YES NVRAM  administratively down down

Vlan7                  192.168.20.2    YES manual up                    down

Vlan8                  192.168.30.2    YES manual up                    down

Vlan9                  192.168.40.2    YES manual up                    down

GigabitEthernet1/0/1   unassigned      YES unset  up                    down

GigabitEthernet1/0/2   unassigned      YES unset  up                    down

GigabitEthernet1/0/3   unassigned      YES unset  administratively down down

GigabitEthernet1/0/4   unassigned      YES unset  administratively down down

GigabitEthernet1/0/5   unassigned      YES unset  down                  down

GigabitEthernet1/0/6   unassigned      YES unset  administratively down down

GigabitEthernet1/0/7   unassigned      YES unset  administratively down down

GigabitEthernet1/0/8   unassigned      YES unset  administratively down down

GigabitEthernet1/0/9   unassigned      YES unset  administratively down down

GigabitEthernet1/0/10  unassigned      YES unset  administratively down down

GigabitEthernet1/0/11  unassigned      YES unset  administratively down down

GigabitEthernet1/0/12  unassigned      YES unset  administratively down down

GigabitEthernet1/0/13  unassigned      YES unset  administratively down down

GigabitEthernet1/0/14  unassigned      YES unset  administratively down down

GigabitEthernet1/0/15  unassigned      YES unset  administratively down down

GigabitEthernet1/0/16  unassigned      YES unset  administratively down down

GigabitEthernet1/0/17  unassigned      YES unset  administratively down down

GigabitEthernet1/0/18  unassigned      YES unset  administratively down down

GigabitEthernet1/0/19  unassigned      YES unset  administratively down down

GigabitEthernet1/0/20  unassigned      YES unset  administratively down down

GigabitEthernet1/0/21  unassigned      YES unset  administratively down down

GigabitEthernet1/0/22  unassigned      YES unset  administratively down down

GigabitEthernet1/0/23  unassigned      YES unset  administratively down down

GigabitEthernet1/0/24  unassigned      YES unset  administratively down down

GigabitEthernet1/0/25  unassigned      YES unset  administratively down down

GigabitEthernet1/0/26  unassigned      YES unset  administratively down down

GigabitEthernet1/0/27  unassigned      YES unset  administratively down down

GigabitEthernet1/0/28  unassigned      YES unset  administratively down down

Port-channel1          unassigned      YES unset  down                  down

S1#show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Gi1/0/2, Gi1/0/3, Gi1/0/4

Gi1/0/5, Gi1/0/7, Gi1/0/8

Gi1/0/9, Gi1/0/10, Gi1/0/11

Gi1/0/12, Gi1/0/13, Gi1/0/14

Gi1/0/15, Gi1/0/16, Gi1/0/17

Gi1/0/18, Gi1/0/19, Gi1/0/20

Gi1/0/21, Gi1/0/22, Gi1/0/23

Gi1/0/24, Gi1/0/25, Gi1/0/26

Gi1/0/27, Gi1/0/28

7    London                           active    Gi1/0/6

8    Swansea                          active

9    Management                       active

10   Birmingham                       active

11   Native                           active    Gi1/0/1

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

7    enet  100007     1500  -      -      -        -    -        0      0

8    enet  100008     1500  -      -      -        -    -        0      0

9    enet  100009     1500  -      -      -        -    -        0      0

10   enet  100010     1500  -      -      -        -    -        0      0

11   enet  100011     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

Primary Secondary Type              Ports

------- --------- ----------------- ------------------------------------------

S1#

# Commands on a R1:

Last input never, output never, output hang never

Last clearing of "show interface" counters never

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

0 packets input, 0 bytes, 0 no buffer

Received 0 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 packets output, 0 bytes, 0 underruns

0 output errors, 1 interface resets

0 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

Vlan9 is up, line protocol is down

S1(config)#do show ip brief

^

% Invalid input detected at '^' marker.

S1(config)#do show ip ?

access-lists    List IP access lists

accounting      The active IP accounting database

admission       Network Admission Control information

aliases         IP alias table

arp             IP ARP table

cache           IP fast-switching route cache

cef             Cisco Express Forwarding

device          Show IP Tracking Hosts

dhcp            Show items in the DHCP database

helper-address  helper-address table

host            IP host information

http            HTTP information

igmp            IGMP extension

interface       IP interface status and configuration

local           IP local options

masks           Masks associated with a network

nat             IP NAT information

port-map        Port to Application Mapping (PAM) information

protocols       IP routing protocol process parameters and statistics

redirects       IP redirects

rib             Routing Information Base

rip             IP RIP show commands

route           IP routing table

sockets         Open IP sockets

source          IP source

ssh             Information on SSH

static          Static operation

traffic         IP protocol statistics

verify          Ip verify

S1(config)#end

S1#sh

*Aug 21 11:16:20.149: %SYS-5-CONFIG_I: Configured from console by consoleow

% Type "show ?" for a list of subcommands

S1#dhow

Translating "dhow"

% Unknown command or computer name, or unable to find computer address

S1#show

% Type "show ?" for a list of subcommands

S1#show ?

access-lists       List access lists

aliases            Display alias commands

auto               Show Automation Template

banner             Display banner information

configuration      Contents of Non-Volatile memory

controllers        Interface controller status

debugging          State of each debugging option

exception          exception informations

history            Display the session command history

hosts              IP domain-name, lookup style, nameservers, and host table

interfaces         Interface status and configuration

location           Display the system location

logging            Show the contents of logging buffers

parser             Display parser information

privilege          Show current privilege level

queue              Show queue contents

queueing           Show queueing configuration

running-config     Current operating configuration

startup-config     Contents of startup configuration

trace              Show trace options

hardware

aaa                Show AAA values

adjacency          Adjacent nodes

alignment          Show alignment information

archive            Archive functions

arp                ARP table

authentication     Shows Auth Manager stats, registrations or sessions

beep               Show BEEP information

bluetooth          bluetooth show commands

boot               show boot attributes

buffers            Buffer pool statistics

bulkstat           Bulkstat show commands

caaa               Subscriber Policy Entity Information

cable-diagnostics  Show Cable Diagnostics Results

cache              Shows Device-Sensor Cache Informations

calendar           Display the hardware calendar

call-home          Show command for call home

capability         Capability Information

cca                CCA information

cdp                CDP information

cef                CEF address family independent status

cisp               Shows CISP information

class-map          Show CPL Class Map

clock              Display the system clock

cns                CNS agents

context            Show context information about recent crash(s)

crypto             Encryption module

dampening          Display dampening information

data-corruption    Show data errors

dcm                Data Collection Manager Core Details

derived-config     Derived operating configuration

device             Display device classifier information

device-sensor      Shows Device Sensor Information

dhcp               Dynamic Host Configuration Protocol status

diagnostic         Show command for diagnostic

dot1x              Dot1x information

dtp                DTP information

dying-gasp         Dying Gasp information

eap                Shows EAP registration/session information

eee                Show Energy Efficient Ethernet

env                Environmental facilities

epm                EPM information

errdisable         Error disable

etherchannel       EtherChannel information

ethernet           Ethernet parameters

fallback           Display Fallback configuration

file               Show filesystem information

fips               FIPS information

flash1:            display information about flash1: file system

flash:             display information about flash: file system

flowcontrol        show flow control information

format             Show format information

html               HTML helper commands

id-manager         ID pool manager

idb                List of Interface Descriptor Blocks

identity           Identity profiles and policies

idprom             show IDPROMs for interfaces

if-mgr             if-mgr information

inventory          Show the physical inventory

ip                 IP information

ipc                Interprocess communications commands

ipv6               IPv6 information

kron               Kron Subsystem

l2                 Layer 2

lacp               Port channel information

license            Displays all the RTU licenses.

line               TTY line information

link               Show Link

lldp               LLDP information

login              Display Secure Login Configurations and State

loopdetect         loopdetect information

mab                MAB information

mac                MAC configuration

macro              Show command macros

memory             Memory statistics

mls                mls global commands

monitor            Monitoring different system events

mtm                MTM

network-policy     Network Policy profile information

nmsp               nmsp show commands

ntp                Network time protocol

object-group       List object groups

odm-format         Show the schema used for ODM input file

pagp               Port channel information

parameter-map      Show parameter map of type

persistent         Show persistent information

platform           platform specific show commands

pm                 Show Port Manager commands

pnp                Display PNP information

policy-manager     Policy Manager

policy-map         Show Policy Map

port-security      Show secure port information

post               Show the results of the system POST

power              Switch Power

pre                PRE show commands

processes          Active process statistics

protocols          Active network routing protocols

psp                Show PSP details

radius             Shows radius information

region             Region Manager Status

registry           Function registry information

reload             Scheduled reload information

rep                Resilient Ethernet Protocol

resource           Resource group statistics

rhosts             Remote-host+user equivalences

rib                Routing Information Base

rmon               rmon statistics

route-tag          route-tag information

sasl               show SASL information

scalable-queue     Scalable Queue statistics

sched-event        Scheduler event information

sdm                Switch database management

service-template   Show service-templates/identity policies

sessions           Information about Telnet connections

setup              Show express setup settings

sflow              sFlow information

snmp               snmp statistics

sockets            Socket Details

spanning-tree      Spanning tree topology

ssh                Status of SSH server connections

stacks             Process stack utilization

standby            Hot Standby Router Protocol (HSRP) information

storm-control      Show storm control configuration

subsys             Show subsystem information

switch             show information about the stack ring

system             Show the system configuration

tacacs             Shows tacacs+ server statistics

tcp                Status of TCP connections

tech-support       Show system information for Tech-Support

template           Template information

terminal           Display terminal configuration parameters

test_rib_access    RIB_ACCESS TEST info

time-range         Time range

topology           Topology instance information

traffic-shape      traffic rate shaping configuration

udld               UDLD information

udp                UDP Details

usb                USB Interface

users              Display information about terminal lines

version            System hardware and software status

vlan               VTP VLAN status

vtp                VTP information

wsma               Show Web Services Management Agents information

xos                Cross-OS Library Information and Traces

xsd-format         Show the ODM XSD for the command

S1#show interface

Vlan1 is administratively down, line protocol is down

Hardware is EtherSVI, address is a0bc.6fe8.07c0 (bia a0bc.6fe8.07c0)

MTU 1500 bytes, BW 1000000 Kbit/sec, DLY 10 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation ARPA, loopback not set

Keepalive not supported

ARP type: ARPA, ARP Timeout 04:00:00

Last input never, output never, output hang never

Last clearing of "show interface" counters never

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

0 packets input, 0 bytes, 0 no buffer

Received 0 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 packets output, 0 bytes, 0 underruns

0 output errors, 1 interface resets

0 unknown protocol drops

0 output buffer failures, 0 output buffers swapped out

Vlan9 is up, line protocol is down

S1#show interface ?

Async              Async interface

Auto-Template      Auto-Template interface

BVI                Bridge-Group Virtual Interface

Bluetooth          Bluetooth interface

CTunnel            CTunnel interface

Dialer             Dialer interface

Filter             Filter interface

Filtergroup        Filter Group interface

GigabitEthernet    GigabitEthernet IEEE 802.3z

GroupVI            Group Virtual interface

Loopback           Loopback interface

Lspvif             LSP virtual interface

Null               Null interface

Port-channel       Ethernet Channel of interfaces

Portgroup          Portgroup interface

Pos-channel        POS Channel of interfaces

Tunnel             Tunnel interface

Vif                PGM Multicast Host interface

Virtual-Template   Virtual Template interface

Virtual-TokenRing  Virtual TokenRing

Vlan               Catalyst Vlans

accounting         Show interface accounting

S1#show interfaces ?

Async              Async interface

Auto-Template      Auto-Template interface

BVI                Bridge-Group Virtual Interface

Bluetooth          Bluetooth interface

CTunnel            CTunnel interface

Dialer             Dialer interface

Filter             Filter interface

Filtergroup        Filter Group interface

GigabitEthernet    GigabitEthernet IEEE 802.3z

GroupVI            Group Virtual interface

Loopback           Loopback interface

Lspvif             LSP virtual interface

Null               Null interface

Port-channel       Ethernet Channel of interfaces

Portgroup          Portgroup interface

Pos-channel        POS Channel of interfaces

Tunnel             Tunnel interface

Vif                PGM Multicast Host interface

Virtual-Template   Virtual Template interface

Virtual-TokenRing  Virtual TokenRing

Vlan               Catalyst Vlans

accounting         Show interface accounting

S1#show interfaces ig

S1#show interfaces gig

S1#show interfaces gigabitEthernet

% Incomplete command.

S1#show interfaces gigabitEthernet0/6

^

% Invalid input detected at '^' marker.

S1#show interfaces gigabitEthernet0/0/6

^

% Invalid input detected at '^' marker.

S1#show interfaces brief

^

% Invalid input detected at '^' marker.

S1#show interface brief

^

% Invalid input detected at '^' marker.

S1#show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Vlan1                  unassigned      YES NVRAM  administratively down down

Vlan9                  192.168.40.2    YES manual up                    down

GigabitEthernet1/0/1   unassigned      YES unset  up                    up

GigabitEthernet1/0/2   unassigned      YES unset  up                    up

GigabitEthernet1/0/3   unassigned      YES unset  down                  down

GigabitEthernet1/0/4   unassigned      YES unset  down                  down

GigabitEthernet1/0/5   unassigned      YES unset  down                  down

GigabitEthernet1/0/6   unassigned      YES unset  up                    up

GigabitEthernet1/0/7   unassigned      YES unset  down                  down

GigabitEthernet1/0/8   unassigned      YES unset  down                  down

GigabitEthernet1/0/9   unassigned      YES unset  down                  down

GigabitEthernet1/0/10  unassigned      YES unset  down                  down

GigabitEthernet1/0/11  unassigned      YES unset  down                  down

GigabitEthernet1/0/12  unassigned      YES unset  down                  down

GigabitEthernet1/0/13  unassigned      YES unset  down                  down

GigabitEthernet1/0/14  unassigned      YES unset  down                  down

GigabitEthernet1/0/15  unassigned      YES unset  down                  down

GigabitEthernet1/0/16  unassigned      YES unset  down                  down

GigabitEthernet1/0/17  unassigned      YES unset  down                  down

GigabitEthernet1/0/18  unassigned      YES unset  down                  down

GigabitEthernet1/0/19  unassigned      YES unset  down                  down

GigabitEthernet1/0/20  unassigned      YES unset  down                  down

GigabitEthernet1/0/21  unassigned      YES unset  down                  down

GigabitEthernet1/0/22  unassigned      YES unset  down                  down

GigabitEthernet1/0/23  unassigned      YES unset  down                  down

GigabitEthernet1/0/24  unassigned      YES unset  down                  down

GigabitEthernet1/0/25  unassigned      YES unset  down                  down

GigabitEthernet1/0/26  unassigned      YES unset  down                  down

GigabitEthernet1/0/27  unassigned      YES unset  down                  down

GigabitEthernet1/0/28  unassigned      YES unset  down                  down

S1#

S1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#interface gi1/0/6

S1(config-if)#swit

S1(config-if)#switchport mode acc

S1(config-if)#switchport mode access

S1(config-if)#sw

S1(config-if)#switchport access vlan 7

S1(config-if)#port?

% Unrecognized command

S1(config-if)#?

Interface configuration commands:

aaa                 Authentication, Authorization and Accounting.

access-session      Access Session specific Interface Configuration Commands

arp                 Set arp type (arpa, probe, snap) or timeout or log

options

authentication      Auth Manager Interface Configuration Commands

auto                Configure Automation

bandwidth           Set bandwidth informational parameter

carrier-delay       Specify delay for interface transitions

cdp                 CDP interface subcommands

channel-group       Etherchannel/port bundling configuration

channel-protocol    Select the channel protocol (LACP, PAgP)

dampening           Enable event dampening

default             Set a command to its defaults

delay               Specify interface throughput delay

description         Interface specific description

dot1x               Interface Config Commands for IEEE 802.1X

down-when-looped    Force looped interface down

duplex              Configure duplex operation.

ethernet            Ethernet interface parameters

exit                Exit from interface configuration mode

fair-queue          Enable Fair Queuing on an Interface

flowcontrol         Configure flow operation.

help                Description of the interactive help system

history             Interface history histograms - 60 second, 60 minute and

72 hour

hold-queue          Set hold queue depth

ip                  Interface Internet Protocol config commands

ipv6                IPv6 interface subcommands

keepalive           Enable keepalive

lacp                LACP interface subcommands

link                Interface link related commands

lldp                LLDP interface subcommands

load-interval       Specify interval for load calculation for an interface

location            Interface location information

logging             Configure logging for interface

loopdetect          Configure loopdetect feature setting

mab                 MAC Authentication Bypass Interface Config Commands

mac                 MAC interface commands

macro               Command macro

mdix                Set Media Dependent Interface with Crossover

mls                 mls interface commands

neighbor            interface neighbor configuration mode commands

network-policy      Network Policy

nmsp                NMSP interface configuration

no                  Negate a command or set its defaults

pagp                PAgP interface subcommands

power               Power configuration

priority-queue      Priority Queue

random-detect       Enable Weighted Random Early Detection (WRED) on an

Interface

rep                 Resilient Ethernet Protocol characteristics

routing             Per-interface routing configuration

service-policy      Configure CPL Service Policy

sflow               Enable sflow on the interface

shutdown            Shutdown the selected interface

snmp                Modify SNMP interface parameters

source              Get config from another source

spanning-tree       Spanning Tree Subsystem

speed               Configure speed operation.

srr-queue           Configure shaped round-robin transmit queues

storm-control       storm configuration

subscriber          Subscriber inactivity timeout value.

switchport          Set switching mode characteristics

timeout             Define timeout values for this interface

topology            Configure routing topology on the interface

transmit-interface  Assign a transmit interface to a receive-only interface

tx-ring-limit       Configure PA level transmit ring limit

udld                Configure UDLD enabled or disabled and ignore global UDLD

setting

vtp                 Enable VTP on this interface

S1(config-if)#exit

S1(config)#interface gi1/0/6

S1(config-if)#secu

^

% Invalid input detected at '^' marker.

S1(config-if)#secu

S1(config-if)#port

S1(config-if)#port?

% Unrecognized command

S1(config-if)#secu

S1(config-if)#security

S1(config-if)#security-pol

S1(config-if)#security-policy ?

% Unrecognized command

S1(config-if)#security-policy

^

% Invalid input detected at '^' marker.

S1(config-if)#servi

S1(config-if)#service-policy ?

input   Assign policy-map to the input of an interface

output  Assign policy-map to the output of an interface

type    Configure CPL Service Policy

S1(config-if)#exit

S1(config)#interface range gi1/0/1-2

S1(config-if-range)#?

Interface configuration commands:

aaa                 Authentication, Authorization and Accounting.

access-session      Access Session specific Interface Configuration Commands

arp                 Set arp type (arpa, probe, snap) or timeout or log

options

auto                Configure Automation

bandwidth           Set bandwidth informational parameter

carrier-delay       Specify delay for interface transitions

cdp                 CDP interface subcommands

channel-group       Etherchannel/port bundling configuration

channel-protocol    Select the channel protocol (LACP, PAgP)

dampening           Enable event dampening

default             Set a command to its defaults

delay               Specify interface throughput delay

description         Interface specific description

down-when-looped    Force looped interface down

duplex              Configure duplex operation.

ethernet            Ethernet interface parameters

exit                Exit from interface configuration mode

fair-queue          Enable Fair Queuing on an Interface

flowcontrol         Configure flow operation.

help                Description of the interactive help system

history             Interface history histograms - 60 second, 60 minute and

72 hour

hold-queue          Set hold queue depth

ip                  Interface Internet Protocol config commands

ipv6                IPv6 interface subcommands

keepalive           Enable keepalive

lacp                LACP interface subcommands

link                Interface link related commands

lldp                LLDP interface subcommands

load-interval       Specify interval for load calculation for an interface

location            Interface location information

logging             Configure logging for interface

loopdetect          Configure loopdetect feature setting

mac                 MAC interface commands

macro               Command macro

mdix                Set Media Dependent Interface with Crossover

mls                 mls interface commands

neighbor            interface neighbor configuration mode commands

network-policy      Network Policy

nmsp                NMSP interface configuration

no                  Negate a command or set its defaults

pagp                PAgP interface subcommands

power               Power configuration

S1(config-if-range)#cha

S1(config-if-range)#channel-gr

S1(config-if-range)#channel-group ?

<1-6>  Channel group number

auto   Enable LACP auto on this interface

S1(config-if-range)#channel-group 1

% Incomplete command.

S1(config-if-range)#channel-group 1?

<1-6>

S1(config-if-range)#channel-group 2

% Incomplete command.

S1(config-if-range)#channel-group 2?

<1-6>

S1(config-if-range)#channel-group 1-2

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group ?

<1-6>  Channel group number

auto   Enable LACP auto on this interface

S1(config-if-range)#channel-group 1?

<1-6>

S1(config-if-range)#channel-group 1-6

^

% Invalid input detected at '^' marker.

S1(config-if-range)#?

Interface configuration commands:

aaa                 Authentication, Authorization and Accounting.

access-session      Access Session specific Interface Configuration Commands

arp                 Set arp type (arpa, probe, snap) or timeout or log

options

auto                Configure Automation

bandwidth           Set bandwidth informational parameter

carrier-delay       Specify delay for interface transitions

cdp                 CDP interface subcommands

channel-group       Etherchannel/port bundling configuration

channel-protocol    Select the channel protocol (LACP, PAgP)

dampening           Enable event dampening

default             Set a command to its defaults

delay               Specify interface throughput delay

description         Interface specific description

down-when-looped    Force looped interface down

duplex              Configure duplex operation.

ethernet            Ethernet interface parameters

exit                Exit from interface configuration mode

fair-queue          Enable Fair Queuing on an Interface

flowcontrol         Configure flow operation.

help                Description of the interactive help system

S1(config-if-range)#channel-group 1 6

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1 2

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1 0

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 14

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1

% Incomplete command.

S1(config-if-range)#channel-group 1-6

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1

% Incomplete command.

S1(config-if-range)#channel-group 1?

<1-6>

S1(config-if-range)#channel-group 1-6

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1 ?

mode  Etherchannel Mode of the interface

S1(config-if-range)#channel-group 1 L?

% Unrecognized command

S1(config-if-range)#channel-group 1 p?

% Unrecognized command

S1(config-if-range)#channel-group 1 pagp

^

% Invalid input detected at '^' marker.

S1(config-if-range)#channel-group 1 pagp?

% Unrecognized command

S1(config-if-range)#channel-group 1 p?

% Unrecognized command

S1(config-if-range)#exit

S1(config)#inter

S1(config)#interface range gi1/0/3-4

S1(config-if-range)#shut

S1(config-if-range)#shutdown

S1(config-if-range)#ex

*Aug 21 11:39:32.292: %LINK-5-CHANGED: Interface GigabitEthernet1/0/3, changed state to administratively down

*Aug 21 11:39:32.293: %LINK-5-CHANGED: Interface GigabitEthernet1/0/4, changed state to administratively downit

S1(config)#exit

S1#

*Aug 21 11:39:37.603: %SYS-5-CONFIG_I: Configured from console by console

S1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#interface range gi1/0/6-28

S1(config-if-range)#shutd

S1(config-if-range)#shutdown

S1(config-if-range)#

*Aug 21 11:40:00.385: %LINK-5-CHANGED: Interface GigabitEthernet1/0/6, changed state to administratively down

*Aug 21 11:40:00.446: %LINK-5-CHANGED: Interface GigabitEthernet1/0/7, changed state to administratively down

*Aug 21 11:40:00.448: %LINK-5-CHANGED: Interface GigabitEthernet1/0/8, changed state to administratively down

*Aug 21 11:40:00.450: %LINK-5-CHANGED: Interface GigabitEthernet1/0/9, changed state to administratively down

*Aug 21 11:40:00.451: %LINK-5-CHANGED: Interface GigabitEthernet1/0/10, changed state to administratively down

*Aug 21 11:40:00.453: %LINK-5-CHANGED: Interface GigabitEthernet1/0/11, changed state to administratively down

*Aug 21 11:40:00.454: %LINK-5-CHANGED: Interface GigabitEthernet1/0/12, changed state to administratively down

*Aug 21 11:40:00.456: %LINK-5-CHANGED: Interface GigabitEthernet1/0/13, changed state to administratively down

*Aug 21 11:40:00.458: %LINK-5-CHANGED: Interface GigabitEthernet1/0/14, changed state to administratively down

*Aug 21 11:40:00.459: %LINK-5-CHANGED: Interface GigabitEthernet1/0/15, changed state to administratively down

*Aug 21 11:40:00.461: %LINK-5-CHANGED: Interface GigabitEthernet1/0/16, changed state to administratively down

*Aug 21 11:40:00.462: %LINK-5-CHANGED: Interface GigabitEthernet1/0/17, changed state to administratively down

*Aug 21 11:40:00.464: %LINK-5-CHANGED: Interface GigabitEthernet1/0/18, changed state to administratively down

*Aug 21 11:40:00.466: %LINK-5-CHANGED: Interface GigabitEthernet1/0/19, changed state to administratively down

*Aug 21 11:40:00.467: %LINK-5-CHANGED: Interface GigabitEthernet1/0/20, changed state to administratively down

*Aug 21 11:40:00.469: %LINK-5-CHANGED: Interface GigabitEthernet1/0/21, changed state to administratively down

*Aug 21 11:40:00.470: %LINK-5-CHANGED: Interface GigabitEthernet1/0/22, changed state to administratively down

*Aug 21 11:40:00.472: %LINK-5-CHANGED: Interface GigabitEthernet1/0/23, changed state to administratively down

*Aug 21 11:40:00.474: %LINK-5-CHANGED: Interface GigabitEthernet1/0/24, changed state to administratively down

*Aug 21 11:40:00.475: %LINK-5-CHANGED: Interface GigabitEthernet1/0/25, changed state to administratively down

*Aug 21 11:40:00.477: %LINK-5-CHANGED: Interface GigabitEthernet1/0/26, changed state to administratively down

*Aug 21 11:40:00.478: %LINK-5-CHANGED: Interface GigabitEthernet1/0/27, changed state to administratively down

*Aug 21 11:40:00.479: %LINK-5-CHANGED: Interface GigabitEthernet1/0/28, changed state to administratively down

*Aug 21 11:40:01.385: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/6, changed state to down

S1(config-if-range)#

S1(config-if-range)#exit

S1(config)#do show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Gi1/0/1, Gi1/0/2, Gi1/0/3

Gi1/0/4, Gi1/0/5, Gi1/0/7

Gi1/0/8, Gi1/0/9, Gi1/0/10

Gi1/0/11, Gi1/0/12, Gi1/0/13

Gi1/0/14, Gi1/0/15, Gi1/0/16

Gi1/0/17, Gi1/0/18, Gi1/0/19

Gi1/0/20, Gi1/0/21, Gi1/0/22

Gi1/0/23, Gi1/0/24, Gi1/0/25

Gi1/0/26, Gi1/0/27, Gi1/0/28

7    London                           active    Gi1/0/6

8    Swansea                          active

9    Management                       active

10   Birmingham                       active

11   Native                           active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

S1(config)#

S1(config)#do write

Building configuration...

[OK]

S1(config)#

User Access Verification

Password:

Password:

R1>ena

Password:

R1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#do show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

Primary Secondary Type              Ports

------- --------- ----------------- ------------------------------------------

R1(config)#exit

R1#

*Aug 21 10:46:06.636: %SYS-5-CONFIG_I: Configured from console by console

R1#

S1(config)#

S1(config)#do show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Gi1/0/1, Gi1/0/2, Gi1/0/3

Gi1/0/4, Gi1/0/5, Gi1/0/7

Gi1/0/8, Gi1/0/9, Gi1/0/10

Gi1/0/11, Gi1/0/12, Gi1/0/13

Gi1/0/14, Gi1/0/15, Gi1/0/16

Gi1/0/17, Gi1/0/18, Gi1/0/19

Gi1/0/20, Gi1/0/21, Gi1/0/22

Gi1/0/23, Gi1/0/24, Gi1/0/25

Gi1/0/26, Gi1/0/27, Gi1/0/28

7    London                           active    Gi1/0/6

8    Swansea                          active

9    Management                       active

10   Birmingham                       active

11   Native                           active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

S1(config)#interface gi/0/1

^

% Invalid input detected at '^' marker.

S1(config)#interface gi1/0/1

S1(config-if)#sw

S1(config-if)#switchport tr

S1(config-if)#switchport trunk mode

^

% Invalid input detected at '^' marker.

S1(config-if)#

S1(config-if)#switchport trunk ?

allowed  Set allowed VLAN characteristics when interface is in trunking mode

native   Set trunking native characteristics when interface is in trunking

mode

pruning  Set pruning VLAN characteristics when interface is in trunking mode

S1(config-if)#switchport ?

access         Set access mode characteristics of the interface

autostate      Include or exclude this port from vlan link up calculation

block          Disable forwarding of unknown uni/multi cast addresses

host           Set port host

mode           Set trunking mode of the interface

nonegotiate    Device will not engage in negotiation protocol on this

interface

port-security  Security related command

priority       Set appliance 802.1p priority

private-vlan   Set the private VLAN configuration

protected      Configure an interface to be a protected port

trunk          Set trunking characteristics of the interface

voice          Voice appliance attributes

<cr>

S1(config-if)#switchport mode ?

access        Set trunking mode to ACCESS unconditionally

dynamic       Set trunking mode to dynamically negotiate access or trunk mode

private-vlan  Set private-vlan mode

trunk         Set trunking mode to TRUNK unconditionally

S1(config-if)#switchport mode tr

S1(config-if)#switchport mode trunk

S1(config-if)#swi

*Aug 21 11:52:26.605: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to downt

S1(config-if)#

*Aug 21 11:52:28.740: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan9, changed state to up

*Aug 21 11:52:29.670: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to up

S1(config-if)#

S1(config-if)#swi

S1(config-if)#switchport ?

access         Set access mode characteristics of the interface

autostate      Include or exclude this port from vlan link up calculation

block          Disable forwarding of unknown uni/multi cast addresses

host           Set port host

mode           Set trunking mode of the interface

nonegotiate    Device will not engage in negotiation protocol on this

interface

port-security  Security related command

priority       Set appliance 802.1p priority

private-vlan   Set the private VLAN configuration

protected      Configure an interface to be a protected port

trunk          Set trunking characteristics of the interface

voice          Voice appliance attributes

<cr>

S1(config-if)#switchport trunk native vlan ?

<1-4094>  VLAN ID of the native VLAN when this port is in trunking mode

S1(config-if)#switchport trunk native vlan 11

S1(config-if)#

*Aug 21 11:53:09.131: %SPANTREE-2-RECV_PVID_ERR: Received BPDU with inconsistent peer vlan id 1 on GigabitEthernet1/0/1 VLAN11.

*Aug 21 11:53:09.131: %SPANTREE-2-BLOCK_PVID_PEER: Blocking GigabitEthernet1/0/1 on VLAN0001. Inconsistent peer vlan.

*Aug 21 11:53:09.131: %SPANTREE-2-BLOCK_PVID_LOCAL: Blocking GigabitEthernet1/0/1 on VLAN0011. Inconsistent local vlan.

*Aug 21 11:53:31.253: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 11:54:28.380: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config-if)#exit

S1(config)#do show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Vlan1                  unassigned      YES NVRAM  administratively down down

Vlan9                  192.168.40.2    YES manual up                    up

GigabitEthernet1/0/1   unassigned      YES unset  up                    up

GigabitEthernet1/0/2   unassigned      YES unset  up                    up

GigabitEthernet1/0/3   unassigned      YES unset  administratively down down

GigabitEthernet1/0/4   unassigned      YES unset  administratively down down

GigabitEthernet1/0/5   unassigned      YES unset  down                  down

GigabitEthernet1/0/6   unassigned      YES unset  administratively down down

GigabitEthernet1/0/7   unassigned      YES unset  administratively down down

GigabitEthernet1/0/8   unassigned      YES unset  administratively down down

GigabitEthernet1/0/9   unassigned      YES unset  administratively down down

GigabitEthernet1/0/10  unassigned      YES unset  administratively down down

GigabitEthernet1/0/11  unassigned      YES unset  administratively down down

GigabitEthernet1/0/12  unassigned      YES unset  administratively down down

GigabitEthernet1/0/13  unassigned      YES unset  administratively down down

GigabitEthernet1/0/14  unassigned      YES unset  administratively down down

GigabitEthernet1/0/15  unassigned      YES unset  administratively down down

GigabitEthernet1/0/16  unassigned      YES unset  administratively down down

GigabitEthernet1/0/17  unassigned      YES unset  administratively down down

GigabitEthernet1/0/18  unassigned      YES unset  administratively down down

GigabitEthernet1/0/19  unassigned      YES unset  administratively down down

--More--

*Aug 21 11:55:24.138: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernetGigabitEthernet1/0/20  unassigned      YES unset  administratively down down

GigabitEthernet1/0/21  unassigned      YES unset  administratively down down

S1(config)#

S1(config)#sw

S1(config)#interface gi1/0/1

S1(config-if)#sw

S1(config-if)#switchport

*Aug 21 11:56:14.655: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).native

^

% Invalid input detected at '^' marker.

S1(config-if)#switchport trunk  native ?

vlan  Set native VLAN when interface is in trunking mode

S1(config-if)#switchport trunk  native vlan ?

<1-4094>  VLAN ID of the native VLAN when this port is in trunking mode

S1(config-if)#switchport trunk ?

allowed  Set allowed VLAN characteristics when interface is in trunking mode

native   Set trunking native characteristics when interface is in trunking

mode

pruning  Set pruning VLAN characteristics when interface is in trunking mode

S1(config-if)#switchport trunk native

% Incomplete command.

S1(config-if)#switchport trunk native ?

vlan  Set native VLAN when interface is in trunking mode

S1(config-if)#switchport trunk native 11 ?

% Unrecognized command

S1(config-if)#switchport trunk native 11?

% Unrecognized command

S1(config-if)#switchport trunk ?

allowed  Set allowed VLAN characteristics when interface is in trunking mode

native   Set trunking native characteristics when interface is in trunking

mode

pruning  Set pruning VLAN characteristics when interface is in trunking mode

S1(config-if)#switchport trunk

*Aug 21 11:57:10.488: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEinterface gi1/0/

% Incomplete command.

S1(config)#switchport trunk native ?

% Unrecognized command

S1(config)#switchport trunk native

^

% Invalid input detected at '^' marker.

S1(config)#switchport trunk native ?

% Unrecognized command

S1(config)#interfa gi1/0/1

S1(config-if)#swi

S1(config-if)#switchport trunk ?

allowed  Set allowed VLAN characteristics when interface is in trunking mode

native   Set trunking native characteristics when interface is in trunking

mode

pruning  Set pruning VLAN characteristics when interface is in trunking mode

S1(config-if)#switchport trunk native ?

vlan  Set native VLAN when interface is in trunking mode

S1(config-if)#switchport trunk native vlan ?

<1-4094>  VLAN ID of the native VLAN when this port is in trunking mode

S1(config-if)#switchport trunk native vlan

*Aug 21 11:58:00.608: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.

S1(config-if)#

S1(config-if)#exit

S1(config)#do show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Gi1/0/2, Gi1/0/3, Gi1/0/4

Gi1/0/5, Gi1/0/7, Gi1/0/8

Gi1/0/9, Gi1/0/10, Gi1/0/11

Gi1/0/12, Gi1/0/13, Gi1/0/14

Gi1/0/15, Gi1/0/16, Gi1/0/17

Gi1/0/18, Gi1/0/19, Gi1/0/20

Gi1/0/21, Gi1/0/22, Gi1/0/23

Gi1/0/24, Gi1/0/25, Gi1/0/26

Gi1/0/27, Gi1/0/28

7    London                           active    Gi1/0/6

8    Swansea                          active

9    Management                       active

10   Birmingham                       active

11   Native                           active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

S1(config)#

*Aug 21 11:58:53.524: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 11:59:42.522: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config)#

S1(config)#interfa gi1/0/1

S1(config-if)#switchport mode ?

access        Set trunking mode to ACCESS unconditionally

dynamic       Set trunking mode to dynamically negotiate access or trunk mode

private-vlan  Set private-vlan mode

trunk         Set trunking mode to TRUNK unconditionally

S1(config-if)#switchport mode trunk ?

<cr>

S1(config-if)#switchport mode trunk

S1(config-if)#swit?

switchport

S1(config-if)#swit

S1(config-if)#switchport ?

access         Set access mode characteristics of the interface

autostate      Include or exclude this port from vlan link up calculation

block          Disable forwarding of unknown uni/multi cast addresses

host           Set port host

mode           Set trunking mode of the interface

nonegotiate    Device will not engage in negotiation protocol on this

interface

port-security  Security related command

priority       Set appliance 802.1p priority

private-vlan   Set the private VLAN configuration

protected      Configure an interface to be a protected port

trunk          Set trunking characteristics of the interface

voice          Voice appliance attributes

<cr>

S1(config-if)#switchport

*Aug 21 12:00:31.523: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).access ?

vlan  Set VLAN when interface is in access mode

S1(config-if)#switchport access vlan ?

<1-4094>  VLAN ID of the VLAN when this port is in access mode

name      Set VLAN when interface is in access mode

S1(config-if)#switchport access vlan 11

S1(config-if)#

S1(config-if)#exit

S1(config)#do show vlan brief

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Gi1/0/2, Gi1/0/3, Gi1/0/4

Gi1/0/5, Gi1/0/7, Gi1/0/8

Gi1/0/9, Gi1/0/10, Gi1/0/11

Gi1/0/12, Gi1/0/13, Gi1/0/14

Gi1/0/15, Gi1/0/16, Gi1/0/17

Gi1/0/18, Gi1/0/19, Gi1/0/20

Gi1/0/21, Gi1/0/22, Gi1/0/23

Gi1/0/24, Gi1/0/25, Gi1/0/26

Gi1/0/27, Gi1/0/28

7    London                           active    Gi1/0/6

8    Swansea                          active

9    Management                       active

10   Birmingham                       active

11   Native                           active

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

S1(config)#

*Aug 21 12:01:26.395: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config)#

*Aug 21 12:02:17.696: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:03:12.451: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:04:05.631: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:05:01.218: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:05:49.556: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:06:42.795: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:07:39.154: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:08:36.760: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:09:27.303: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:10:20.081: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config)#

S1(config)#

*Aug 21 12:11:10.847: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config)#

S1(config)#interface range gi1/0/1-2

S1(config-if-range)#.

^

% Invalid input detected at '^' marker.

S1(config-if-range)#chaa

S1(config-if-range)#chann

S1(config-if-range)#channel-gro

S1(config-if-range)#channel-group 1 mode active

Creating a port-channel interface Port-channel 1

S1(config-if-range)#

*Aug 21 12:12:04.316: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:12:04.934: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Gi1/0/1 and will be suspended (trunk mode of Gi1/0/2 is access, Gi1/0/1 is trunk)

*Aug 21 12:12:04.940: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan9, changed state to down

*Aug 21 12:12:05.868: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to down

*Aug 21 12:12:05.932: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/2, changed state to downexit

S1(config)#

S1(config)#

*Aug 21 12:12:12.966: %EC-5-L3DONTBNDL2: Gi1/0/1 suspended: LACP currently not enabled on the remote port.

*Aug 21 12:14:00.479: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Gi1/0/1 and will be suspended (trunk mode of Gi1/0/2 is access, Gi1/0/1 is trunk)

*Aug 21 12:14:00.546: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Gi1/0/1 and will be suspended (trunk mode of Gi1/0/2 is access, Gi1/0/1 is trunk)

*Aug 21 12:14:04.540: %SPANTREE-2-RECV_PVID_ERR: Received BPDU with inconsistent peer vlan id 1 on Port-channel1 VLAN11.

*Aug 21 12:14:04.540: %SPANTREE-2-BLOCK_PVID_PEER: Blocking Port-channel1 on VLAN0001. Inconsistent peer vlan.

*Aug 21 12:14:04.540: %SPANTREE-2-BLOCK_PVID_LOCAL: Blocking Port-channel1 on VLAN0011. Inconsistent local vlan.

*Aug 21 12:14:04.543: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan9, changed state to up

*Aug 21 12:14:05.534: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to up

*Aug 21 12:14:05.536: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:06.528: %LINK-3-UPDOWN: Interface Port-channel1, changed state to up

*Aug 21 12:14:06.535: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:07.528: %LINEPROTO-5-UPDOWN: Line protocol on Interface Port-channel1, changed state to up

*Aug 21 12:14:07.536: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:08.534: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:09.536: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:10.539: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:11.536: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:12.534: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:14:13.536: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:15:10.888: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:16:05.482: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:16:54.873: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config)#

S1(config)#interface gi1/0/1

S1(config-if)#sw

S1(config-if)#switchport trunk mode

^

% Invalid input detected at '^' marker.

S1(config-if)#switchport trunk mo

*Aug 21 12:17:52.486: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1

^

% Invalid input detected at '^' marker.

S1(config-if)#switchport mode trunk

S1(config-if)#switchport trunk encapsulation dot1q

^

% Invalid input detected at '^' marker.

S1(config-if)#

*Aug 21 12:18:51.362: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config-if)#

S1(config-if)#?

Interface configuration commands:

aaa                 Authentication, Authorization and Accounting.

arp                 Set arp type (arpa, probe, snap) or timeout or log

options

auto                Configure Automation

bandwidth           Set bandwidth informational parameter

carrier-delay       Specify delay for interface transitions

cdp                 CDP interface subcommands

channel-group       Etherchannel/port bundling configuration

channel-protocol    Select the channel protocol (LACP, PAgP)

dampening           Enable event dampening

default             Set a command to its defaults

delay               Specify interface throughput delay

description         Interface specific description

down-when-looped    Force looped interface down

duplex              Configure duplex operation.

ethernet            Ethernet interface parameters

exit                Exit from interface configuration mode

fair-queue          Enable Fair Queuing on an Interface

flowcontrol         Configure flow operation.

help                Description of the interactive help system

history             Interface history histograms - 60 second, 60 minute and

72 hour

hold-queue          Set hold queue depth

ip                  Interface Internet Protocol config commands

ipv6                IPv6 interface subcommands

keepalive           Enable keepalive

lacp                LACP interface subcommands

link                Interface link related commands

lldp                LLDP interface subcommands

load-interval       Specify interval for load calculation for an interface

location            Interface location information

logging             Configure logging for interface

loopdetect          Configure loopdetect feature setting

mac                 MAC interface commands

macro               Command macro

mdix                Set Media Dependent Interface with Crossover

mls                 mls interface commands

neighbor            interface neighbor configuration mode commands

network-policy      Network Policy

nmsp                NMSP interface configuration

no                  Negate a command or set its defaults

pagp                PAgP interface subcommands

power               Power configuration

priority-queue      Priority Queue

random-detect       Enable Weighted Random Early Detection (WRED) on an

Interface

rep                 Resilient Ethernet Protocol characteristics

routing             Per-interface routing configuration

service-policy      Configure CPL Service Policy

sflow               Enable sflow on the interface

shutdown            Shutdown the selected interface

snmp                Modify SNMP interface parameters

source              Get config from another source

spanning-tree       Spanning Tree Subsystem

speed               Configure speed operation.

srr-queue           Configure shaped round-robin transmit queues

storm-control       storm configuration

subscriber          Subscriber inactivity timeout value.

switchport          Set switching mode characteristics

timeout             Define timeout values for this interface

topology            Configure routing topology on the interface

transmit-interface  Assign a transmit interface to a receive-only interface

tx-ring-limit       Configure PA level transmit ring limit

udld                Configure UDLD enabled or disabled and ignore global UDLD

setting

vtp                 Enable VTP on this interface

S1(config-if)#swit

S1(config-if)#switchport trunk ?

allowed  Set allowed VLAN characteristics when interface is in trunking mode

native   Set trunking native characteristics when interface is in trunking

mode

pruning  Set pruning VLAN characteristics when interface is in trunking mode

S1(config-if)#switchport trunk native ?

vlan  Set native VLAN when interface is in trunking mode

S1(config-if)#switchport trunk native vlan ?

<1-4094>  VLAN ID of the native VLAN when this port is in trunking mode

S1(config-if)#switchport trunk native vlan 11

S1(config-if)#

S1(config-if)#

*Aug 21 12:19:42.934: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config-if)#switchport trunk al

S1(config-if)#switchport trunk allowed ?

vlan  Set allowed VLANs when interface is in trunking mode

S1(config-if)#switchport trunk allowed vlan ?

WORD    VLAN IDs of the allowed VLANs when this port is in trunking mode

add     add VLANs to the current list

all     all VLANs

except  all VLANs except the following

none    no VLANs

remove  remove VLANs from the current list

S1(config-if)#switchport trunk allowed vlan 7,8,9,10,11

S1(config-if)#

*Aug 21 12:20:18.350: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Po1 and will be suspended (access vlan of Gi1/0/2 is 1, Po1 is 11)

*Aug 21 12:20:18.350: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Po1 and will be suspended (access vlan of Gi1/0/2 is 1, Po1 is 11)

*Aug 21 12:20:18.357: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan9, changed state to down

*Aug 21 12:20:19.345: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to down

*Aug 21

S1(config-if)#12:20:19.346: %LINEPROTO-5-UPDOWN: Line protocol on Interface Port-channel1, changed state to down

*Aug 21 12:20:20.350: %LINK-3-UPDOWN: Interface Port-channel1, changed state to down

*Aug 21 12:20:26.057: %SPANTREE-2-RECV_PVID_ERR: Received BPDU with inconsistent peer vlan id 1 on Port-channel1 VLAN11.

*Aug 21 12:20:26.057: %SPANTREE-2-BLOCK_PVID_LOCAL: Blocking Port-channel1 on VLAN0011. Inconsistent local vlan.

*Aug 21 12:20:26.060: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan9, changed state to up

*Aug 21 12:20:27.047: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet1/0/1, changed state to up

*Aug 21 12:20:27.054: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:28.045: %LINK-3-UPDOWN: Interface Port-channel1, changed state to up

*Aug 21 12:20:28.052: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:29.045: %LINEPROTO-5-UPDOWN: Line protocol on Interface Port-channel1, changed state to up

*Aug 21 12:20:29.053: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:30.052: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:31.051: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:32.052: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:33.051: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:34.052: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:20:35.051: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

*Aug 21 12:21:30.906: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

S1(config-if)#

*Aug 21 12:22:19.494: %CDP-4-NATIVE_VLAN_MISMATCH: Native VLAN mismatch discovered on GigabitEthernet1/0/1 (11), with S2.swre-lab.com GigabitEthernet1/0/1 (1).

User Access Verification

Password:

Password:

Password:

% Bad passwords

R1 con0 is now available

Press RETURN to get started.

User Access Verification

Password:

R1>ena

Password:

Password:

R1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#

R1(config)#

R1(config)#ip dhcp-exclude address

^

% Invalid input detected at '^' marker.

R1(config)#ip dhcp ?

aaa                        Configure aaa attributes

auto-broadcast             Configure auto broadcast feature

binding                    DHCP address bindings

bootp                      BOOTP specific configuration

class                      Configure DHCP classes

compatibility              Compatibility configuration

conflict                   DHCP address conflict parameters

database                   Configure DHCP database agents

drop-inform                Drop inform message if enabled

event-notification         DHCP event notifications

excluded-address           Prevent DHCP from assigning certain addresses

global-options             Configure global DHCP options

limit                      Limit DHCP Lease

limited-broadcast-address  Use all 1's broadcast address

monitor                    Monitor DHCP packets

ping                       Specify ping parameters used by DHCP

pool                       Configure DHCP address pools

relay                      DHCP relay agent parameters

remember                   Remember released bindings

route                      Specify the type of routes for clients on

unnumbered interfaces

smart-relay                Enable Smart Relay feature

snooping                   DHCP Snooping

subscriber-id              Global subscriber-id configuration

support                    Configure support for certain features

update                     Configure dynamic updates

use                        Configure use of certain parameters during

allocation

R1(config)#ip dhcp my_pool

^

% Invalid input detected at '^' marker.

R1(config)#ip dhcp pool my_pool

R1(dhcp-config)#network 192.168.20.1 ?

/nn or A.B.C.D  Network mask or prefix length

<cr>            <cr>

R1(dhcp-config)#network 192.168.20.1

R1(dhcp-config)#network 192.168.30.1

R1(dhcp-config)#network 192.168.40.1

R1(dhcp-config)#ip defa

R1(dhcp-config)#ip default-router ?

% Unrecognized command

R1(dhcp-config)#?

DHCP pool configuration commands:

accounting           Send Accounting Start/Stop messages

address              Configure a reserved address

authorization        Obtain address allocation information from AAA

bootfile             Boot file name

class                Specify a DHCP class

client-identifier    Client identifier

client-name          Client name

default-router       Default routers

dns-server           DNS servers

domain-name          Domain name

exit                 Exit from DHCP pool configuration mode

hardware-address     Client hardware address

host                 Client IP address and mask

import               Programatically importing DHCP option parameters

lease                Address lease time

netbios-name-server  NetBIOS (WINS) name servers

netbios-node-type    NetBIOS node type

network              Network number and mask

next-server          Next server in boot process

no                   Negate a command or set its defaults

option               Raw DHCP options

origin               Configure the origin of the pool

relay                Function as a DHCP relay

remember             Remember released bindings

renew                Configure renewal policy

reserved-only        Only allocate reserved addresses

server               Configure the server ID option value

subnet               Subnet allocation commands

update               Dynamic updates

utilization          Configure various utilization parameters

vrf                  Associate this pool with a VRF

R1(dhcp-config)#defa

R1(dhcp-config)#default-router 192.168.40.1

R1(dhcp-config)#dns

R1(dhcp-config)#dns-server 8.8.8.8 ?

Hostname or A.B.C.D  Server's name or IP address

<cr>                 <cr>

R1(dhcp-config)#dns-server 8.8.8.8

R1(dhcp-config)#doma

R1(dhcp-config)#domain-name Google.com

R1(dhcp-config)#lease 7

R1(dhcp-config)#

R1(dhcp-config)#exit

R1(config)#

*Aug 21 12:41:32.284: %EC-5-L3DONTBNDL2: Gi1/0/1 suspended: LACP currently not enabled on the remote port.

User Access Verification

Password:

S1>ena

Password:

S1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#

S1(config)#inter

S1(config)#interface gi1/0/6

S1(config-if)#swi

S1(config-if)#switchport port

S1(config-if)#switchport port-security max

S1(config-if)#switchport port-security maximum 3

S1(config-if)#exit

S1(config)#do write

Building configuration...

[OK]

S1(config)#inter

S1(config)#interface range  gi1/0/1-2

S1(config-if-range)#sw

S1(config-if-range)#switchport trunk

S1(config-if-range)#switchport trunk native vlan 11

S1(config-if-range)#

*Aug 21 12:48:10.126: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Po1 and will be suspended (access vlan of Gi1/0/2 is 1, Po1 is 11)

*Aug 21 12:48:10.126: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Po1 and will be suspended (access vlan of Gi1/0/2 is 1, Po1 is 11)

*Aug 21 12:48:17.898: %EC-5-L3DONTBNDL2: Gi1/0/1 suspended: LACP currently not enabled on the remote port.switchport trunk allowed ?

vlan  Set allowed VLANs when interface is in trunking mode

S1(config-if-range)#switchport trunk allowed ?

vlan  Set allowed VLANs when interface is in trunking mode

S1(config-if-range)#switchport trunk allowed vlan 7,8,9,10,11

S1(config-if-range)#

*Aug 21 12:48:42.724: %EC-5-CANNOT_BUNDLE2: Gi1/0/2 is not compatible with Po1 and will be suspended (access vlan of Gi1/0/2 is 1, Po1 is 11)

*Aug 21 12:48:50.610: %EC-5-L3DONTBNDL2: Gi1/0/1 suspended: LACP currently not enabled on the remote port.

*Aug 21 12:53:03.007: %EC-5-L3DONTBNDL2: Gi1/0/1 suspended: LACP currently not enabled on the remote port.

*Aug 21 12:53:05.994: %EC-5-L3DONTBNDL2: Gi1/0/2 suspended: LACP currently not enabled on the remote port.

S1 con0 is now available

Press RETURN to get started.

*Aug 21 13:00:11.112: %SYS-5-CONFIG_I: Configured from console by console

User Access Verification

Password:

S1>ena

Password:

Password:

S1#conf term

Enter configuration commands, one per line.  End with CNTL/Z.

S1(config)#interface vlan 7

S1(config-if)#1

*Aug 21 13:01:00.122: %LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan7, changed state to dowip add

S1(config-if)#ip address 192.168.20.2 255.255.255.0

S1(config-if)#no shut

S1(config-if)#no shutdown

S1(config-if)#

S1(config-if)#interface vlan 8

S1(config-if)#ip address 192.168.20.2 255.255.255.0

*Aug 21 13:01:22.337: %LINEPROTO-5-UPDOWN: Line protocol on Inp address 192.168.30.2 255.255.255.0

S1(config-if)#no shutdown

S1(config-if)#

S1(config-if)#interface vlan 9

S1(config-if)#ip address 192.168.40.2 255.255.255.0

S1(config-if)#no shutdown

S1(config-if)#

S1(config-if)#exit

S1(config)#

S1(config)#interfa gi1/0/1

S1(config-if)#sw

S1(config-if)#switchport m

S1(config-if)#switchport mode t

S1(config-if)#switchport mode trunk

S1(config-if)#s

S1(config-if)#sw

S1(config-if)#switchport tr

S1(config-if)#switchport trunk native vlan 11

S1(config-if)#

S1(config-if)#switchport trunk allow

S1(config-if)#switchport trunk allowed vlan 7,8,9,10,11

S1(config-if)#

S1(config-if)#exit

S1(config)#do write

Building configuration...

[OK]

S1(config)#

S1(config)#inte

S1(config)#interface gi1/0/2

S1(config-if)#switchport mode trunk

S1(config-if)#switchport trunk native vlan 11

S1(config-if)#switchport trunk allowed vlan 7,8,9,10,11

S1(config-if)#exit

S1(config)#do write

Building configuration...

[OK]

S1(config)#

S1(config)#

S1(config)#

S1(config)#end

S1#

S1#show

*Aug 21 13:16:23.772: %SYS-5-CONFIG_I: Configured from console by con

S1#

S1#show run

S1#show running-config

Building configuration...

Current configuration : 4465 bytes

!

! Last configuration change at 13:16:23 UTC Wed Aug 21 2024

!

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

!

hostname S1

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$xXvoADXyT.K4eJ$HgBhlbsOgNbxdd/lF9ShWA8QggT8NSUw3MTEhCkfu0w

!

username cisco secret 9 $9$f8OZZv7OS1SmQD$Aka52NM3k758RfB0/3z8hp7BG2gZ5B3fU65ETt83FFc

no aaa new-model

switch 1 provision c1000-24t-4g-l

system mtu routing 1500

!

!

no ip domain-lookup

ip domain-name swre-lab.com

!

!

!

!

!

!

crypto pki trustpoint TP-self-signed-1877477248

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1877477248

revocation-check none

rsakeypair TP-self-signed-1877477248

!

!

crypto pki certificate chain TP-self-signed-1877477248

certificate self-signed 01

3082022B 30820194 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 31383737 34373732 3438301E 170D3234 30383231 31303236

32355A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D31 38373734

37373234 3830819F 300D0609 2A864886 F70D0101 01050003 818D0030 81890281

8100C618 5BB0DAFE D2AA3C79 22B86DED 95A07A9C 694CD83C 52351104 60F655D7

71B201A5 BE2F231C 1B81D4C7 CFBC9C74 F5D5939A 6A4EBC13 727273B7 C2407993

125256D0 EDB5F144 D7C0C541 D1403600 4C9EE0D9 437DC610 AD7DD500 62A62EB9

AB0C451F 0D4D0C0F 37A13A8C DD273945 8A6568CC 902C4E23 0B66B736 F2A6DE4B

31D90203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF 301F0603

551D2304 18301680 14571337 76A72C30 3A6E0D10 5F97B01E EE04FEFB CE301D06

03551D0E 04160414 57133776 A72C303A 6E0D105F 97B01EEE 04FEFBCE 300D0609

2A864886 F70D0101 05050003 818100BD F3CDA9FB 11953CF9 BC925511 8ADA5B31

BFECC7DD 44869044 9BB6C8D6 248B9F21 5893435C C441DCBB 6B6CD3C3 14C4F6ED

559409E2 37B36F87 50159A57 0DF77EF7 3F1FC0B8 06121061 2BEC14FB 64E5731A

036A18DD 048617C5 E47286A0 C33602A9 373F0A58 9FB06A0F C67003C2 47B4A12E

8BAF6BD3 91939A4E 653BC6BC 17C5D4

quit

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

!

!

!

!

!

interface Port-channel1

switchport access vlan 11

switchport trunk allowed vlan 7-11

switchport trunk native vlan 11

switchport mode trunk

!

interface GigabitEthernet1/0/1

switchport access vlan 11

switchport trunk allowed vlan 7-11

switchport trunk native vlan 11

switchport mode trunk

channel-group 1 mode active

!

interface GigabitEthernet1/0/2

switchport trunk allowed vlan 7-11

switchport trunk native vlan 11

switchport mode trunk

channel-group 1 mode active

!

interface GigabitEthernet1/0/3

shutdown

!

interface GigabitEthernet1/0/4

shutdown

!

interface GigabitEthernet1/0/5

!

interface GigabitEthernet1/0/6

switchport access vlan 7

switchport mode access

switchport port-security maximum 3

shutdown

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

shutdown

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

shutdown

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

shutdown

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/0/25

shutdown

!

interface GigabitEthernet1/0/26

shutdown

!

interface GigabitEthernet1/0/27

shutdown

!

interface GigabitEthernet1/0/28

shutdown

!

interface Vlan1

no ip address

shutdown

!

interface Vlan7

ip address 192.168.20.2 255.255.255.0

!

interface Vlan8

ip address 192.168.30.2 255.255.255.0

!

interface Vlan9

ip address 192.168.40.2 255.255.255.0

!

ip default-gateway 192.168.40.1

ip http server

ip http secure-server

!

!

!

!

line con 0

password 7 045802150C2E1D1C5A

login

line vty 0 4

password 7 045802150C2E1D1C5A

login local

transport input none

line vty 5 15

password 7 104D000A061843595F

login local

transport input none

!

end

S1#show ip inter

S1#show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Vlan1                  unassigned      YES NVRAM  administratively down down

Vlan7                  192.168.20.2    YES manual up                    down

Vlan8                  192.168.30.2    YES manual up                    down

Vlan9                  192.168.40.2    YES manual up                    down

GigabitEthernet1/0/1   unassigned      YES unset  up                    down

GigabitEthernet1/0/2   unassigned      YES unset  up                    down

GigabitEthernet1/0/3   unassigned      YES unset  administratively down down

GigabitEthernet1/0/4   unassigned      YES unset  administratively down down

GigabitEthernet1/0/5   unassigned      YES unset  down                  down

GigabitEthernet1/0/6   unassigned      YES unset  administratively down down

GigabitEthernet1/0/7   unassigned      YES unset  administratively down down

GigabitEthernet1/0/8   unassigned      YES unset  administratively down down

GigabitEthernet1/0/9   unassigned      YES unset  administratively down down

GigabitEthernet1/0/10  unassigned      YES unset  administratively down down

GigabitEthernet1/0/11  unassigned      YES unset  administratively down down

GigabitEthernet1/0/12  unassigned      YES unset  administratively down down

GigabitEthernet1/0/13  unassigned      YES unset  administratively down down

GigabitEthernet1/0/14  unassigned      YES unset  administratively down down

GigabitEthernet1/0/15  unassigned      YES unset  administratively down down

GigabitEthernet1/0/16  unassigned      YES unset  administratively down down

GigabitEthernet1/0/17  unassigned      YES unset  administratively down down

GigabitEthernet1/0/18  unassigned      YES unset  administratively down down

GigabitEthernet1/0/19  unassigned      YES unset  administratively down down

GigabitEthernet1/0/20  unassigned      YES unset  administratively down down

GigabitEthernet1/0/21  unassigned      YES unset  administratively down down

GigabitEthernet1/0/22  unassigned      YES unset  administratively down down

GigabitEthernet1/0/23  unassigned      YES unset  administratively down down

GigabitEthernet1/0/24  unassigned      YES unset  administratively down down

GigabitEthernet1/0/25  unassigned      YES unset  administratively down down

GigabitEthernet1/0/26  unassigned      YES unset  administratively down down

GigabitEthernet1/0/27  unassigned      YES unset  administratively down down

GigabitEthernet1/0/28  unassigned      YES unset  administratively down down

Port-channel1          unassigned      YES unset  down                  down

S1#show vlan

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Gi1/0/2, Gi1/0/3, Gi1/0/4

Gi1/0/5, Gi1/0/7, Gi1/0/8

Gi1/0/9, Gi1/0/10, Gi1/0/11

Gi1/0/12, Gi1/0/13, Gi1/0/14

Gi1/0/15, Gi1/0/16, Gi1/0/17

Gi1/0/18, Gi1/0/19, Gi1/0/20

Gi1/0/21, Gi1/0/22, Gi1/0/23

Gi1/0/24, Gi1/0/25, Gi1/0/26

Gi1/0/27, Gi1/0/28

7    London                           active    Gi1/0/6

8    Swansea                          active

9    Management                       active

10   Birmingham                       active

11   Native                           active    Gi1/0/1

1002 fddi-default                     act/unsup

1003 token-ring-default               act/unsup

1004 fddinet-default                  act/unsup

1005 trnet-default                    act/unsup

VLAN Type  SAID       MTU   Parent RingNo BridgeNo Stp  BrdgMode Trans1 Trans2

---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------

1    enet  100001     1500  -      -      -        -    -        0      0

7    enet  100007     1500  -      -      -        -    -        0      0

8    enet  100008     1500  -      -      -        -    -        0      0

9    enet  100009     1500  -      -      -        -    -        0      0

10   enet  100010     1500  -      -      -        -    -        0      0

11   enet  100011     1500  -      -      -        -    -        0      0

1002 fddi  101002     1500  -      -      -        -    -        0      0

1003 tr    101003     1500  -      -      -        -    -        0      0

1004 fdnet 101004     1500  -      -      -        ieee -        0      0

1005 trnet 101005     1500  -      -      -        ibm  -        0      0

Primary Secondary Type              Ports

------- --------- ----------------- ------------------------------------------

S1#

User Access Verification

Password:

R1>ena

Password:

R1#show run

R1#show running-config

Building configuration...

Current configuration : 6765 bytes

!

! Last configuration change at 11:46:16 UTC Wed Aug 21 2024

!

version 17.9

service timestamps debug datetime msec

service timestamps log datetime msec

service password-encryption

! Call-home is enabled by Smart-Licensing.

service call-home

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname R1

!

boot-start-marker

boot-end-marker

!

!

no aaa new-model

!

!

!

!

!

!

!

no ip domain lookup

ip domain name swre-lab.com

!

ip dhcp pool my_pool

network 192.168.40.0 255.255.255.0

default-router 192.168.40.1

dns-server 8.8.8.8

domain-name Google.com

lease 7

!

!

!

login on-success log

!

!

!

!

!

!

!

subscriber templating

vtp version 1

multilink bundle-name authenticated

!

!

!

crypto pki trustpoint TP-self-signed-90536024

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-90536024

revocation-check none

rsakeypair TP-self-signed-90536024

!

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

!

!

crypto pki certificate chain TP-self-signed-90536024

certificate self-signed 01

3082032C 30820214 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

2F312D30 2B060355 04031324 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 39303533 36303234 301E170D 32343038 32313038 34373135

5A170D33 34303832 31303834 3731355A 302F312D 302B0603 55040313 24494F53

2D53656C 662D5369 676E6564 2D436572 74696669 63617465 2D393035 33363032

34308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201 0A028201

0100AFA9 A66E81E5 464C140F 96EF0582 0054443E 097D6403 1DE87A7E A9190B14

11573960 101A7B7D B9D1CB5A C17EF704 D205CF27 F029CB86 CBC376E9 F28FBDDC

7C637F8F 376FE4AC 952FA026 53921CD2 4734B4F6 E2F20CAA 67EEAF5C F7070A1A

2AEF859B 2968FF2B 6F9470E9 CC26FF6B B78823B1 5AF9DC11 34D930E4 6E9932B5

DC32E6E2 5FEC7675 9C893E8F 58E2767F DEA76E5C A64DAB6A 20978DF6 739ED864

44EE97D9 1340AD09 6028E85F 880EFB8C 04E22AD2 9280B2D4 38DF8EAB 8436937B

AB3312C8 D5E1DAB4 7C6627BA 6362D4A5 A867D519 0B0B37AC 6D696B12 CC080789

254DF03A FF54148C 396B6F7F 1E04F91B 56E6A662 A5ACD2A9 B6E8DA72 B485A7FD

A2DD0203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF 301F0603

551D2304 18301680 143BB2D2 A22EFA5B F56BA133 08A53E28 DE48CC28 77301D06

03551D0E 04160414 3BB2D2A2 2EFA5BF5 6BA13308 A53E28DE 48CC2877 300D0609

2A864886 F70D0101 05050003 82010100 A8AD36FA C4CF17F4 5040430F 2C4B8C60

0268F484 76F9BB49 46408BE2 1AF7924C 5F263FA0 B7DB747E 4D1B1ABA D59866C1

98A90888 223387E6 304BD1FD 26FBAB33 06BFE9C4 B19AD22D 734FA52A AEB7A626

00BC6DCA 84721B42 E2B5C97E 146FB2F7 4B90D9E0 D752F087 EC6F63CC 688DA18A

90C20B27 A4EC4B79 5D4FD6C1 F8E08722 2042681A BBE8F2C9 B4F790AA A7A3C32F

ACF16427 6538C11D CBD48E07 00EA7E93 C32778B1 7EB0500D 9CB76C6F 95778648

CB3E1BE5 056B6E43 59590021 65174A94 B31F36F7 541E88F2 DFF1D175 481771FD

C47B1EE4 570262CD 16DF309F CD71B7A1 A7369E5B FEB47264 6B13DD41 98CD5BBD

AC6471A9 C63B3E9B F49FDFB1 3041A3BE

quit

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

30820321 30820209 A0030201 02020101 300D0609 2A864886 F70D0101 0B050030

32310E30 0C060355 040A1305 43697363 6F312030 1E060355 04031317 43697363

6F204C69 63656E73 696E6720 526F6F74 20434130 1E170D31 33303533 30313934

3834375A 170D3338 30353330 31393438 34375A30 32310E30 0C060355 040A1305

43697363 6F312030 1E060355 04031317 43697363 6F204C69 63656E73 696E6720

526F6F74 20434130 82012230 0D06092A 864886F7 0D010101 05000382 010F0030

82010A02 82010100 A6BCBD96 131E05F7 145EA72C 2CD686E6 17222EA1 F1EFF64D

CBB4C798 212AA147 C655D8D7 9471380D 8711441E 1AAF071A 9CAE6388 8A38E520

1C394D78 462EF239 C659F715 B98C0A59 5BBB5CBD 0CFEBEA3 700A8BF7 D8F256EE

4AA4E80D DB6FD1C9 60B1FD18 FFC69C96 6FA68957 A2617DE7 104FDC5F EA2956AC

7390A3EB 2B5436AD C847A2C5 DAB553EB 69A9A535 58E9F3E3 C0BD23CF 58BD7188

68E69491 20F320E7 948E71D7 AE3BCC84 F10684C7 4BC8E00F 539BA42B 42C68BB7

C7479096 B4CB2D62 EA2F505D C7B062A4 6811D95B E8250FC4 5D5D5FB8 8F27D191

C55F0D76 61F9A4CD 3D992327 A8BB03BD 4E6D7069 7CBADF8B DF5F4368 95135E44

DFC7C6CF 04DD7FD1 02030100 01A34230 40300E06 03551D0F 0101FF04 04030201

06300F06 03551D13 0101FF04 05300301 01FF301D 0603551D 0E041604 1449DC85

4B3D31E5 1B3E6A17 606AF333 3D3B4C73 E8300D06 092A8648 86F70D01 010B0500

03820101 00507F24 D3932A66 86025D9F E838AE5C 6D4DF6B0 49631C78 240DA905

604EDCDE FF4FED2B 77FC460E CD636FDB DD44681E 3A5673AB 9093D3B1 6C9E3D8B

D98987BF E40CBD9E 1AECA0C2 2189BB5C 8FA85686 CD98B646 5575B146 8DFC66A8

467A3DF4 4D565700 6ADF0F0D CF835015 3C04FF7C 21E878AC 11BA9CD2 55A9232C

7CA7B7E6 C1AF74F6 152E99B7 B1FCF9BB E973DE7F 5BDDEB86 C71E3B49 1765308B

5FB0DA06 B92AFE7F 494E8A9E 07B85737 F3A58BE1 1A48A229 C37C1E69 39F08678

80DDCD16 D6BACECA EEBC7CF9 8428787B 35202CDC 60E4616A B623CDBD 230E3AFB

418616A9 4093E049 4D10AB75 27E86F73 932E35B5 8862FDAE 0275156F 719BB2F0

D697DF7F 28

quit

!

!

license udi pid C8200L-1N-4T sn FCZ2818R0N0

memory free low-watermark processor 62972

!

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

enable secret 9 $9$BBQX.6ZUm0/SyU$CSTCkmK7K8UrGAQVdElScJDGshUYArbv7ywQhhiOqnk

!

username cisco secret 9 $9$Br.nUoILTclkj.$Vey59xFWHPZ6JQEis3mf8vVL4F0Yshthw5haa0mSH9s

!

redundancy

mode none

!

!

!

!

!

!

!

!

interface Loopback0

ip address 209.165.201.1 255.255.255.0

!

interface GigabitEthernet0/0/0

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet0/0/1

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet0/0/1.7

encapsulation dot1Q 7

ip address 192.168.20.1 255.255.255.0

!

interface GigabitEthernet0/0/1.8

encapsulation dot1Q 8

ip address 192.168.30.1 255.255.255.0

!

interface GigabitEthernet0/0/1.9

encapsulation dot1Q 9

ip address 192.168.40.1 255.255.255.0

!

interface GigabitEthernet0/0/1.11

!

interface GigabitEthernet0/0/2

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet0/0/3

no ip address

shutdown

negotiation auto

!

interface Serial0/1/0

!

interface Serial0/1/1

!

ip http server

ip http authentication local

ip http secure-server

ip forward-protocol nd

!

!

!

!

!

!

control-plane

!

!

line con 0

password 7 110A1016141D5A5E57

login

stopbits 1

line aux 0

line vty 0 4

password 7 070C285F4D06485744

login local

transport input ssh

line vty 5 15

password 7 070C285F4D06485744

login local

transport input ssh

!

call-home

! If contact email address in call-home is configured as sch-smart-licensing@cisco.com

! the email address configured in Cisco Smart License Portal will be used as contact email address to send SCH notifications.

contact-email-addr sch-smart-licensing@cisco.com

profile "CiscoTAC-1"

active

destination transport-method http

!

!

!

!

!

!

end

R1#show ip inter

R1#show ip interface brief

Any interface listed with OK? value "NO" does not have a valid configuration

Interface              IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0/0   unassigned      YES unset  administratively down down

GigabitEthernet0/0/1   unassigned      YES unset  administratively down down

GigabitEthernet0/0/1.7 192.168.20.1    YES manual administratively down down

GigabitEthernet0/0/1.8 192.168.30.1    YES manual administratively down down

GigabitEthernet0/0/1.9 192.168.40.1    YES manual administratively down down

Gi0/0/1.11             unassigned      YES unset  administratively down down

GigabitEthernet0/0/2   unassigned      YES unset  administratively down down

GigabitEthernet0/0/3   unassigned      YES unset  administratively down down

Serial0/1/0            unassigned      NO  unset  down                  down

Serial0/1/1            unassigned      NO  unset  down                  down

Loopback0              209.165.201.1   YES manual up                    up

R1#

Commands on S2:

S2>ena

Password:

Password:

S2#show run

S2#show running-config

S2#show running-config

Building configuration...

Current configuration : 2700 bytes

!

! Last configuration change at 12:31:40 UTC Wed Aug 21 2024

!

version 15.2

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname S2

!

boot-start-marker

boot-end-marker

!

enable secret 9 $9$tIhBrcoOMQBIEJ$r3WQRJW8JU3sWG/gqgu2dlen3bEfqDgw/VtGG.ZWbJw

!

username cisco secret 9 $9$jmam0.Q3UkQY2w$NUgUXO3zn2gv0jmEkArDDaJA/UIWtf3mKQtUfE                                                                                                                                                             xYslA

no aaa new-model

switch 1 provision c1000-24t-4g-l

system mtu routing 1500

!

!

no ip domain-lookup

ip domain-name swre-lab.com

!

!

!

!

!

!

!

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

vlan internal allocation policy ascending

!

!

!

!

!

!

interface Port-channel1

switchport mode trunk

!

interface GigabitEthernet1/0/1

switchport trunk allowed vlan 7-11

switchport trunk native vlan 11

switchport mode trunk

channel-group 1 mode active

!

interface GigabitEthernet1/0/2

switchport trunk allowed vlan 7-11

switchport trunk native vlan 11

switchport mode trunk

channel-group 1 mode active

!

interface GigabitEthernet1/0/3

shutdown

!

interface GigabitEthernet1/0/4

shutdown

!

interface GigabitEthernet1/0/5

shutdown

!

interface GigabitEthernet1/0/6

shutdown

!

interface GigabitEthernet1/0/7

shutdown

!

interface GigabitEthernet1/0/8

shutdown

!

interface GigabitEthernet1/0/9

shutdown

!

interface GigabitEthernet1/0/10

shutdown

!

interface GigabitEthernet1/0/11

shutdown

!

interface GigabitEthernet1/0/12

shutdown

!

interface GigabitEthernet1/0/13

shutdown

!

interface GigabitEthernet1/0/14

shutdown

!

interface GigabitEthernet1/0/15

shutdown

!

interface GigabitEthernet1/0/16

shutdown

!

interface GigabitEthernet1/0/17

shutdown

!

interface GigabitEthernet1/0/18

switchport access vlan 8

switchport mode access

switchport port-security maximum 3

!

interface GigabitEthernet1/0/19

shutdown

!

interface GigabitEthernet1/0/20

shutdown

!

interface GigabitEthernet1/0/21

shutdown

!

interface GigabitEthernet1/0/22

shutdown

!

interface GigabitEthernet1/0/23

shutdown

!

interface GigabitEthernet1/0/24

shutdown

!

interface GigabitEthernet1/0/25

shutdown

!

interface GigabitEthernet1/0/26

shutdown

!

interface GigabitEthernet1/0/27

shutdown

!

interface GigabitEthernet1/0/28

shutdown

!

interface Vlan1

no ip address

!

interface Vlan7

ip address 192.168.20.2 255.255.255.0

!

interface Vlan8

ip address 192.168.30.2 255.255.255.0

!

interface Vlan9

ip address 192.168.40.2 255.255.255.0

!

ip default-gateway 192.168.40.1

ip http server

ip http secure-server

!

!

!

!

line con 0

password cisco123

line vty 0 4

password cisco123

login local

transport input none

line vty 5 15

password cisco123

login local

transport input none

!

end

S2#show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Vlan1                  unassigned      YES unset  up                    down

Vlan7                  192.168.20.2    YES manual up                    down

Vlan8                  192.168.30.2    YES manual up                    up

Vlan9                  192.168.40.2    YES manual up                    down

GigabitEthernet1/0/1   unassigned      YES unset  up                    down

GigabitEthernet1/0/2   unassigned      YES unset  up                    down

GigabitEthernet1/0/3   unassigned      YES unset  administratively down down

GigabitEthernet1/0/4   unassigned      YES unset  administratively down down

GigabitEthernet1/0/5   unassigned      YES unset  administratively down down

GigabitEthernet1/0/6   unassigned      YES unset  administratively down down

GigabitEthernet1/0/7   unassigned      YES unset  administratively down down

GigabitEthernet1/0/8   unassigned      YES unset  administratively down down

GigabitEthernet1/0/9   unassigned      YES unset  administratively down down

GigabitEthernet1/0/10  unassigned      YES unset  administratively down down

GigabitEthernet1/0/11  unassigned      YES unset  administratively down down

GigabitEthernet1/0/12  unassigned      YES unset  administratively down down

GigabitEthernet1/0/13  unassigned      YES unset  administratively down down

GigabitEthernet1/0/14  unassigned      YES unset  administratively down down

GigabitEthernet1/0/15  unassigned      YES unset  administratively down down

GigabitEthernet1/0/16  unassigned      YES unset  administratively down down

GigabitEthernet1/0/17  unassigned      YES unset  administratively down down

GigabitEthernet1/0/18  unassigned      YES unset  up                    up

GigabitEthernet1/0/19  unassigned      YES unset  administratively down down

GigabitEthernet1/0/20  unassigned      YES unset  administratively down down

GigabitEthernet1/0/21  unassigned      YES unset  administratively down down

GigabitEthernet1/0/22  unassigned      YES unset  administratively down down

GigabitEthernet1/0/23  unassigned      YES unset  administratively down down

GigabitEthernet1/0/24  unassigned      YES unset  administratively down down

GigabitEthernet1/0/25  unassigned      YES unset  administratively down down

GigabitEthernet1/0/26  unassigned      YES unset  administratively down down

GigabitEthernet1/0/27  unassigned      YES unset  administratively down down

GigabitEthernet1/0/28  unassigned      YES unset  administratively down down

Port-channel1          unassigned      YES unset  down                  down

S2#