Router 1

Router>enable

Router#config t

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#hostname R1

R1(config)#no ip domain-lookup

R1(config)#line console 0

R1(config-line)#password cisco

R1(config-line)#login

R1(config-line)#logging synchronous

R1(config-line)#line aux 0

R1(config-line)#password cisco

R1(config-line)#login

R1(config-line)#

R1#

R1#enable secret class

^

% Invalid input detected at '^' marker.

R1#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#enable secret class

R1(config)#banner motd <Authorized Access Only>

Enter TEXT message.  End with the character '<'.

<Authorized access only>

R1(config)#interface gigabitethernet 0/0

R1(config-if)#ip address 200.100.1.1 255.255.255.0

R1(config-if)#no shutdown

R1#

R1#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#interface Serial0/3/0

R1(config-if)#ip address 200.100.12.10 255.255.255.252

R1(config-if)#clock rate 128000

R1(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to down

R1(config-if)#

R1(config)#interface Serial 0/3/1

R1(config-if)#ip address 200.100.13.1 255.255.255.252

R1(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/3/1, changed state to down

R1(config-if)#end

R1#

%SYS-5-CONFIG_I: Configured from console by console

R1#

R1#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#interface g0/0

R1(config-if)#ip address 200.100.1.1 255.255.255.0

R1(config-if)#description "Connected to R2_Primary Link"

R1(config-if)#no shutdown

R1(config-if)#exit

R1(config)#

R1(config)#router  ospf 1

R1(config-router)#network 200.100.1.1 0.0.0.255 area 0

R1(config-router)#network 200.100.12.1 0.0.0.3 area 0

R1(config-router)#network 200.100.13.1 0.0.0.3 area 0

R1(config-router)#end

R1#show ip ospf interface

GigabitEthernet0/0 is up, line protocol is up

Internet address is 200.100.1.1/24, Area 0

Process ID 1, Router ID 200.100.13.1, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 200.100.13.1, Interface address 200.100.1.1

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:00

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

R1#

R1#show ip ospf neighbor

R1#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

200.100.1.0/24 is variably subnetted, 2 subnets, 2 masks

C       200.100.1.0/24 is directly connected, GigabitEthernet0/0

L       200.100.1.1/32 is directly connected, GigabitEthernet0/0

R1#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#interface lo 0

R1(config-if)#

%LINK-5-CHANGED: Interface Loopback0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Loopback0, changed state to up

R1(config-if)#ip add 1.1.1.1 255.255.255.255

R1(config-if)#no shutdown

R1(config-if)#end

R1#

%SYS-5-CONFIG_I: Configured from console by console

R1(config)#router ospf 1

R1(config-router)#passive-interface g0/0

R1(config-router)#end

R1#

%SYS-5-CONFIG_I: Configured from console by console

R1#show ip ospf interface g0/0

GigabitEthernet0/0 is up, line protocol is up

Internet address is 200.100.1.1/24, Area 0

Process ID 1, Router ID 200.100.13.1, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State WAITING, Priority 1

No designated router on this network

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

No Hellos (Passive interface)

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

R1#

R1#config

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#interface serial0/3/0

R1(config-if)#bandwidth 128

R1(config-if)#end

R1#

%SYS-5-CONFIG_I: Configured from console by console

R1#show ip route ospf

R1#show ip ospf interface brief

^

% Invalid input detected at '^' marker.

R1#show ip ospf interface

GigabitEthernet0/0 is up, line protocol is up

Internet address is 200.100.1.1/24, Area 0

Process ID 1, Router ID 200.100.13.1, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State WAITING, Priority 1

No designated router on this network

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

No Hellos (Passive interface)

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

R1#

R1>enable

Password:

R1#config

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line.  End with CNTL/Z.

R1(config)#interface serial 0/3/0

R1(config-if)#bandwidth 1000

R1(config-if)#end

R1#

%SYS-5-CONFIG_I: Configured from console by console

R1#

R1>show ip ospf database

OSPF Router with ID (200.100.13.1) (Process ID 1)

Router Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum Link count

200.100.13.1    200.100.13.1    569         0x80000004 0x00262f 3

200.100.23.1    200.100.23.1    546         0x80000005 0x002f51 4

200.100.23.2    200.100.23.2    379         0x80000006 0x005041 5

R1>

R1>show ip ospf database

OSPF Router with ID (200.100.13.1) (Process ID 1)

Router Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum Link count

200.100.13.1    200.100.13.1    569         0x80000004 0x00262f 3

200.100.23.1    200.100.23.1    546         0x80000005 0x002f51 4

200.100.23.2    200.100.23.2    379         0x80000006 0x005041 5

R1>show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

1.0.0.0/32 is subnetted, 1 subnets

C       1.1.1.1/32 is directly connected, Loopback0

200.100.1.0/24 is variably subnetted, 2 subnets, 2 masks

C       200.100.1.0/24 is directly connected, GigabitEthernet0/0

L       200.100.1.1/32 is directly connected, GigabitEthernet0/0

O    200.100.2.0/24 [110/129] via 200.100.13.2, 00:09:49, Serial0/3/1

O    200.100.3.0/24 [110/65] via 200.100.13.2, 00:10:23, Serial0/3/1

200.100.12.0/24 is variably subnetted, 3 subnets, 2 masks

O       200.100.12.0/30 [110/228] via 200.100.13.2, 00:09:49, Serial0/3/1

C       200.100.12.8/30 is directly connected, Serial0/3/0

L       200.100.12.10/32 is directly connected, Serial0/3/0

200.100.13.0/24 is variably subnetted, 2 subnets, 2 masks

C       200.100.13.0/30 is directly connected, Serial0/3/1

L       200.100.13.1/32 is directly connected, Serial0/3/1

200.100.23.0/30 is subnetted, 1 subnets

O       200.100.23.0/30 [110/128] via 200.100.13.2, 00:09:59, Serial0/3/1

Router2

Router>enable

Router#config t

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#hostname R2

R2(config)#no ip domain-lookup

R2(config)#line console 0

R2(config-line)#password cisco

R2(config-line)#login

R2(config-line)#logging synchornous

^

% Invalid input detected at '^' marker.

R2(config-line)#logging synchronous

R2(config-line)#line aux 0

R2(config-line)#

R2(config-line)#password cisco

R2(config-line)#login

R2(config-line)#line vty 0 4

R2(config-line)#login

% Login disabled on line 388, until 'password' is set

% Login disabled on line 389, until 'password' is set

% Login disabled on line 390, until 'password' is set

% Login disabled on line 391, until 'password' is set

% Login disabled on line 392, until 'password' is set

R2(config-line)#exit

R2(config)#enable secret class

R2(config)#banner motd

% Incomplete command.

R2(config)#banner motd <Authorized Access Only>

Enter TEXT message.  End with the character '<'.

Authorized accesss only

exit

<Authorized access only>

R2(config)#interface gigabitethernet 0/0

R2(config-if)#ip address 200.100.2.1 255.255.255.0

R2(config-if)#no shutdown

R2(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

R2(config-if)#interface serial 0/3/0

R2(config-if)#ip address 200.100.12.2 255.255.255.252

R2(config-if)#no shutdon

^

% Invalid input detected at '^' marker.

R2(config-if)#no shutdown

R2(config-if)#

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to up

R2(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/0, changed state to up

R2(config-if)#interface serial 0/3/1

R2(config-if)#ip address 200.100.23.1 255.255.255.252

R2(config-if)#clock rate 128000

R2(config-if)#no shutdown

%LINK-5-CHANGED: Interface Serial0/3/1, changed state to down

R2(config-if)#end

R2#

%SYS-5-CONFIG_I: Configured from console by console

R2#R2#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#router ospf 1

R2(config-router)#network 200.100.2.1 0.0.0.255 area 0

R2(config-router)#network 200.100.12.2 0.0.0.3 area 0

R2(config-router)#network 200.100.23.1 0.0.0.3 area 0

R2(config-router)#end

R2#

%SYS-5-CONFIG_I: Configured from console by console

R2#show ip ospf neighbor

R2#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

200.100.2.0/24 is variably subnetted, 2 subnets, 2 masks

C       200.100.2.0/24 is directly connected, GigabitEthernet0/0

L       200.100.2.1/32 is directly connected, GigabitEthernet0/0

200.100.12.0/24 is variably subnetted, 2 subnets, 2 masks

C       200.100.12.0/30 is directly connected, Serial0/3/0

L       200.100.12.2/32 is directly connected, Serial0/3/0

R2#show ip ospf interface

GigabitEthernet0/0 is up, line protocol is up

Internet address is 200.100.2.1/24, Area 0

Process ID 1, Router ID 200.100.23.1, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 200.100.23.1, Interface address 200.100.2.1

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:00

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

Serial0/3/0 is up, line protocol is up

Internet address is 200.100.12.2/30, Area 0

Process ID 1, Router ID 200.100.23.1, Network Type POINT-TO-POINT, Cost: 64

Transmit Delay is 1 sec, State POINT-TO-POINT, Priority 0

No designated router on this network

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:09

Index 2/2, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Suppress hello for 0 neighbor(s)

R2#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#interface lo 0

R2(config-if)#

%LINK-5-CHANGED: Interface Loopback0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Loopback0, changed state to up

R2(config-if)#ip add 2.2.2.2 255.255.255.255

R2(config-if)#no shutdown

R2(config-if)#end

R2#

%SYS-5-CONFIG_I: Configured from console by console

R2#R2#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#router ospf 1

R2(config-router)#passive-interface 0/0

^

% Invalid input detected at '^' marker.

R2(config-router)#end

R2#

%SYS-5-CONFIG_I: Configured from console by console

R2#show ip ospf passive-interface

^

% Invalid input detected at '^' marker.

R2#show ip ospf interface

GigabitEthernet0/0 is up, line protocol is up

Internet address is 200.100.2.1/24, Area 0

Process ID 1, Router ID 200.100.23.1, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 200.100.23.1, Interface address 200.100.2.1

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:08

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

Serial0/3/0 is up, line protocol is up

Internet address is 200.100.12.2/30, Area 0

Process ID 1, Router ID 200.100.23.1, Network Type POINT-TO-POINT, Cost: 64

Transmit Delay is 1 sec, State POINT-TO-POINT, Priority 0

No designated router on this network

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:07

Index 2/2, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Suppress hello for 0 neighbor(s)

R2#

R2#

R2#show ip osfp database

^

% Invalid input detected at '^' marker.

R2#show ospf database

^

% Invalid input detected at '^' marker.

R2#show ip ospf database

OSPF Router with ID (200.100.23.1) (Process ID 1)

Router Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum Link count

200.100.23.1    200.100.23.1    371         0x80000002 0x0047cb 2

R2#

R2#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#interface serial 0/3/0

R2(config-if)#bandwidth 1000

R2(config-if)#end

R2#

%SYS-5-CONFIG_I: Configured from console by console

Router3

Router#config t

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#hostname R3

R3(config)#non ip domain-lookup

^

% Invalid input detected at '^' marker.

R3(config)#no ip domain-lookup

R3(config)#line console 0

R3(config-line)#passoword

^

% Invalid input detected at '^' marker.

R3(config-line)#password cisco

R3(config-line)#login

R3(config-line)#logging synchronous

R3(config-line)#line aux 0

R3(config-line)#login

% Login disabled on line 0, until 'password' is set

R3(config-line)#password cisco

R3(config-line)#login

R3(config-line)#line vty 0 4

R3(config-line)#paswword cisco

^

% Invalid input detected at '^' marker.

R3(config-line)#password cisco

R3(config-line)#login

R3(config-line)#exit

R3(config)#enable secret class

R3(config)#banner motd <Authorized Access Only>

Enter TEXT message.  End with the character '<'.

<Authorized access only>

R3(config)#interface gigabitethernet 0/0

R3(config-if)#ip address 200.100.3.1 255.255.255.0

R3(config-if)#no shutdown

R3(config-if)#

%LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up

R3(config-if)#interface serial 0/3/0

R3(config-if)#up address 200.100.13.2 255.255.255.252

^

% Invalid input detected at '^' marker.

R3(config-if)#ip address 200.100.13.2 255.255.255.252

R3(config-if)#clock rate 128000

R3(config-if)#no shutdown

R3(config-if)#

%LINK-5-CHANGED: Interface Serial0/3/0, changed state to up

R3(config-if)#interface serial

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/0, changed state to up

R3(config-if)#interface serial 0/3/1

R3(config-if)#ip address 200.100.23.2 255.255.255.252

R3(config-if)#no shutdown

R3(config-if)#

%LINK-5-CHANGED: Interface Serial0/3/1, changed state to up

R3(config-if)#end

R3#

%SYS-5-CONFIG_I: Configured from console by console

R3#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/3/1, changed state to up

R3#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R3(config)#router ospf 1

R3(config-router)#network 200.100.3.1 0.0.0.255 area 0

R3(config-router)#network 200.100.13.2 0.0.0.3 area 0

R3(config-router)#ne

01:30:35: %OSPF-5-ADJCHG: Process 1, Nbr 200.100.13.1 on Serial0/3/0 from LOADING to FULL, Loading Done

R3(config-router)#network 200.100.23.2 0.0.0.3 area 0

R3(config-router)#

01:30:58: %OSPF-5-ADJCHG: Process 1, Nbr 200.100.23.1 on Serial0/3/1 from LOADING to FULL, Loading Done

R3(config-router)#end

R3#

%SYS-5-CONFIG_I: Configured from console by console

R3#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R3(config)#interface lo 0

R3(config-if)#

%LINK-5-CHANGED: Interface Loopback0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Loopback0, changed state to up

R3(config-if)#ip add 3.3.3.3 255.255.255.255

R3(config-if)#no shutdown

R3(config-if)#end

R3#

%SYS-5-CONFIG_I: Configured from console by console

R3#router ospf 1

^

% Invalid input detected at '^' marker.

R3#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R3(config)#router ospf 1

R3(config-router)#passive-interface 0/0

^

% Invalid input detected at '^' marker.

R3(config-router)#bandwidth 1000

^

% Invalid input detected at '^' marker.

R3(config-router)#end

R3#

%SYS-5-CONFIG_I: Configured from console by console

R3#config t

Enter configuration commands, one per line.  End with CNTL/Z.

R3(config)#interface serial 0/3/0

R3(config-if)#bandwidth 1000

R3(config-if)#end

R3#

%SYS-5-CONFIG_I: Configured from console by console

R3#show ip ospf neigbor

^

% Invalid input detected at '^' marker.

R3#shoe ip ospf neighbor

^

% Invalid input detected at '^' marker.

R3#show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

200.100.23.1      0   FULL/  -        00:00:39    200.100.23.1    Serial0/3/1

200.100.13.1      0   FULL/  -        00:00:36    200.100.13.1    Serial0/3/0

R3#shoe ip route

^

% Invalid input detected at '^' marker.

R3#show ospf interface

^

% Invalid input detected at '^' marker.

R3#show ip ospf interface

GigabitEthernet0/0 is up, line protocol is up

Internet address is 200.100.3.1/24, Area 0

Process ID 1, Router ID 200.100.23.2, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 200.100.23.2, Interface address 200.100.3.1

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:04

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

Serial0/3/0 is up, line protocol is up

Internet address is 200.100.13.2/30, Area 0

Process ID 1, Router ID 200.100.23.2, Network Type POINT-TO-POINT, Cost: 100

Transmit Delay is 1 sec, State POINT-TO-POINT, Priority 0

No designated router on this network

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:06

Index 2/2, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1 , Adjacent neighbor count is 1

Adjacent with neighbor 200.100.13.1

Suppress hello for 0 neighbor(s)

Serial0/3/1 is up, line protocol is up

Internet address is 200.100.23.2/30, Area 0

Process ID 1, Router ID 200.100.23.2, Network Type POINT-TO-POINT, Cost: 64

Transmit Delay is 1 sec, State POINT-TO-POINT, Priority 0

No designated router on this network

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:00

Index 3/3, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1 , Adjacent neighbor count is 1

Adjacent with neighbor 200.100.23.1

Suppress hello for 0 neighbor(s)

R3#

R3#show ip ospf database

OSPF Router with ID (200.100.23.2) (Process ID 1)

Router Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum Link count

200.100.13.1    200.100.13.1    277         0x80000004 0x00262f 3

200.100.23.1    200.100.23.1    254         0x80000005 0x002f51 4

200.100.23.2    200.100.23.2    87          0x80000006 0x005041 5

R3#

C:\>ping 200.100.2.3

Pinging 200.100.2.3 with 32 bytes of data:

Reply from 200.100.2.3: bytes=32 time=3ms TTL=125

Reply from 200.100.2.3: bytes=32 time=4ms TTL=125

Reply from 200.100.2.3: bytes=32 time=6ms TTL=125

Reply from 200.100.2.3: bytes=32 time=10ms TTL=125

Ping statistics for 200.100.2.3:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 3ms, Maximum = 10ms, Average = 5ms

C:\>