SOUND – PART 3 – FORMULAE AND TERMINOLGY

- Absorption coefficient (a) = Absorbed sound energy

Incident sound energy

Unit:  none; as the value is a ratio.

- Absorption of surface = Area of surface x Absorption coefficient of that surface.

Unit:  m2 sabins or ‘absorption units’

- Total absorption =   (Area x Absorption coefficient)

Unit:  m2 sabins or ‘absorption units’

- Reverberation time can be calculated for rooms without excessive absorption using the Sabine formula.

t = 0.16V

A

Where t   = reverberation time (s)

V   = Volume of the room (m3)

A   = Total absorption of room surfaces (m2 sabins)

=  (surface area x absorption coefficient)

- The Transmission Coefficient (T) is the measure of the amount of sound energy transmitted through the partition.

T = Transmitted sound energy

Incident sound energy

Unit : none . It is a ratio

- The sound reduction index (SRI) is a measure of the insulation against the direct transmission of airborne sound.

SRI = 10 log 10 (1/T)

Where T = transmission coefficient for the partition under test.

Unit : Decibels (dB)

- For composite partition, the transmission coefficients can be averaged by area of the part, using the following formula.

T0 = ( T1 x A1) + (T2 x A2) + (T3 x A3)

A1 + A2 + A3

Where T0 = overall transmission coefficient

T1 = transmission coefficient of one component

A1 = area of that component etc.