University of Wales Trinity Saint David

Wales Institute of Science and Art

Engine Performance Characteristics

Powertrain Technology (ACHA4029)

Dafydd Pughe

2401243

Date (03/2026)

Word Count : 1910

Contents

1.	Introduction:	1

2.	Methodology:	2

3.	Raw Data	4

4.	Test Results	7

5.	Calculations	10

5.1. Torque Equation	10

5.2. Power Equation	10

5.3. Corrected Power Equation	10

5.4. SFC Equation	11

5.5. BMEP Equation	11

5.6. Overall Thermal Efficiency Equation	11

6.	Charts/Graphs	12

6.1.	SFC against Engine Speed	12

6.2.	Torque against Engine Power	13

6.3.	Brake power against Engine speed.	14

6.4.	Thermal Efficiency against Power	15

7.	Evaluation	16

8.	References & List of Figures and Tables	17

List of Figures:	18

List of Tables:	18

# Introduction:

This reports concentrates on running a car engine on a dynamometer, collecting data from the experiment and figuring out calculations. Comparing sets of data to contrast why the factors that was measured are important for the vehicles performance and detect if the engine has lost power from factory on factors such as thermal efficiency. Factors such as torque, power, corrected power, BMEP and SFC will be concentrated on and calculating the figure for those factors will be made. The engine that the data was collected from was a 1.6 Honda V.Tec from the Honda CRX 1988. (Honda, 1988).

Figure 1 - Engine Specifics used for testing

# Methodology:

Due to the Engine Dyno being unavailable for usage at the time, the experiment had to be done using fixed figures provided to the students. To learn about the dyno that would’ve been used, there is a video on YouTube providing the information needed for the students to watch and understand before going through the experiment. By watching these videos, it gives a visual aspect to see what goes on during this experiment if the dyno was in working order.

The dyno that the measuring was meant to be used on was a Schenk WS230. (Workshops, 2020).  The dyno works by absorbing the power from the engine and transform into heat energy to collect the data. The dyno can be controlled manually or automatically, when controlled manually the dyno controller can control the throttle position control to the speed of the engine and desired torque.

Figure 2- Diagram of how the dyno works

In the diagram above, (Schenck, 2001) , it shows how the dyno works displaying key aspects. The torque acts upon the lever arm of the dyno where the measured force is corresponding with the braking moment. From the demonstration YouTube video, the lever arm was measured at 815.776mm from pivoting point to the hook eye of the load cell.

The engine is attached to the dyno by the driveshaft followed to the crankshaft with many cables and sensors connected to carry the signals from the engine to read the data that is produced. The supply board that is connected to the engine carries out fuel pumped by a lift pump that is carried up by a fuel flow meter to the swirl pot, which feds to main fuel pump via the regulator. The sensors that are placed of the engine are the coolant temperatures and the flow meter and lambda sensors. Using a computer, the throttle can be controlled by the throttle actuator which records the throttle position.

(Workshops, 2020)

# Raw Data

Table 1 - Raw Data Results Table

| Load | Dyno Speed | Fuel Flow Rate | Lambda |
| --- | --- | --- | --- |
| Newtons | rpm | l/min | Lambda |
| 102.28 | 1000.00 | 0.07 | 1.27 |
| 102.76 | 1050.00 | 0.07 | 1.27 |
| 103.69 | 1100.00 | 0.07 | 1.27 |
| 104.21 | 1150.00 | 0.07 | 1.28 |
| 105.81 | 1200.00 | 0.07 | 1.29 |
| 107.51 | 1250.00 | 0.08 | 1.30 |
| 109.49 | 1300.00 | 0.08 | 1.30 |
| 111.80 | 1350.00 | 0.08 | 1.29 |
| 114.27 | 1400.00 | 0.08 | 1.29 |
| 115.78 | 1450.00 | 0.08 | 1.28 |
| 116.99 | 1500.00 | 0.09 | 1.28 |
| 117.79 | 1550.00 | 0.09 | 1.28 |
| 118.47 | 1600.00 | 0.10 | 1.28 |
| 118.77 | 1650.00 | 0.10 | 1.29 |
| 118.66 | 1700.00 | 0.10 | 1.30 |
| 118.86 | 1750.00 | 0.10 | 1.31 |
| 119.58 | 1800.00 | 0.11 | 1.31 |
| 119.93 | 1850.00 | 0.11 | 1.32 |
| 120.18 | 1900.00 | 0.12 | 1.32 |
| 120.95 | 1950.00 | 0.12 | 1.31 |
| 122.16 | 2000.00 | 0.13 | 1.32 |
| 122.85 | 2050.00 | 0.13 | 1.32 |
| 123.80 | 2100.00 | 0.13 | 1.32 |
| 124.79 | 2150.00 | 0.13 | 1.32 |
| 125.14 | 2200.00 | 0.14 | 1.32 |
| 126.03 | 2250.00 | 0.14 | 1.30 |
| 126.66 | 2300.00 | 0.14 | 1.30 |
| 126.82 | 2350.00 | 0.15 | 1.28 |
| 126.69 | 2400.00 | 0.15 | 1.27 |
| 126.55 | 2450.00 | 0.16 | 1.27 |
| 126.44 | 2500.00 | 0.16 | 1.27 |
| 126.24 | 2550.00 | 0.16 | 1.28 |
| 126.16 | 2600.00 | 0.17 | 1.28 |
| 125.85 | 2650.00 | 0.17 | 1.28 |
| 124.91 | 2700.00 | 0.17 | 1.28 |
| 123.93 | 2750.00 | 0.18 | 1.28 |
| 123.69 | 2800.00 | 0.18 | 1.27 |
| 123.39 | 2850.00 | 0.18 | 1.26 |
| 122.52 | 2900.00 | 0.19 | 1.25 |
| 121.84 | 2950.00 | 0.19 | 1.24 |
| 121.71 | 3000.00 | 0.20 | 1.23 |
| 121.12 | 3050.00 | 0.21 | 1.22 |
| 121.12 | 3100.00 | 0.21 | 1.21 |
| 120.85 | 3150.00 | 0.22 | 1.19 |
| 120.60 | 3200.00 | 0.22 | 1.18 |
| 120.84 | 3250.00 | 0.22 | 1.17 |
| 121.63 | 3300.00 | 0.23 | 1.16 |
| 122.18 | 3350.00 | 0.23 | 1.15 |
| 122.93 | 3400.00 | 0.23 | 1.14 |
| 123.48 | 3450.00 | 0.24 | 1.13 |
| 123.43 | 3500.00 | 0.24 | 1.12 |
| 124.21 | 3550.00 | 0.25 | 1.11 |
| 124.07 | 3600.00 | 0.26 | 1.10 |
| 123.97 | 3650.00 | 0.26 | 1.09 |
| 123.82 | 3700.00 | 0.26 | 1.10 |
| 123.77 | 3750.00 | 0.27 | 1.10 |
| 123.51 | 3800.00 | 0.27 | 1.10 |
| 122.67 | 3850.00 | 0.27 | 1.11 |
| 122.16 | 3900.00 | 0.28 | 1.12 |
| 121.18 | 3950.00 | 0.28 | 1.12 |
| 120.00 | 4000.00 | 0.28 | 1.12 |
| 118.73 | 4050.00 | 0.29 | 1.12 |
| 117.44 | 4100.00 | 0.29 | 1.12 |
| 116.50 | 4150.00 | 0.29 | 1.12 |
| 115.93 | 4200.00 | 0.29 | 1.12 |
| 115.03 | 4250.00 | 0.30 | 1.12 |
| 114.39 | 4300.00 | 0.30 | 1.12 |
| 114.02 | 4350.00 | 0.31 | 1.12 |
| 113.87 | 4400.00 | 0.31 | 1.12 |
| 113.99 | 4450.00 | 0.31 | 1.12 |
| 113.31 | 4500.00 | 0.31 | 1.12 |
| 113.07 | 4550.00 | 0.32 | 1.11 |
| 112.20 | 4600.00 | 0.32 | 1.11 |
| 111.06 | 4650.00 | 0.32 | 1.10 |
| 109.97 | 4700.00 | 0.32 | 1.10 |
| 108.65 | 4750.00 | 0.32 | 1.09 |
| 107.08 | 4800.00 | 0.32 | 1.08 |
| 105.24 | 4850.00 | 0.33 | 1.08 |
| 103.63 | 4900.00 | 0.33 | 1.08 |
| 102.34 | 4950.00 | 0.33 | 1.09 |
| 101.11 | 5000.00 | 0.33 | 1.09 |
| 99.31 | 5050.00 | 0.33 | 1.09 |
| 97.58 | 5100.00 | 0.33 | 1.09 |
| 95.85 | 5150.00 | 0.33 | 1.09 |
| 94.80 | 5200.00 | 0.33 | 1.09 |
| 93.10 | 5250.00 | 0.33 | 1.09 |
| 91.79 | 5300.00 | 0.33 | 1.08 |
| 90.59 | 5350.00 | 0.33 | 1.08 |
| 89.64 | 5400.00 | 0.34 | 1.08 |
| 88.93 | 5450.00 | 0.34 | 1.08 |
| 88.65 | 5500.00 | 0.35 | 1.08 |

Figure 3 - Additional Raw Data

By using the set of raw data above showing the load of the engine, the dyno speed measured in Rpm, the fuel flow rate and the lambda. The calculations for the test results could be made. Due to the dyno not being in working order at the time, the results were provided for this experiment. (Harrison, 2026).

# Test Results

Table 2 - Table of Calculated Test Results

| Torque | Power | Corrected Power | SFC | BMEP | Overall Thermal Efficiency |
| --- | --- | --- | --- | --- | --- |
| Nm | Kw | Kw | g/Kw.h | Kpa | % |
| 83.43 | 8.74 | 8.79 | 362.59 | 659.41 | 22.86 |
| 83.83 | 9.22 | 9.27 | 353.47 | 662.51 | 23.45 |
| 84.59 | 9.74 | 9.80 | 340.37 | 668.52 | 24.35 |
| 85.01 | 10.24 | 10.30 | 319.99 | 671.88 | 25.90 |
| 86.32 | 10.85 | 10.91 | 308.66 | 682.20 | 26.85 |
| 87.70 | 11.48 | 11.55 | 309.46 | 693.16 | 26.78 |
| 89.32 | 12.16 | 12.24 | 292.37 | 705.91 | 28.35 |
| 91.20 | 12.89 | 12.97 | 277.82 | 720.80 | 29.83 |
| 93.22 | 13.67 | 13.75 | 274.95 | 736.73 | 30.15 |
| 94.45 | 14.34 | 14.43 | 265.76 | 746.50 | 31.19 |
| 95.43 | 14.99 | 15.08 | 266.12 | 754.25 | 31.15 |
| 96.09 | 15.60 | 15.69 | 268.90 | 759.42 | 30.82 |
| 96.65 | 16.19 | 16.29 | 266.50 | 763.83 | 31.10 |
| 96.89 | 16.74 | 16.85 | 265.43 | 765.77 | 31.23 |
| 96.80 | 17.23 | 17.34 | 261.27 | 765.02 | 31.72 |
| 96.96 | 17.77 | 17.88 | 260.97 | 766.34 | 31.76 |
| 97.55 | 18.39 | 18.50 | 267.48 | 771.00 | 30.99 |
| 97.84 | 18.95 | 19.07 | 271.00 | 773.26 | 30.58 |
| 98.04 | 19.51 | 19.63 | 270.95 | 774.82 | 30.59 |
| 98.67 | 20.15 | 20.27 | 269.24 | 779.80 | 30.78 |
| 99.65 | 20.87 | 21.00 | 270.70 | 787.59 | 30.62 |
| 100.21 | 21.51 | 21.65 | 267.43 | 792.03 | 30.99 |
| 100.99 | 22.21 | 22.35 | 265.23 | 798.16 | 31.25 |
| 101.80 | 22.92 | 23.06 | 263.77 | 804.60 | 31.42 |
| 102.08 | 23.52 | 23.67 | 263.47 | 806.81 | 31.46 |
| 102.81 | 24.22 | 24.38 | 262.02 | 812.55 | 31.63 |
| 103.33 | 24.89 | 25.04 | 262.10 | 816.63 | 31.62 |
| 103.46 | 25.46 | 25.62 | 264.85 | 817.67 | 31.29 |
| 103.35 | 25.97 | 26.14 | 266.28 | 816.82 | 31.13 |
| 103.24 | 26.49 | 26.65 | 264.44 | 815.93 | 31.34 |
| 103.15 | 27.00 | 27.17 | 265.97 | 815.19 | 31.16 |
| 102.98 | 27.50 | 27.67 | 266.07 | 813.91 | 31.15 |
| 102.92 | 28.02 | 28.20 | 269.87 | 813.41 | 30.71 |
| 102.66 | 28.49 | 28.67 | 270.81 | 811.38 | 30.61 |
| 101.90 | 28.81 | 28.99 | 272.78 | 805.35 | 30.38 |
| 101.10 | 29.11 | 29.30 | 275.75 | 799.01 | 30.06 |
| 100.90 | 29.59 | 29.77 | 273.85 | 797.47 | 30.27 |
| 100.66 | 30.04 | 30.23 | 274.27 | 795.54 | 30.22 |
| 99.95 | 30.35 | 30.54 | 278.57 | 789.93 | 29.75 |
| 99.39 | 30.70 | 30.90 | 282.20 | 785.53 | 29.37 |
| 99.29 | 31.19 | 31.39 | 287.09 | 784.70 | 28.87 |
| 98.81 | 31.56 | 31.76 | 293.80 | 780.93 | 28.21 |
| 98.81 | 32.08 | 32.28 | 294.68 | 780.93 | 28.13 |
| 98.59 | 32.52 | 32.72 | 298.96 | 779.17 | 27.72 |
| 98.38 | 32.97 | 33.17 | 297.50 | 777.53 | 27.86 |
| 98.58 | 33.55 | 33.76 | 299.17 | 779.10 | 27.70 |
| 99.23 | 34.29 | 34.50 | 295.93 | 784.21 | 28.01 |
| 99.67 | 34.97 | 35.19 | 294.07 | 787.75 | 28.18 |
| 100.29 | 35.71 | 35.93 | 295.16 | 792.60 | 28.08 |
| 100.73 | 36.39 | 36.62 | 291.89 | 796.09 | 28.40 |
| 100.69 | 36.90 | 37.14 | 295.02 | 795.79 | 28.09 |
| 101.32 | 37.67 | 37.90 | 293.23 | 800.80 | 28.27 |
| 101.21 | 38.16 | 38.39 | 300.80 | 799.90 | 27.55 |
| 101.13 | 38.65 | 38.90 | 299.83 | 799.26 | 27.64 |
| 101.01 | 39.14 | 39.38 | 304.08 | 798.28 | 27.26 |
| 100.97 | 39.65 | 39.90 | 300.88 | 797.97 | 27.55 |
| 100.76 | 40.09 | 40.35 | 302.25 | 796.31 | 27.42 |
| 100.07 | 40.35 | 40.60 | 305.32 | 790.91 | 27.15 |
| 99.65 | 40.70 | 40.95 | 305.06 | 787.59 | 27.17 |
| 98.86 | 40.89 | 41.15 | 311.26 | 781.32 | 26.63 |
| 97.90 | 41.01 | 41.26 | 312.26 | 773.71 | 26.54 |
| 96.86 | 41.08 | 41.34 | 314.50 | 765.52 | 26.35 |
| 95.81 | 41.13 | 41.39 | 316.10 | 757.19 | 26.22 |
| 95.04 | 41.30 | 41.56 | 319.29 | 751.12 | 25.96 |
| 94.57 | 41.60 | 41.86 | 319.04 | 747.45 | 25.98 |
| 93.84 | 41.77 | 42.03 | 320.16 | 741.67 | 25.89 |
| 93.32 | 42.02 | 42.28 | 323.63 | 737.52 | 25.61 |
| 93.01 | 42.37 | 42.64 | 325.05 | 735.11 | 25.50 |
| 92.89 | 42.80 | 43.07 | 325.08 | 734.17 | 25.50 |
| 92.99 | 43.33 | 43.60 | 321.83 | 734.91 | 25.75 |
| 92.43 | 43.56 | 43.83 | 324.86 | 730.53 | 25.51 |
| 92.24 | 43.95 | 44.23 | 324.66 | 729.03 | 25.53 |
| 91.53 | 44.09 | 44.37 | 326.00 | 723.36 | 25.42 |
| 90.60 | 44.12 | 44.39 | 326.96 | 716.05 | 25.35 |
| 89.71 | 44.16 | 44.43 | 326.83 | 709.04 | 25.36 |
| 88.64 | 44.09 | 44.37 | 331.56 | 700.53 | 25.00 |
| 87.35 | 43.91 | 44.18 | 332.63 | 690.36 | 24.92 |
| 85.85 | 43.60 | 43.88 | 336.13 | 678.52 | 24.66 |
| 84.54 | 43.38 | 43.65 | 339.94 | 668.16 | 24.38 |
| 83.48 | 43.27 | 43.55 | 341.55 | 659.79 | 24.27 |
| 82.48 | 43.19 | 43.46 | 342.50 | 651.87 | 24.20 |
| 81.02 | 42.85 | 43.11 | 346.44 | 640.32 | 23.92 |
| 79.60 | 42.51 | 42.78 | 348.97 | 629.15 | 23.75 |
| 78.19 | 42.17 | 42.43 | 353.17 | 617.97 | 23.47 |
| 77.34 | 42.11 | 42.38 | 356.67 | 611.23 | 23.24 |
| 75.95 | 41.75 | 42.02 | 357.06 | 600.24 | 23.21 |
| 74.88 | 41.56 | 41.82 | 359.92 | 591.80 | 23.03 |
| 73.90 | 41.40 | 41.66 | 359.52 | 584.09 | 23.05 |
| 73.12 | 41.35 | 41.61 | 368.70 | 577.93 | 22.48 |
| 72.54 | 41.40 | 41.66 | 373.68 | 573.34 | 22.18 |
| 72.32 | 41.65 | 41.91 | 376.84 | 571.54 | 21.99 |

# Calculations

The calculations that must be done is to discover the values for 6 key factors of the engine. These factors are the torque (Nm), Power (Kw), Corrected power (Kw), SFC (g/Kw.h), BMEP (Kpa) and the Overall Thermal Efficiency (%). These factors of the engine are important for comparison using graphs. The calculations needed for this was provided to the students on Teams. (Harrison, 2020)

## 5.1. Torque Equation

To get the value for torque, using the value of load (N) and dividing the value with the radius of which the load acts (m), the value for the radius was including in the YouTube video provided by the university which came to 815.776 mm which means that the value had to by 1000 to covert from milometers to meters.

T=WR     T=Wload×(815.776÷1000)

## 5.2. Power Equation

To get the value for Power, using the values of Dyno Speed (RPM) and the value of Torque (Nm) to get the correct value. The value given from the equation came out in Watts, meaning that the value had to be divided by 1000 to covert to Kw.

P= 2πNT60÷1000

## 5.3. Corrected Power Equation

To calculate the value for Corrected Power, using the value of Baro which came to 1018.9 mBar that was given. The value of standard pressure of bar came to 1.01325 bar which was converted to 1013.25 mBar for this equation. The value of power also had to be included in the equation. The ambient temperature (Ta) in kelvin for this equation came to 300k.

Ps=1013.251018.19×Ta/293×P

## 5.4. SFC Equation

To calculate SFC, the value of V was the fuel flow rate was given. The value for Power was calculated in from a previous equation.

SFC= V ×60 ×0.75P×1000

## 5.5. BMEP Equation

To calculate the value for BMEP, using the value for Torque which was calculated in a previous equation. The value for cubic volume of the engine cylinders came to 0.00159m³.

BMEP= 4π ×T0.00159 ÷1000

## 5.6. Overall Thermal Efficiency Equation

To calculate the value for the overall thermal efficiency, using the given value of fuel flow rate and the previous value of corrected power. Using those values we can multiply with the value of calorific value of fuel which is 40000 KJ/kg. By multiplying by 60, this changes the time from minutes to seconds.

Thermal Efficiency= Corrected PowerFuel Flow Rate/60×0.75×40000×100

# Charts/Graphs

By using the calculated data shown above in Table 2, using graph to compare and contrast the data was made. The graphs below show essential factors of the engines performance.

## SFC against Engine Speed

Figure 4 - Graph showing SFC against Engine Speed

The graph above shows SFC (specific fuel consumption) against engine speed. SFC is the measurement of fuel efficiency when power gets introduced. (Fuel Consumption, 2025). The graph shows that when the engine starts to run from 1000 rpm the fuel consumptions starts to decrees when reaching the middle between 1000 and 2000 rpm, from that point the fuel consumption stays at a consistent level until it starts to rise at around 3000 rpm. The peak fuel consumption of the engine it when the engine is at its peak speed at 5500 rpm. When the engine is running at low rpm, the fuel consumption is less efficient meaning that more fuel is required per unit power. A factor to consider at low rpm and less efficient fuel consumption could be due to poor air-fuel mixing at slower combustion or heat loss to cylinder walls. When the engine is at a moderate speed, 1500 to 2500 rpm, the fuel consumption is at its best efficiency, a factor to consider at this stage of engine speed is that the engine is near its optimal torque region. By increasing the engine speed further that 2500 rpm, the fuel consumption gets less efficient when the engine starts to work harder. When comparing the results, the best SFC is when 1500 and 2500 rpm when the SFC is at it lowest making it more efficient, to compare with the SFC reaching high levels around start of the engine producing power and when the engine is reaching its peak speed. By the engine having a high SFC, it needs more fuel to produce the speed needed, making the process less efficient. (Anon., 2018)

## Torque against Engine Power

Figure 5 - Graph showing Torque against Engine Speed

The graph above shows the torque that the engine provides during the dyno run against the speed of the engine. By looking at the graph above, it shows from start speed at 1000 rpm that the engine builds torque until 3000rpm. The graph also shows that there are 2 peaks of the graph at around the 2250 rpm and 3500 rpm showing its peak torque distribution. The graph shows that during 3000 rpm that it is past its peak torque distribution but slowly rises after. When the engine reaches higher speed at around 4000rpm, it continuously loses torque at the engines peak speed. At low rpm, between 1000 and 1500, the engine starts to work and build speed where the airflow into the cylinders is weak meaning combustion from the mixture of fuel and air is less efficient. When the engine speed starts to rise between 1500 and 2500 rpm, the engine reaches its peak torque distribution. The fuel starts to mix better with the air producing the power more efficient. Between 2500 and 4000 rpm the torque stays at a constant rate meaning that the engine maintains high volumetric efficiency at its mid range of running. After reaching 4000 rpm the engine starts to produce less torque after reaching its peak, at this point the airflow gets restricted in the intake and exhaust meaning torque decreases. (X Engineer, 2026).

## Brake power against Engine speed.

Figure 6 - Graph showing Brake Power against Engine Speed

The graph above shows the brake power that the engine in producing against the engines speed. It shows that the engine continuously builds its speed from start at 1000 rpm to 4500 rpm before starting to produce less power, but before the engines peak speed it starts to gain speed again. The peak brake power that the engine produces is 44.16 Kw at 4700 rpm before starting to slowly produce less power. The rate that the power increases is pretty consistent from start to 3000 rpm before a small rate of change at 3200 rpm then starts to build more power. With the engine speed increasing, the brake power that is produced from the engine is also increasing. The torque from the engine helps increase the power from start to 4000 rpm where the engine reaches its peak torque. Due to the air-fuel mix improving through the engine power increase, there is a better combustion efficiency which helps producing more brake power.

## Thermal Efficiency against Power

Figure 7 - Graph showing Overall Thermal Efficiency against Power

The graph above shows the thermal efficiency of the engine while compared to the power the engine is producing. Thermal efficiency can affect how the engine performs and production of power by recording the engine heat when converting into engine power, by having a higher efficiency the engine performs much better by using the heat to produce power. When the temperature is low, the engine struggles to product the correct amount of power and can affect the results. (How Engineering Works, 2017).  The thermal efficiency gradually rises when the power increases. When the engine produces 17.5 Kw of power the thermal efficiency starts to decrease and stays at a steady level until the power reaches 30 Kw. After reaching 30 Kw of power the thermal efficiency decreases again to around 27.5% and stays at that level until 40 Kw of power. The engine reaches its lowest thermal efficiency when its reached its peak power output at 44 Kw. The reason of the change in thermal efficiency of the engine could lead to different factors at different power outputs.

# Evaluation

To conclude this report, the performance of the 1.6L VTEC engine from the 1988 Honda CRX was analysed using a dyno to determine and calculate the torque, brake power, corrected power, SFC, BMEP and the overall thermal efficiency. The results that was calculated indicated that the torque the engine produced increased when the engine speed was low, the torque peaked when reaching the mid range but then reduced when the engine reached its peak speed. This could indicate that there was limitations in the air-fuel mix and also the volumetric efficiency of the engine. When the engine speed started to speed up, the brake power increased reached a peak of 44.16 Kw when reaching 4700 rpm.

SFC was at its lowest when the engine was running between 1500-2500 rpm, showing that the engine was at its most efficient speed. When the engine reached higher speed, the SFC started to rise again.

The findings in this experiment highlighted the relationship between engine speed, power output and fuel efficiency. By these findings being highlighted it showed that the engine is at its peak performance when reaching its mid range speed at around 2500 rpm.

During the assignment, there were some improvements that could be done if this was to be repeated again in the future. One improvement that can be done is to use the dynamometer to gain knowledge and use the data that was collected personally rath than the data being given to the students.

# References & List of Figures and Tables

Anon., 2018. We Engineers. [Online] 
Available at: https://electricalknowledgecommunity.blogspot.com/2018/05/what-is-specific-fuel-consumption-in.html
[Accessed 28 May 2018].

Fuel Consumption, 2025. Science Direct. [Online] 
Available at: https://fuelconsumption.co.za/sfc-specific-fuel-consumption/
[Accessed 9 Febuary 2025].

Harrison, A., 2020. Handout Master 2020 Part 1. s.l.:s.n.

Harrison, A., 2026. Engine Dynamometer. s.l.:s.n.

Honda, 1988. Honda Civiv Coupe CRX Chassis Maintence and Repair. s.l.:s.n.

How Engineering Works, 2017. How Engineering Works. [Online] 
Available at: https://www.howengineeringworks.com/questions/what-is-thermal-efficiency-4/
[Accessed 2017].

Schenck, 2001. Technical Specification Eddy-Current Dynometer W Series. Darmstadt: s.n.

Workshops, U. W., 2020. YouTube. [Online] 
Available at: https://www.youtube.com/watch?v=-WajOAp2LUE&t=432s
[Accessed 16 October 2020].

Workshops, U. W., 2020. YouTube. [Online] 
Available at: https://www.youtube.com/watch?v=VD9OPoaAcJU
[Accessed 16 Octob 2020].

X Engineer, 2026. X Engineer. [Online] 
Available at: https://x-engineer.org/power-vs-torque/
[Accessed 2026].

## List of Figures:

Figure 1 - Engine Specifics used for testing	1

Figure 2- Diagram of how the dyno works	2

Figure 3 - Additional Raw Data	6

Figure 4 - Graph showing SFC against Engine Speed	12

Figure 5 - Graph showing Torque against Engine Speed	13

Figure 6 - Graph showing Brake Power against Engine Speed	14

Figure 7 - Graph showing Overall Thermal Efficiency against Power	15

## List of Tables:

Table 1 - Raw Data Results Table	4

Table 2 - Calculated Test Results	7