| Activity to be Assessed |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bridge Foundations & Pier construction (working with concrete) |  |  |  |  |  | 1 |  |  |  |  |  |  |  |
| Persons Undertaking or Affected by the Activity |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Public Contractor Service User Employees Other |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazard |  | Associated Risk |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Skin contact with wet concrete | Serious burns or irritation to skin |  | 4 |  |  | 4 |  |  | 16 (red) |  |  |  |
| 2 | Accidental contact with the eye whilst handling concrete | Serious irritation to eyes |  | 4 |  |  | 4 |  |  | 16 (red) |  |  |  |
| 3 | Contact with rotating drum | Being knocked down or injured |  | 2 |  |  | 3 |  |  | 6G (orange) |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Appropriate gloves & safety goggles must be worn at all times, to avoid contact with eyes or hands. Gloves should be long and tight fitting to prevent concrete entering the glove from the wrist and being trapped between the gloves. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Suitable footwear, which are high enough to prevent concrete entering by the top of the boot. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Waterproof trousers should be worn, for protection, if kneeling on surfaces with wet concrete. Time spent kneeling should be minimised but for additional protection knee pads may also be utilised. |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Any contact with concrete should be washed off with soap immediately, using the on-site washing facilities. |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | The mixer is to be placed in a safe area where non-essential workers are not in direct contact with the rotating drum. Any rotating machinery is to be monitored, whilst in operation. Additionally, no hand tools such as shovels are to be used, whilst the mixing is in operation. The mixer is to be stopped and disconnected, before cleaning is undertaken. |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Skin contact with wet concrete |  |  | 1 |  |  | 4 |  |  |  | 4 (green) |  |  |
| 2 | Accidental contact with the eye whilst handling concrete |  |  | 1 |  |  | 4 |  |  |  | 4 (green) |  |  |
| 3 | Contact with rotating drum |  |  | 1 |  |  | 3 |  |  |  | 3 (green) |  |  |
| Name: Adam Hassan Position: Principal Contractor |  |  | Signed: ADAM HASSAN |  | Date: 11/04/22 |  |  |  |  |  |  |  |  |
| Reviews |  |  | Key |  |  |  |  |  |  |  |  |  |  |
| Review Date: Reviewed By: |  |  | Likelihood | Severity | Severity of Injury |  | 5 | 5Y | 10R |  | 15 | 20 | 25 |
| Review Date: Reviewed By: |  |  | 1 Very Unlikely | 1 Nuisance |  |  | 4 | 4 | 8 |  | 12 | 16 | 20 |
| Review Date: Reviewed By: |  |  | 2 Unlikely | 2 Minor |  |  | 3 | 3 | 6G |  | 9 | 12 | 15 |
| Review Date: Reviewed By: |  |  | 3 Likely | 3 Medical Treatment |  |  | 2 | 2 | 4 |  | 6Y | 8 | 10Y |
| Review Date: Reviewed By: |  |  | 4 Very Likely | 4 Major |  |  | 1 | 1 | 2 |  | 3 | 4 | 5G |
| Review Date: Reviewed By: |  |  |  |  |  |  | 0 | 1 | 2 |  | 3 | 4 | 5 |
| Review Date: Reviewed By: |  |  | 5 Certainty | 5 Fatal |  |  |  | Likelihood of Injury |  |  |  |  |  |
| Review Date: Reviewed By: |  |  |  |  | High Risk Medium Risk Low Risk |  |  |  |  |  |  |  |  |

| Activity to be Assessed |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cofferdam Erection & Dewatering |  |  |  |  |  | 2 |  |  |  |  |  |  |  |
| Persons Undertaking or Affected by the Activity |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor Public Service User Employees Other |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazard |  | Associated Risk |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Contact with moving Barge crane | Damage to equipment/plant or workers being trapped |  | 3 |  |  | 3 |  |  | 9 (orange) |  |  |  |
| 2 | Cofferdam Leakage | Collapse of the cofferdam system |  | 2 |  |  | 5 |  |  | 10R (red) |  |  |  |
| 3 | People Falling into Cofferdam | Serious injury or harm falling from height into cofferdam |  | 3 |  |  | 5 |  |  | 15 (red) |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | The shape of the cofferdam has been carefully designed and planned to suit the working space and construction plant, as well as the final pier structure. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Daily inspections are to be carried out on the cofferdam, to check for leaks and structural integrity |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | All cofferdam works are remotely executed, by working at a safe distance using the barge and crane, eliminating workers from working too close to the cofferdam or being at risk of falls from height. |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Contact with moving equipment/plant |  |  | 1 |  |  | 3 |  |  |  | 3 (green) |  |  |
| 2 | Cofferdam Leakage |  |  | 1 |  |  | 5 |  |  |  | 5Y (orange) |  |  |
| 3 | People Falling into Cofferdam |  |  | 1 |  |  | 5 |  |  |  | 5Y (orange) |  |  |
| Name: Adam Hassan Position: Principal Contractor |  |  | Signed: ADAM HASSAN |  | Date: 11/04/22 |  |  |  |  |  |  |  |  |
| Reviews |  |  | Key |  |  |  |  |  |  |  |  |  |  |
| Review Date: Reviewed By: |  |  | Likelihood | Severity | Severity of Injury |  | 5 | 5Y | 10R |  | 15 | 20 | 25 |
| Review Date: Reviewed By: |  |  | 1 Very Unlikely | 1 Nuisance |  |  | 4 | 4 | 8 |  | 12 | 16 | 20 |
| Review Date: Reviewed By: |  |  | 2 Unlikely | 2 Minor |  |  | 3 | 3 | 6G |  | 9 | 12 | 15 |
| Review Date: Reviewed By: |  |  | 3 Likely | 3 Medical Treatment |  |  | 2 | 2 | 4 |  | 6Y | 8 | 10Y |
| Review Date: Reviewed By: |  |  | 4 Very Likely | 4 Major |  |  | 1 | 1 | 2 |  | 3 | 4 | 5G |
| Review Date: Reviewed By: |  |  |  |  |  |  | 0 | 1 | 2 |  | 3 | 4 | 5 |
| Review Date: Reviewed By: |  |  | 5 Certainty | 5 Fatal |  |  |  | Likelihood of Injury |  |  |  |  |  |
| Review Date: Reviewed By: |  |  |  |  | High Risk Medium Risk Low Risk |  |  |  |  |  |  |  |  |

| Activity to be Assessed |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bridge Deck & Tower Construction (Operating Barge & Crawler Crane) |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| Persons Undertaking or Affected by the Activity |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor Public Service User Employees Other |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazard |  | Associated Risk |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Toppling of crane | May fall on equipment, people or the structure |  | 3 |  |  | 5 |  |  | 15 (red) |  |  |  |
| 2 | Crane overload | Falling of the Load may cause severe injury or potential death |  | 3 |  |  | 5 |  |  | 15 (red) |  |  |  |
| 3 | Mechanical Failure | Plant failure during operation may cause the load to suddenly swing and drop. |  | 3 |  |  | 4 |  |  | 12 (red) |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Outriggers must be extended and chocked before raising any loads. Additionally, do not operate in adverse weather, specifically when there are strong winds. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Health and safety, as well as operational usage training must be completed by all relevant personnel. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Do not exceed the barge or cranes operational load capacity. Operational capacities will be identified upon training. |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Routine inspections, maintenance and repairs (if necessary) shall be conducted before and after the use of both the barge and crane to ensure all mechanical systems are functioning sufficiently and safely. |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Toppling of crane |  |  | 1 |  |  | 5 |  |  |  | 5Y (orange) |  |  |
| 2 | Crane overload |  |  | 1 |  |  | 5 |  |  |  | 5Y (orange) |  |  |
| 3 | Mechanical Failure |  |  | 1 |  |  | 4 |  |  |  | 4 (green) |  |  |
| Name: Adam Hassan Position: Principal Contractor |  |  | Signed: ADAM HASSAN |  | Date: 11/04/22 |  |  |  |  |  |  |  |  |
| Reviews |  |  | Key |  |  |  |  |  |  |  |  |  |  |
| Review Date: Reviewed By: |  |  | Likelihood | Severity | Severity of Injury |  | 5 | 5Y | 10R |  | 15 | 20 | 25 |
| Review Date: Reviewed By: |  |  | 1 Very Unlikely | 1 Nuisance |  |  | 4 | 4 | 8 |  | 12 | 16 | 20 |
| Review Date: Reviewed By: |  |  | 2 Unlikely | 2 Minor |  |  | 3 | 3 | 6G |  | 9 | 12 | 15 |
| Review Date: Reviewed By: |  |  | 3 Likely | 3 Medical Treatment |  |  | 2 | 2 | 4 |  | 6Y | 8 | 10Y |
| Review Date: Reviewed By: |  |  | 4 Very Likely | 4 Major |  |  | 1 | 1 | 2 |  | 3 | 4 | 5G |
| Review Date: Reviewed By: |  |  |  |  |  |  | 0 | 1 | 2 |  | 3 | 4 | 5 |
| Review Date: Reviewed By: |  |  | 5 Certainty | 5 Fatal |  |  |  | Likelihood of Injury |  |  |  |  |  |
| Review Date: Reviewed By: |  |  |  |  | High Risk Medium Risk Low Risk |  |  |  |  |  |  |  |  |

| Activity to be Assessed |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Suspended Cable Fixings: Working at height above water; via Mobile Elevated Working Platforms (MEWPs) |  |  |  |  |  | 4 |  |  |  |  |  |  |  |
| Persons Undertaking or Affected by the Activity |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor Public Service User Employees Other |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazard |  | Associated Risk |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Entrapment of operator | Operator may become trapped against platform controls |  | 3 |  |  | 4 |  |  | 12 (red) |  |  |  |
| 2 | MEWP overturning | Workers / Equipment being carried away by currents |  | 4 |  |  | 4 |  |  | 16 (red) |  |  |  |
|  |  | Workers falling into water & catching hypothermia, drowning or getting ‘cold shock’. |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Falling objects | Objects may fall, causing harm to workers in working spaces below |  | 4 |  |  | 4 |  |  | 16 (red) |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | An anti-entrapment device, consisting of sensor bars/pads, will fitted to MEWPs and will be activated should an operator be pushed (trapped) onto them. The MEWP will instantly stop moving when the sensors are activated, avoiding the possibility of an incident and preventing harm to the operator. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | MEWPs will be fitted with guard rails and toe bars, preventing workers from falling. Additionally, harnesses will be required, whilst working on the platforms. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Outriggers must be extended and chocked before raising the platform. Additionally, spreader plates will also be required, for increased stability of the platform. |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | High winds may tilt platforms, making them unstable. A maximum wind speed, for safe operation conditions is to be set. Additionally, storms and snowfalls may damage equipment/plant, therefore all they must be checked before they are used and after severe weather. |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Appropriate PPE is to be worn by all workers on site, at all times. E.g., hardhats, high visibility vests & work boots. |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | All workers working MEWPs will be required to wear tool belts, so that all tools can be secured around the waste, when not in use. In addition, all workers will receive manual handling training, if necessary, upon induction. |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Entrapment of operator |  |  | 1 |  |  | 4 |  |  |  | 4 (green) |  |  |
| 2 | MEWP overturning |  |  | 1 |  |  | 4 |  |  |  | 4 (green) |  |  |
| 3 | Falling objects |  |  | 1 |  |  | 4 |  |  |  | 4 (green) |  |  |
| Name: Adam Hassan Position: Principal Contractor |  |  | Signed: ADAM HASSAN |  | Date: 11/04/22 |  |  |  |  |  |  |  |  |
| Reviews |  |  | Key |  |  |  |  |  |  |  |  |  |  |
| Review Date: Reviewed By: |  |  | Likelihood | Severity | Severity of Injury |  | 5 | 5Y | 10R |  | 15 | 20 | 25 |
| Review Date: Reviewed By: |  |  | 1 Very Unlikely | 1 Nuisance |  |  | 4 | 4 | 8 |  | 12 | 16 | 20 |
| Review Date: Reviewed By: |  |  | 2 Unlikely | 2 Minor |  |  | 3 | 3 | 6G |  | 9 | 12 | 15 |
| Review Date: Reviewed By: |  |  | 3 Likely | 3 Medical Treatment |  |  | 2 | 2 | 4 |  | 6Y | 8 | 10Y |
| Review Date: Reviewed By: |  |  | 4 Very Likely | 4 Major |  |  | 1 | 1 | 2 |  | 3 | 4 | 5G |
| Review Date: Reviewed By: |  |  |  |  |  |  | 0 | 1 | 2 |  | 3 | 4 | 5 |
| Review Date: Reviewed By: |  |  | 5 Certainty | 5 Fatal |  |  |  | Likelihood of Injury |  |  |  |  |  |
| Review Date: Reviewed By: |  |  |  |  | High Risk Medium Risk Low Risk |  |  |  |  |  |  |  |  |

| Activity to be Assessed |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Renewable Energy Fixings (Working with Electrical Equipment) |  |  |  |  |  | 5 |  |  |  |  |  |  |  |
| Persons Undertaking or Affected by the Activity |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor Public Service User Employees Other |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazard |  | Associated Risk |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Contact with live parts | Electric shock and burns |  | 3 |  |  | 4 |  |  | 12 (red) |  |  |  |
| 2 | Faulty equipment | Fires may be caused by faulty equipment or installations |  | 2 |  |  | 5 |  |  | 15 (red) |  |  |  |
| 3 | Loose cables | Tripping over wiring and being caused injury. |  | 4 |  |  | 3 |  |  | 12 (orange) |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | All persons will be given proper training and supervision. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Ensure appliances are switched off and unplugged before cleaning of adjusting them. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Check that sockets are not overloaded and that unfused adaptors are not used. |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | All electrical equipment should be maintained and checked before use. Plugs, connectors, cables and visible burns, due to overheating, should be inspected. Additionally, cables repaired with tape are not secure. |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | If possible, electrical equipment should be substituted with battery operated tools/equipment. However, if this is not applicable then ensure there are no trailing cables. |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Sufficient lighting should be provided at all times for improved visibility. |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Contact with live parts |  |  | 1 |  |  | 4 |  |  |  | 4 (green) |  |  |
| 2 | Faulty equipment |  |  | 1 |  |  | 5 |  |  |  | 5Y (orange) |  |  |
| 3 | Loose cables |  |  | 2 |  |  | 3 |  |  |  | 6G (orange) |  |  |
| Name: Adam Hassan Position: Principal Contractor |  |  | Signed: ADAM HASSAN |  | Date: 11/04/22 |  |  |  |  |  |  |  |  |
| Reviews |  |  | Key |  |  |  |  |  |  |  |  |  |  |
| Review Date: Reviewed By: |  |  | Likelihood | Severity | Severity of Injury |  | 5 | 5Y | 10R |  | 15 | 20 | 25 |
| Review Date: Reviewed By: |  |  | 1 Very Unlikely | 1 Nuisance |  |  | 4 | 4 | 8 |  | 12 | 16 | 20 |
| Review Date: Reviewed By: |  |  | 2 Unlikely | 2 Minor |  |  | 3 | 3 | 6G |  | 9 | 12 | 15 |
| Review Date: Reviewed By: |  |  | 3 Likely | 3 Medical Treatment |  |  | 2 | 2 | 4 |  | 6Y | 8 | 10Y |
| Review Date: Reviewed By: |  |  | 4 Very Likely | 4 Major |  |  | 1 | 1 | 2 |  | 3 | 4 | 5G |
| Review Date: Reviewed By: |  |  |  |  |  |  | 0 | 1 | 2 |  | 3 | 4 | 5 |
| Review Date: Reviewed By: |  |  | 5 Certainty | 5 Fatal |  |  |  | Likelihood of Injury |  |  |  |  |  |
| Review Date: Reviewed By: |  |  |  |  | High Risk Medium Risk Low Risk |  |  |  |  |  |  |  |  |

| Activity to be Assessed |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Excavation works: Exposure to Contaminated Land |  |  |  |  |  | 6 |  |  |  |  |  |  |  |
| Persons Undertaking or Affected by the Activity |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor Public Service User Employees Other |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazard |  | Associated Risk |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Exposure to contaminated land | Possible respiratory illness |  | 2 |  |  | 3 |  |  | 6G (orange) |  |  |  |
| 2 | Contact with contaminated Material | Irritation to eyes and/or skin |  | 2 |  |  | 3 |  |  | 6G (orange) |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Keep all unnecessary personnel away from the hazard area and execute any excavation work with the crane and clamshell attachment, reducing the exposure to workers. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Additionally, any contaminated material is to be kept segregated from all other material/plant and personnel, ready to disposed off appropriately, off-site. |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Activity Hazards |  |  |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Exposure to contaminated land |  |  | 1 |  |  | 3 |  |  |  | 3 (green) |  |  |
| 2 | Contact with contaminated Material |  |  | 1 |  |  | 3 |  |  |  | 3 (green) |  |  |
| Name: Adam Hassan Position: Principal Contractor |  |  | Signed: ADAM HASSAN |  | Date: 11/04/22 |  |  |  |  |  |  |  |  |
| Reviews |  |  | Key |  |  |  |  |  |  |  |  |  |  |
| Review Date: Reviewed By: |  |  | Likelihood | Severity | Severity of Injury |  | 5 | 5Y | 10R |  | 15 | 20 | 25 |
| Review Date: Reviewed By: |  |  | 1 Very Unlikely | 1 Nuisance |  |  | 4 | 4 | 8 |  | 12 | 16 | 20 |
| Review Date: Reviewed By: |  |  | 2 Unlikely | 2 Minor |  |  | 3 | 3 | 6G |  | 9 | 12 | 15 |
| Review Date: Reviewed By: |  |  | 3 Likely | 3 Medical Treatment |  |  | 2 | 2 | 4 |  | 6Y | 8 | 10Y |
| Review Date: Reviewed By: |  |  | 4 Very Likely | 4 Major |  |  | 1 | 1 | 2 |  | 3 | 4 | 5G |
| Review Date: Reviewed By: |  |  |  |  |  |  | 0 | 1 | 2 |  | 3 | 4 | 5 |
| Review Date: Reviewed By: |  |  | 5 Certainty | 5 Fatal |  |  |  | Likelihood of Injury |  |  |  |  |  |
| Review Date: Reviewed By: |  |  |  |  | High Risk Medium Risk Low Risk |  |  |  |  |  |  |  |  |

| Activity to be Assessed |  |  |  |  |  | Assessment Number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Working Outdoors (Adverse Weather Conditions) |  |  |  |  |  | 7 |  |  |  |  |  |  |  |
| Persons Undertaking or Affected by the Activity |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor Public Service User Employees Other |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identified Hazard |  | Associated Risk |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Strong Wind | Injury to workers / damage to equipment from being knocked down |  | 4 |  |  | 4 |  |  | 16 (red) |  |  |  |
| 2 | Rain / Snow | Injury from slips and falls from wet/icy conditions |  | 4 |  |  | 4 |  |  | 16 (red) |  |  |  |
| 3 | Extreme Sun / Heat | Sunburn or heat stroke in hot sunny weather |  | 3 |  |  | 4 |  |  | 12 (red) |  |  |  |
| Existing Control Measures / Additional Control Measures Required |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Anti-slip mats must be used where surfaces are affected by rain, snow or possible spillages. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | Suitable life jackets must be worn, whilst working near or over water. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Waterproof grip footwear must be worn to reduce the risk of slipping. |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | A lifebuoy with rope of adequate length attached is available, where a person may fall into the water. |  |  |  |  |  |  |  |  |  |  |  |  |
| Reassessment of Hazards |  |  |  | Likelihood |  |  | Severity |  |  |  | Risk Level |  |  |
| 1 | Strong Wind |  |  | 4 |  |  | 1 |  |  |  | 4 (green) |  |  |
| 2 | Rain / Snow |  |  | 4 |  |  | 1 |  |  |  | 4 (green) |  |  |
| 3 | Extreme Sun / Heat |  |  | 4 |  |  | 1 |  |  |  | 4 (green) |  |  |
| Name: Adam Hassan Position: Principal Contractor |  |  | Signed: ADAM HASSAN |  | Date: 11/04/22 |  |  |  |  |  |  |  |  |
| Reviews |  |  | Key |  |  |  |  |  |  |  |  |  |  |
| Review Date: Reviewed By: |  |  | Likelihood | Severity | Severity of Injury |  | 5 | 5Y | 10R |  | 15 | 20 | 25 |
| Review Date: Reviewed By: |  |  | 1 Very Unlikely | 1 Nuisance |  |  | 4 | 4 | 8 |  | 12 | 16 | 20 |
| Review Date: Reviewed By: |  |  | 2 Unlikely | 2 Minor |  |  | 3 | 3 | 6G |  | 9 | 12 | 15 |
| Review Date: Reviewed By: |  |  | 3 Likely | 3 Medical Treatment |  |  | 2 | 2 | 4 |  | 6Y | 8 | 10Y |
| Review Date: Reviewed By: |  |  | 4 Very Likely | 4 Major |  |  | 1 | 1 | 2 |  | 3 | 4 | 5G |
| Review Date: Reviewed By: |  |  |  |  |  |  | 0 | 1 | 2 |  | 3 | 4 | 5 |
| Review Date: Reviewed By: |  |  | 5 Certainty | 5 Fatal |  |  |  | Likelihood of Injury |  |  |  |  |  |
| Review Date: Reviewed By: |  |  |  |  | High Risk Medium Risk Low Risk |  |  |  |  |  |  |  |  |

References for Risk Assessments:

| Assessment Number | Reference |
| --- | --- |
| 1 Bridge Foundations & Pier construction (working with concrete) | - https://www.hse.gov.uk/construction/healthrisks/hazardous-substances/cement.htm#:~:text=Wet%20cement%20is%20highly%20alkaline,chemical%20burns%20to%20the%20eyes. - https://www.totalconcrete.co.uk/wp-content/uploads/2019/05/risk-assessment.pdf |
| 2 Cofferdam Erection & Dewatering | - https://www.aisolutions.co.uk/community/Knowledge/Topic/250/1004/Cofferdams-and-Caissons - https://www.hse.gov.uk/construction/safetytopics/temporary-works.htm |
| 3 Bridge Deck & Tower Construction (Operating Barge & Crawler Crane) | - https://www.hse.gov.uk/construction/safetytopics/lifting-operations.htm#planning - https://www.highspeedtraining.co.uk/hub/crane-safety-hazards-control-measures/ - https://learninglegacy.hs2.org.uk/wp-content/uploads/2021/09/4478-BS-7121-Part-3-2017.pdf |
| 4 Suspended Cable Fixings: Working at height above water; via Mobile Elevated Working Platforms (MEWPs) | - https://www.hse.gov.uk/construction/safetytopics/mewp.htm |
| 5 Renewable Energy Fixings (Working with Electrical Equipment) | - https://www.hse.gov.uk/toolbox/electrical.htm |
| 6 Excavation works: Exposure to Contaminated Land | - https://www.hse.gov.uk/pubns/indg136.pdf - https://www.hse.gov.uk/construction/safetytopics/excavations.htm |
| 7 Working Outdoors (Adverse Weather Conditions) | - https://www.hse.gov.uk/temperature/outdoor.htm |

References

BSI, 2017. BS 7121-3:2017 - Code of practice for safe use of cranes | Part 3: Mobile cranes, BSI.

Available online at https://learninglegacy.hs2.org.uk/wp-content/uploads/2021/09/4478-BS-7121-Part-3-2017.pdf

[Accessed 17th April 2022]

HSE, 2022. Construction hazardous substances: Cement, HSE.

Available online at https://www.hse.gov.uk/construction/healthrisks/hazardous-substances/cement.htm#:~:text=Wet%20cement%20is%20highly%20alkaline,chemical%20burns%20to%20the%20eyes.

[Accessed 14 April 2022]

HSE, 2022. Lifting operations, HSE.

Available online at https://www.hse.gov.uk/construction/safetytopics/lifting-operations.htm#planning

[Accessed 15th April 2022]

HSE, 2022. Temporary Works, HSE.

Available online at https://www.hse.gov.uk/construction/safetytopics/temporary-works.htm

[Accessed 16th April 2022]

HSE, 2022. Mobile elevating work platforms (MEWPs), HSE.

Available online at  https://www.hse.gov.uk/construction/safetytopics/mewp.htm

[Accessed 25th April 2022]

HSE, 2022. Electrical safety, HSE.

Available online at https://www.hse.gov.uk/toolbox/electrical.htm

[Accessed 30th April 2022]

HSE, 2022. Working with substances hazardous to health | A brief guide to COSHH, HSE.

Available online at https://www.hse.gov.uk/pubns/indg136.pdf

[Accessed 2nd May 2022]

HSE, 2022. Excavations, HSE.

Available online at https://www.hse.gov.uk/construction/safetytopics/excavations.htm

[Accessed 5th May 2022]

HSE, 2022. Outdoor working, HSE.

Available online at https://www.hse.gov.uk/temperature/outdoor.htm

[Accessed 5th May 2022]

Martinelli K., 2018. Crane Safety Hazards and Control Measures,

Available online at https://www.highspeedtraining.co.uk/hub/crane-safety-hazards-control-measures/

[Accessed 17th April 2022]

Safety matters, n.d. Cofferdams and Caissons, Safety matters.

Available online at https://www.aisolutions.co.uk/community/Knowledge/Topic/250/1004/Cofferdams-and-Caissons

[Accessed 16th April 2022]

Total concrete Ltd, 2019. Risk assessment, Total concrete Ltd.

Available online at https://www.totalconcrete.co.uk/wp-content/uploads/2019/05/risk-assessment.pdf

[Accessed 14th April 2022]